3GPP TSG-RAN WG4 Meeting #114	R4-2501429

Athens, GR, 17th – 21nd February, 2025

Source:	Samsung

Title:	Initial simulation results on spatial channel model

Agenda item:	7.16.2

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved, which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#108 in June 2025. The objectives are as below:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In last RAN4#113 meeting, parameter assumptions, CDL methodologies and AAV configurations has been agreed for initial simulation [2].

In this contribution, we share our initial simulation results for CDL-based model and baseline TDL channel model.

- Simulation assumptions

  - Agreed simualtion assumptions

Since this is the initial simulation results and the normalization method of AAV/beamforing is not finally defined, in order to align the implementation of CDL based channel modeling with companies, we only select parts of the agreed simulation assumptions [2] for 4 layers as below.

| Parameter |  | SU-MIMO PDSCH (FR1) | SU-MIMO PMI (FR1) |  | MU-MIMO PDSCH (FR1) |
| --- | --- | --- | --- | --- | --- |
| Duplex mode |  | TDD |  |  |  |
| TDD Slot Configuration Pattern |  | 7DS2U |  |  |  |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |  |  |  |
| UE receiver type |  | IRC Baseline | IRC Baseline |  | IRC, E-IRC |
| Number of Tx Ports |  | 8,4 | 84 |  | 4 |
| Number of Rx Ports/Antennas |  | 8,4 | 4 |  | 4 |
| Number of layers |  | 8,4 | 4,2 |  | 2+2 |
| PMI |  | Random, FFS Fixed | Follow, Random |  | UE1: Random UE2: Random and not equal to UE1, Orthogonal |
| Waveform |  | CP-OFDM with normal CP |  |  |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |  |  |
| MCS |  | 13 (64 QAM table) |  |  |  |
| PDSCH configuration | Mapping type | Type A |  |  |  |
|  | k0 | 0 |  |  |  |
|  | Starting symbol (S) | 2 |  |  |  |
|  | Length (L) | 12 |  |  |  |
|  | PDSCH aggregation factor | 1 |  |  |  |
|  | Resource allocation type | Type 0 |  |  |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |  |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |  |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |  |  |
|  | Number of additional DMRS | 1 |  |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |  |  |
| Codebook configuration | CodebookType | type-I-SP | type-I-SP, eType-II |  | type-I-SP |
|  | Codebook configuration | For 8Tx (N1,N2,O1,O2) = (4,1,4,1) For 4Tx (N1,N2,O1,O2) = (2,1,4,1) | For typeI-SP: (N1,N2,O1,O2) = (4,1,4,1) For eType-II: (N1,N2,O1,O2) = (4,1,4,1) [ paramCombination-r16=5 (L=4, pν=1/4, β=3/4) ] |  | (N1,N2,O1,O2) = (2,1,4,1) |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) For Fixed precoding: Wideband PMI | For typeI-SP (random and follow): Wideband PMI For eType-II with follow and random: TBD |  | For Random precoding: for every PRB Bundle (size=2) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |  |  |  |
|  | Number of CSI-RS ports (X) | 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer | 8 CSI-RS Ports (4,1) 4 CSI-RS Ports (2,1) |  | TBD |
|  | Density (ρ) | 1 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |  |  |  |
| CQI/RI/PMI delay |  | N/A | 6.5 ms | N/A |  |
| Number of HARQ Processes |  | 8 |  |  |  |
| Maximum HARQ transmissions |  | 4 |  |  |  |
| Channel Models |  | The purpose of the study item. |  |  |  |
| Testing Metric |  | Full Throughput Curves |  |  |  |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |  |  |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |  |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |  |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |  |  |
|  | BS Antenna Polarisation | (+45, -45) |  |  |  |
|  | BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |  |  |  |
|  | UE Antenna Polarisation | (0, 90) |  |  |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |  |  |
|  | Antenna Panel Placement | YZ Plane |  |  |  |

  - Other parameters and configurations

Channel Model Configurations

For the channel model configurations, we use option A1 and baseline TDLC300 as below.

- Option A1:  CDL (TR 38.753) with Table 7.2.1-8 in TR 38.827

- Baseline: TDLC300 defined in TS38.101-4

AAV Configurations

For AAV configurations of CDL based channel modeling, below options are used for initial simulation

- Option 1X: with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,8,2,8,4) for 4Tx CSI-RS Ports

- Option 1Y: with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y)

- Option 3: with AAV with the following configuration:

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

Doppler frequencies

- 10Hz

- 100Hz

Correlation matrix for baseline TDLC cases

- TDLC 300-100 ULA Low

- TDLC 300-10 ULA Low

- TDLC 300-100 X-Pol Medium

- Simulation results

Results for PDSCH demodulation performance with random PMI, Doppler 10Hz

Averaged AAV/beamforming gain:

- 7.41dB for CDL-option1X

- 1.96dB for CDL-option 1Y

SNR working points:

SNR working points at 30%, 50%, 70% fractions of maximum throughput could be summarized as below table.

| Fraction of maximum throughput | TDLC300-10 Low corr | CDL option1X | CDL option1Y | CDL option3 |
| --- | --- | --- | --- | --- |
| 30% | 5.4 dB | 7.8 dB | 6.0 dB | 5.4 dB |
| 50% | 10.1 dB | 14.0 dB | 14.0 dB | 14.0 dB |
| 70% | 13.4 dB | 14.8 dB | 15.2 dB | 14.9 dB |

Results for PDSCH demodulation performance with random PMI, Doppler 100Hz

Averaged AAV/beamforming gain:

- 6.71dB for CDL-option1X

- 0.63dB for CDL-option1Y

SNR working points:

SNR working points at 30%, 50%, 70% fractions of maximum throughput could be summarized as below table.

| Fraction of maximum throughput | TDLC300-100 Low corr | TDLC300-100 XPMed corr | CDL option1X | CDL option1Y | CDL option3 |
| --- | --- | --- | --- | --- | --- |
| 30% | 5.4 | 6.7 | 8.4 | 3.8 | 2.7 |
| 50% | 10.5 | 13.4 | 12.0 | 8.0 | 8.0 |
| 70% | 15.2 | 17.6 | 16.8 | 18.4 | 18.1 |

- Reference

[1] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104 meeting, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[2] R4-2419782, Way Forward for [113][320]NR_SCM, RAN4#113 meeting, Nokia

[3] R4-2416591, Way Forward Minutes for [112bis][318] NR_SCM, RAN4#112-bis meeting, Nokia

[4] R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, RAN4#112 meeting, Nokia