3GPP TSG-RAN WG4 Meeting #114	R4-2501431

Athens, GR, 17th – 21nd February, 2025

Source:	Samsung

Title:	Discussion on UE demodulation and CSI performance requirements for 8Rx with MMSE-IRC

Agenda item:	7.18.2

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the revised WID [1] for “NR demodulation performance: Phase 5” has been approved. According the plan, this WI is targeting to be finished in RAN#109 meeting on Sep 2025.

Generally, the enhancements for UE demodulation performance focus on two scenarios, the first one is the inter-cell interference with MMSE-IRC receiver scenario, the second one is the intra-cell inter-user interference with MMSE-IRC receiver scenario.

RAN4#112 meeting is the first meeting for this topic, several high-level agreements are achieved [2]. Cover CQI reporting requirements under ICI for 8Rx is agreed and most parameters for simulation assumptions are agreed in RAN4#112bis [3] and RAN4#113 [4].

In this contribution, we provide our further analysis and views on the open issues for PDSCH demodulation performance parts and simulation assumptions for CQI part.

- Discussion

  - General

Definition of MMSE-IRC receiver capability for 8Rx UE

In last RAN4#113 meeting, several options are proposed as below for MMSE-IRC receiver capability definition.

Definition of MMSE-IRC receiver capability for 8Rx UE - Proposals - Option 1: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver. - Option 2: Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI. - Option 3: Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

As we know, for Rel-18 8Rx, two separate UE capabilities based on UE declaration are defined in TS38.101-4 and TS38.306

Section 3.1 in TS38.101-4

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Section 5.2 in TS38.306

SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

Considering there is no processing difference for UE implementation with MMSE-IRC receiver when handling SU-MIMO and MU-MIMO, and the 8Rx feature is introduced for non-handheld UE in Rel-18, we think update current definition in TS38.101-4 and TS38.306 as similar as option 4 is more appropriate and clearer than introducing new capabilities for MU-MIMO or new applicability rule.

Proposal 1: For the capability defined in TS 38.101-4, update current definition as below

Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

8Rx UE MMSE-IRC receiver coverage

In previous meetings, no agreement for defining one set or two sets of requirements.

Issue 1-1-2: 8Rx UE MMSE-IRC receiver coverage - Proposals - Option 1: RAN4 to define two sets of requirements (baseline 8Rx receiver and simplified 8Rx receiver). - Option 2: Set the criteria for defining two sets of requirements as: If the span after removing a reasonable number or outliers (at least 5 results should be left) is >2.5dB, define two sets of requirements. The evaluation of the span shall be done for each configuration agreed for requirement definition. - Option 3: Postpone defining criteria for decision of one or two sets of requirements until all test cases are finalized.

Per our understanding, RAN4 should try our best to define only one set of test requirements to cover both receivers, no matter the UE receiver is baseline or simplified which should depends on UE implementation, and the decision should be made after checking latest simulation results updated in this meeting.

Proposal 2: For the criteria of both inter-cell interference scenario and intra-cell inter-user interference scenario, RAN4 should try our best to define only one set of test requirements to cover both receivers, no matter the UE receiver is baseline or simplified which should depends on UE implementation, and the decision should be made after checking latest simulation results updated in this meeting.

  - PDSCH requirements for inter-cell interference with MMSE-IRC receiver scenario

Number of interference cells

For the number of interference cells, actually in theory, the UE behavior does not change for both scenarios (HomNet and Hetnet), target UE only cares about the interference INR value.

Number of interference cells - Background: - We have an existing agreement from RAN#112bis, to reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx, with TBD for applicability rules to reduce UE efforts [R4-2416555] - Proposals: - Option 1: Both 1 interference cell and 2 interference cells

Per our understanding, the agreement in RAN4#112bis meeting as below is targeting on the discussion for interference profile and power level, not on the number of interference cells. We assume that’s the reason some companies prefer to think there is no agreement for the number of interference cells.

Agreement in R4-2416555 [3]

Interference profile and power levels Agreement: - Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx. - TBD: Further discuss applicability rules to potentially reduce UE effort to only comply to either 1 or 2 interference cells.

Observation 1: Previous agreement in RAN4#112bis meeting is targeting on the discussion for interference profile and power level, not on the number of interference cells.

In our view, define requirement only for HomNet is sufficient for UE performance verification under inter-cell interference scenario. If requirements are planned to be defined for both HomNet and Hetnet scenarios, we’d like to introduce a new applicability rule to skip Hetnet test case once HomNet test case has passed.

Proposal 3: Define requirement only for HomNet is sufficient for UE performance verification under inter-cell interference scenario. If requirements are planned to be defined for both HomNet and Hetnet scenarios, we’d like to introduce a new applicability rule to skip Hetnet test case once HomNet test case has passed.

Rank

For the Rank of serving cell, companies agreed to evaluate rank 1,2 & 4 in initial simulation analysis and further discuss the down-selection for requirement definition. However, considering this is the rank discussion for inter-cell interference scenario, which means UE is supposed to be distributed at the cell edge where in practical not suitable for high rank.

Rank and MCS Agreement: - Use the following for evaluation. TBD on requirements: - Rank1: MCS17 - Rank2: MCS17 - Rank4: MCS13 - 2nd priority simulations can be removed for next meeting for both baseline and simplified receiver.

Observation 2: This is the rank discussion for inter-cell interference scenario, in which UE is supposed to be distributed at the cell edge where in practical not suitable for high rank.

Proposal 4: For the rank of serving cell for PDSCH demodulation performance with 8Rx inter-cell interference scenario, do not prefer to consider rank 4 for requirements definition.

  - Applicability rule and release independent

Test applicability to skip 2/4Rx requirements

In order to balance the test coverage and test workload, for both demodulation performance tests and CQI reporting tests, we prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 5: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Test applicability for FDD/TDD

Furthermore, we also prefer to introduce similar applicability rules as below in Rel-17 for UE supporting both duplex mode TDD and FDD as legacy 2Rx/4Rx demodulation performance tests, for the purpose to reduce the workload.

- 5.1.1.10 Applicability of requirements for PDSCH with inter cell interference Tests; Applicability notes. All tests in Clause 5.2.2.1.15, 5.2.3.1.15, 5.2.2.2.16 and 5.2.3.2.16; If UE supporting both duplex mode TDD and FDD with 2RX, only test 1-1 in clause 5.2.2.1.15 and test 1-2 in clause 5.2.2.2.16 will be applied. If UE supporting both duplex mode TDD and FDD with 4RX, only test 1-1 in clause 5.2.3.1.15 and test 1-2 in clause 5.2.3.2.16 will be applied..

Proposal 6: For 8Rx inter-cell interference scenario, for demodulation performance tests, prefer to introduce similar applicability rules defined in section 5.1.1.10 for UE supporting both duplex mode TDD and FDD as legacy 2Rx/4Rx. If test requirement is defined only for one of homNet and Hetnet scenarios, for UE with 8Rx supporting both duplex mode TDD and FDD, only need to pass test cases of one duplex mode.

Test applicability for HomNet and Hetnet scenarios

In order to aviod heavy test workload, if test requirements are defined for both HomNet and Hetnet scenarios, we’d like to introduce a new applicability rule to skip Hetnet test case once HomNet test case has passed.

Proposal 7: If requirements are defined for both HomNet and Hetnet scenarios, we’d like to introduce a new applicability rule to skip Hetnet test case once HomNet test case has passed.

Release independent

Since 8Rx is introduced in Rel-18, we think it is reasonable to define the new requirements for 8Rx should be release independent from Rel-18.

Proposal 8: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18.

- Conclusion

In this contribution, we provide analysis and views on NR demodulation performance: Phase 5.

The observations and proposals could be summarized as:

Proposal 1: For the capability defined in TS 38.101-4, update current definition as below

Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Proposal 2: For the criteria of both inter-cell interference scenario and intra-cell inter-user interference scenario, RAN4 should try our best to define only one set of test requirements to cover both receivers, no matter the UE receiver is baseline or simplified which should depends on UE implementation, and the decision should be made after checking latest simulation results updated in this meeting.

Observation 1: Previous agreement in RAN4#112bis meeting is targeting on the discussion for interference profile and power level, not on the number of interference cells.

Proposal 3: Define requirement only for HomNet is sufficient for UE performance verification under inter-cell interference scenario. If requirements are planned to be defined for both HomNet and Hetnet scenarios, we’d like to introduce a new applicability rule to skip Hetnet test case once HomNet test case has passed.

Observation 2: This is the rank discussion for inter-cell interference scenario, in which UE is supposed to be distributed at the cell edge where in practical not suitable for high rank.

Proposal 4: For the rank of serving cell for PDSCH demodulation performance with 8Rx inter-cell interference scenario, do not prefer to consider rank 4 for requirements definition.

Proposal 5: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 6: For 8Rx inter-cell interference scenario, for demodulation performance tests, prefer to introduce similar applicability rules defined in section 5.1.1.10 for UE supporting both duplex mode TDD and FDD as legacy 2Rx/4Rx. If test requirement is defined only for one of homNet and Hetnet scenarios, for UE with 8Rx supporting both duplex mode TDD and FDD, only need to pass test cases of one duplex mode.

Proposal 7: If requirements are defined for both HomNet and Hetnet scenarios, we’d like to introduce a new applicability rule to skip Hetnet test case once HomNet test case has passed.

Proposal 8: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18.

- Reference

[1] RP-241297, Revised WID on NR demodulation performance: Phase 5, RAN#104 meeting, China Telecom, NTT DOCOMO

[2] R4-2413570, WF on NR demodulation performance: Phase 5, RAN4#112 meeting, China Telcom

[3] R4-2416555, WF for [112bis][320] NR_demod_Ph5_Part2_UE, RAN4#112bis meeting, Nokia

[4] R4-2419784, Way forward for [113][322] NR_demod_Ph5_Part2_UE, RAN4#113 meeting, Nokia

[5] R4-2418553, Simulation result summary for ICI and MU-MIMO with 8RX, RAN4#113 meeting, Apple