# 3GPP TSG-RAN WG4 Meeting # 114	R4-2501435

Athens, Greece, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on UL CA_n41C MSD test point and its capture in TS

Agenda Item:	6.3.3

Document for:	Approval

Introduction

In the RAN4#113 meeting, RAN4 reached the way forward [1] on UL CA_n41C MSD test point and its capture in TS for CA_n40A-n41C DL with CA_n41C UL and CA_n41C-n79A DL with CA_n41C UL. There are some open issues on how to capture test points in the TS.

Capturing test points in the TS Background: For the optional second test points using fully allocated CCS for contiguous intra-band UL CA, companies have used different table formats and intended to different TS tables with some using the 2UL IMD table format and others the 1UL crossband isolation table format. Observation: - Whether the allocation is 1RB+1RB or contiguous full allocation, the interference products are related to the IMD range of the two CCs. Especially the IMD4 (+3, -1) is only related to IMDs falling close to the Harmonic 2 and thus are not exactly cross-band isolation. - Whether the allocation is 1RB+1RB or contiguous full allocation, the same 1UL PA non-linearity are tested. - The current 1RB+1RB IMDs MSD may not really belong to the 2UL section as this is 1UL band. - The 1RB=1RB test point should stay mandatory Agreement: - Applicable to 1UL band case with contiguous intra-band ULCA unless otherwise stated. - Companies are encouraged to propose how to capture the two test points in the same table so that it is they are not confused as two separate issues. - The 1RB+1RB is the mandatory baseline test point. - For H2 and H3 related interference, the victim channel is chosen the closest to the related harmonic frequency. - The second optional test point uses contiguous fully allocated intra-band contiguous ULCA case. - FFS if a new table is needed but if so, the legacy 1UL 2CC test points should be moved to that table for both contiguous and non-contiguous intra-band UL CA - FFS any further restrictions on the second test point.

In this paper, we’d like to share our views on how to capture the UL CA_n41C MSD test point in TS.

Discussion

During the discussion, there are two options to capture the two test points.

Option 1: Both 1RB+1RB test case and fully contiguous RB allocation test case are captured into one table.

Option 2: 1RB+1RB test case is captured into the legacy IMD table and fully contiguous RB allocation test case is captured into the cross band isolation MSD table.

Some companies intend to consider option 1 based on the argument that they are not confused as two separate issues. However, in our opinion, the technical interference mechanisms between 1RB+1RB test case and fully contiguous RB allocation test case are totally different.

Referring to the measurement results [2], the discrete IMD interference can be identified easily for 1RB + 1RB test case. However, for the fully contiguous RB allocation test case, it’s very hard to say the interference is only from IMD3 or IMD4 since the interference come from the averaged interference of IMD3+IMD5+IMD7….

Observation 1: the discrete IMD interference can be identified easily for 1RB + 1RB test case.

Observation 2: for the fully contiguous RB allocation test case, it’s very hard to say the interference is only from IMD3 or IMD4 since the interference come from the averaged interference of IMD3+IMD5+IMD7….

Since the technical interference mechanisms between 1RB+1RB test case and fully contiguous RB allocation test case are different, it’s reasonable to consider these two different configurations as two separate issues.

The reason why we have two different configurations is that operators and vendors may have different demands. For 1RB+1RB test case, vendors can specify it as worst case. However, from operators’ perspective, it’s meaningful to verify the fully contiguous RB allocation test case for the system performance, which is the mainly scheduling scenario.

Observation 3: different demands between operators and vendors lead to the compromise that two test cases for UL CA_n41C MSD are proposed. It’s very hard to consider the two test cases to address one same issue.

If we introduce fully contiguous RB allocation test case for UL CA_n41C into the legacy IMD MSD table, it may confuse people why fully contiguous RB allocation test case is considered as IMD case.

Proposal 1: introduce fully contiguous RB allocation test case for UL CA_n41C into the MSD table due to cross band isolation.

Summary

Observation 1: the discrete IMD interference can be identified easily for 1RB + 1RB test case.

Observation 2: for the fully contiguous RB allocation test case, it’s very hard to say the interference is only from IMD3 or IMD4 since the interference come from the averaged interference of IMD3+IMD5+IMD7….

Observation 3: different demands between operators and vendors lead to the compromise that two test cases for Proposal 1: to introduce fully contiguous RB allocation test case for UL CA_n41C into the MSD table due to cross band isolation.

- References

[1] R4-2420471, WF on UL CA_n41C MSD test point and its capture in TS, Skyworks Solution Inc., Huawei, Qualcomm

[2] R4-2407372, On UL configuration for intra-band ULCA IMDs, Skyworks Solutions Inc.