3GPP TSG-RAN WG4 Meeting # 114	R4-2501442

# Athens, Greece, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | DraftCR | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-2 to introduce CA_n258R and CA_n258S |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_SUL_R19-Core |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce CA_n258R and CA_n258S. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce CA_n258R and CA_n258S. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec can’t support CA_n258R and CA_n258S. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.101-2>>

5.5A.1	Configurations for intra-band contiguous CA

Table 5.5A.1-1: NR CA configurations, bandwidth combination sets, and fallback group defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set / Fallback group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | BW channel (MHz) | Maximum aggregated BW (MHz) | BCS | Fallback group |
| CA_n257B | CA_n257B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n257C | CA_n257B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  |  |  |  |  | 1200 | 0 |  |
| CA_n257D | CA_n257D | 50, 100, 200 | 200 |  |  |  |  |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n257E | CA_n257D/E | 50, 100, 200 | 200 | 200 |  |  |  |  |  |  |  |  |  | 600 | 0 |  |
| CA_n257F | CA_n257D/E/F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  |  |  |  |  | 800 | 0 |  |
| CA_n257G | CA_n257G | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n257H | CA_n257G/H | 50, 100 | 100 | 100 |  |  |  |  |  |  |  |  |  | 300 | 0 |  |
| CA_n257I | CA_n257G/H/I | 50, 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  | 400 | 0 |  |
| CA_n257J | CA_n257G/H/I/J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  | 500 | 0 |  |
| CA_n257K | CA_n257G/H/I/J/K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  | 600 | 0 |  |
| CA_n257L | CA_n257G/H/I/J/K/L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  | 700 | 0 |  |
| CA_n257M | CA_n257G/H/I/J/K/L/M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  | 800 | 0 |  |
| CA_n257O | CA_n257O | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n257P | CA_n257O/P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  | 300 | 0 |  |
| CA_n257Q | CA_n257O/P/Q | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 400 | 0 |  |
| CA_n258B | CA_n258B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n258C | CA_n258B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  |  |  |  |  | 1200 | 0 |  |
| CA_n258D | CA_n258D | 50, 100, 200 | 200 |  |  |  |  |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n258E | CA_n258D/E | 50, 100, 200 | 200 | 200 |  |  |  |  |  |  |  |  |  | 600 | 0 |  |
| CA_n258F | CA_n258D/E/F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  |  |  |  |  | 800 | 0 |  |
| CA_n258R | CA_n258D/E/F/R | 50, 100, 200 | 200 | 200 | 200 | 200 |  |  |  |  |  |  |  | 1000 | 0 |  |
| CA_n258S | CA_n258D/E/F/R/S | 50, 100, 200 | 200 | 200 | 200 | 200 | 200 |  |  |  |  |  |  | 1200 | 0 |  |
| CA_n258G | CA_n258G | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n258H | CA_n258G/H | 50, 100 | 100 | 100 |  |  |  |  |  |  |  |  |  | 300 | 0 |  |
| CA_n258I | CA_n258G/H/I | 50, 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  | 400 | 0 |  |
| CA_n258J | CA_n258G/H/I/J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  | 500 | 0 |  |
| CA_n258K | CA_n258G/H/I/J/K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  | 600 | 0 |  |
| CA_n258L | CA_n258G/H/I/J/K/L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  | 700 | 0 |  |
| CA_n258M | CA_n258G/H/I/J/K/L/M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  | 800 | 0 |  |
| CA_n258O | CA_n258O | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n258P | CA_n258O/P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  | 300 | 0 |  |
| CA_n258Q | CA_n258O/P/Q | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 400 | 0 |  |
| CA_n258R2 | CA_n258R2 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |  |  |  | 400 | 0 | 5 |
| CA_n258R3 | CA_n258R2/R3 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |  |  | 600 | 0 |  |
| CA_n258R4 | CA_n258R2/R3/R4 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |  | 800 | 0 |  |
| CA_n258R5 | CA_n258R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  | 1000 | 0 |  |
| CA_n258R6 | CA_n258R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  | 1200 | 0 |  |
| CA_n258R7 | CA_n258R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  | 1400 | 0 |  |
| CA_n258R8 | CA_n258R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  | 16004 | 0 |  |
| CA_n258R9 | CA_n258R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  | 18004 | 0 |  |
| CA_n258R10 | CA_n258R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  | 20004 | 0 |  |
| CA_n258R11 | CA_n258R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  | 22004 | 0 |  |
| CA_n258R12 | CA_n258R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 24004 | 0 |  |
| CA_n259B | CA_n259B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n259C | CA_n259B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  |  |  |  |  | 1200 | 0 |  |
| CA_n259G | CA_n259G | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n259H | CA_n259G/H | 50, 100 | 100 | 100 |  |  |  |  |  |  |  |  |  | 300 | 0 |  |
| CA_n259I | CA_n259G/H/I | 50, 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  | 400 | 0 |  |
| CA_n259J | CA_n259G/H/I/J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  | 500 | 0 |  |
| CA_n259K | CA_n259G/H/I/J/K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  | 600 | 0 |  |
| CA_n259L | CA_n259G/H/I/J/K/L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  | 700 | 0 |  |
| CA_n259M | CA_n259G/H/I/J/K/L/M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  | 800 | 0 |  |
| CA_n260B | CA_n260B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n260C | CA_n260B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  |  |  |  |  | 1200 | 0 |  |
| CA_n260D | CA_n260D | 50, 100, 200 | 200 |  |  |  |  |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n260E | CA_n260D/E | 50, 100, 200 | 200 | 200 |  |  |  |  |  |  |  |  |  | 600 | 0 |  |
| CA_n260F | CA_n260D/E/F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  |  |  |  |  | 800 | 0 |  |
| CA_n260G | CA_n260G | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n260H | CA_n260G/H | 50, 100 | 100 | 100 |  |  |  |  |  |  |  |  |  | 300 | 0 |  |
| CA_n260I | CA_n260G/H/I | 50, 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  | 400 | 0 |  |
| CA_n260J | CA_n260G/H/I/J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  | 500 | 0 |  |
| CA_n260K | CA_n260G/H/I/J/K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  | 600 | 0 |  |
| CA_n260L | CA_n260G/H/I/J/K/L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  | 700 | 0 |  |
| CA_n260M | CA_n260G/H/I/J/K/L/M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  | 800 | 0 |  |
| CA_n260O | CA_n260O | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n260P | CA_n260O/P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  | 300 | 0 |  |
| CA_n260Q | CA_n260O/P/Q | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 400 | 0 |  |
| CA_n260R2 | CA_n260R2 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |  |  |  | 400 | 0 | 5 |
| CA_n260R3 | CA_n260R2/R3 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |  |  | 600 | 0 |  |
| CA_n260R4 | CA_n260R2/R3/R4 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  |  | 800 | 0 |  |
| CA_n260R5 | CA_n260R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  |  | 1000 | 0 |  |
| CA_n260R6 | CA_n260R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  |  | 1200 | 0 |  |
| CA_n260R7 | CA_n260R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  |  | 1400 | 0 |  |
| CA_n260R8 | CA_n260R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  |  | 16004 | 0 |  |
| CA_n260R9 | CA_n260R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  |  | 18004 | 0 |  |
| CA_n260R10 | CA_n260R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  |  | 20004 | 0 |  |
| CA_n260R11 | CA_n260R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  | 22004 | 0 |  |
| CA_n260R12 | CA_n260R2/R3/R4/R55 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 100, 200 | 24004 | 0 |  |
| CA_n261B | CA_n261B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n261C | CA_n261B | 50 | 400 | 400 |  |  |  |  |  |  |  |  |  | 850 | 0 |  |
| CA_n261D | CA_n261D | 50, 100, 200 | 200 |  |  |  |  |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n261E | CA_n261D/E | 50, 100, 200 | 200 | 200 |  |  |  |  |  |  |  |  |  | 600 | 0 |  |
| CA_n261F | CA_n261D/E/F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  |  |  |  |  | 800 | 0 |  |
| CA_n261G | CA_n261G | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n261H | CA_n261G/H | 50, 100 | 100 | 100 |  |  |  |  |  |  |  |  |  | 300 | 0 |  |
| CA_n261I | CA_n261G/H/I | 50, 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  | 400 | 0 |  |
| CA_n261J | CA_n261G/H/I/J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  | 500 | 0 |  |
| CA_n261K | CA_n261G/H/I/J/K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  | 600 | 0 |  |
| CA_n261L | CA_n261G/H/I/J/K/L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  | 700 | 0 |  |
| CA_n261M | CA_n261G/H/I/J/K/L/M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  | 800 | 0 |  |
| CA_n261O | CA_n261O | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n261P | CA_n261O/P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  |  | 300 | 0 |  |
| CA_n261Q | CA_n261O/P/Q | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  |  |  |  | 400 | 0 |  |
| CA_n262G | CA_n262G | 50, 100 | 100 |  |  |  |  |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n262H | CA_n262G/H | 50, 100 | 100 | 100 |  |  |  |  |  |  |  |  |  | 300 | 0 |  |
| CA_n262I | CA_n262G/H/I | 50, 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  | 400 | 0 |  |
| CA_n262J | CA_n262G/H/I/J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  | 500 | 0 |  |
| CA_n262K | CA_n262G/H/I/J/K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  | 600 | 0 |  |
| CA_n262L | CA_n262G/H/I/J/K/L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  | 700 | 0 |  |
| CA_n262M | CA_n262G/H/I/J/K/L/M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  | 800 | 0 |  |
| CA_n263B | CA_n263A | 400 | 400 |  |  |  |  |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n263C | CA_n263A | 400 | 400 | 400 |  |  |  |  |  |  |  |  |  | 1200 | 0 | 1 |
| CA_n263G | CA_n263A | 100 | 100 |  |  |  |  |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n263H | CA_n263A | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  | 300 | 0 | 3 |
| CA_n263I | CA_n263A | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  | 400 | 0 | 3 |
| CA_n263J | CA_n263A | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  | 500 | 0 | 3 |
| CA_n263K | CA_n263A | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |  | 600 | 0 | 3 |
| CA_n263L | CA_n263A | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  | 700 | 0 | 3 |
| CA_n263M | CA_n263A | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  | 800 | 0 | 3 |
| NOTE 1: Void NOTE 2: For the NR CA configuration with more than two component carries, the bandwidths in a BCS which may introduce combinations more than requested unintentionally should be listed in a row separately. NOTE 3: In this release of the specification, contiguous DL CA configurations within FR2-2 may only contain multiples of the same channel bandwidth. NOTE 4: In Rel-18 maximum aggregated downlink BW is limited to 1600 MHz. NOTE 5: In Rel-18 maximum aggregated uplink BW is limited to 800 MHz. NOTE 6: The delimiter “/” is only used in the uplink configurations for the sake of simplicity. For example, CA_nyA/B/C denotes CA_nyA, CA_nyB and CA_nyC, where ny is a FR2 NR band and A, B and C are the corresponding bandwidth classes respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<End of Change>>