# 3GPP TSG-RAN WG4 Meeting # 114	R4-2501456

# Athens, Greece, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on VSAT requirements for NR NTN Ku band

Agenda Item:	7.11.4

Document for:	Approval

Introduction

In the RAN4#113 meeting, RAN4 made some progress on the Maximum EIRP and TRP requirements for Ku band [1].

- Issue 1-2-2: Maximum EIRP and TRP requirement - The following aspects will be consider if setting the maximum EIRP & TRP of the NTN Ku band in 38.101-5 - An absolute maximum upper limit on 88.2 dBm EIRP fulfilling GSO scenarios, while allowing separating consideration for non-GSO scenarios - Upper EIRP limits in the regulation - Enable flexibility for different application and scenarios - Issue 1-2-3: Maximum EIRP requirements and the minimum antenna size for the CEPT Ku band - For the CEPT Ku band, the following requirements from ECC regulation will be considered, FFS on how to reflect to the specifications and the VSAT types. - NTN VSAT operating with GSO 14.0 – 14.25 GHz Max EIRP: 60 dBW (from ECC Decision(17)04) - NTN VSAT operating with GSO 14.25 – 14.5 GHz Max EIRP: 50 dBW (from ECC Decision(03)04) - NTN VSAT operating with GSO 14.0 – 14.25 GHz Max EIRP: 60 dBW (from ECC Decision(06)03) - Mobile VSAT / Land ESIM 14.0 – 14.5 GHz Max EIRP: 54.5 dBW (from ECC Decision(18)04) - Mobile VSAT / Maritime ESIM 14.0 – 14.5 GHz minimum antenna size: 0.6 m (from ECC Decision (05)10)

In this paper, we’d like to share our views on these open issues.

Discussion

For the Maximum EIRP and TRP requirement, RAN4 could consider some flexibilities for different scenarios. For example, an absolute maximum upper limit on 88.2 dBm EIRP fulfilling GSO scenarios can be specified. For Ku VSAT targeting NGSO scenarios, RAN4 can consider a separate maximum upper limit.

Proposal 1: When defining the Maximum EIRP and TRP requirement, RAN4 could consider some flexibilities for different scenarios. For example:

An absolute maximum upper limit on 88.2 dBm EIRP fulfilling GSO scenarios can be specified.

A separate maximum upper limit can be specified for Ku VSAT targeting NGSO scenarios.

For the CEPT Ku band, the following requirements from ECC regulation will be considered in RAN4.

- NTN VSAT operating with GSO 14.0 – 14.25 GHz Max EIRP: 60 dBW (from ECC Decision(17)04) - NTN VSAT operating with GSO 14.25 – 14.5 GHz Max EIRP: 50 dBW (from ECC Decision(03)04) - NTN VSAT operating with GSO 14.0 – 14.25 GHz Max EIRP: 60 dBW (from ECC Decision(06)03) - Mobile VSAT / Land ESIM 14.0 – 14.5 GHz Max EIRP: 54.5 dBW (from ECC Decision(18)04) - Mobile VSAT / Maritime ESIM 14.0 – 14.5 GHz minimum antenna size: 0.6 m (from ECC Decision (05)10)

One option is to specify the 50dBW which can meet all the ECC decision requirements. Another option is to consider Pemax which can limit the VSAT EIRP by network indication.

Proposal 2: RAN4 can discuss the following two options on how to reflect ECC decision Max EIRP requirements into the specification.

Option 1: to specify the 50dBW which can meet all the ECC decision Max EIRP requirements.

Option 2: to consider Pemax which can limit the VSAT EIRP by network indication.

Summary

Proposal 1: When defining the Maximum EIRP and TRP requirement, RAN4 could consider some flexibilities for different scenarios. For example:

An absolute maximum upper limit on 88.2 dBm EIRP fulfilling GSO scenarios can be specified.

A separate maximum upper limit can be specified for Ku VSAT targeting NGSO scenarios.

Proposal 2: RAN4 can discuss the following two options on how to reflect ECC decision Max EIRP requirements into the specification.

Option 1: to specify the 50dBW which can meet all the ECC decision Max EIRP requirements.

Option 2: to consider Pemax which can limit the VSAT EIRP by network indication.

- References

[1] R4-2419790, Way Forward for [113][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL