3GPP TSG-RAN WG4 Meeting # 114	R4-2501468

# Athens, Greece, 17th - 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5359 | rev | - | Current version: | 16.22.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_RRM_enh-Core)Correction on RRC based BWP switch delay for multiple CCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-Core |  |  |  |  |  | Date: |  |  | 2025-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There is no clear definition in 8.6.3A of NR Slot length, although there is definition in single CC scenario in 8.6.3. It is ambiguous of whether the same definition can be used or not. - For simultaneous RRC based BWP switch delay, the definition is the smallest SCS among all SCS values - For non-simultaneous RRC based BWP switch delay, the BWP switch does not involve SCS change, the definition is the smallest SCS among all the CCs in the second CG - According to 38.306, D can be multiple values, DRRC*(N-1) or DRRC*(M-1) may be not the integral times of “NR Slot legnth”. Ceil function is needed to avoid fractional value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add definition of “NR slot length” in 8.6.3A.1 & 8.6.3A.2 - Add ceil funtion in formula. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is ambiguity of the RRC based BWP switch delay for multi-CCs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.3A.1 8.6.3A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------Start Change #1-------------------------

8.6.3A	RRC based BWP switch delay on multiple CCs

The requirements in this clause only apply to the case when the same type of BWP switch (RRC based BWP switch) is performed on multiple CCs simultaneously or over partially overlapping time period.

The requirements in this clause shall apply:

- Active BWP switching or parameter change of its active BWPs for SpCell

- Parameter change of its active BWPs except parameter firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id for SCells

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs

Requirements in this clause apply only if RRC based BWP switching on multiple CCs for NR-CA is triggered by a single RRC command.

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWPs, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWPs on the serving cells on which BWP switch occurs on the first DL or UL slot right after a time duration  of  TRRCprocessingDelay+TBWPswitchDelayRRC+DRRC*(N-1)NR slot lengthTRRCprocessingDelay+TBWPswitchDelayRRC+DRRC*(N-1)NR slot lengthslots which begins from the beginning of DL slot n, where

DL slot n is the last slot overlapping with the PDSCH containing the RRC command, and

NR Slot length is determined by the smallest SCS among all SCS values of all involved CCs before and after BWP switch if the BWP switch involves changing of SCS, and

TRRCprocessingDelay and TBWPswitchDelayRRC are defined in clause 8.6.3, and

DRRC=0 for UE which is capable of type 1 BWP switching delay depending on UE capability bwp-SwitchingDelay [2]. DRRC=D for UE which is capable of type 2 BWP switching delay depending on UE capability bwp-SwitchingDelay [2], where D is the incremental delay for each additional CC involved in simultaneous BWP switch and depends on UE capability [TS 38.306, 14].

N is the number of CCs within the NR-CA configured for performing simultaneous BWP switch.

The UE is not required to transmit UL signals or receive DL signals during the time defined by TRRCprocessingDelay+TBWPswitchDelayRRC+DRRC*(N-1)  on the cells where RRC-based BWP switch occurs.

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs

In non-simultaneous case, the RRC-based BWP switch on multiple CCs is triggered over partially overlapping time period in different Cell groups. The delay requirements in this clause apply only if:

BWP switching on multiple CCs in different cell groups are triggered by separate RRC commands, and

UE is operating in NR-DC (FR1+FR2), and

UE is capable of per-FR gap, and

BWP switch does not involve SCS change.

For non-simultaneous RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWPs, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWPs on the serving cells on which BWP switch occurs on the first DL or UL slot right after a time duration  of  TWaiting+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRC*(M-1)NR slot lengthTWaiting+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRC*(M-1)NR slot length slots which begins from the beginning of DL slot n, where

DL slot n is the last slot containing the RRC command,

TWaiting is the waiting time for RRC based BWP switch which is upper bounded by the ongoing BWP switch time in the first CG defined in clause 8.6.3A.1,

M is the number of CCs within the NR-CA configured for performing simultaneous BWP switch in the second CG; M=1 if the BWP switch is performed on single CC,

NR Slot length is determined by the smallest SCS among all the CCs in the second CG, and

TRRCprocessingDelay and TBWPswitchDelayRRC are defined in clause 8.6.3, and

DRRC is defined in clause 8.6.3A.1.

The UE is not required to transmit UL signals or receive DL signals during the time defined by TRRCprocessingDelay+TBWPswitchDelayRRC  +DRRC*(M-1)  on the cells in the second CG where RRC-based BWP switch occurs.

----------------------------------------End Change #1-------------------------