3GPP TSG-RAN WG4 Meeting #114	R4-2501474
Athens, Greece, 17th – 21nd February, 2025

Agenda item:	7.10.2.2.1

Source: 	OPPO

Title: 	Discussion on Tx requirements for NR-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Tx RF requirements with NR NTN operating in FR1-NTN bands were discussed and some agreements are approved in WF [1]:

Agreements: Issue 2-1: MOP - Agreement: - Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC2 for NR-NTN Non-Redcap HPUE Issue 2-2: MPR/A-MPR - Agreement: - MPR for NTN HPUE should be further evaluated according to the agreed ACLR value, PA model and other limiting factor for MPR evaluation. Issue 2-3: SEM - Way forward: - Follow previous agreement that re-use existing SEM from PC3 to other PCs as starting point. - Further discuss if SEM can be relaxed for the best power gains for NTN with co-ex study outcomes. Issue 2-4: ACLR - Way forward: - Further discuss the ACLR requirements for NR-NTN HPUE - Take into account the co-ex study outcomes in this #113 meeting with previous agreements. Open issues Issue 2-5: NR-NTN PC2 UE - Proposals - Option 1: In Rel-19 RAN WG4 can focus on 26dBm 1TX (PC2) for handheld devices. (P1a/R4-2417747) - Option 2: For handheld UE, 1Tx and 2Tx are covered with existing TN 26dBm and 23dBm PAs respectively. (P1/R4-2419521) Issue 2-6: NR-NTN PC1.5 UE - Proposals PC1.5 UE; 1Tx; 2Tx. Handheld; To specify: Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015, P1/R4-2419521 FFS:; To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532 Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015 FFS:. Non-handheld; To specify: Not to specify: P1/R4-2419668, P3/R4-2419072 FFS: P5/R4-2418200, P1/R4-2419521; To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532, P2/R4-2419521 Not to specify: P1/R4-2419668, P3/R4-2419072, P1b/R4-2417747 FFS:. Issue 2-7: NR-NTN PC1 UE - Proposals PC1; 1Tx; 2Tx; 3Tx; 4Tx. Handheld; To specify: Not to specify: P1/R4-2419521 FFS:; To specify: Not to specify: P5/R4-2418200, P1/R4-2419521 FFS:; To specify: P1/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200; To specify: Not to specify: FFS:. Non-handheld Only non-handheld: P2/R4-2419015, P1/R4-2419072; To specify: P1a/R4-2417747, P2/R4-2419521 Not to specify: FFS:; To specify: Not to specify: P5/R4-2418200 FFS:; To specify: P2/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200; To specify: P4/R4-2419072 Not to specify: FFS:. Issue 3-1-1: General considerations - Proposals - Option 1: For power class 2, there is no differentiation of handheld and non-handheld UE. (P2b/R4-2418201) - Option 2: The uplink configuration for reference sensitivity normally doesn’t depend on the power class, so for PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration as for PC3 NR NTN UE. (P2/R4-2419717)

This contribution further discussed the UE Tx RF requirements for NR NTN HPUE.

- Discussion

In the R-19, the purpose of NTN HPUE is to introduce PC1, PC1.5 and PC2 for NTN bands and

- PC2 will apply to both handheld and non-handheld UE

- PC1 and PC1.5 are prioritized for non-handheld UE

In previous meeting, the architecture of NTN HPUE was discussed, and it agreed for PC2 (NR NTN), 1 Tx and 2 Tx for PC2 NR NTN UE are to be considered for requirements and the architecture for other power class need further study.

PC1.5 for TN was introduced only for TDD currently and the typical architecture is 2Tx and 4Tx also is introduced. Although it doesn’t need duplexer for TDD bands, it still needs 2Tx for PC1.5 for handheld UE. That is, there is no single PA whose output power are enough to support 29dBm considering the front-end loss. For FDD bands, the insertion loss of duplexer also needs consider into the PA’s output power, it’s hard to achieve by 1PA. In additional, there is no suitable duplexer for PC 1.5 handheld UE. Therefore, it’s impossible for 1 Tx to support PC1.5 handheld UE.

Although it’s feasible for 2Tx to support PC1.5 NTN handheld UE based on PC2 with 1Tx. Here we can review the MPR for PC1.5 with 2Tx and PC2 with 1Tx in TN for 16QAM as shown in Table 2-1, and compare the maximum output power between PC1.5 and PC2 considering MPR as Table 2-2:

Table 2-1: Maximum power reduction (MPR) for TN 16QAM

|  | Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| PC1.5 with 2Tx | CP-OFDM | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
| PC2 with 1Tx | CP-OFDM | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |

Table 2-2: Compared the maximum output power considering the MPR

|  | Modulation |  | Maximum output power(dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| PC1.5 with 2Tx | CP-OFDM | 16 QAM | 22.5 | 24.5 | 26.5 |
| PC2 with 1Tx | CP-OFDM | 16 QAM | 22.5 | 23 | 24 |
| Delta between PC1.5 and PC2 | CP-OFDM | 16 QAM | 0 | 1.5 | 2.5 |

Based on Table 2-2, PC1.5 with 2Tx can get the 2.5dB diversity gain compared with PC2 only under the inner RB allocations. Although, the above analysis is based on MPR of TN, the ACLR of NTN for PC2 and PC1.5 is not significantly different from that of TN from the coexistence simulation results in last meeting. The MPR for NTN will very similar with the value of TN based on the same SEM and similar ACLR, it’s unnecessary for 2 Tx to support PC1.5 handheld UE.

Proposal 1: PC1.5 only apply to non-handheld UE in Rel-19.

About PC1, it applied to non-handheld UE only, the main limitation is still because of the higher power PA and duplexer.

Proposal 2: PC1 only apply to non-handheld UE in Rel-19.

- Conclusion

In this paper we give further discussion on the TX requirement for HPUE NTN. The proposals are as following:

Proposal 1: PC1.5 only apply to non-handheld UE in Rel-19.

Proposal 2: PC1 only apply to non-handheld UE in Rel-19.

- Reference

[1] R4-2420403, WF on co-existence study and HPUE for NR-NTN, Samsung, RAN4#113 2024-11