3GPP TSG-RAN WG4 Meeting #113	R4-2501484

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	On introducing requirements for CB-MSG3 in TS 36.133

Agenda item:	7.30.3

Document for:	Discussion

Introduction

In RAN2 #126, RAN2 has reached agreements in the Rel-19 WI related to NB-IoT over NTN (IoT_NTN_Ph3-Core) to introduce the contention-based message 3 (CB-MSG3), where the UE skips the Msg1 and Msg2 in the Random Access procedure, for faster data dump. This was indicated by RAN2 in their LS to RAN4 (R2-2405769).

RAN4 has initially provided its views on the matter analyzing the potential impacts on the timing requirements of such feature.

The work has progressed in RAN2 and several agreements were reached consolidating the feature. In this document we analyze the potential impact of these features in RAN4.

Discussion

RAN2 has agreed upon the introduction of CB-MSG3 and has reached several agreements related to this feature (the list below is non-exhaustive and provide only a few examples of agreements reached by RAN2):

- Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

In special, the parts highlighted in blue indicate procedural changes on the UE, akin to the procedures described for the Random Access procedure (Section 6.6A). When the full procedure is established in RAN2, RAN4 would need to provide specific requirements on the UE behaviour (RSRP thresholds, resource selection, fallback triggers, etc).

Therefore, we propose to include the requirements for CB-MSG3 transmission in a new subclause of 6.6A

Include procedural requirements for CB-MSG3 transmission in a new subclause of 6.6A in TS36.133.

The suggested format would include:

- UE behavior when contention resolution timer expires: whether the UE will select a new preamble depends on the new behaviour to be defined in RAN2. Another problem is that the transmission of a new PRACH after a failed contention based is now dependent upon the expiration of the backoff timer. However, the backoff timer is never initiated as the UE does not monitor for Msg2.

- Updates on the Correct behaviour when receiving a message over Temporary C-RNTI

- Updates, if needed, on the PRACH transmission power when occurring as fallback from CB-MSG3 transmission.

Conclusion

In this paper we analyzed the need for updates on the RAN4 requirements to introduce requirements related to CB-MSG3 feature and the following proposal was made:

Proposal 1: Include procedural requirements for CB-MSG3 transmission in a new subclause of 6.6A in TS36.133.

- References

- R2-2405769, “LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN2#126, Fukuoka, Japan, May 20 – May 24, 2024.

- R4-2414114, “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”,

- R1-2407548 “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024

- TS 36.133 “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”, 3GPP, Release 18.6.0

- TS 36.211 “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”, 3GPP, Release 18.6.0