3GPP TSG-RAN WG4 Meeting #113	R4-2501487

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	On the impact of extended SSB periodicity on the main NTN cell

Agenda item:	7.29.4.2

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The WID contain a handful of objectives and among them, there is one topic on Downlink Coverage Enhancements. One of the main features discussed to be implemented as part of this feature is an extended SSB periodicity for the main (SA) cell in NTN, to match the periodic “active” and “inactive” periods of the cell. The switching between “active” and “inactive” is created in NTN to handle the limited power availability in the satellite.

The discussions of this WI in RAN4 were initiated in the meeting RAN4 #112. Discussions ensued in the following meetings, including RAN4#113 and as the outcome of the discussion a way-forward (WF) document was approved which contains a set of agreements and other way-forward (for further discussion) guidance for currently identified open issues [3].

In this document we provide our views related to current open issues on “Downlink Coverage Enhancements”.

Discussion

Recent agreements in RAN1 and in RAN plenary have limited the extension of the SSB periodicity up to 160 ms. The reason for this choice is that this SSB periodicity is already implemented in earlier versions of the specification – not requiring significant modification on chipset developments -   and can be configured by available signalling. Although 160 ms is a valid SSB periodicity to be configured, it was not previously expected as the SSB periodicity of a main cell in standalone scenarios (as is the case for NTN).

Therefore, there is currently discussions in RAN4 about whether some requirements need to be adapted to handle the extended periodicity. The list of current open issues identified in [3] is given by:

Issue 1-1: SSB periodicity enhancement Agreement (online): For SSB periodicity up to 160ms: - No further discussion on the impact on initial access from the perspective of RRM requirement - Discuss whether to Modify the default values for SSB and SMTC periodicities for the following if the UE is not provided with SMTC configuration or measurement object on the corresponding frequency by NW - RRC Re-establishment: TSMTC,i assumption - RRC connection release with redirection to NR: Trs assumption - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync: Trs assumption - Further discuss the details on how to update the requirement. - Further discuss the following for SSB periodicity > 20ms: - Cell Reselection: for the SSB-periodicity of 160ms, whether the values for Tmeasure_NR_xxx_enh need to be updated for DRX Cycles equal to 0.32 s and other DRX cycles - Paging interruption in RRC Idle/Inactive: whether/how to adjust the requirements when SSB periods =160 ms in NGSO scenarios - RRC re-establishment: RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync - Further discuss whether and how to update the requirement

Mobility requirements when SMTC is not configured for the target frequency

The first set of requirements require discussion on RAN4 about whether they need to be modified, and they were first  introduced to discussion by [6].  In TS 38.133, the following requirements are affected by this discussion:

- HO (RACH/RACH-less, CHO): if the UE is not explicitly provided with SMTC configuration on the target frequency the requirement in this clause is applied with Trs = 5ms, assuming a periodicity of 5 ms for the SSB transmission from the target object.

- For RRC Re-establishment for Satellite Access a similar definition is made, where the SSB periodicity is assumed not larger than 20 ms if SMTC for the interfrequency layer is not configured.

- For RRC Connection release with redirection to NR, a SSB periodicity not larger than 20 ms is assumed. Otherwise, there is no requirement applicable.

For case A, it seems that even for the Rel-17/18 NTN applications, where a likely implementation is the SSB periodicity equal to 20 ms, there would be no requirements when no SMTC is configured for the target frequency (as 20 ms are above the 5 ms described in the  requirement). Therefore, if the same thinking is adopted in the feature with extended periodicity, there is no need to update the requirements.

For Cases B and C, the SSB periodicity is set to default value of 20 ms. In the absence of the SMTC configuration the UE cannot possibly distinguish whether the neighbor cell in a  neighbor frequency is using the extended SSB periodicity or not.

Not update the requirements related to the absence of SMTC configuration when extended periodicity is used.

Paging interruption

Paging interruption requirements state that:

#### 4.2C.2.5 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms. Where, If the target cell belongs to the same satellite as the current one, and if the target cell is known, then K = 2. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is GEO, and if the target cell is known, then K = 2. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO, then K = 5 if the target cell is known.

The multiplying factor is added to create to allow the UE to have enough time to acquire DL synchronization to the target cell, in order to be capable to receive the paging messages. However, if a non-GEO target cell is used the current multiplying factor is equal to 5, which was justifiable with short SSB configurations, however, if the same scaling factor is maintained when the SSB periods is equal to 160 ms, the total interruption time will be given by: 5*160=900ms + TSI-NR might be expected, which may exceed 1 second of interruption in a system with already very long natural interruption times due to cell DTX.

Therefore, we propose that:

Adjust the paging interruption requirements when SSB periods equal or larger 160 ms are used in NGSO scenarios.

Conclusion

In this paper we analyzed some of the requirements when SSB with extended periodicity is adopted in NR NTN, and we make the following proposals:

Proposal 1: Not update the requirements related to the absence of SMTC configuration when extended periodicity is used.

Proposal 2: Adjust the paging interruption requirements when SSB periods equal or larger 160 ms are used in NGSO scenarios.

- References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2420105, " WF on RRM requirements for NR_NTN_Ph3_Part2", Moderator (Qualcomm Incorporated), RAN4 #114 Orlando, US, 18th – 22nd November, 2024.

- R2-2407623, “LS on common TA in a regenerative payload scenario”, China Mobile, Maastricht, Netherlands, 19th – 23rd August 2024

- R1-2407538, “Draft Reply LS on DL coverage enhancements”, China Mobile, RAN1 #118, Maastricht, The Netherlands, 19th – 23rd August, 2024

- R4-2418569, “On R19 other NTN RRM requirements”, Apple, RAN4 #114 Orlando, US, 18th – 22nd November, 2024.

- R4-2419151, “Discussion on other RRM requirements for Rel-19 NTN”, Huawei, HiSilicon,