3GPP TSG-RAN WG4 Meeting #114											R4-2501492
Athens, GR, 17th – 21st February, 2025

Title:	R19 MPR for PC1.5 contig CA

Source:	OPPO

Agenda item:	7.1.1.1.2

Document for:	Approval

- Introduction

This paper discusses the Rel-19 PC1.5 contiguous CA MPR based on WF [1] and also requirement clarifications in [2][3].

- Contiguous RB allocation MRP

The MPR simulation results have been shared in previous meetings in [4] for contiguous CA. And large number of simulations have been done to sweep more cases, which can be found in:

- Annex A for inner RB /CP-OFDM

- Annex B for outer RB /CP-OFDM

- Annex C for inner RB /DFT-S-OFDM

- Annex D for outer RB /DFT-S -OFDM

The configurations are:

- CA class C

- CA power class PC1.5

- Two PC2 PA

- CP-OFDM /DFT-S-OFDM

- scs is 30khz

- LO number is 1LO+TxD

- Modulations are same in both CCs

- -28dBc carrier leakage and IQ image, -60dBc CIM3

- EVM/IBE are not verified if two CCs both have configured RBs

Table 1 summarized the MPR simulation results for contiguous RB allocations with necessary additional implementation margins being added.

Table 1 MPR for PC1.5 intra-band contiguous CA with contig RB allocations

| Modulation |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.5 | 5 |
|  | QPSK | 3.5 | 5 |
|  | 16QAM | 4 | 5.5 |
|  | 64QAM | 4 | 5.5 |
|  | 256QAM | 5.5 | 6 |
| CP-OFDM | QPSK | 5 | 6.5 |
|  | 16QAM | 5 | 6.5 |
|  | 64QAM | 5 | 7 |
|  | 256QAM | 6 | 7.5 |

The simulated values are relatively smaller than what have been defined for PC2 with 2T. The difference may come from whether the worst case have been swept, PA model differences, etc. Due to time reasons, the analysis cannot be done before this meeting, we may continue to look at it during or after this meeting.

Proposal 1:         Consider MPR results for contiguous RB allocations in table 1.

- Non-Contig RB allocation MRP

For non-contiguous RB allocations we do not have time to do more simulation than the results provided in last meeting in [4]. Therefore, the previous proposed results in [4] are reproduced below for discussion.

The configurations are:

- CA class C

- CA power class PC1.5

- Two PC2 PA

- CP-OFDM /DFT-S-OFDM

- scs is 15khz or 30khz

- LO number is 1LO+TxD

- Modulations are same in both CCs

Above MPR results can be summarized as below table 2 with some necessary margins added.

Table 2 MPR for non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 5 | 6.5 | 8.5 |
|  | QPSK | 5 | 7 | 8.5 |
|  | 16QAM | 5 | 7.5 | 8.5 |
|  | 64QAM | 5 | 7.5 | 8.5 |
|  | 256QAM | 5 | 8 | 9 |
| CP-OFDM | QPSK | 5 | 8.5 | 9 |
|  | 16QAM | 5 | 8.5 | 9 |
|  | 64QAM | 5 | 8.5 | 9 |
|  | 256QAM | 5 | 8.5 | 9 |

Proposal 2:         Consider MPR results for non-contiguous RB allocations in two CCs in table 2.

- Conclusion

This paper shared more MPR simulation results for PC1.5 intra-band contiguous CA, got below proposals.

Proposal 1:         Consider MPR results for contiguous RB allocations in table 1.

Table 1 MPR for PC1.5 intra-band contiguous CA with contig RB allocations

| Modulation |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.5 | 5 |
|  | QPSK | 3.5 | 5 |
|  | 16QAM | 4 | 5.5 |
|  | 64QAM | 4 | 5.5 |
|  | 256QAM | 5.5 | 6 |
| CP-OFDM | QPSK | 5 | 6.5 |
|  | 16QAM | 5 | 6.5 |
|  | 64QAM | 5 | 7 |
|  | 256QAM | 6 | 7.5 |

Proposal 2:         Consider MPR results for non-contiguous RB allocations in two CCs in table 2.

Table 2 MPR for non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 5 | 6.5 | 8.5 |
|  | QPSK | 5 | 7 | 8.5 |
|  | 16QAM | 5 | 7.5 | 8.5 |
|  | 64QAM | 5 | 7.5 | 8.5 |
|  | 256QAM | 5 | 8 | 9 |
| CP-OFDM | QPSK | 5 | 8.5 | 9 |
|  | 16QAM | 5 | 8.5 | 9 |
|  | 64QAM | 5 | 8.5 | 9 |
|  | 256QAM | 5 | 8.5 | 9 |

- Reference

[1] R4-2414277 WF on HPUE for CA in TN, Samsung, #112 2024-08

[2] R4-2412349 R19 MPR for PC1.5 contiguous CA, OPPO, #112 2024-08

[3] R4-2412350 R19 MPR for PC1.5 NC CA, OPPO, RAN4#112 2024-08

[4] R4-2417914 R19 MPR for PC1.5 contiguous CA, OPPO, RAN4#113 2024-11

Annex A: Inner contig RB /CP-OFDM MPR simulation

| CBW1 | CBW2 | NRB1 | NRB2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 90 | 273 | 245 | 1 | 1 | 0 | 1 | 30 | inner | 2.0 | 1.9 | 1.8 | 2.0 | 2.6 |
| 100 | 90 | 273 | 245 | 2 | 1 | 0 | 1 | 30 | inner | 1.6 | 1.6 | 1.8 | 2.0 | 2.8 |
| 100 | 90 | 273 | 245 | 1 | 133 | 0 | 1 | 30 | inner | 2.0 | 1.9 | 2.0 | 2.0 | 2.6 |
| 100 | 90 | 273 | 245 | 2 | 133 | 0 | 1 | 30 | inner | 1.4 | 1.2 | 1.4 | 1.9 | 2.8 |
| 100 | 90 | 273 | 245 | 3 | 133 | 0 | 1 | 30 | inner | 1.4 | 1.4 | 1.2 | 1.5 | 2.6 |
| 100 | 90 | 273 | 245 | 4 | 133 | 0 | 1 | 30 | inner | 0.9 | 0.7 | 1.0 | 1.3 | 2.7 |
| 100 | 90 | 273 | 245 | 5 | 133 | 0 | 1 | 30 | inner | 0.6 | 0.7 | 1.0 | 1.6 | 2.6 |
| 100 | 90 | 273 | 245 | 1 | 134 | 0 | 1 | 30 | inner | 2.2 | 1.9 | 2.0 | 2.0 | 2.6 |
| 100 | 90 | 273 | 245 | 2 | 134 | 0 | 1 | 30 | inner | 1.0 | 1.0 | 1.1 | 1.9 | 2.8 |
| 100 | 90 | 273 | 245 | 3 | 134 | 0 | 1 | 30 | inner | 1.4 | 1.4 | 1.2 | 1.5 | 2.6 |
| 100 | 90 | 273 | 245 | 4 | 134 | 0 | 1 | 30 | inner | 0.8 | 0.7 | 1.0 | 1.3 | 2.7 |
| 100 | 90 | 273 | 245 | 5 | 134 | 0 | 1 | 30 | inner | 0.6 | 0.7 | 1.0 | 1.6 | 2.6 |
| 100 | 90 | 273 | 245 | 1 | 135 | 0 | 1 | 30 | inner | 2.0 | 1.9 | 2.0 | 2.0 | 2.6 |
| 100 | 90 | 273 | 245 | 2 | 135 | 0 | 1 | 30 | inner | 1.0 | 1.0 | 1.1 | 1.9 | 2.8 |
| 100 | 90 | 273 | 245 | 3 | 135 | 0 | 1 | 30 | inner | 1.4 | 1.4 | 1.2 | 1.5 | 2.6 |
| 100 | 90 | 273 | 245 | 4 | 135 | 0 | 1 | 30 | inner | 0.9 | 0.9 | 1.0 | 1.3 | 2.7 |
| 100 | 90 | 273 | 245 | 5 | 135 | 0 | 1 | 30 | inner | 0.6 | 0.7 | 1.0 | 1.6 | 2.6 |
| 100 | 90 | 273 | 245 | 1 | 136 | 0 | 1 | 30 | inner | 2.2 | 1.9 | 2.0 | 2.0 | 2.6 |
| 100 | 90 | 273 | 245 | 2 | 136 | 0 | 1 | 30 | inner | 1.2 | 1.2 | 1.2 | 1.9 | 2.8 |
| 100 | 90 | 273 | 245 | 3 | 136 | 0 | 1 | 30 | inner | 1.4 | 1.2 | 1.2 | 1.5 | 2.6 |
| 100 | 90 | 273 | 245 | 4 | 136 | 0 | 1 | 30 | inner | 0.9 | 0.9 | 1.0 | 1.3 | 2.7 |
| 100 | 90 | 273 | 245 | 5 | 136 | 0 | 1 | 30 | inner | 0.6 | 0.7 | 1.0 | 1.6 | 2.6 |
| 100 | 90 | 273 | 245 | 1 | 137 | 0 | 1 | 30 | inner | 2.2 | 1.9 | 2.0 | 2.0 | 2.6 |
| 100 | 90 | 273 | 245 | 2 | 137 | 0 | 1 | 30 | inner | 1.4 | 1.4 | 1.4 | 1.9 | 2.8 |
| 100 | 90 | 273 | 245 | 3 | 137 | 0 | 1 | 30 | inner | 1.2 | 1.2 | 1.0 | 1.5 | 2.6 |
| 100 | 90 | 273 | 245 | 4 | 137 | 0 | 1 | 30 | inner | 1.1 | 0.9 | 1.0 | 1.3 | 2.7 |
| 100 | 90 | 273 | 245 | 5 | 137 | 0 | 1 | 30 | inner | 0.6 | 0.7 | 1.0 | 1.6 | 2.6 |
| 100 | 90 | 273 | 245 | 100 | 134 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 108 | 134 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 2.0 | 2.7 |
| 100 | 90 | 273 | 245 | 120 | 134 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 125 | 134 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 135 | 134 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 2.0 | 2.7 |
| 100 | 90 | 273 | 245 | 100 | 135 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 108 | 135 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 2.0 | 2.7 |
| 100 | 90 | 273 | 245 | 120 | 135 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 125 | 135 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 135 | 135 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 2.0 | 2.7 |
| 100 | 90 | 273 | 245 | 100 | 136 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 108 | 136 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 2.0 | 2.7 |
| 100 | 90 | 273 | 245 | 120 | 136 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 125 | 136 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 135 | 136 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 2.0 | 2.7 |
| 100 | 90 | 273 | 245 | 100 | 137 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 108 | 137 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 2.0 | 2.7 |
| 100 | 90 | 273 | 245 | 120 | 137 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 125 | 137 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 2.0 | 2.7 |
| 100 | 90 | 273 | 245 | 135 | 137 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 2.0 | 2.9 |
| 100 | 90 | 273 | 245 | 1 | 254 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 0.9 | 1.5 | 2.6 |
| 100 | 90 | 273 | 245 | 2 | 254 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 0.9 | 1.9 | 2.8 |
| 100 | 90 | 273 | 245 | 3 | 254 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.5 | 2.6 |
| 100 | 90 | 273 | 245 | 4 | 254 | 0 | 1 | 30 | inner | 0.4 | 0.7 | 1.0 | 1.3 | 2.7 |
| 100 | 90 | 273 | 245 | 1 | 255 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 0.9 | 1.5 | 2.6 |
| 100 | 90 | 273 | 245 | 2 | 255 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 0.9 | 1.9 | 2.8 |
| 100 | 90 | 273 | 245 | 3 | 255 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 1.0 | 1.5 | 2.6 |
| 100 | 90 | 273 | 245 | 1 | 256 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 0.9 | 1.5 | 2.6 |
| 100 | 90 | 273 | 245 | 2 | 256 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 0.9 | 1.9 | 2.8 |
| 100 | 90 | 273 | 245 | 1 | 257 | 0 | 1 | 30 | inner | 0.3 | 0.7 | 0.9 | 1.5 | 2.6 |
| 60 | 100 | 162 | 273 | 10 | 152 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 120 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 130 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 140 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 150 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 160 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 170 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 120 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 130 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 140 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 150 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 160 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 170 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 120 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 130 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 140 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 150 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 160 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 170 | 0 | 30 | inner | 0.2 | 0.0 | 0.2 | 0.2 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 120 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 130 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 140 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 150 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 160 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 120 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 130 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 140 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 150 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 160 | 0 | 30 | inner | 0.2 | 0.0 | 0.2 | 0.2 | 0.2 |
| 60 | 100 | 162 | 273 | 60 | 102 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 120 | 0 | 30 | inner | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 130 | 0 | 30 | inner | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| 60 | 100 | 162 | 273 | 60 | 102 | 140 | 0 | 30 | inner | 0.2 | 0.4 | 0.4 | 0.4 | 0.2 |
| 60 | 100 | 162 | 273 | 70 | 92 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.2 | 0.2 | 0.2 |
| 60 | 100 | 162 | 273 | 70 | 92 | 100 | 0 | 30 | inner | 0.2 | 0.2 | 0.2 | 0.2 | 0.4 |
| 60 | 100 | 162 | 273 | 70 | 92 | 110 | 0 | 30 | inner | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 80 | 82 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 60 | 0 | 30 | inner | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| 60 | 100 | 162 | 273 | 80 | 82 | 70 | 0 | 30 | inner | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 80 | 82 | 80 | 0 | 30 | inner | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 90 | 72 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 90 | 72 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 90 | 72 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 90 | 72 | 40 | 0 | 30 | inner | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 90 | 72 | 50 | 0 | 30 | inner | 0.7 | 0.4 | 0.4 | 0.4 | 0.7 |
| 60 | 100 | 162 | 273 | 100 | 62 | 10 | 0 | 30 | inner | 0.4 | 0.2 | 0.2 | 0.5 | 0.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 20 | 0 | 30 | inner | 0.4 | 0.7 | 0.5 | 0.7 | 0.5 |

Annex B: Outer contig RB /CP-OFDM MPR simulation

| CBW1 | CBW2 | NRB1 | NRB2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 90 | 273 | 245 | 3 | 1 | 0 | 1 | 30 | outer | 1.2 | 1.2 | 1.0 | 1.5 | 2.9 |
| 100 | 90 | 273 | 245 | 4 | 1 | 0 | 1 | 30 | outer | 3.0 | 3.0 | 2.9 | 2.6 | 3.3 |
| 100 | 90 | 273 | 245 | 5 | 1 | 0 | 1 | 30 | outer | 3.2 | 3.2 | 3.2 | 3.2 | 3.8 |
| 100 | 90 | 273 | 245 | 6 | 1 | 0 | 1 | 30 | outer | 3.5 | 3.5 | 3.6 | 3.5 | 3.8 |
| 100 | 90 | 273 | 245 | 80 | 1 | 0 | 1 | 30 | outer | 2.7 | 2.7 | 2.7 | 2.7 | 3.8 |
| 100 | 90 | 273 | 245 | 81 | 1 | 0 | 1 | 30 | outer | 2.7 | 2.7 | 2.7 | 2.7 | 4.2 |
| 100 | 90 | 273 | 245 | 90 | 1 | 0 | 1 | 30 | outer | 2.7 | 2.7 | 2.7 | 2.7 | 3.8 |
| 100 | 90 | 273 | 245 | 96 | 1 | 0 | 1 | 30 | outer | 2.7 | 2.7 | 2.7 | 2.5 | 3.8 |
| 100 | 90 | 273 | 245 | 100 | 1 | 0 | 1 | 30 | outer | 2.7 | 2.7 | 2.4 | 2.7 | 3.8 |
| 100 | 90 | 273 | 245 | 108 | 1 | 0 | 1 | 30 | outer | 2.4 | 2.5 | 2.4 | 2.4 | 3.8 |
| 100 | 90 | 273 | 245 | 120 | 1 | 0 | 1 | 30 | outer | 2.4 | 2.4 | 2.4 | 2.4 | 3.8 |
| 100 | 90 | 273 | 245 | 125 | 1 | 0 | 1 | 30 | outer | 2.4 | 2.4 | 2.4 | 2.5 | 3.5 |
| 100 | 90 | 273 | 245 | 135 | 1 | 0 | 1 | 30 | outer | 2.4 | 2.7 | 2.4 | 2.4 | 3.5 |
| 100 | 90 | 273 | 245 | 144 | 1 | 0 | 1 | 30 | outer | 2.7 | 2.4 | 2.7 | 2.4 | 3.2 |
| 100 | 90 | 273 | 245 | 150 | 1 | 0 | 1 | 30 | outer | 2.4 | 2.4 | 2.7 | 2.7 | 2.9 |
| 100 | 90 | 273 | 245 | 250 | 1 | 0 | 1 | 30 | outer | 2.7 | 2.4 | 2.7 | 2.7 | 3.0 |
| 100 | 90 | 273 | 245 | 270 | 1 | 0 | 1 | 30 | outer | 2.7 | 2.4 | 2.7 | 2.9 | 3.2 |
| 100 | 90 | 273 | 245 | 0 | 1 | 89 | 121 | 30 | outer | 0.7 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 0 | 1 | 96 | 121 | 30 | outer | 1.4 | 1.4 | 1.2 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 0 | 1 | 107 | 121 | 30 | outer | 2.0 | 2.0 | 2.0 | 2.2 | 2.7 |
| 100 | 90 | 273 | 245 | 0 | 1 | 112 | 121 | 30 | outer | 2.0 | 2.0 | 2.0 | 2.0 | 3.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 121 | 121 | 30 | outer | 2.4 | 2.4 | 2.2 | 2.4 | 2.9 |
| 100 | 90 | 273 | 245 | 0 | 1 | 89 | 120 | 30 | outer | 0.7 | 0.7 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 0 | 1 | 96 | 120 | 30 | outer | 1.4 | 1.2 | 1.2 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 0 | 1 | 107 | 120 | 30 | outer | 2.0 | 2.0 | 2.0 | 2.2 | 2.7 |
| 100 | 90 | 273 | 245 | 0 | 1 | 112 | 120 | 30 | outer | 2.0 | 2.0 | 2.0 | 2.0 | 2.7 |
| 100 | 90 | 273 | 245 | 0 | 1 | 121 | 120 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.4 | 2.9 |
| 100 | 90 | 273 | 245 | 0 | 1 | 89 | 122 | 30 | outer | 0.9 | 0.9 | 1.0 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 0 | 1 | 96 | 122 | 30 | outer | 1.4 | 1.4 | 1.4 | 1.6 | 2.7 |
| 100 | 90 | 273 | 245 | 0 | 1 | 107 | 122 | 30 | outer | 2.0 | 2.0 | 2.0 | 2.2 | 2.7 |
| 100 | 90 | 273 | 245 | 0 | 1 | 112 | 122 | 30 | outer | 2.2 | 2.2 | 2.0 | 2.0 | 2.7 |
| 100 | 90 | 273 | 245 | 0 | 1 | 121 | 122 | 30 | outer | 2.4 | 2.4 | 2.4 | 2.4 | 2.9 |
| 100 | 90 | 273 | 245 | 270 | 3 | 0 | 1 | 30 | outer | 2.4 | 2.4 | 2.4 | 2.7 | 3.2 |
| 100 | 90 | 273 | 245 | 270 | 3 | 1 | 1 | 30 | outer | 2.4 | 2.4 | 2.4 | 2.4 | 2.4 |
| 100 | 90 | 273 | 245 | 270 | 3 | 2 | 1 | 30 | outer | 2.4 | 2.4 | 2.4 | 2.4 | 2.4 |
| 100 | 90 | 273 | 245 | 270 | 3 | 3 | 1 | 30 | outer | 2.4 | 2.4 | 2.4 | 2.4 | 2.4 |
| 100 | 90 | 273 | 245 | 270 | 3 | 4 | 1 | 30 | outer | 2.4 | 2.4 | 2.4 | 2.4 | 2.4 |
| 100 | 90 | 273 | 245 | 144 | 129 | 201 | 1 | 30 | outer | 2.0 | 2.0 | 2.0 | 2.0 | 2.0 |
| 100 | 90 | 273 | 245 | 144 | 129 | 215 | 1 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 100 | 90 | 273 | 245 | 144 | 129 | 218 | 1 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 100 | 90 | 273 | 245 | 144 | 129 | 224 | 1 | 30 | outer | 2.4 | 2.5 | 2.4 | 2.4 | 2.4 |
| 100 | 90 | 273 | 245 | 144 | 129 | 242 | 1 | 30 | outer | 2.4 | 2.5 | 2.4 | 2.7 | 2.7 |
| 100 | 90 | 273 | 245 | 135 | 138 | 145 | 1 | 30 | outer | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| 100 | 90 | 273 | 245 | 135 | 138 | 161 | 1 | 30 | outer | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| 100 | 90 | 273 | 245 | 135 | 138 | 179 | 1 | 30 | outer | 1.4 | 1.4 | 1.6 | 1.4 | 1.4 |
| 100 | 90 | 273 | 245 | 135 | 138 | 193 | 1 | 30 | outer | 2.0 | 2.0 | 2.0 | 2.0 | 2.0 |
| 100 | 90 | 273 | 245 | 135 | 138 | 201 | 1 | 30 | outer | 2.0 | 2.0 | 2.0 | 2.0 | 2.0 |
| 100 | 90 | 273 | 245 | 135 | 138 | 215 | 1 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 100 | 90 | 273 | 245 | 135 | 138 | 218 | 1 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 100 | 90 | 273 | 245 | 135 | 138 | 224 | 1 | 30 | outer | 2.4 | 2.4 | 2.4 | 2.4 | 2.4 |
| 100 | 90 | 273 | 245 | 135 | 138 | 242 | 1 | 30 | outer | 2.4 | 2.5 | 2.4 | 2.4 | 2.4 |
| 100 | 90 | 273 | 245 | 15 | 258 | 179 | 1 | 30 | outer | 1.0 | 0.9 | 0.9 | 0.9 | 0.9 |
| 100 | 90 | 273 | 245 | 15 | 258 | 193 | 1 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 100 | 90 | 273 | 245 | 15 | 258 | 201 | 1 | 30 | outer | 1.6 | 1.6 | 1.6 | 1.6 | 1.6 |
| 100 | 90 | 273 | 245 | 15 | 258 | 215 | 1 | 30 | outer | 2.0 | 2.0 | 2.0 | 2.0 | 2.0 |
| 100 | 90 | 273 | 245 | 15 | 258 | 218 | 1 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 100 | 90 | 273 | 245 | 15 | 258 | 224 | 1 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 100 | 90 | 273 | 245 | 15 | 258 | 242 | 1 | 30 | outer | 2.4 | 2.4 | 2.4 | 2.5 | 2.5 |
| 100 | 90 | 273 | 245 | 273 | 1 | 245 | 1 | 30 | outer | 2.4 | 2.5 | 2.4 | 2.4 | 2.4 |
| 100 | 90 | 273 | 245 | 0 | 1 | 1 | 244 | 30 | outer | 2.2 | 1.9 | 2.0 | 2.0 | 2.6 |
| 100 | 90 | 273 | 245 | 270 | 3 | 242 | 1 | 30 | outer | 2.4 | 2.4 | 2.4 | 2.7 | 2.4 |
| 60 | 100 | 162 | 273 | 10 | 152 | 180 | 0 | 30 | outer | 0.4 | 0.2 | 0.2 | 0.2 | 0.2 |
| 60 | 100 | 162 | 273 | 10 | 152 | 190 | 0 | 30 | outer | 0.6 | 0.7 | 0.6 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 10 | 152 | 200 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.6 | 0.9 |
| 60 | 100 | 162 | 273 | 10 | 152 | 210 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 10 | 152 | 220 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 10 | 152 | 230 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 10 | 152 | 240 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 10 | 152 | 250 | 0 | 30 | outer | 2.1 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 10 | 152 | 260 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 10 | 152 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.6 | 2.5 |
| 60 | 100 | 162 | 273 | 10 | 152 | 273 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 20 | 142 | 180 | 0 | 30 | outer | 0.4 | 0.2 | 0.4 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 20 | 142 | 190 | 0 | 30 | outer | 0.6 | 0.7 | 0.6 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 20 | 142 | 200 | 0 | 30 | outer | 0.6 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 20 | 142 | 210 | 0 | 30 | outer | 1.2 | 1.2 | 0.9 | 0.9 | 1.2 |
| 60 | 100 | 162 | 273 | 20 | 142 | 220 | 0 | 30 | outer | 1.2 | 1.2 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 20 | 142 | 230 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 20 | 142 | 240 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 20 | 142 | 250 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 20 | 142 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 20 | 142 | 270 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 20 | 142 | 273 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 30 | 132 | 180 | 0 | 30 | outer | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 30 | 132 | 190 | 0 | 30 | outer | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 30 | 132 | 200 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 30 | 132 | 210 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 30 | 132 | 220 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 30 | 132 | 230 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 30 | 132 | 240 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 30 | 132 | 250 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 30 | 132 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 30 | 132 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.6 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 30 | 132 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.6 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 40 | 122 | 170 | 0 | 30 | outer | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| 60 | 100 | 162 | 273 | 40 | 122 | 180 | 0 | 30 | outer | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 40 | 122 | 190 | 0 | 30 | outer | 0.7 | 0.7 | 0.6 | 0.6 | 0.6 |
| 60 | 100 | 162 | 273 | 40 | 122 | 200 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 40 | 122 | 210 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 40 | 122 | 220 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 40 | 122 | 230 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 2.2 | 1.5 |
| 60 | 100 | 162 | 273 | 40 | 122 | 240 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 40 | 122 | 250 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 40 | 122 | 260 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 40 | 122 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 40 | 122 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 50 | 112 | 170 | 0 | 30 | outer | 0.2 | 0.4 | 0.2 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 50 | 112 | 180 | 0 | 30 | outer | 0.4 | 0.4 | 0.4 | 0.6 | 0.7 |
| 60 | 100 | 162 | 273 | 50 | 112 | 190 | 0 | 30 | outer | 0.6 | 0.7 | 0.7 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 50 | 112 | 200 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 50 | 112 | 210 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 50 | 112 | 220 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 50 | 112 | 230 | 0 | 30 | outer | 2.1 | 1.5 | 2.2 | 2.2 | 1.5 |
| 60 | 100 | 162 | 273 | 50 | 112 | 240 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 50 | 112 | 250 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.1 |
| 60 | 100 | 162 | 273 | 50 | 112 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.6 | 2.5 |
| 60 | 100 | 162 | 273 | 50 | 112 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.6 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 50 | 112 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 60 | 102 | 150 | 0 | 30 | outer | 0.4 | 0.4 | 0.2 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 60 | 102 | 160 | 0 | 30 | outer | 0.4 | 0.4 | 0.2 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 60 | 102 | 170 | 0 | 30 | outer | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 60 | 102 | 180 | 0 | 30 | outer | 0.7 | 0.4 | 0.6 | 0.4 | 0.7 |
| 60 | 100 | 162 | 273 | 60 | 102 | 190 | 0 | 30 | outer | 0.7 | 0.7 | 0.7 | 0.6 | 0.7 |
| 60 | 100 | 162 | 273 | 60 | 102 | 200 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 1.2 | 0.9 |
| 60 | 100 | 162 | 273 | 60 | 102 | 210 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 60 | 102 | 220 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 60 | 102 | 230 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.1 | 2.1 |
| 60 | 100 | 162 | 273 | 60 | 102 | 240 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 60 | 102 | 250 | 0 | 30 | outer | 2.1 | 2.2 | 2.5 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 60 | 102 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 60 | 102 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 60 | 102 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 70 | 92 | 120 | 0 | 30 | outer | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 70 | 92 | 130 | 0 | 30 | outer | 0.4 | 0.4 | 0.6 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 70 | 92 | 140 | 0 | 30 | outer | 0.7 | 0.4 | 0.6 | 0.4 | 0.4 |
| 60 | 100 | 162 | 273 | 70 | 92 | 150 | 0 | 30 | outer | 0.4 | 0.7 | 0.6 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 70 | 92 | 160 | 0 | 30 | outer | 0.6 | 0.7 | 0.6 | 0.6 | 0.7 |
| 60 | 100 | 162 | 273 | 70 | 92 | 170 | 0 | 30 | outer | 0.6 | 0.7 | 0.6 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 70 | 92 | 180 | 0 | 30 | outer | 0.6 | 0.7 | 0.7 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 70 | 92 | 190 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 70 | 92 | 200 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 70 | 92 | 210 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 70 | 92 | 220 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 70 | 92 | 230 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 70 | 92 | 240 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 70 | 92 | 250 | 0 | 30 | outer | 2.5 | 2.2 | 2.1 | 2.1 | 2.1 |
| 60 | 100 | 162 | 273 | 70 | 92 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 70 | 92 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 70 | 92 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 80 | 82 | 90 | 0 | 30 | outer | 0.7 | 0.7 | 0.6 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 80 | 82 | 100 | 0 | 30 | outer | 0.7 | 0.7 | 0.6 | 0.6 | 0.7 |
| 60 | 100 | 162 | 273 | 80 | 82 | 110 | 0 | 30 | outer | 0.6 | 0.7 | 0.6 | 0.6 | 0.7 |
| 60 | 100 | 162 | 273 | 80 | 82 | 120 | 0 | 30 | outer | 0.7 | 0.7 | 0.6 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 80 | 82 | 130 | 0 | 30 | outer | 0.6 | 0.9 | 0.6 | 0.6 | 0.7 |
| 60 | 100 | 162 | 273 | 80 | 82 | 140 | 0 | 30 | outer | 0.6 | 0.7 | 0.6 | 0.7 | 0.6 |
| 60 | 100 | 162 | 273 | 80 | 82 | 150 | 0 | 30 | outer | 0.9 | 0.7 | 0.9 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 80 | 82 | 160 | 0 | 30 | outer | 0.9 | 0.7 | 0.9 | 0.7 | 0.9 |
| 60 | 100 | 162 | 273 | 80 | 82 | 170 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 80 | 82 | 180 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 80 | 82 | 190 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 80 | 82 | 200 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 80 | 82 | 210 | 0 | 30 | outer | 1.2 | 1.5 | 1.5 | 1.5 | 1.2 |
| 60 | 100 | 162 | 273 | 80 | 82 | 220 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 80 | 82 | 230 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 80 | 82 | 240 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 2.1 |
| 60 | 100 | 162 | 273 | 80 | 82 | 250 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 80 | 82 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.6 | 2.5 |
| 60 | 100 | 162 | 273 | 80 | 82 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 80 | 82 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 90 | 72 | 60 | 0 | 30 | outer | 0.7 | 0.7 | 0.6 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 90 | 72 | 70 | 0 | 30 | outer | 0.7 | 0.9 | 0.6 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 90 | 72 | 80 | 0 | 30 | outer | 0.9 | 0.7 | 0.9 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 90 | 72 | 90 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 90 | 72 | 100 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 90 | 72 | 110 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 90 | 72 | 120 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 90 | 72 | 130 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 90 | 72 | 140 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 90 | 72 | 150 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 90 | 72 | 160 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 90 | 72 | 170 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 90 | 72 | 180 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 90 | 72 | 190 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 90 | 72 | 200 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 90 | 72 | 210 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 90 | 72 | 220 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 90 | 72 | 230 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.2 | 2.1 |
| 60 | 100 | 162 | 273 | 90 | 72 | 240 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.1 |
| 60 | 100 | 162 | 273 | 90 | 72 | 250 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 90 | 72 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 90 | 72 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 90 | 72 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 100 | 62 | 30 | 0 | 30 | outer | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 100 | 62 | 40 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 100 | 62 | 50 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 100 | 62 | 60 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 100 | 62 | 70 | 0 | 30 | outer | 0.9 | 0.9 | 1.2 | 1.2 | 0.9 |
| 60 | 100 | 162 | 273 | 100 | 62 | 80 | 0 | 30 | outer | 0.9 | 1.2 | 1.2 | 1.2 | 0.9 |
| 60 | 100 | 162 | 273 | 100 | 62 | 90 | 0 | 30 | outer | 0.9 | 0.9 | 1.2 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 100 | 62 | 100 | 0 | 30 | outer | 1.2 | 0.9 | 1.2 | 1.2 | 0.9 |
| 60 | 100 | 162 | 273 | 100 | 62 | 110 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 100 | 62 | 120 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 100 | 62 | 130 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 100 | 62 | 140 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 100 | 62 | 150 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 100 | 62 | 160 | 0 | 30 | outer | 1.2 | 1.2 | 1.5 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 100 | 62 | 170 | 0 | 30 | outer | 1.5 | 1.2 | 1.2 | 1.5 | 1.2 |
| 60 | 100 | 162 | 273 | 100 | 62 | 180 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 100 | 62 | 190 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 100 | 62 | 200 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 100 | 62 | 210 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 100 | 62 | 220 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 100 | 62 | 230 | 0 | 30 | outer | 2.1 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 100 | 62 | 240 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 100 | 62 | 250 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 100 | 62 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.2 |
| 60 | 100 | 162 | 273 | 100 | 62 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 100 | 62 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 10 | 0 | 30 | outer | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 110 | 52 | 20 | 0 | 30 | outer | 0.9 | 1.2 | 1.2 | 0.9 | 1.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 30 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 40 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 50 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 60 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 70 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 80 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 90 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 100 | 0 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 110 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 120 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 130 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 140 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 150 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 160 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 170 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 180 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 190 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 200 | 0 | 30 | outer | 2.2 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 210 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 220 | 0 | 30 | outer | 1.5 | 1.5 | 2.1 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 230 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 240 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 2.1 |
| 60 | 100 | 162 | 273 | 110 | 52 | 250 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 110 | 52 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 10 | 0 | 30 | outer | 1.5 | 1.5 | 1.2 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 20 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 30 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.2 | 1.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 40 | 0 | 30 | outer | 1.5 | 1.2 | 1.4 | 1.2 | 1.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 50 | 0 | 30 | outer | 1.5 | 1.5 | 1.2 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 60 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 70 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 80 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 90 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 100 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 110 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 120 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 130 | 0 | 30 | outer | 1.5 | 2.2 | 1.5 | 2.2 | 1.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 140 | 0 | 30 | outer | 1.5 | 2.2 | 1.5 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 150 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 160 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 170 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 180 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 190 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 200 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 210 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 220 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 230 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 240 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 120 | 42 | 250 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.2 | 2.1 |
| 60 | 100 | 162 | 273 | 120 | 42 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 120 | 42 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 120 | 42 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 130 | 32 | 10 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 130 | 32 | 20 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 130 | 32 | 30 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 130 | 32 | 40 | 0 | 30 | outer | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| 60 | 100 | 162 | 273 | 130 | 32 | 50 | 0 | 30 | outer | 2.2 | 2.2 | 1.5 | 2.1 | 1.5 |
| 60 | 100 | 162 | 273 | 130 | 32 | 60 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 70 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 1.5 |
| 60 | 100 | 162 | 273 | 130 | 32 | 80 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 90 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 100 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 110 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 120 | 0 | 30 | outer | 2.1 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 130 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 140 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 150 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 160 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 170 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 180 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 190 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 200 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 210 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 220 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 230 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 240 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 250 | 0 | 30 | outer | 2.5 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 130 | 32 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.1 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 130 | 32 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 130 | 32 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 140 | 22 | 10 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 20 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 30 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 40 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 50 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 60 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 70 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 80 | 0 | 30 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 90 | 0 | 30 | outer | 2.5 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 100 | 0 | 30 | outer | 2.1 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 110 | 0 | 30 | outer | 2.2 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 120 | 0 | 30 | outer | 2.1 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 130 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 140 | 0 | 30 | outer | 2.1 | 2.6 | 2.1 | 2.5 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 150 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 140 | 22 | 160 | 0 | 30 | outer | 2.2 | 2.6 | 2.5 | 2.5 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 170 | 0 | 30 | outer | 2.5 | 2.2 | 2.6 | 2.5 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 180 | 0 | 30 | outer | 2.2 | 2.5 | 2.5 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 190 | 0 | 30 | outer | 2.1 | 2.2 | 2.1 | 2.2 | 2.6 |
| 60 | 100 | 162 | 273 | 140 | 22 | 200 | 0 | 30 | outer | 2.5 | 2.5 | 2.2 | 2.5 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 210 | 0 | 30 | outer | 2.1 | 2.5 | 2.2 | 2.5 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 220 | 0 | 30 | outer | 2.2 | 2.5 | 2.1 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 140 | 22 | 230 | 0 | 30 | outer | 2.1 | 2.2 | 2.2 | 2.1 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 240 | 0 | 30 | outer | 2.5 | 2.2 | 2.1 | 2.2 | 2.1 |
| 60 | 100 | 162 | 273 | 140 | 22 | 250 | 0 | 30 | outer | 2.1 | 2.2 | 2.2 | 2.2 | 2.2 |
| 60 | 100 | 162 | 273 | 140 | 22 | 260 | 0 | 30 | outer | 2.1 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 140 | 22 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 140 | 22 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 150 | 12 | 10 | 0 | 30 | outer | 2.6 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 20 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 30 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 40 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 50 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 60 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 70 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 80 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 90 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 100 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 110 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 120 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 130 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 140 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 150 | 12 | 150 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 160 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 170 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 180 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 190 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 200 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 210 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 220 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 150 | 12 | 230 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 240 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 250 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 150 | 12 | 260 | 0 | 30 | outer | 2.1 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 150 | 12 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 150 | 12 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 160 | 2 | 10 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 20 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 30 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 40 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 50 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 60 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 70 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 80 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 90 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 100 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 110 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 120 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 160 | 2 | 130 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 140 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 150 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 160 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 170 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 180 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 190 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 200 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 160 | 2 | 210 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 160 | 2 | 220 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 160 | 2 | 230 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 160 | 2 | 240 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 160 | 2 | 250 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 160 | 2 | 260 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 160 | 2 | 270 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 |
| 60 | 100 | 162 | 273 | 160 | 2 | 273 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 10 | 0 | 30 | outer | 2.6 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 20 | 0 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 30 | 0 | 30 | outer | 3.0 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 40 | 0 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 50 | 0 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 60 | 0 | 30 | outer | 2.6 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 70 | 0 | 30 | outer | 2.6 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 80 | 0 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 90 | 0 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 100 | 0 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 110 | 0 | 30 | outer | 2.6 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 120 | 0 | 30 | outer | 2.6 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 130 | 0 | 30 | outer | 2.6 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 140 | 0 | 30 | outer | 2.6 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 150 | 0 | 30 | outer | 2.6 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 160 | 0 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 170 | 0 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 180 | 0 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 190 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 200 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 210 | 0 | 30 | outer | 2.6 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 220 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 230 | 0 | 30 | outer | 2.6 | 2.6 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 240 | 0 | 30 | outer | 2.5 | 2.6 | 2.6 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 250 | 0 | 30 | outer | 2.5 | 2.5 | 2.5 | 2.6 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 260 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 270 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.5 | 2.6 |
| 60 | 100 | 162 | 273 | 162 | 0 | 273 | 0 | 30 | outer | 2.5 | 2.6 | 2.5 | 2.6 | 2.6 |

Annex C: Inner contig RB /DFT-S-OFDM MPR simulation

| CBW1 | CBW2 | NRB1 | NRB2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 90 | 273 | 245 | 1 | 133 | 0 | 1 | 30 | inner | 0.0 | 0.2 | 0.8 | 0.8 | 2.1 |
| 100 | 90 | 273 | 245 | 2 | 133 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.1 |
| 100 | 90 | 273 | 245 | 3 | 133 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 4 | 133 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 5 | 133 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 1 | 134 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.8 | 1.0 | 2.1 |
| 100 | 90 | 273 | 245 | 2 | 134 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.1 |
| 100 | 90 | 273 | 245 | 3 | 134 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 4 | 134 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 5 | 134 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 1 | 135 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.8 | 1.0 | 2.1 |
| 100 | 90 | 273 | 245 | 2 | 135 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.1 |
| 100 | 90 | 273 | 245 | 3 | 135 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 4 | 135 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 5 | 135 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 1 | 136 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.8 | 1.0 | 2.1 |
| 100 | 90 | 273 | 245 | 2 | 136 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.1 |
| 100 | 90 | 273 | 245 | 3 | 136 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 4 | 136 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 5 | 136 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 1 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.8 | 1.0 | 2.1 |
| 100 | 90 | 273 | 245 | 2 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.1 |
| 100 | 90 | 273 | 245 | 3 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 4 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 5 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 100 | 134 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 108 | 134 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 120 | 134 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 125 | 134 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 135 | 134 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 100 | 135 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 108 | 135 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 120 | 135 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 125 | 135 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 135 | 135 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 100 | 136 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 108 | 136 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 120 | 136 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 125 | 136 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 135 | 136 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 100 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 108 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 120 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 125 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 135 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.3 |
| 100 | 90 | 273 | 245 | 1 | 254 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.6 | 2.1 |
| 100 | 90 | 273 | 245 | 2 | 254 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.1 |
| 100 | 90 | 273 | 245 | 3 | 254 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 4 | 254 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 1 | 255 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.6 | 2.1 |
| 100 | 90 | 273 | 245 | 2 | 255 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.1 |
| 100 | 90 | 273 | 245 | 3 | 255 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 1 | 256 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.6 | 2.1 |
| 100 | 90 | 273 | 245 | 2 | 256 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.8 | 2.1 |
| 100 | 90 | 273 | 245 | 1 | 257 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.4 | 0.6 | 2.1 |
| 60 | 100 | 162 | 273 | 10 | 152 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 120 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 130 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 140 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 150 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 160 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 10 | 152 | 170 | 0 | 30 | inner | 0.0 | 0.0 | 0.2 | 0.3 | 0.3 |
| 60 | 100 | 162 | 273 | 20 | 142 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 120 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 130 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 140 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 150 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 160 | 0 | 30 | inner | 0.0 | 0.0 | 0.3 | 0.3 | 0.3 |
| 60 | 100 | 162 | 273 | 20 | 142 | 170 | 0 | 30 | inner | 0.1 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 30 | 132 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 120 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 130 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 140 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 150 | 0 | 30 | inner | 0.0 | 0.0 | 0.3 | 0.3 | 0.3 |
| 60 | 100 | 162 | 273 | 30 | 132 | 160 | 0 | 30 | inner | 0.0 | 0.2 | 0.3 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 30 | 132 | 170 | 0 | 30 | inner | 0.1 | 0.4 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 40 | 122 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 120 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 130 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 140 | 0 | 30 | inner | 0.0 | 0.0 | 0.1 | 0.3 | 0.3 |
| 60 | 100 | 162 | 273 | 40 | 122 | 150 | 0 | 30 | inner | 0.0 | 0.2 | 0.3 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 40 | 122 | 160 | 0 | 30 | inner | 0.1 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 50 | 112 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 120 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 130 | 0 | 30 | inner | 0.0 | 0.0 | 0.3 | 0.3 | 0.3 |
| 60 | 100 | 162 | 273 | 50 | 112 | 140 | 0 | 30 | inner | 0.0 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 50 | 112 | 150 | 0 | 30 | inner | 0.0 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 50 | 112 | 160 | 0 | 30 | inner | 0.1 | 0.4 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 60 | 102 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 110 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| 60 | 100 | 162 | 273 | 60 | 102 | 120 | 0 | 30 | inner | 0.0 | 0.0 | 0.3 | 0.3 | 0.3 |
| 60 | 100 | 162 | 273 | 60 | 102 | 130 | 0 | 30 | inner | 0.0 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 60 | 102 | 140 | 0 | 30 | inner | 0.0 | 0.1 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 70 | 92 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 60 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 70 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 80 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 90 | 0 | 30 | inner | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 |
| 60 | 100 | 162 | 273 | 70 | 92 | 100 | 0 | 30 | inner | 0.0 | 0.0 | 0.3 | 0.3 | 0.3 |
| 60 | 100 | 162 | 273 | 70 | 92 | 110 | 0 | 30 | inner | 0.0 | 0.2 | 0.3 | 0.3 | 0.5 |
| 60 | 100 | 162 | 273 | 80 | 82 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 30 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 40 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 50 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 60 | 0 | 30 | inner | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 70 | 0 | 30 | inner | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 |
| 60 | 100 | 162 | 273 | 80 | 82 | 80 | 0 | 30 | inner | 0.1 | 0.0 | 0.3 | 0.3 | 0.3 |
| 60 | 100 | 162 | 273 | 90 | 72 | 10 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 90 | 72 | 20 | 0 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 60 | 100 | 162 | 273 | 90 | 72 | 30 | 0 | 30 | inner | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| 60 | 100 | 162 | 273 | 90 | 72 | 40 | 0 | 30 | inner | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 |
| 60 | 100 | 162 | 273 | 90 | 72 | 50 | 0 | 30 | inner | 0.1 | 0.0 | 0.3 | 0.3 | 0.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 10 | 0 | 30 | inner | 0.0 | 0.2 | 0.3 | 0.4 | 0.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 20 | 0 | 30 | inner | 0.2 | 0.3 | 0.4 | 0.4 | 0.4 |

Annex D: Outer contig RB /DFT-S-OFDM MPR simulation

| CBW1 | CBW2 | NRB1 | NRB2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 90 | 273 | 245 | 3 | 1 | 0 | 1 | 30 | outer | 15.8 | 0.0 | 0.5 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 4 | 1 | 0 | 1 | 30 | outer | 15.8 | 1.2 | 1.8 | 1.8 | 2.2 |
| 100 | 90 | 273 | 245 | 5 | 1 | 0 | 1 | 30 | outer | 0.6 | 1.7 | 2.2 | 2.2 | 2.5 |
| 100 | 90 | 273 | 245 | 6 | 1 | 0 | 1 | 30 | outer | 0.4 | 1.7 | 2.3 | 2.5 | 2.8 |
| 100 | 90 | 273 | 245 | 80 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.6 | 1.1 | 1.1 | 2.3 |
| 100 | 90 | 273 | 245 | 81 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.6 | 1.1 | 1.1 | 2.0 |
| 100 | 90 | 273 | 245 | 90 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.6 | 1.1 | 1.1 | 2.0 |
| 100 | 90 | 273 | 245 | 96 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.6 | 1.1 | 1.1 | 2.0 |
| 100 | 90 | 273 | 245 | 100 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.4 | 0.9 | 1.1 | 2.0 |
| 100 | 90 | 273 | 245 | 108 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.4 | 0.9 | 1.1 | 2.0 |
| 100 | 90 | 273 | 245 | 120 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.4 | 0.9 | 0.9 | 2.0 |
| 100 | 90 | 273 | 245 | 125 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.4 | 0.9 | 0.9 | 2.0 |
| 100 | 90 | 273 | 245 | 135 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.4 | 0.7 | 0.9 | 2.0 |
| 100 | 90 | 273 | 245 | 144 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.4 | 0.7 | 0.9 | 2.0 |
| 100 | 90 | 273 | 245 | 150 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.2 | 0.7 | 0.9 | 2.0 |
| 100 | 90 | 273 | 245 | 250 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.2 | 0.9 | 0.9 | 2.0 |
| 100 | 90 | 273 | 245 | 270 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.2 | 0.9 | 0.9 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 89 | 121 | 30 | outer | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 96 | 121 | 30 | outer | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 107 | 121 | 30 | outer | 0.0 | 0.0 | 0.5 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 112 | 121 | 30 | outer | 0.0 | 0.2 | 0.7 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 121 | 121 | 30 | outer | 0.0 | 0.2 | 0.7 | 0.9 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 89 | 120 | 30 | outer | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 96 | 120 | 30 | outer | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 107 | 120 | 30 | outer | 0.0 | 0.0 | 0.5 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 112 | 120 | 30 | outer | 0.0 | 0.2 | 0.7 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 121 | 120 | 30 | outer | 0.0 | 0.2 | 0.7 | 0.9 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 89 | 122 | 30 | outer | 0.0 | 0.0 | 0.4 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 96 | 122 | 30 | outer | 0.0 | 0.0 | 0.4 | 0.8 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 107 | 122 | 30 | outer | 0.0 | 0.2 | 0.7 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 112 | 122 | 30 | outer | 0.0 | 0.2 | 0.7 | 0.7 | 2.0 |
| 100 | 90 | 273 | 245 | 0 | 1 | 121 | 122 | 30 | outer | 0.0 | 0.2 | 0.7 | 0.9 | 2.0 |
| 100 | 90 | 273 | 245 | 270 | 3 | 0 | 1 | 30 | outer | 0.0 | 0.2 | 0.7 | 0.9 | 2.0 |
| 100 | 90 | 273 | 245 | 270 | 3 | 1 | 1 | 30 | outer | 1.8 | 0.5 | 1.0 | 0.9 | 0.9 |
| 100 | 90 | 273 | 245 | 270 | 3 | 2 | 1 | 30 | outer | 1.3 | 0.5 | 0.9 | 0.9 | 1.0 |
| 100 | 90 | 273 | 245 | 270 | 3 | 3 | 1 | 30 | outer | 1.1 | 0.5 | 0.9 | 0.9 | 1.0 |
| 100 | 90 | 273 | 245 | 270 | 3 | 4 | 1 | 30 | outer | 1.1 | 0.5 | 0.9 | 0.9 | 0.9 |
| 100 | 90 | 273 | 245 | 144 | 129 | 201 | 1 | 30 | outer | 0.3 | 0.6 | 1.1 | 1.1 | 1.1 |
| 100 | 90 | 273 | 245 | 144 | 129 | 215 | 1 | 30 | outer | 0.3 | 0.8 | 1.3 | 1.3 | 1.3 |
| 100 | 90 | 273 | 245 | 144 | 129 | 218 | 1 | 30 | outer | 0.3 | 0.8 | 1.3 | 1.3 | 1.3 |
| 100 | 90 | 273 | 245 | 144 | 129 | 224 | 1 | 30 | outer | 0.3 | 0.8 | 1.3 | 1.3 | 1.3 |
| 100 | 90 | 273 | 245 | 144 | 129 | 242 | 1 | 30 | outer | 0.4 | 1.0 | 1.3 | 1.5 | 1.5 |
| 100 | 90 | 273 | 245 | 135 | 138 | 145 | 1 | 30 | outer | 0.0 | 0.1 | 0.3 | 0.3 | 0.3 |
| 100 | 90 | 273 | 245 | 135 | 138 | 161 | 1 | 30 | outer | 0.0 | 0.2 | 0.6 | 0.6 | 0.6 |
| 100 | 90 | 273 | 245 | 135 | 138 | 179 | 1 | 30 | outer | 0.1 | 0.5 | 0.7 | 0.9 | 0.9 |
| 100 | 90 | 273 | 245 | 135 | 138 | 193 | 1 | 30 | outer | 0.1 | 0.6 | 0.9 | 1.1 | 1.1 |
| 100 | 90 | 273 | 245 | 135 | 138 | 201 | 1 | 30 | outer | 0.2 | 0.6 | 1.1 | 1.1 | 1.1 |
| 100 | 90 | 273 | 245 | 135 | 138 | 215 | 1 | 30 | outer | 0.3 | 0.8 | 1.1 | 1.3 | 1.3 |
| 100 | 90 | 273 | 245 | 135 | 138 | 218 | 1 | 30 | outer | 0.3 | 0.8 | 1.3 | 1.3 | 1.3 |
| 100 | 90 | 273 | 245 | 135 | 138 | 224 | 1 | 30 | outer | 0.3 | 0.8 | 1.3 | 1.3 | 1.3 |
| 100 | 90 | 273 | 245 | 135 | 138 | 242 | 1 | 30 | outer | 0.4 | 0.8 | 1.3 | 1.5 | 1.5 |
| 100 | 90 | 273 | 245 | 15 | 258 | 179 | 1 | 30 | outer | 0.6 | 0.0 | 0.1 | 0.2 | 0.2 |
| 100 | 90 | 273 | 245 | 15 | 258 | 193 | 1 | 30 | outer | 0.7 | 0.1 | 0.3 | 0.3 | 0.4 |
| 100 | 90 | 273 | 245 | 15 | 258 | 201 | 1 | 30 | outer | 0.6 | 0.2 | 0.4 | 0.4 | 0.5 |
| 100 | 90 | 273 | 245 | 15 | 258 | 215 | 1 | 30 | outer | 0.7 | 0.3 | 0.7 | 0.7 | 0.8 |
| 100 | 90 | 273 | 245 | 15 | 258 | 218 | 1 | 30 | outer | 0.6 | 0.3 | 0.7 | 0.7 | 0.8 |
| 100 | 90 | 273 | 245 | 15 | 258 | 224 | 1 | 30 | outer | 0.7 | 0.3 | 0.7 | 0.7 | 0.8 |
| 100 | 90 | 273 | 245 | 15 | 258 | 242 | 1 | 30 | outer | 0.6 | 0.5 | 0.9 | 0.9 | 0.9 |
| 100 | 90 | 273 | 245 | 273 | 1 | 245 | 1 | 30 | outer | 0.6 | 1.2 | 1.9 | 1.9 | 1.9 |
| 100 | 90 | 273 | 245 | 0 | 1 | 1 | 244 | 30 | outer | 0.0 | 0.4 | 0.8 | 1.0 | 2.1 |
| 100 | 90 | 273 | 245 | 270 | 3 | 242 | 1 | 30 | outer | 0.6 | 1.2 | 1.9 | 1.9 | 1.9 |
| 60 | 100 | 162 | 273 | 10 | 152 | 180 | 0 | 30 | outer | 0.3 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 10 | 152 | 190 | 0 | 30 | outer | 0.9 | 0.4 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 10 | 152 | 200 | 0 | 30 | outer | 0.9 | 0.4 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 10 | 152 | 210 | 0 | 30 | outer | 0.9 | 0.4 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 10 | 152 | 220 | 0 | 30 | outer | 0.9 | 0.4 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 10 | 152 | 230 | 0 | 30 | outer | 0.6 | 0.2 | 0.5 | 0.7 | 0.5 |
| 60 | 100 | 162 | 273 | 10 | 152 | 240 | 0 | 30 | outer | 0.9 | 0.4 | 0.7 | 0.7 | 0.8 |
| 60 | 100 | 162 | 273 | 10 | 152 | 250 | 0 | 30 | outer | 0.9 | 0.4 | 0.7 | 0.7 | 0.8 |
| 60 | 100 | 162 | 273 | 10 | 152 | 260 | 0 | 30 | outer | 0.6 | 0.4 | 0.7 | 0.7 | 0.8 |
| 60 | 100 | 162 | 273 | 10 | 152 | 270 | 0 | 30 | outer | 0.9 | 0.4 | 0.7 | 0.7 | 0.8 |
| 60 | 100 | 162 | 273 | 10 | 152 | 273 | 0 | 30 | outer | 0.9 | 0.4 | 0.7 | 0.7 | 0.8 |
| 60 | 100 | 162 | 273 | 20 | 142 | 180 | 0 | 30 | outer | 0.1 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 20 | 142 | 190 | 0 | 30 | outer | 0.3 | 0.4 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 20 | 142 | 200 | 0 | 30 | outer | 0.6 | 0.4 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 20 | 142 | 210 | 0 | 30 | outer | 0.6 | 0.4 | 0.5 | 0.5 | 0.8 |
| 60 | 100 | 162 | 273 | 20 | 142 | 220 | 0 | 30 | outer | 0.6 | 0.4 | 0.5 | 0.8 | 0.5 |
| 60 | 100 | 162 | 273 | 20 | 142 | 230 | 0 | 30 | outer | 0.6 | 0.4 | 0.5 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 20 | 142 | 240 | 0 | 30 | outer | 0.6 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 20 | 142 | 250 | 0 | 30 | outer | 0.6 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 20 | 142 | 260 | 0 | 30 | outer | 0.3 | 0.4 | 0.8 | 0.8 | 1.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 270 | 0 | 30 | outer | 0.3 | 0.7 | 0.8 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 20 | 142 | 273 | 0 | 30 | outer | 0.3 | 0.6 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 180 | 0 | 30 | outer | 0.3 | 0.4 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 30 | 132 | 190 | 0 | 30 | outer | 0.3 | 0.4 | 0.5 | 0.8 | 0.5 |
| 60 | 100 | 162 | 273 | 30 | 132 | 200 | 0 | 30 | outer | 0.3 | 0.4 | 0.5 | 0.7 | 0.8 |
| 60 | 100 | 162 | 273 | 30 | 132 | 210 | 0 | 30 | outer | 0.3 | 0.4 | 0.5 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 30 | 132 | 220 | 0 | 30 | outer | 0.3 | 0.4 | 0.5 | 0.7 | 0.8 |
| 60 | 100 | 162 | 273 | 30 | 132 | 230 | 0 | 30 | outer | 0.3 | 0.4 | 0.8 | 0.7 | 0.8 |
| 60 | 100 | 162 | 273 | 30 | 132 | 240 | 0 | 30 | outer | 0.3 | 0.4 | 0.8 | 0.7 | 0.8 |
| 60 | 100 | 162 | 273 | 30 | 132 | 250 | 0 | 30 | outer | 0.3 | 0.4 | 0.8 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 260 | 0 | 30 | outer | 0.3 | 0.6 | 1.0 | 1.0 | 1.1 |
| 60 | 100 | 162 | 273 | 30 | 132 | 270 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 30 | 132 | 273 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 170 | 0 | 30 | outer | 0.3 | 0.4 | 0.5 | 0.8 | 0.5 |
| 60 | 100 | 162 | 273 | 40 | 122 | 180 | 0 | 30 | outer | 0.1 | 0.4 | 0.5 | 0.5 | 0.8 |
| 60 | 100 | 162 | 273 | 40 | 122 | 190 | 0 | 30 | outer | 0.1 | 0.4 | 0.5 | 0.5 | 0.8 |
| 60 | 100 | 162 | 273 | 40 | 122 | 200 | 0 | 30 | outer | 0.1 | 0.4 | 0.5 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 40 | 122 | 210 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 40 | 122 | 220 | 0 | 30 | outer | 0.3 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 40 | 122 | 230 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 40 | 122 | 240 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 250 | 0 | 30 | outer | 0.1 | 0.6 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 260 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 270 | 0 | 30 | outer | 0.3 | 0.6 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 40 | 122 | 273 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 170 | 0 | 30 | outer | 0.1 | 0.4 | 0.5 | 0.8 | 0.5 |
| 60 | 100 | 162 | 273 | 50 | 112 | 180 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.7 |
| 60 | 100 | 162 | 273 | 50 | 112 | 190 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 50 | 112 | 200 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.7 |
| 60 | 100 | 162 | 273 | 50 | 112 | 210 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 50 | 112 | 220 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 230 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 240 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 250 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 260 | 0 | 30 | outer | 0.3 | 0.6 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 270 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 50 | 112 | 273 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.3 |
| 60 | 100 | 162 | 273 | 60 | 102 | 150 | 0 | 30 | outer | 0.1 | 0.2 | 0.5 | 0.5 | 0.8 |
| 60 | 100 | 162 | 273 | 60 | 102 | 160 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.7 | 0.8 |
| 60 | 100 | 162 | 273 | 60 | 102 | 170 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.7 | 0.8 |
| 60 | 100 | 162 | 273 | 60 | 102 | 180 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 60 | 102 | 190 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 60 | 102 | 200 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 210 | 0 | 30 | outer | 0.1 | 0.4 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 220 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 230 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 240 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 250 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 260 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 60 | 102 | 270 | 0 | 30 | outer | 0.3 | 0.6 | 1.0 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 60 | 102 | 273 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 70 | 92 | 120 | 0 | 30 | outer | 0.0 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 70 | 92 | 130 | 0 | 30 | outer | 0.0 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 70 | 92 | 140 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.5 | 0.8 |
| 60 | 100 | 162 | 273 | 70 | 92 | 150 | 0 | 30 | outer | 0.1 | 0.4 | 0.5 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 70 | 92 | 160 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 70 | 92 | 170 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 70 | 92 | 180 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 70 | 92 | 190 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 1.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 200 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 210 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 220 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 230 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 240 | 0 | 30 | outer | 0.3 | 0.6 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 70 | 92 | 250 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 70 | 92 | 260 | 0 | 30 | outer | 0.3 | 0.6 | 1.0 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 70 | 92 | 270 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 70 | 92 | 273 | 0 | 30 | outer | 0.3 | 0.7 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 80 | 82 | 90 | 0 | 30 | outer | 0.1 | 0.2 | 0.3 | 0.5 | 0.3 |
| 60 | 100 | 162 | 273 | 80 | 82 | 100 | 0 | 30 | outer | 0.0 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 80 | 82 | 110 | 0 | 30 | outer | 0.0 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 80 | 82 | 120 | 0 | 30 | outer | 0.0 | 0.2 | 0.5 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 80 | 82 | 130 | 0 | 30 | outer | 0.1 | 0.4 | 0.5 | 0.5 | 0.8 |
| 60 | 100 | 162 | 273 | 80 | 82 | 140 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 80 | 82 | 150 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 80 | 82 | 160 | 0 | 30 | outer | 0.3 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 80 | 82 | 170 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 1.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 180 | 0 | 30 | outer | 0.1 | 0.4 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 190 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 200 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 210 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 80 | 82 | 220 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.3 |
| 60 | 100 | 162 | 273 | 80 | 82 | 230 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.3 |
| 60 | 100 | 162 | 273 | 80 | 82 | 240 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 80 | 82 | 250 | 0 | 30 | outer | 0.3 | 0.7 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 80 | 82 | 260 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 80 | 82 | 270 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 80 | 82 | 273 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 90 | 72 | 60 | 0 | 30 | outer | 0.1 | 0.2 | 0.3 | 0.3 | 0.3 |
| 60 | 100 | 162 | 273 | 90 | 72 | 70 | 0 | 30 | outer | 0.1 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 90 | 72 | 80 | 0 | 30 | outer | 0.1 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 90 | 72 | 90 | 0 | 30 | outer | 0.0 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 90 | 72 | 100 | 0 | 30 | outer | 0.0 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 90 | 72 | 110 | 0 | 30 | outer | 0.0 | 0.4 | 0.7 | 0.5 | 0.8 |
| 60 | 100 | 162 | 273 | 90 | 72 | 120 | 0 | 30 | outer | 0.0 | 0.4 | 0.5 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 90 | 72 | 130 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 90 | 72 | 140 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 90 | 72 | 150 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 90 | 72 | 160 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 90 | 72 | 170 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 90 | 72 | 180 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 90 | 72 | 190 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 90 | 72 | 200 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 90 | 72 | 210 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.3 |
| 60 | 100 | 162 | 273 | 90 | 72 | 220 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 90 | 72 | 230 | 0 | 30 | outer | 0.3 | 0.7 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 90 | 72 | 240 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 90 | 72 | 250 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 90 | 72 | 260 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 90 | 72 | 270 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 90 | 72 | 273 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 30 | 0 | 30 | outer | 0.4 | 0.2 | 0.3 | 0.3 | 0.4 |
| 60 | 100 | 162 | 273 | 100 | 62 | 40 | 0 | 30 | outer | 0.4 | 0.2 | 0.3 | 0.5 | 0.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 50 | 0 | 30 | outer | 0.1 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 100 | 62 | 60 | 0 | 30 | outer | 0.1 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 100 | 62 | 70 | 0 | 30 | outer | 0.1 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 100 | 62 | 80 | 0 | 30 | outer | 0.0 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 100 | 62 | 90 | 0 | 30 | outer | 0.0 | 0.4 | 0.5 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 100 | 62 | 100 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 100 | 62 | 110 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 100 | 62 | 120 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 100 | 62 | 130 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 100 | 62 | 140 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 1.0 | 1.1 |
| 60 | 100 | 162 | 273 | 100 | 62 | 150 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 100 | 62 | 160 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 100 | 62 | 170 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 100 | 62 | 180 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 100 | 62 | 190 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 200 | 0 | 30 | outer | 0.3 | 0.7 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 210 | 0 | 30 | outer | 0.3 | 0.7 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 220 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 230 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 100 | 62 | 240 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 250 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 260 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 270 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 100 | 62 | 273 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 110 | 52 | 10 | 0 | 30 | outer | 0.8 | 0.4 | 0.7 | 0.8 | 0.7 |
| 60 | 100 | 162 | 273 | 110 | 52 | 20 | 0 | 30 | outer | 0.7 | 0.3 | 0.6 | 0.6 | 0.6 |
| 60 | 100 | 162 | 273 | 110 | 52 | 30 | 0 | 30 | outer | 0.4 | 0.2 | 0.5 | 0.5 | 0.6 |
| 60 | 100 | 162 | 273 | 110 | 52 | 40 | 0 | 30 | outer | 0.3 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 50 | 0 | 30 | outer | 0.1 | 0.2 | 0.5 | 0.5 | 0.5 |
| 60 | 100 | 162 | 273 | 110 | 52 | 60 | 0 | 30 | outer | 0.1 | 0.4 | 0.5 | 0.5 | 0.8 |
| 60 | 100 | 162 | 273 | 110 | 52 | 70 | 0 | 30 | outer | 0.1 | 0.4 | 0.5 | 0.5 | 0.8 |
| 60 | 100 | 162 | 273 | 110 | 52 | 80 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 110 | 52 | 90 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 110 | 52 | 100 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 110 | 52 | 110 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 110 | 52 | 120 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 1.0 | 1.1 |
| 60 | 100 | 162 | 273 | 110 | 52 | 130 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.1 |
| 60 | 100 | 162 | 273 | 110 | 52 | 140 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 110 | 52 | 150 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 110 | 52 | 160 | 0 | 30 | outer | 0.3 | 0.6 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 110 | 52 | 170 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 110 | 52 | 180 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 110 | 52 | 190 | 0 | 30 | outer | 0.3 | 0.7 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 110 | 52 | 200 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 110 | 52 | 210 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 110 | 52 | 220 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 110 | 52 | 230 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 110 | 52 | 240 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 110 | 52 | 250 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 110 | 52 | 260 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 110 | 52 | 270 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 110 | 52 | 273 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 120 | 42 | 10 | 0 | 30 | outer | 1.1 | 0.4 | 0.7 | 0.7 | 0.7 |
| 60 | 100 | 162 | 273 | 120 | 42 | 20 | 0 | 30 | outer | 0.7 | 0.5 | 0.6 | 0.6 | 0.6 |
| 60 | 100 | 162 | 273 | 120 | 42 | 30 | 0 | 30 | outer | 0.4 | 0.5 | 0.5 | 0.5 | 0.6 |
| 60 | 100 | 162 | 273 | 120 | 42 | 40 | 0 | 30 | outer | 0.4 | 0.2 | 0.8 | 0.5 | 0.8 |
| 60 | 100 | 162 | 273 | 120 | 42 | 50 | 0 | 30 | outer | 0.3 | 0.4 | 0.5 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 120 | 42 | 60 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 120 | 42 | 70 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 120 | 42 | 80 | 0 | 30 | outer | 0.1 | 0.4 | 0.7 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 120 | 42 | 90 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 120 | 42 | 100 | 0 | 30 | outer | 0.1 | 0.4 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 120 | 42 | 110 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.0 | 1.1 |
| 60 | 100 | 162 | 273 | 120 | 42 | 120 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 120 | 42 | 130 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 120 | 42 | 140 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.1 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 150 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 160 | 0 | 30 | outer | 0.3 | 0.7 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 170 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 180 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 190 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 200 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 210 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 220 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 230 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 240 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 250 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 260 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 270 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 120 | 42 | 273 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 130 | 32 | 10 | 0 | 30 | outer | 1.1 | 0.6 | 0.7 | 1.0 | 0.7 |
| 60 | 100 | 162 | 273 | 130 | 32 | 20 | 0 | 30 | outer | 0.7 | 0.2 | 0.6 | 0.8 | 0.6 |
| 60 | 100 | 162 | 273 | 130 | 32 | 30 | 0 | 30 | outer | 0.4 | 0.5 | 0.5 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 130 | 32 | 40 | 0 | 30 | outer | 0.4 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 130 | 32 | 50 | 0 | 30 | outer | 0.3 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 130 | 32 | 60 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 130 | 32 | 70 | 0 | 30 | outer | 0.1 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 130 | 32 | 80 | 0 | 30 | outer | 0.1 | 0.4 | 1.0 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 130 | 32 | 90 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 130 | 32 | 100 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.0 | 1.1 |
| 60 | 100 | 162 | 273 | 130 | 32 | 110 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.1 |
| 60 | 100 | 162 | 273 | 130 | 32 | 120 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.1 |
| 60 | 100 | 162 | 273 | 130 | 32 | 130 | 0 | 30 | outer | 0.3 | 0.6 | 1.0 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 140 | 0 | 30 | outer | 0.3 | 0.7 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 150 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 160 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 170 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 180 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 190 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 200 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 210 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 220 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 230 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 240 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 250 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 260 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 270 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 130 | 32 | 273 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 140 | 22 | 10 | 0 | 30 | outer | 1.1 | 0.6 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 140 | 22 | 20 | 0 | 30 | outer | 0.7 | 0.5 | 0.9 | 0.9 | 0.9 |
| 60 | 100 | 162 | 273 | 140 | 22 | 30 | 0 | 30 | outer | 0.4 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 140 | 22 | 40 | 0 | 30 | outer | 0.4 | 0.4 | 0.8 | 0.8 | 0.8 |
| 60 | 100 | 162 | 273 | 140 | 22 | 50 | 0 | 30 | outer | 0.3 | 0.4 | 0.8 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 140 | 22 | 60 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 140 | 22 | 70 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 140 | 22 | 80 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 140 | 22 | 90 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 140 | 22 | 100 | 0 | 30 | outer | 0.3 | 0.7 | 1.0 | 1.0 | 1.4 |
| 60 | 100 | 162 | 273 | 140 | 22 | 110 | 0 | 30 | outer | 0.3 | 0.7 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 140 | 22 | 120 | 0 | 30 | outer | 0.3 | 0.7 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 140 | 22 | 130 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 140 | 22 | 140 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 140 | 22 | 150 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 140 | 22 | 160 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 140 | 22 | 170 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 140 | 22 | 180 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 140 | 22 | 190 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.3 |
| 60 | 100 | 162 | 273 | 140 | 22 | 200 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 140 | 22 | 210 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.3 |
| 60 | 100 | 162 | 273 | 140 | 22 | 220 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 140 | 22 | 230 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.0 |
| 60 | 100 | 162 | 273 | 140 | 22 | 240 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.0 |
| 60 | 100 | 162 | 273 | 140 | 22 | 250 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 140 | 22 | 260 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.0 |
| 60 | 100 | 162 | 273 | 140 | 22 | 270 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 140 | 22 | 273 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 150 | 12 | 10 | 0 | 30 | outer | 1.1 | 0.6 | 1.0 | 1.0 | 1.0 |
| 60 | 100 | 162 | 273 | 150 | 12 | 20 | 0 | 30 | outer | 0.7 | 0.5 | 0.9 | 0.9 | 0.8 |
| 60 | 100 | 162 | 273 | 150 | 12 | 30 | 0 | 30 | outer | 0.4 | 0.5 | 1.1 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 150 | 12 | 40 | 0 | 30 | outer | 0.4 | 0.7 | 1.1 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 150 | 12 | 50 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 150 | 12 | 60 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 150 | 12 | 70 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 150 | 12 | 80 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 150 | 12 | 90 | 0 | 30 | outer | 0.3 | 0.7 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 150 | 12 | 100 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 150 | 12 | 110 | 0 | 30 | outer | 0.3 | 0.9 | 1.4 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 150 | 12 | 120 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.3 |
| 60 | 100 | 162 | 273 | 150 | 12 | 130 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.3 |
| 60 | 100 | 162 | 273 | 150 | 12 | 140 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 150 | 12 | 150 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 150 | 12 | 160 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 150 | 12 | 170 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.3 |
| 60 | 100 | 162 | 273 | 150 | 12 | 180 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 150 | 12 | 190 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.3 |
| 60 | 100 | 162 | 273 | 150 | 12 | 200 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 2.0 |
| 60 | 100 | 162 | 273 | 150 | 12 | 210 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.0 |
| 60 | 100 | 162 | 273 | 150 | 12 | 220 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.0 |
| 60 | 100 | 162 | 273 | 150 | 12 | 230 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.0 |
| 60 | 100 | 162 | 273 | 150 | 12 | 240 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.0 | 2.0 |
| 60 | 100 | 162 | 273 | 150 | 12 | 250 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.0 |
| 60 | 100 | 162 | 273 | 150 | 12 | 260 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 2.0 |
| 60 | 100 | 162 | 273 | 150 | 12 | 270 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.0 |
| 60 | 100 | 162 | 273 | 150 | 12 | 273 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.0 |
| 60 | 100 | 162 | 273 | 160 | 2 | 10 | 0 | 30 | outer | 1.1 | 0.9 | 1.2 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 160 | 2 | 20 | 0 | 30 | outer | 0.7 | 0.8 | 1.2 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 160 | 2 | 30 | 0 | 30 | outer | 0.4 | 0.7 | 1.1 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 160 | 2 | 40 | 0 | 30 | outer | 0.4 | 0.7 | 1.1 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 160 | 2 | 50 | 0 | 30 | outer | 0.1 | 0.7 | 1.1 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 160 | 2 | 60 | 0 | 30 | outer | 0.1 | 0.7 | 1.0 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 160 | 2 | 70 | 0 | 30 | outer | 0.3 | 1.0 | 1.4 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 160 | 2 | 80 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 160 | 2 | 90 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 160 | 2 | 100 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 160 | 2 | 110 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 160 | 2 | 120 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.4 |
| 60 | 100 | 162 | 273 | 160 | 2 | 130 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 160 | 2 | 140 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 160 | 2 | 150 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 160 | 2 | 160 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.1 |
| 60 | 100 | 162 | 273 | 160 | 2 | 170 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 160 | 2 | 180 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.0 |
| 60 | 100 | 162 | 273 | 160 | 2 | 190 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.0 | 2.0 |
| 60 | 100 | 162 | 273 | 160 | 2 | 200 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 1.3 | 2.0 |
| 60 | 100 | 162 | 273 | 160 | 2 | 210 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.0 | 2.0 |
| 60 | 100 | 162 | 273 | 160 | 2 | 220 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.0 | 2.0 |
| 60 | 100 | 162 | 273 | 160 | 2 | 230 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.0 | 2.1 |
| 60 | 100 | 162 | 273 | 160 | 2 | 240 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.0 | 2.1 |
| 60 | 100 | 162 | 273 | 160 | 2 | 250 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.0 | 2.1 |
| 60 | 100 | 162 | 273 | 160 | 2 | 260 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.0 | 2.0 |
| 60 | 100 | 162 | 273 | 160 | 2 | 270 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.0 | 2.0 |
| 60 | 100 | 162 | 273 | 160 | 2 | 273 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.0 | 2.0 |
| 60 | 100 | 162 | 273 | 162 | 0 | 10 | 0 | 30 | outer | 1.1 | 0.9 | 1.2 | 1.3 | 1.3 |
| 60 | 100 | 162 | 273 | 162 | 0 | 20 | 0 | 30 | outer | 0.7 | 0.8 | 1.2 | 1.2 | 1.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 30 | 0 | 30 | outer | 0.4 | 0.7 | 1.1 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 40 | 0 | 30 | outer | 0.4 | 0.7 | 1.1 | 1.1 | 1.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 50 | 0 | 30 | outer | 0.1 | 0.7 | 1.1 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 60 | 0 | 30 | outer | 0.4 | 0.7 | 1.4 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 70 | 0 | 30 | outer | 0.4 | 0.7 | 1.4 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 80 | 0 | 30 | outer | 0.3 | 1.0 | 1.4 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 90 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 100 | 0 | 30 | outer | 0.3 | 1.0 | 1.4 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 110 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 120 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 130 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 140 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 150 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 160 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 170 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 2.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 180 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 1.4 |
| 60 | 100 | 162 | 273 | 162 | 0 | 190 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 2.1 | 2.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 200 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.1 | 2.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 210 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 2.1 | 2.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 220 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.1 | 2.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 230 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 2.1 | 2.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 240 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 2.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 250 | 0 | 30 | outer | 0.3 | 0.9 | 1.4 | 2.1 | 2.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 260 | 0 | 30 | outer | 0.3 | 0.9 | 1.3 | 2.0 | 2.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 270 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 1.4 | 2.1 |
| 60 | 100 | 162 | 273 | 162 | 0 | 273 | 0 | 30 | outer | 0.3 | 1.0 | 1.3 | 2.1 | 2.1 |