3GPP TSG-RAN WG4 Meeting #114	R4-2501497

Athens, Greece, February 17th – 21st 2025

Agenda Item:	7.20.3

Source:	Ericsson

Title:	On 2-sided compression study and simulations

Document for:	Discussion

# 1	Introduction

During RAN4#113, it was agreed to follow two tracks for investigating the option 3 for CSI compression as follows:

Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset)

- The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s).

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

Following RAN4#113, several companies have uploaded encoder and decoder models and datasets. The decoders provided by CATT, Huawei and Samsung were selected as example test decoders for following track 1.

In this contribution, we provide some initial results for track 1, and encoder/decoders based on a mixed dataset for track 2.

The simulation campaign does not address all aspects that need to be considered if option 3 or option 4 would be adopted as solutions in RAN4. In this contribution, we discuss some further aspects that should be considered.

# 2	Simulation results for track 1

According to track 1, companies should attempt to train an encoder based on one of the selected “test decoders” and examine whether a similar performance can be obtained with the encoder+test decoder pair compared to what can be obtained with the original encoder/decoder pair.

Three companies’ decoders were selected after RAN4#113 as test decoder examples; these were from CATT, Huawei, and Samsung.

After examining the models, one observation we make is that there is some ambiguity how quantization is dealt with:

- Whether there is quantization

- If so, what kind of quantization

- Whether compression (Sigmoid) is applied

- Whether the training is quantizer aware

The approach to quantization should probably be aligned, in particular if some test decoders are found not to perform well. Some discussion has taken place in between the meetings on aligning quantization assumptions, but it would be good to agree and confirm an assumption at RAN4#114.

Align assumptions on model quantization

## 2.1	Inference of companies’ model with the agreed test set

To make sure that our simulation tools, including the model assumption (e.g., on the quantization details), align with the model shared by companies, we conducted an inference of the agreed models using the mixed test set (70K), including testing with each individual company test set (7x10K each). The evaluation results can be seen in Table 1 and Table 2 for the unquantized model and the quantized model, respectively. Note that each of these models are presumed to be trained with their respected dataset (e.g., Samsung model is trained with Samsung dataset).

Table 1	SGCS performance of companies’ unquantized model tested with various test set

| Test set | CATT model | Samsung model | Ericsson model |
| --- | --- | --- | --- |
| CATT | 0.8108 | 0.5697 | 0.4916 |
| Ericsson | 0.2995 | 0.7248 | 0.7776 |
| Mediatek | 0.3451 | 0.6541 | 0.6364 |
| Nokia | 0.3195 | 0.7514 | 0.7593 |
| Oppo | 0.3398 | 0.7605 | 0.7640 |
| Vivo | 0.2884 | 0.7198 | 0.7372 |
| Qualcomm | 0.1437 | 0.5296 | 0.5156 |
| Mixed | 0.3638 | 0.6729 | 0.6688 |

Table 2	SGCS performance of companies’ quantized model tested with various test set

| Test set | CATT model | Huawei model | Ericsson model |
| --- | --- | --- | --- |
| CATT | 0.7455 | 0.3946 | 0.4367 |
| Ericsson | 0.2370 | 0.5286 | 0.7137 |
| Mediatek | 0.3017 | 0.3963 | 0.5565 |
| Nokia | 0.2508 | 0.5559 | 0.7032 |
| Oppo | 0.2643 | 0.5793 | 0.7138 |
| Vivo | 0.2331 | 0.5127 | 0.6731 |
| Qualcomm | 0.1339 | 0.4585 | 0.4597 |
| Mixed | 0.3095 | 0.4894 | 0.6081 |

From the above tables, we can observe a noticeable variance in the SGCS performance when the testing is conducted for each test set for different models. This may suggest that a model from a company (trained with a training dataset of the respective company) may not work well with other company dataset. For sure, training with a mixed dataset may alleviate, to some extent, this problem. However, it may be problematic to setup, e.g., performance requirement when a model performance varies substantially between supposedly similar test and training sets even when the evaluation assumption is the same (this will also be discussed a bit in the evaluation for Track 2). Therefore, it is encouraged that companies simulate their model with other company test set and see whether there is a substantial performance degradation occurs or whether the model is robust across the test sets (which should actually be similar).

There can be substantial difference in SGCS when a model (encoder+decoder) created by one company is tested using a testset from another.

Further discuss/consider the source of variation between different testsets, since this may be problematic if setting requirements.

## 2.2	Training an encoder with agreed (frozen) decoders

In the below evaluation, we take the decoder from Samsung’s model. The model is trained without quantization and there is no sigmoid included.

Note that we have used the same encoder structure as previously agreed. Once a test decoder is established in the specification, it may be desirable for UE vendors to create encoders with a different structure. It may be interesting to investigate how sensitive the performance with the test decoder is to different encoder structures.

### 2.2.1	Training with Ericsson dataset

In this evaluation, these three following cases are being compared:

- Encoder 1 – Decoder 1

Here, the encoder and the decoder part come from Samsung model which is trained with Samsung’s dataset.

- Encoder 2 – Decoder 2

Here, the encoder and the decoder part come from Ericsson model which is trained with Ericsson’s dataset. Note that in the model, sigmoid is included in the model. The results of unquantized model using and not using sigmoid, however, shall be similar.

- Encoder 3 – Decoder 1

Here, the decoder part come from the Samsung model which is trained with Samsung’s dataset. The encoder is trained with Decoder 1 (which is frozen during the training) using Ericsson training dataset.

The evaluation can be seen in the following figure.

Figure 1	Performance of encoder-training with frozen decoder and Ericsson dataset

From the above figure, we can observe that training with a frozen decoder is generally feasible. The performance gain/loss compared to a more specialized model may vary depending on the dataset. The gain/loss of training with a frozen decoder is better captured in the evaluation in Section 2.2.2.

Our initial finding is that training an encoder based on a fixed decoder is feasible.

### 2.2.2	Training with mixed dataset

This evaluation is conducted in a similar manner with the evaluation of Section 2.2.1. The difference lies on the dataset used for the training and testing. In general, the following setup is being considered.

- Encoder 1 – Decoder 1

Here, the encoder and the decoder part come from Samsung model which is trained with Samsung’s dataset.

- Encoder 2 – Decoder 2

Here, the encoder and the decoder part come from joint encoder decoder training with mixed dataset.

- Encoder 3 – Decoder 1

Here, the decoder part come from the Samsung model which is trained with Samsung’s dataset. The encoder is trained with Decoder 1 (which is frozen during the training) using mixed dataset.

The evaluation of the models with various test set can be seen in the following table figure.

Figure 2	Performance of encoder-training with frozen decoder and mixed dataset. Unquantized model is used.

From the above figure, we can observe the following:

- The performance for the Encoder 2 – Decoder 2 pair is generally the best. This is because the model is trained with mixed dataset to begin with.

- For the case of the Encoder 1 – Decoder 1 pair has a good performance (e.g., above 0.7), the performance of that pair is better than the rest encoder – decoder pair setup. This is because the Encoder 1 – Decoder 1 pair is trained with a more specific dataset which somewhat match with the respected test set.

- Train an encoder with mixed dataset (although the decoder is trained with a specific dataset) may be generally improve the performance as it gives better generalization capability. However, for the case of the model is tested with a test set that is similar with the dataset used for the training of the original model, a slight performance degradation is observed.

In the below figure, a similar setup with the above evaluation is done. However, the quantized model is used instead of the unquantized model.

Figure 3	Performance of encoder-training with frozen decoder and mixed dataset. Quantized model is used

# 3.	Simulation results for track 2

According to track 2, as a first step a mixed dataset should be created and used to train an encoder/decoder. One or more of the decoders should be selected as an example test decoder. Following this step, companies should use the test decoder(s) and create encoders, similar to track 1.

To create the mixed dataset, it was agreed during reflector discussion to take 100K samples from N companies that have uploaded datasets (7 until our evaluation is conducted). From each 100K samples, 10K samples are used for the test set. I.e., there will be a total of 630K samples for training and 70K samples for testing, collected from 7 companies’ dataset.

To further progress track 2, we trained a model (i.e., an encoder-decoder) pair with this mixed dataset. The model can be uploaded to the 3GPP server if needed. Besides being tested with the mixed test set, the model is also tested with an individual 10K test set from each company. The evaluation results can be seen in the following table. Note that in the following table, there is no sigmoid operation in the encoder for the unquantized model.

Table 3	SGCS performances of models trained with mixed dataset

| Test set | Un-quantized | % gap to mixed dataset | Quantized | % gap to mixed dataset |
| --- | --- | --- | --- | --- |
| Mixed | 0.7460 | 0.0% | 0.6932 | 0.0% |
| CATT | 0.6984 | -6.4% | 0.6528 | -5.8% |
| Ericsson | 0.7203 | -3.4% | 0.6679 | -3.6% |
| Mediatek | 0.7616 | 2.1% | 0.7041 | 1.6% |
| Nokia | 0.7425 | -0.5% | 0.6897 | -0.5% |
| Oppo | 0.7523 | 0.8% | 0.7063 | 1.9% |
| Vivo | 0.7169 | -3.9% | 0.6590 | -4.9% |
| Qualcomm | 0.8299 | 11.2% | 0.7728 | 11.5% |

From the above table, we can observe that there is substantial variation when the model is tested with each individual company test set. This may suggest that there is certain difference on the dataset distribution or characteristic, and possibly on the data pre-processing mechanism (e.g., the normalization process).

There is substantial variation between the datasets used to create the mixed dataset.

In the below, we plot a norm across subband for TX port index = 0 for each 10K test set.

Figure 4	Norm across subbands for TX port with index = 0

From the above figure, we can observe that there is a difference on the dataset characteristic/distribution from companies. In particular, sample 1-10000 seems to have a different CDF for TX port index 0, which may indicate a different approach to normalization.

Align on the normalization used for the test dataset.

Note that the above figure is only one mechanism to assess the dataset characteristic/distribution, and there may be other mechanisms which may explain the substantial difference on the SGCS performance when the model is tested with different test set.

# 4. Additional considerations for option 3

Option 3 is standardization of a test decoder in the 3GPP specifications. The current simulation work is related to assessing whether it is realistic, at least in a simulation environment, to fix a test decoder and then for companies to be able to create their own encoders with satisfactory performance. The simulation activity can both provide an initial view on the viability of option 3 as well as a learning curve on how a real standardization activity could proceed. Apart from the simulation activity, a number of other considerations arise when considering the viability of option 3.

For exchanging models during the study phase, ONNX has been used, as this enables swapping of models independently of AI tools used by different companies. This is useful for a study phase, however for permanent capture of a test decoder in the specifications it may not be suitable. ONNX is an open source tool that is subject to variation between versions and that is not under the control of 3GPP. A 3GPP specification needs to be open access and not be subject to becoming invalid due to variations in the platform used for representing the model. The 3GPP specification should also not be tied to a specific AI/ML platform; both because it restricts implementation freedom and also because this would again link the 3GPP specification with a third-party tool.

Thus, there may be a need to develop a 3GPP specification method for a test decoder in the TS.

The study on option 3 should consider that a 3GPP internal method for capturing test decoder AI models is needed. Possible means to achieve this may be discussed if time allows.

The 3GPP representation may also need a process for updating of the test decoder. This may be handled by the CR and release process, although some thought may be needed on managing backward compatibility. For example, if encoders would not be directly compatible with an updated decoder, then a mismatch could occur between the version of a decoder assumed by the network and the version against which UE encoders have been trained; such mismatches should be avoided or handled by signalling.

The study should consider how a procedure for updating the test decoder would work, considering in particular whether updates should always be backward compatible or whether signalling of versions could be needed.

It is possible that requirements may be derived for several types of cell, e.g., corresponding to an indoor-hotspot-type of deployment or an outdoor-macro-type of deployment. If the cell type would e.g. be signalled, then there exists the possibility of standardizing a test decoder specific to each condition. This approach would require the UE to operate more than one encoder. Further discussion may be needed on how many test decoders to specify, and how test decoders should be linked to signalling, and may depend on RAN1 design.

The study should consider / note that different test decoders may be needed.

Moving to the process of deciding on the decoder during standardization, for the simulation exercise some arbitrary assumptions were made on the decoder structure in order to obtain an alignment to check the decoder development process using simulation. During a real standardization, there would be a need to spend time and detail on agreeing the decoder structure. The starting point would be the backbone, scale and complexity of the overall model. Some overall goals would need to be agreed, such as performance target, complexity target, number of models, degree of generalizability over a wide test-set. There may also be a need to consider the extent to which different types of test decoder backbone and structure could support variation in encoder and in decoder implementations. It might be expected that a large amount of time might be needed for this discussion.

For the current study phase, it may be useful to discuss around what characteristics may be of importance when deciding on a real decoder structure. Some candidates for discussion are outlined in proposal 7.

The study should consider what key characteristics should drive the discussion on structure for a real standardization exercise. Some characteristics include:

- Backbone (CNN, transformer etc.)

- Complexity target

- Number of models (depending on scenarios etc.)

- Acceptable performance target

- Ability of the agreed structure for the test decoder to allow for variation of real encoder/decoder implementation

It should be noted that a discussion is ongoing in RAN1 relating to model specification options. Although the purpose of the RAN1 discussion is different (inter-vendor collaboration rather than standardization of a test decoder), a discussion in RAN4 on test decoder structure may do well to co-ordinate with the RAN1 discussions and conclusions, in particular considering that it may be the case that aligning the test decoder structure with a structure used for collaboration may be needed.

Conclude that, if Option 3 is supported for performance testing, then the fully specified test decoder is the same as the decoder in the reference model for Direction C in RAN1 inter-vendor training collaboration discussion.

# 5. Additional considerations for option 4

With option 4, rather than standardizing a test decoder directly, the specification contains sufficient information that TE vendors can individually train a test decoder, and correctly trained test decoders will be interoperable and provide the same performance with any compliant encoder.

The agreements so far have not captured what needs to be specified for option 4. To provide interoperability, some kind of information on the ordering of the latent space needs to be specified. To provide the same performance, it is necessary that the same training dataset is used by all TE vendors. There are two ways to capture the latent space ordering; either (i) A dataset for which each encoder input is labelled with an encoder output (option 4a) or (ii) a dataset and a standardized encoder (option 4b).

A latent space ordering and dataset may not be sufficient. It is not clear whether the backbone and structure for the test decoder would need to be captured in the specification. If not captured, then the implication is that different TE vendors could use different structure/backbone and still obtain interoperability and the same performance. Whether it is possible for different structures to provide the same performance and interoperability needs to be investigated as part of the study.

For option 4, the study needs to clarify whether the decoder structure should be specified or left for the TE vendor to decide.

If comparable performance cannot be obtained by different model structures/backbone and a detailed model structure needs to be specified, then option 4 has very little advantage over option 3. Basically, instead of capturing both structure and parameters of a test decoder, option 4 would capture structure and dataset with latent space, which would require more storage and introduce an additional step and uncertainty of the TE training procedure.

If the structure is specified for option 4, then there is little difference between option 3 and option 4.

Assuming that it is possible for different structure and parameters to be used for the test decoder while obtaining the same performance and interoperability, then there needs to be a means to limit the amount of variation in the structure. Obviously, for example there will be a lower limit to the complexity of the test decoder. One means to do this is to provide a standardized verification encoder and dataset for the test decoder to ensure that the test decoder performance is interoperable and at the correct level. This would verify test decoder implementations. With such a verification, there would not be a direct need to specify any model structure, since any test decoder that could pass the verification would suffice.

Instead of specifying a structure, a reference encoder and acceptable verification performance could be specified for option 4.

We use the terminology “reference encoder” and “verification encoder” here as follows:

- A “reference encoder” is a standardized encoder that could be used on a training dataset in order to get the latent space.

- A “verification encoder” is a standardized encoder that could be used on a verification dataset that could be used to check the performance of the decoder.

In reality, the encoder would be the same in either case; the terminology difference is to differentiate the usage.

A further aspect that needs to be considered for option 4 is the extent to which the training process needs to be specified. Different implementations of the training procedure between different TE vendors could lead to divergence in test decoder performance. To avoid divergence, one way would be to specify the training procedure.

If the training procedure is not specified for option 4, there could be divergence between different TE decoders.

Another way to avoid the need to specify details of the training procedure would be to specify a verification encoder and dataset in order to ensure test decoder interoperability and performance. Thus, as long as the training procedure could ensure a test decoder that would pass the verification, the training procedure could be left open for the test equipment vendor.

The need to specify training procedure could be mitigated by specifying an encoder and acceptable performance level when verifying the test decoder.

Thus, a verification encoder and dataset would avoid the need for specifying e.g. model structure and training procedure. However, a verification encoder and dataset would need to be carefully standardized, and would need to ensure performance over a large enough range of different circumstances. Since specifying the structure would make option 4 very similar to option 3, we propose to focus option 4 on specifying a reference encoder and verification.

For option 4, focus on specifying the following,

- Dataset, with possibly the latent space

- (Reference) encoder, at least for verification

- Verification dataset and acceptable verification performance.

If a verification encoder, dataset and performance are not specified for option 4, then a structure and training procedure should be specified. Option 4 is then very similar to option 3.

A further area for discussion when considering option 4 is on how to capture information in the specification. If a dataset and encoder would be specified, and/or a verification encoder then there would be a need to capture the encoder model in the TS. As described for option 3, whilst ONNX is suitable for exchanging models during the study, it is outside of 3GPP and may not be suitable for standardizing an encoder. Further work may be needed on defining a means within the 3GPP domain of capturing an encoder model within the specifications.

For option 4, there would also be a need to provide a means to capture the reference/verification encoder in a 3GPP TS.

Apart from the possibility of capturing an encoder, there may be a need to capture a dataset or a dataset together with the latent space. This would necessitate standardization of a format for capturing the dataset within the 3GPP specifications, however it is not expected that this would be as challenging as capturing a model structure.

If the model structure or the training procedure is captured or partially captured in the 3GPP specifications, then it is in addition necessary to find a means to capture those things. In principle, however this can be achieved by existing specification processes.

Again, as noted above, a discussion is ongoing in RAN1 relating to model specification options. Although the purpose of the RAN1 discussion is different (inter-vendor collaboration rather than standardization of a test decoder), a discussion in RAN4 on reference or verification encoder structure may do well to co-ordinate with the RAN1 discussions and conclusions, in particular considering that it may be the case that aligning the verification encoder structure with a structure used for collaboration may be needed.

Conclude that, if Option 4 is supported for performance testing, and if a reference or verification encoder is specified, then the fully specified verification encoder is the same as the encoder in the reference model for Direction C in RAN1 inter-vendor training collaboration discussion.

# Conclusion

In the previous sections we made the following observations:

Observation 1	There can be substantial difference in SGCS when a model (encoder+decoder) created by one company is tested using a testset from another.

Observation 2	Our initial finding is that training an encoder based on a fixed decoder is feasible.

Observation 3	There is substantial variation between the datasets used to create the mixed dataset.

Observation 4	For option 4, the study needs to clarify whether the decoder structure should be specified or left for the TE vendor to decide.

Observation 5	If the structure is specified for option 4, then there is little difference between option 3 and option 4.

Observation 6	Instead of specifying a structure, a reference encoder and acceptable verification performance could be specified for option 4.

Observation 7	If the training procedure is not specified for option 4, there could be divergence between different TE decoders.

Observation 8	The need to specify training procedure could be mitigated by specifying an encoder and acceptable performance level when verifying the test decoder.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Align assumptions on model quantization

Proposal 2	Further discuss/consider the source of variation between different testsets, since this may be problematic if setting requirements.

Proposal 3	Align on the normalization used for the test dataset.

Proposal 4	The study on option 3 should consider that a 3GPP internal method for capturing test decoder AI models is needed. Possible means to achieve this may be discussed if time allows.

Proposal 5	The study should consider how a procedure for updating the test decoder would work, considering in particular whether updates should always be backward compatible or whether signalling of versions could be needed.

Proposal 6	The study should consider / note that different test decoders may be needed.

Proposal 7	The study should consider what key characteristics should drive the discussion on structure for a real standardization exercise. Some characteristics include:

	Backbone (CNN, transformer etc.)

	Complexity target

	Number of models (depending on scenarios etc.)

	Acceptable performance target

	Ability of the agreed structure for the test decoder to allow for variation of real encoder/decoder implementation

Proposal 8	Conclude that, if Option 3 is supported for performance testing, then the fully specified test decoder is the same as the decoder in the reference model for Direction C in RAN1 inter-vendor training collaboration discussion.

Proposal 9	For option 4, focus on specifying the following,

	Dataset, with possibly the latent space

	(Reference) encoder, at least for verification

	Verification dataset and acceptable verification performance.

Proposal 10	If a verification encoder, dataset and performance are not specified for option 4, then a structure and training procedure should be specified. Option 4 is then very similar to option 3.

Proposal 11	For option 4, there would also be a need to provide a means to capture the reference/verification encoder in a 3GPP TS.

Proposal 12	Conclude that, if Option 4 is supported for performance testing, and if a reference or verification encoder is specified, then the fully specified verification encoder is the same as the encoder in the reference model for Direction C in RAN1 inter-vendor training collaboration discussion.