3GPP TSG-RAN WG4 Meeting #114	R4-2501517

# Athens, Greece, 17th – 21st Feb, 2025

Title: 	Discussions on spatial channel models

Source: 	Huawei, HiSilicon

Agenda item:	7.16.2

Document for:	Discussion

Background

In last meeting, WF R4-2419782 [1] related to spatial channel models was approved, this contribution provides our views.

- TDL based approach

The main problem for legacy TDL+ correlation channel model is rank deficiency, which is not suitable for PDSCH requirements definition with high layers. To increase the spatial degree of freedom, two kinds of enhanced TDL methodologies are proposed by some companies, one is proposed by [2], the idea of which is to introduce per tap per correlation. The pro is that the model is very typical and proposed by a lot of literatures, and very close to the real MIMO channel model in the real deployment. However, the con is huge computational complexity, which may be unacceptable by some companies.  The other one is proposed by [3], the idea of which is to extent existing two cluster TDL channel model to multi-cluster channel model, where each cluster corresponds to one TX-RX beam. The pro is that it’s easy to be implemented by TE, the cons is that each TX-RX beam have same delay, which may be artificial and may not reflect the real channel characterisers. Next we propose a new approach to combine such two approaches, which is easy to be implemented and can also highly increase the spatial degree of freedom.

Instead of deriving the correlation matrix per tap from formula, we implement multi beams in one TDL channel, where each beam corresponds to some taps and each beam is steering of other beams with steering matrixes in B.2.3.2.3A of 38.101-4 The formula is:

ht,τn=Dθk1,k2rx,n*ht,τn*Dθk1,k2tx,n

Where ht,τn (NRx x NTx) is channel coefficient for tap n with legacy correlation matrix applied in time domain. (XPL high is used in our simulation), n=0,1,2,…,N-1, N is number of taps per TDL channel, i.e. 12 for simplified TDL channel used for performance requirements definition. Pn normalized power of tap n. Dθk1,k2rx,n and Dθk1,k2tx,n are Rx beam steering matrix and Tx beam steering matrix respectively for tap n. ht,τn is final channel coefficient for tap n after applying beam steering.

Dθk1,k2rx,n=1001⨂Dθk1rx,nN1⨂Dθk2rx,nN2

Dθk1,k2tx,n=1001⨂Dθk1tx,nN1⨂Dθk2tx,nN2

-	Dθk1rx,nN1, Dθk1tx,nN2is the steering matrix for tap n in first dimension with same polarization for Rx beam and Tx beam respectively

-	Dθk2rx,nN1, Dθk2tx,nN2is the steering matrix for tap n in second dimension with same polarization for Rx beam and Tx beam respectively

-	 is the number of antenna elements in first dimension with same polarization,

-	 is the number of antenna elements in second dimension with same polarization,

-	For antenna array with only one direction, number of antenna element in second direction equals 1，

For 1 antenna element of the same polarization in one direction, .

For 2 antenna elements of the same polarization in one direction, .

For 3 antenna elements of the same polarization in one direction,.

For 4 antenna elements of the same polarization in one direction, .

Table 2-1 provides our proposed enhanced TDLC300 channel with 2 TX-RX beams configurations.

Table 2-1: Enhanced TDLC300 channel with 2 TX-RX beams

| Tap | Delay(ns) | Power(dB) | Tx-Rx Beam | Beam Power(dB) |
| --- | --- | --- | --- | --- |
| 1 | 0 | -6.9 | Beam 1 θk1=0 θk2=0 | 0 |
| 2 | 65 | 0 |  |  |
| 3 | 70 | -7.7 |  |  |
| 4 | 190 | -2.5 |  |  |
| 5 | 195 | -2.4 | Beam 2 θk1=π θk2=π | -1.52 |
| 6 | 200 | -9.9 |  |  |
| 7 | 240 | -8.0 |  |  |
| 8 | 325 | -6.6 |  |  |
| 9 | 520 | -7.1 |  |  |
| 10 | 1045 | -13.0 |  |  |
| 11 | 1510 | -14.2 |  |  |
| 12 | 2595 | -16.0 |  |  |

Table 2-2 provides our proposed enhanced TDLC300 channel with 3 TX-RX beams configurations.

Table 2-2: Enhanced TDLC300 channel with 3 TX-RX beams

| Tap | Delay(ns) | Power | Beam steering | Beam Power(dB) |
| --- | --- | --- | --- | --- |
| 1 | 0 | -6.9 | Beam1 θk1=0 θk2=0 | 0 |
| 2 | 65 | 0 |  |  |
| 3 | 70 | -7.7 |  |  |
| 4 | 190 | -2.5 | Beam2 θk1=2π/3 θk2=2π/3 | -0.82 |
| 5 | 195 | -2.4 |  |  |
| 6 | 200 | -9.9 | Beam3 θk1=4π/3 θk2=4π/3 | -2.41 |
| 7 | 240 | -8.0 |  |  |
| 8 | 325 | -6.6 |  |  |
| 9 | 520 | -7.1 |  |  |
| 10 | 1045 | -13.0 |  |  |
| 11 | 1510 | -14.2 |  |  |
| 12 | 2595 | -16.0 |  |  |

Table 2-3 provides our proposed enhanced TDLC300 channel with 4 TX-RX beams configurations.

Table 2-3: Enhanced TDLC300 channel with 4 TX-RX beams

| Tap | Delay(ns) | Power | Beam steering | Beam Power(dB) |
| --- | --- | --- | --- | --- |
| 1 | 0 | -6.9 | Beam1 θk1=0 θk2=0 | 0 |
| 2 | 65 | 0 |  |  |
| 3 | 70 | -7.7 | Beam2 θk1=π/2 θk2=π/2 | -2.16 |
| 4 | 190 | -2.5 |  |  |
| 5 | 195 | -2.4 | Beam3 θk1=π θk2=π | -2.50 |
| 6 | 200 | -9.9 |  |  |
| 7 | 240 | -8.0 | Beam4 θk1=3π/2 θk2=3π/2 | -2.45 |
| 8 | 325 | -6.6 |  |  |
| 9 | 520 | -7.1 |  |  |
| 10 | 1045 | -13.0 |  |  |
| 11 | 1510 | -14.2 |  |  |
| 12 | 2595 | -16.0 |  |  |

Table 2-4 provides our proposed enhanced TDLC300 channel with 6 TX-RX beams configurations.

Table 2-4: Enhanced TDLC300 channel with 6 TX-RX beams

| Tap | Delay(ns) | Power | Beam | Beam Power(dB) |
| --- | --- | --- | --- | --- |
| 1 | 0 | -6.9 | Beam1 θk1=0 θk2=0 | -7.58 |
| 2 | 65 | 0 | Beam2 θk1= π/3 θk2=π/3 | 0 |
| 3 | 70 | -7.7 |  |  |
| 4 | 190 | -2.5 | Beam3 θk1=2π/3 θk2=2π/3 | -3.18 |
| 5 | 195 | -2.4 | Beam4 θk1=π θk2=π | -2.37 |
| 6 | 200 | -9.9 |  |  |
| 7 | 240 | -8.0 | Beam5 θk1=4π/3 θk2=4π/3 | -4.91 |
| 8 | 325 | -6.6 |  |  |
| 9 | 520 | -7.1 | Beam6 θk1=5π/3 θk2=5π/3 | -5.79 |
| 10 | 1045 | -13.0 |  |  |
| 11 | 1510 | -14.2 |  |  |
| 12 | 2595 | -16.0 |  |  |

Figure 2-1 shows Angular domain of Enhanced TDLC channel with different number of beams (8T8R). It can be observed that spatial freedom domain is improved as number of Tx-Rx beams is increased.

| Legacy TDLC channel | Enhanced TDLC300 channel with 2 TX-RX beams |
| --- | --- |
| Enhanced TDLC300 channel with 3 TX-RX beams | Enhanced TDLC300 channel with 6 TX-RX beams |

Figure 2-1: Angular domain of Enhanced TDLC channel with different number of beams

To verify the performance of our proposed enhanced TDLC channel, we did some simulations for Rank4 and Rank 8 respectively, in the simulation, MCS13 was used, 4T4R for Rank4 and 8T8R for Rank8 were simulated.

Figure 2-2: Simulation results for enhanced TDLC channel with multi beams

For Rank4, TDLC300-XPL-High, TDLC300-XPL-Low, enhanced TDLC channel with 2beams and 3 beams are simulated. It can be observed that for TDLC300-XPL-High, due to the high rank deficiency, 70% of maxTP is unachievable, TDL300-XPL-Low has best performance since low correlation has spatial isotropic characters. Enhanced TDLC channel with 2 beams and 3beams have similar performance and much better performance than TDL300-XPL-High, which means spatial freedom of degree is highly increased when multi-beams are implemented. It can be concluded that for TDLC300, 2 beams enough to check the 4 layers performance under spatial channel model.

For Rank8, TDLC300-XPL-High, TDLC300-XPL-Low and enhanced TDLC channel with 3beams, 4beams and 6beams were simulated. Similar observation as rank4, for TDLC300-XPL-High, due to the high rank deficiency, 70% of maxTP is unachievable, TDL300-XPL-Low has best performance since low correlation has spatial isotropic characters, for enhanced TDLC300 channel, performance will be better when more beams are implemented. It is predicted that performance will converge on low correlation when more beams are introduced. It can be concluded that for TDLC300, implementation with 4 beams is enough to check performance with 8 layers under spatial channel model.

Figure 2-3 shows the performance for two different codewords with Rank8, 8T8R, it can be observed that there is performance difference between two different codewords.

Figure 2-3: Simulation results for two codewords with Rank8

Based on above analysis, the proposed enhanced TDL channel with multi-beams can solve the problems with rank deficiency. Furthermore, each beam corresponds to different tap(s) with different delay, which is more practical than the multi-clusters methodology proposed in [3] and easier to be implemented than per-tap per correlation methodology proposed in [2]. Hence, this method could be one of candidate solutions for R19 SCM.

Proposal 1: RAN4 to introduce enhanced TDL channel with multi-beams as one of candidate solutions for R19 SCM.

Based on our initial evaluation, we propose the following:

Proposal 2: For enhanced TDL channel, RAN4 to use following configurations for simulation:

4T4R,4Layers: Enhanced TDLC300-100, XPL, high with 2 Tx-Rx beams as defined in Table 2-1

8T8R, 8Layers: Enhanced TDLC300-100, XPL, high with 4 Tx-Rx beams as defined in Table 2-3 and 6 Tx-Rx beams as defined in Table 2-4

PMI test: Enhanced TDLC300-100, XPL, high with 2 Tx-Rx beams as defined in Table 2-1

- CDL based approach

SNR definition

We have following agreement on SNR definition for CDL channel:

#### SNR Definition with CDL Agreement: Define normalised receive signal power as the following: The sum of signal power is normalised across all Rx RF chains using a single normalisation factor. The normalisation factor is calculated considering the long term average PDP for the spatial CDL channel and AAV [spatial response/combining gain]. The normalisation factor will be aligned and can be included in TR 38.753.

The agreement is very general, to be intuitive, the normalisation factor should be formulated so it would be more beneficial to align the implementation of CDL channel across companies.  Assume H(k,t)∈CNR×NTis channel coefficient in at subcarrier k, OFDM symbol l. NRx is number of Rx and  NTx is number of Tx ports. The normalization factor should be:

C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2}

Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation. Given the above normalization factor, the average power at each Tx-Rx channel coefficient is 1. I.e. The average power at each Rx is 1, which reaches the same condition as TDL channel. Our measurement results for normalized factor are shown in Table 8-1

Proposal 3: RAN4 to use following as normalization factor:

C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2}

Where H(k,t)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and  NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.

Left over issues for CDL channel

It should be mentioned that there are so many left issues for CDL channel to be discussed:

The most important precondition for rationality of CDL channel is alignment of target SNR and normalization factor. RAN4 confirms the rationality of CDL channel only if both target SNR and normalization factor are aligned across all companies.

Proposal 4: RAN4 confirms the rationality of CDL channel only if both target SNR and normalization factor are aligned.

CDL parameters table defined in 38.827 are only available for specific scenarios. For example, CDL-UMa-C is only available for TDD mode, 3.5GHz carrier frequency. However, RAN4 demodulation and CSI cases are defined as band agnostic, hence, it is necessary to exploit a new CDL parameter table which can be applicable for all bands.

Proposal 5: RAN4 demodulation experts are proposed to exploit new CDL parameter table which can be available for both FDD and TDD.

Angular information for CDL channel is fixed, which conflicts the traditional PMI test setup that beam is steering during the test. That causes the problem that fixed PMI may outperform closed-loop MIMO. Hence RAN4 shall introduce a new test setup which makes optimal PMI change during the test like beam steering approach for TDL channel.

Proposal 6: RAN4 shall introduce a new test setup which makes optimal PMI change during the test like beam steering approach for TDL channel.

CDL-UMa-C channel has 23taps with delay resolution higher than 5ns, which exceeds TE capability as discussed in Rel-15, RAN4 shall discuss how to simplify the existing CDL channel model to reduce the number of clusters like RAN4 did for simplified TDL in 38.101-4.

Proposal 7: RAN4 shall discuss how to simplify the existing CDL channel model to reduce the number of clusters like RAN4 did for simplified TDL in 38.101-4.

CDL vs TDL

From baseband performance perspective, the proposed enhanced TDL channel has all spatial component related characteristics of CDL channels. Based on our simulation results shown in clause 4, for both PDSCH tests and PMI tests, enhanced TDL channel has comparable performance with CDL channel. Furthermore, compared to CDL channel, enhanced TDL channel is simpler to be implemented and easier for simulation alignment. Hence, it can be concluded that enhanced TDL is a good substitute of CDL channel in terms of spatial channel model.

Observation 1: Enhanced TDL channel has all spatial component related characteristics of CDL channel from demodulation perspective and comparable performance with CDL channel.

Observation 2: Enhanced TDL channel is simper to be implemented and easier for simulation alignment.

Proposal 8: Capture the following conclusion in TR: Enhanced TDL is a good substitute of CDL channel in terms of spatial channel model.

- Conclusion

This contribution provides our views on spatial channel model, the proposals are:

Proposal 1: RAN4 to introduce enhanced TDL channel with multi-beams as one of candidate solutions for R19 SCM.

Proposal 2: For enhanced TDL channel, RAN4 to use following configurations for simulation:

4T4R,4Layers: Enhanced TDLC300-100, XPL, high with 2 Tx-Rx beams as defined in Table 2-1

8T8R, 8Layers: Enhanced TDLC300-100, XPL, high with 4 Tx-Rx beams as defined in Table 2-3 and 6 Tx-Rx beams as defined in Table 2-4

PMI test: Enhanced TDLC300-100, XPL, high with 2 Tx-Rx beams as defined in Table 2-1

Proposal 3: RAN4 to use following as normalization factor:

C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2}

Where H(k,t)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and  NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.

Proposal 4: RAN4 confirms the rationality of CDL channel only if both target SNR and normalization factor are aligned.

Proposal 5: RAN4 demodulation experts are proposed to exploit new CDL parameter table which can be available for both FDD and TDD.

Proposal 6: RAN4 shall introduce a new test setup which makes optimal PMI change during the test like beam steering approach for TDL channel.

Proposal 7: RAN4 shall discuss how to simplify the existing CDL channel model to reduce the number of clusters like RAN4 did for simplified TDL in 38.101-4.

Observation 1: Enhanced TDL channel has all spatial component related characteristics of CDL channel from demodulation perspective and comparable performance with CDL channel.

Observation 2: Enhanced TDL channel is simper to be implemented and easier for simulation alignment.

Proposal 8: Capture the following conclusion in TR: Enhanced TDL is a good substitute of CDL channel in terms of spatial channel model.

- Reference

[1]  R4-2419782 Way Forward for [113][320] NR_SCM