3GPP TSG-RAN WG4 Meeting #114	  R4-2501532

Athens, Greece, February 17 – 21, 2025

Agenda Item:	7.20.3

Source:	Huawei, HiSilicon

Title:	Discussion on testing issues for CSI compression two-sided models

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #113 [1], we discuss the remaining issues related to the RRM core requirement for AIML positioning.

# Feasibility verification of potential testing Option 3

RAN4 #112 WF [2]: Option 3 next steps - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

According to RAN4 previous agreement, companies provide simulation results under a common model structure using their own training dataset. It is expected that after a round of parameters refinement, good alignment can be achieved. However, only the intermediate KPI SGCS is collected and the criteria of performance alignment has not been determined yet. Since the eventual goal is to verify whether RAN4 can achieve an aligned AI model for defining requirement of AI-CSI compression, the model that we are trying to align should fulfill the baseline requirement regarding to RAN4 test metric. Otherwise, even if we can achieve an aligned AI model, the feasibility can still not be justified because the aligned model cannot be used for requirement definition. Therefore, we suggest to consider not only the SGCS but also the throughput performance gain related alignment.

Proposal 1: Take throughput performance gain into account for defining the criteria of overall feasibility for potential testing options.

We understand that at current practice stage, SGCS alignment is much easier than throughput alignment, since performance alignment of R16 Type-II among companies has not been achieved yet. From progress perspective, it is suggested to go with SGCS alignment first then to consider throughput alignment.

Proposal 2: Two steps are identified for defining the performance alignment.

- Step 1: The SGCS margin is within x

  - FFS: the value of x

- Step 2: The throughput margin is within y

  - FFS: the value of y

RAN4 #113 WF [1]: Issue 1-2: Option 3 next step - decoder(s) selection Agreement: choose the decoders with Low, medium, high SGCS among the decoders submitted by companies Continue the feasibility study on the two tracks below: Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

Currently, the performance alignment discussion is skipped. Companies are encouraged to share both encoders and decoders via MCC platform in ONNX version.

According to RAN4 #113 WF [1], decoders with Low, medium, high SGCS among the decoders submitted by companies are selected for Step 4 performance cross checking. Based on the agreements, the decoders from Samsung, Huawei, Hisilicon and CATT were chosen.

Two tracks for following feasibility verification steps are identified in RAN4. For track 1, the training dataset is generated by each company, which means that the dataset is not aligned among companies. For track 2, the training dataset is generated by mixing the dataset from different companies, which means that the dataset is aligned among companies.

Assuming an encoder with the agreed model structure, we provide the simulation results both in track 1 and track 2 in Table 1.

Proposal 3: SGCS performance for Track 1 and Track 2 with agreed model structure are provided in Table 1.

Table 1. SGCS results for Track 1 and Track 2 with agreed model structure

|  | Track 1: Training dataset @ Huawei, Hisilicon |  |  | Track 2: Training dataset @ mixed 700K |
| --- | --- | --- | --- | --- |
|  | w/o training | Fixed decoder, train encoder | Fixed encoder, train decoder | w/ training |
| Huawei, Hisilicon | 0.69 | 0.69 | 0.69 | 0.69 |
| Samsung (Q-unaware) | 0.5327 | 0.5736 | 0.6570 | - |
| CATT (Q-aware) | 0.1460 | 0.4752 | 0.4798 | - |
| CATT (Q-unaware) | 0.1423 | 0.4121 | 0.3887 | - |

It is observed that relatively acceptable performance is achieved in Track 2. In Track 1, the SGCS performance is low especially against decoder with lowest performance from CATT. We analysed that it may result from the difference of training dataset distribution between Huawei, Hisilicon and CATT.

Observation 1: SGCS performance in Track 1 is low, which may result from the difference of training dataset distribution among companies.

In the following RAN4 meeting, one or more decoders are suggested to be selected. Companies can develop encoders/decoders to check encoder/decoder performance with own dataset checked against encoders/decoder(s).

Proposal 4: In track 2, RAN4 to select one or more encoders/decoders for companies to develop decoders/encoders and to check the performance with own dataset.

It is agreed in R18 38.843 that the assumption of encoder should consider the complexity limitation at UE. However, the complexity of agreed CNN model structure at the feasibility study stage is much higher than that of R16 eType-II, as shown in Table 2.

Table 2: Complexity of MLP, CNN and Transformer

|  | MLP | CNN | Transformer |
| --- | --- | --- | --- |
| AI | 6 MFLOPs | 94 MFLOPs | 23MFLOPs |
| R16 Type-II | 0.37 MFLOPs |  |  |

We understand that for reducing the workload in the feasibility verification stage, the same model structures of encoder and decoder are assumed. While for defining requirements, there is no need to follow this design. For example, we can use MLP for encoder, while CNN/Transformer for decoder.

Proposal 5: Different model structures of encoder and decoder can be considered for reference model design, e.g., MLP for encoder while Transformer for decoder.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: Take throughput performance gain into account for defining the criteria of overall feasibility for potential testing options.

Proposal 2: Two steps are identified for defining the performance alignment.

- Step 1: The SGCS margin is within x

  - FFS: the value of x

- Step 2: The throughput margin is within y

  - FFS: the value of y

Proposal 3: SGCS performance for Track 1 and Track 2 with agreed model structure are provided in Table 1.

Table 1. SGCS results for Track 1 and Track 2 with agreed model structure

|  | Track 1: Training dataset @ Huawei, Hisilicon |  |  | Track 2: Training dataset @ mixed 700K |
| --- | --- | --- | --- | --- |
|  | w/o training | Fixed decoder, train encoder | Fixed encoder, train decoder | w/ training |
| Huawei, Hisilicon | 0.69 | 0.69 | 0.69 | 0.69 |
| Samsung (Q-unaware) | 0.5327 | 0.5736 | 0.6570 | - |
| CATT (Q-aware) | 0.1460 | 0.4752 | 0.4798 | - |
| CATT (Q-unaware) | 0.1423 | 0.4121 | 0.3887 | - |

Observation 1: SGCS performance in Track 1 is low, which may result from the difference of training dataset distribution among companies.

Proposal 4: In track 2, RAN4 to select one or more encoders/decoders for companies to develop decoders/encoders and to check the performance with own dataset.

Proposal 5: Different model structures of encoder and decoder can be considered for reference model design, e.g., MLP for encoder while Transformer for decoder.

# References

- R4-2420513, “WF on AI/ML air interface part 1”, 3GPP RAN4 #113, Orlando, USA 18th ‒ 22nd November, 2024.

- R4-2410570, “WF on AI/ML”, 3GPP RAN4 #112, May 20th – 24th, 2024.