3GPP TSG-RAN4 Meeting #114	R4-2501573

# Athens, Greece, February 17- February 21, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2677 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Harmonic Mixing UL allocations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R18_2BDL_xBUL-Core, HPUE_FR1_TDD_NR_CADC_SUL_R18, HPUE_FR1_FDD_NR_CADC_R18 |  |  |  |  |  | Date: |  |  | 2025-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A few bugs in R18 UL BW’s were discovered which should have been modified during R18 clean-up |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting misaligned allocations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalingment remains |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n1053 | 5 | 15 | 25 | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n2 | n713 | 5 | 15 | 25 | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n2 | n713 | 5 | 15 | 25 | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| n3 | n5 | 5 | 15 | 25 | 5 | 4 | NOTE 7 | UL1/DL2 |
| n3 | n26 | 5 | 15 | 25 | 5 | 3.7 | NOTE 7 | UL1/DL2 |
| n7 | n263 | 5 | 15 | 25 | 5 | 2.0 | NOTE 10 | UL1/DL3 Near miss |
| n7 | n71 | 5 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n25 | n41 | 5 | 15 | 6 | 10 | 1.3 | NOTE 11 | UL4/DL3 |
| n25 | n713 | 5 | 15 | 25 | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n25 | n713 | 5 | 15 | 25 | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| n39 | n41 | 5 | 15 | 6 | 10 | 9.3 | NOTE 11 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 6 | 100 | 2.2 | NOTE 11 | UL4/DL3 |
| n40 | n203 | 10 | 15 | 25 | 5 | 27.8 | NOTE 4 | UL1/DL3 |
| n40 | n203 | 10 | 15 | 25 | 20 | 20.6 | NOTE 4 | UL1/DL3 |
| n40 | n283 | 10 | 15 | 25 | 5 | 37.8 | NOTE 4 | UL1/DL3 |
| n40 | n283 | 10 | 15 | 25 | 20 | 30.1 | NOTE 4 | UL1/DL3 |
| n41 | n53 | 10 | 15 | 25 | 5 | 24.3 | NOTE 4 | UL1/DL3 |
| n41 | n183 | 10 | 15 | 25 | 5 | 26.3 | NOTE 4 | UL1/DL3 |
| n41 | n183 | 10 | 15 | 25 | 15 | 21.3 | NOTE 4 | UL1/DL3 |
| n41 | n39 | 10 | 15 | 8 | 5 | 4.3 | NOTE 12 | UL3/DL4 |
| n41 | n39 | 10 | 15 | 8 | 40 | 0.8 | NOTE 12 | UL3/DL4 |
| n41 | n48 | 10 | 15 | 6 | 5 | 12.3 | NOTE 9 | UL4/DL3 |
| n41 | n48 | 10 | 15 | 6 | 100 | 2.3 | NOTE 9 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 10 | 8.9 | NOTE 9 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 100 | 2.1 | NOTE 9 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 10 | 9.3 | NOTE 9 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 100 | 2.3 | NOTE 9 | UL4/DL3 |
| n46 | n7 | 20 | 15 | 25 | 5 | 10.9 | NOTE 7 | UL1/DL2 |
| n46 | n7 | 20 | 15 | 25 | 50 | 1 | NOTE 7 | UL1/DL2 |
| n46 | n48 | 20 | 15 | 12 | 5 | 26.8 | NOTE 2 | UL2/DL3 |
| n46 | n48 | 20 | 15 | 12 | 100 | 13.5 | NOTE 2 | UL2/DL3 |
| n46 | n77 | 20 | 15 | 12 | 10 | 20.6 | NOTE 2 | UL2/DL3 |
| n46 | n77 | 20 | 15 | 12 | 100 | 10.6 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 20 | 15 | 12 | 10 | 21.1 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 20 | 15 | 12 | 100 | 11.1 | NOTE 2 | UL2/DL3 |
| n48 | n12 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n48 | n12 | 10 | 15 | 25 | 15 | 18 | NOTE 5 | UL1/DL5 |
| n48 | n26 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4 |
| n48 | n26 | 10 | 15 | 25 | 20 | 1 | NOTE 8 | UL1/DL4 |
| n48 | n29 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n48 | n29 | 10 | 15 | 25 | 10 | 27.8 | NOTE 5 | UL1/DL5 |
| n77 | n2 | 10 | 15 | 25 | 5 | 6.7 | NOTE 7 | UL1/DL2 |
| n77 | n2 | 10 | 15 | 25 | 20 | 2.8 | NOTE 7 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 5.7 | NOTE 7 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 20 | 2.2 | NOTE 7 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 0.8 | NOTE 8 | UL1/DL4 |
| n77 | n7 | 10 | 15 | 12 | 5 | 14.7 | NOTE 2 | UL2/DL3 |
| n77 | n7 | 10 | 15 | 12 | 50 | 2.2 | NOTE 2 | UL2/DL3 |
| n77 | n8 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n77 | n8 | 10 | 15 | 25 | 20 | 0.8 | NOTE 8 | UL1/DL4 |
| n77 | n12 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n12 | 10 | 15 | 25 | 15 | 18 | NOTE 5 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 10 | 27.8 | NOTE 5 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 10 | 27.8 | NOTE 5 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 | 5 | 5.6 | NOTE 7 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 40 | 0.3 | NOTE 7 | UL1/DL2 |
| n77 | n26 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4 |
| n77 | n28 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 30 | 11.4 | NOTE 5 | UL1/DL5 |
| n776 | n29 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n776 | n29 | 10 | 15 | 25 | 10 | 27.8 | NOTE 5 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 | 5 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 10 | 7.6 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 11.7 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 100 | 3.6 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 11.7 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 100 | 3.6 | NOTE 2 | UL2/DL3 |
| n776 | n70 | N/A | N/A | N/A | N/A | N/A | NOTE 7 | UL1/DL2 |
| n77 | n85 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n85 | 10 | 15 | 25 | 15 | 18 | NOTE 5 | UL1/DL5 |
| n78 | n2 | 10 | 15 | 25 | 5 | 6.7 | NOTE 7 | UL1/DL2 |
| n78 | n2 | 10 | 15 | 25 | 20 | 2.8 | NOTE 7 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 5 | 5.7 | NOTE 7 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 20 | 2.2 | NOTE 7 | UL1/DL2 |
| n78 | n5 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n78 | n12 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n78 | n26 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4 |
| n78 | n28 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n78 | n40 | 10 | 15 | 12 | 10 | 11.7 | NOTE 2 | UL2/DL3 |
| n78 | n40 | 10 | 15 | 12 | 100 | 3.6 | NOTE 2 | UL2/DL3 |
| n78 | n41 | 10 | 15 | 12 | 10 | 11.7 | NOTE 2 | UL2/DL3 |
| n78 | n41 | 10 | 15 | 12 | 100 | 3.6 | NOTE 2 | UL2/DL3 |
| n78 | n67 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n78 | n67 | 10 | 15 | 25 | 10 | 27.8 | NOTE 5 | UL1/DL5 |
| n79 | n5 | 10 | 15 | 25 | 5 | 27.5 | NOTE 5 | UL1/DL5 |
| n79 | n8 | 10 | 15 | 25 | 5 | 25 | NOTE 5 | UL1/DL5 |
| n96 | n48 | 20 | 15 | 25 | 5 | 31 | NOTE 7 | UL1/DL2 |
| n96 | n48 | 20 | 15 | 25 | 100 | 17.5 | NOTE 7 | UL1/DL2 |
| n102 | n13 | 20 | 15 | 25 | 5 | 30 | NOTE 4 | UL1/DL3 |
| n104 | n78 | 20 | 15 | 25 | 10 | 29 | NOTE 1 | UL1/DL2 |
| n104 | n78 | 20 | 15 | 25 | 100 | 18.8 | NOTE 1 | UL1/DL2 |
| NOTE 1: Void. NOTE 2: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 6: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 9: The requirements should be verified for DL NR-ARFCN of the victim (higher) band (superscript HB) such that fDLHB=fULLB/0.75 and FUL_lowLB+BWChannelLB/2≤fULLB≤FUL_highLB-BWChannelLB/2 with fULLB the UL carrier frequency and BWChannelLB the channel bandwidth configured in the lower band, both in MHz. NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band NOTE 11: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 12: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 13: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n25 | n41 | 5 | 15 | 6 | 10 | 2.213 3.214 | NOTE 11 | UL4/DL3 |
| n25 | n713 | 5 | 15 | 25 | 5 | 29.513 34.514 | NOTE 4 | UL1/DL3 |
| n25 | n713 | 5 | 15 | 25 | 20 | 18.213 23.314 | NOTE 4 | UL1/DL3 |
| n39 | n41 | 5 | 15 | 6 | 5 | 10.6 | NOTE 8 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 6 | 100 | 4.9 | NOTE 8 | UL4/DL3 |
| n40 | n28 | 10 | 15 | 25 | 5 | 40.8 | NOTE 7 | UL1/DL3 |
| n40 | n28 | 10 | 15 | 25 | 20 | 33.3 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 | 5 | 29.3 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 | 15 | 24.3 | NOTE 7 | UL1/DL3 |
| n41 | n39 | 10 | 15 | 8 | 5 | 6.1 | NOTE 9 | UL3/DL4 |
| n41 | n39 | 10 | 15 | 8 | 40 | 1.5 | NOTE 9 | UL3/DL4 |
| n41 | n77 | 10 | 15 | 6 | 10 | 11.6 | NOTE 8 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 100 | 3.6 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 10 | 12.1 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 100 | 3.8 | NOTE 8 | UL4/DL3 |
| n77 | n2 | 10 | 15 | 25 | 5 | 9.2 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 10 | 15 | 25 | 20 | 4.4 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n77 | n7 | 10 | 15 | 12 | 5 | 17.7 | NOTE 3 | UL2/DL3 |
| n77 | n7 | 10 | 15 | 12 | 50 | 3.6 | NOTE 3 | UL2/DL3 |
| n77 | n8 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n8 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n77 | n12 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n12 | 10 | 15 | 25 | 15 | 21 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 | 5 | 7.9 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 40 | 0.5 | NOTE 4 | UL1/DL2 |
| n77 | n28 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 30 | 14.1 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 | 5 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 10 | 10.2 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n772 | n70 | N/A | N/A | N/A | N/A | N/A | NOTE 4 | UL1/DL2 |
| n77 | n85 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n85 | 10 | 15 | 25 | 15 | 21 | NOTE 1 | UL1/DL5 |
| n78 | n3 | 10 | 15 | 25 | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n78 | n5 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n5 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n78 | n26 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n28 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 10 | 15 | 25 | 30 | 11.3 | NOTE 1 | UL1/DL5 |
| n78 | n40 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n78 | n40 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n79 | n8 | 10 | 15 | 25 | 5 | 28.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 6: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: Void NOTE 11: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 12: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 13: Applicable to UE’s supporting PC2 with 1Tx NOTE 14: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a-2: Void

Table 7.3A.4-4b: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n40 | n28 | 10 | 15 | 25 | 5 | 43.8 | NOTE 7 | UL1/DL3 |
| n40 | n28 | 10 | 15 | 25 | 20 | 36.3 | NOTE 7 | UL1/DL3 |
| n41 | n39 | 10 | 15 | 8 | 5 | 10.5 | NOTE 6 | UL3/DL4 |
| n41 | n39 | 10 | 15 | 8 | 40 | 3.0 | NOTE 6 | UL3/DL4 |
| n41 | n77 | 10 | 15 | 6 | 10 | 14.5 | NOTE 8 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 100 | 5.5 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 10 | 14.9 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 100 | 5.8 | NOTE 8 | UL4/DL3 |
| n77 | n2 | 10 | 15 | 25 | 5 | 11.8 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 10 | 15 | 25 | 20 | 6.5 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 10.7 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 40 | 1.4 | NOTE 4 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 | 5 | 10.7 | NOTE 5 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 2.6 | NOTE 5 | UL1/DL4 |
| n77 | n7 | 10 | 15 | 12 | 10 | 17.5 | NOTE 3 | UL2/DL3 |
| n77 | n7 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n77 | n12 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n12 | 10 | 15 | 25 | 15 | 24 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 10 | 33.8 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 10 | 33.8 | NOTE 1 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 | 5 | 10.6 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n77 | n28 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 30 | 17 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 10 | 33.8 | NOTE 1 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 | 5 | 16.1 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 10 | 13 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 17.5 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 100 | 7.9 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 17.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 100 | 7.9 | NOTE 3 | UL2/DL3 |
| n772 | n70 | N/A | N/A | N/A | N/A | N/A | NOTE 4 | UL1/DL2 |
| n77 | n85 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n85 | 10 | 15 | 25 | 15 | 24 | NOTE 1 | UL1/DL5 |
| n78 | n3 | 10 | 15 | 25 | 5 | 10.7 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 40 | 1.4 | NOTE 4 | UL1/DL2 |
| n78 | n8 | 10 | 15 | 25 | 5 | 10.6 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 20 | 2.6 | NOTE 5 | UL1/DL4 |
| n78 | n26 | 10 | 15 | 25 | 5 | 10.1 | NOTE 5 | UL1/DL4 |
| n78 | n28 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 10 | 15 | 25 | 30 | 17 | NOTE 1 | UL1/DL5 |
| n78 | n40 | 10 | 15 | 12 | 10 | 17.5 | NOTE 3 | UL2/DL3 |
| n78 | n40 | 10 | 15 | 12 | 100 | 7.9 | NOTE 3 | UL2/DL3 |
| n79 | n8 | 10 | 15 | 25 | 5 | 31.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 6: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for DL NR-ARFCN of the victim (higher) band (superscript HB) such that fDLHB=fULLB/0.75 and FUL_lowLB+BWChannelLB/2≤fULLB≤FUL_highLB-BWChannelLB/2 with fULLB the UL carrier frequency and BWChannelLB the channel bandwidth configured in the lower band, both in MHz. NOTE 9: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. |  |  |  |  |  |  |  |  |

---End of changes---