3GPP TSG-RAN WG4#114	R4-2501575

Athens, Greece, February 17th – 21st, 2025

Agenda item:	6.8.4

Source: 	Qualcomm France

Title: 	CA_n2A-n77A and CA_n14A-n77A PC2 MSD

Document for:	Approval

# Introduction

MSD analysis for CA_n2A-n77A and CA_n14A-n77A PC2 are provided in this contribution.

- Discussion

These combinations have UL harmonics MSD specified for PC3, with the following requirements.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n2 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 100 | 2.9 | NOTE 5 | UL5/DL1 direct-hit |

We calculated PC2 1Tx and 2Tx MSD using the PC3 MSD’s as reference. We assumed 6dB imbalance between PRX and DRX interference, and calculated the interferences which lead to PC3 MSD’s. For PC2 1 TX case, we added 3dB into interferences and calculated respective PC1 1 TX MSD. For PC2 2 TX case, we added 0.5dB into PC3 interferences and calculated the interference for PRX and DRX assuming both branches are interfered by own TX interference and interference coupled from other TRX path.

We note that the scaling used for PC3 MSD’s for different BW’s assumed 0dB imbalance between PRX and DRX, but because in recent HPUE MSD discussion larger imbalance was considered, we decided to use 6dB for PC2 which basically means the interference for 10MHz and 100MHz PC3 is a tad different.

The results look as follows.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n2 | n77 | 5 | 15 | 12 | 10 | 26.9 | 29.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 12 | 100 | 16.6 | 19.4 | NOTE 2 | UL2/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 10 | 13 | 15.6 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 100 | 4.4 | 5.6 | NOTE 5 | UL5/DL1 direct-hit |

- Conclusion

MSD analysis for CA_n12A-n77A and CA_n14A-n77A PC2 are provided in this contribution.

Proposal 1: Use the following data for CA_n2-n77 and CA_n14-n77 PC2 MSD

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n2 | n77 | 5 | 15 | 12 | 10 | 26.9 | 29.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 12 | 100 | 16.6 | 19.4 | NOTE 2 | UL2/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 10 | 13 | 15.6 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 5 | 100 | 4.4 | 5.6 | NOTE 5 | UL5/DL1 direct-hit |

- Reference