# 3GPP TSG-RAN WG4 Meeting #114  		R4-2501593

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on maintenance of RRM requirements for R18 NR_Mob_enh2

Agenda Item:	5.23.1

Document for:	Discussion

Introduction

In RAN4 #113, the WF and LSs to RAN2 [1], [2], [3] are approved. Moreover, the following agreement is captured in the chairman notes.

Agreement: Conclude FG 39-3-3 and FG 39-3-5 in Issue 1-4-3 (Clarification on LTM L1-RSRP related capabilities) in the next meeting.

Additionally, in this meeting, one LS from RAN1 is received [4].

Based on all above information, we provide our views on maintenance of RRM requirements for R18 NR_Mob_enh2.

Discussion on core part maintenance

<On clarification on serving cell L1 measurement in LTM>

In RAN4 #113, the follow issues are discussed and the agreements are achieved as follows.

Issue 1-2-1: Spec organization < Agreement > Conclude the issue in RAN4#114: - Option 1 (vivo): - Although same UE behaviour on L1-RSRP measurement for current serving cell is assumed from RRM requirements perspective, no matter it is configured by either CSI-ResourceConfig under csi-MeasConfig or LTM-CSI-ResourceConfig-r18 under ltm-Config-r18, it is still proposed to capture LTM L1 measurement on serving cell in clause 9.14 of TS 38.133, so as to better differentiate UE support of either rxTimingDiff-r18 or multiCellL1-measRTD-greaterThan-CP-r18. - To better capture UE capabilities related to the max number of layers/cells/resource UE can measure under LTM configurations, LTM related serving cell L1 measurement shall be captured under 9.14. - Capture all intra-frequency LTM L1 measurement requirements, including serving cell L1 measurement requirements if counted as LTM L1 measurement, in 9.14. The impact to serving cell L1 measurement, which is configured by CSI-ResourceConfig, due to Rx beam sharing with neighbour cell LTM L1 measurement is captured by PL1_sharing in 9.5.4.1. Issue 1-4-3: Clarification on LTM L1-RSRP related capabilities < Agreement > For RAN4 39-3-6: - ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all SSBs from serving cells, including the serving cells that are not configured with LTM L1 measurements, i.e. SSB resources configured by CSI-MeasConfig are also counted.

In our view, these two issues are co-related to each other. As discussed in [5], RAN4 shall clarify the UE behaviour for LTM L1 measurement. Based on discussion in last meeting and the agreement achieved above, our understanding is that UE performs L1 measurement in a similar way, no matter whether the L1 measurement resources are configured under csi-MeasConfig or under ltm-Config-r18.

Observation 1  Based on RAN4 #113 agreements on RAN4 UE FG 39-3-6, most UE vendors prefer to perform LTM L1 measurement, of which the measurement resources are configured under ltm-Config-r18, in a unified manner with legacy L1 measurement, of which the measurement resources are configured under csi-MeasConfig.

Based on observation 1, we would like to further confirm the other 3 UE FGs for LTM L1 measurement which shares the same issue, as discussed in [5]. The current TS 38.306 is cited as follow.

| maxFreqLayersL1-Meas-r18 (R4 39-3-1) Indicates the number of frequency layers for L1-RSRP measurement. This capability signalling comprises of the following parameters: - supportedMaxIntraInterFreqLayersWithoutGaps-r18 indicates the maximum number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 and/or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxIntraInterFreqLayersWithGaps-r18 indicates the maximum number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| maxNeighCellsPerFreqLayerL1-Meas-r18 (R4 39-3-2) Indicates the number of neighbouring cells per frequency layer for L1-RSRP measurement. This capability signalling comprises of the following parameters: - supportedMaxNeighCellsPerFreqLayersWithoutGaps-r18 indicates the max number of neighbour cells UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxNeighCellsPerFreqLayersWithGaps-r18 indicates the max number of neighbour cells UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| supportedMaxCellsWithoutGapsL1-Meas-r18 (R4 39-3-3) Indicates the max number of total cells of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. | BC | No | N/A | N/A |
| supportedMaxSSB-WithinSlotL1-Meas-r18 (R4 39-3-4) Indicates the max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. | BC | No | N/A | N/A |
| maxSSB-PerFreqLayerL1-Meas-r18 (R4 39-3-5) Indicates the maximum number of SSB resources for L1-RSRP measurement per frequency layer UE can measure. This capability signalling comprises of the following parameters: - supportedMaxSSB-PerFreqLayersWithoutGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxSSB-PerFreqLayersWithGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| supportedMaxSSB-L1-Meas-r18 (R4 39-3-6) Indicates the max number of total SSB resources of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. | BC | No | N/A | N/A |

For RAN4 FG 39-3-1, regarding intra-frequency L1 measurements, our understanding is that here the number of SSB frequency layers does NOT only count the frequency layers where LTM L1 measurement are configured, based on observation 1 above. All serving cell SSB frequency layers, including SSB frequency layers that are not configured with any LTM L1 measurements, shall be counted.

Proposal 1  RAN4 to clarify that in RAN4 FG 39-3-1 maxFreqLayersL1-Meas-r18, serving SSB frequency layers configured without LTM L1 measurements are also counted, i.e. it also counts serving SSB frequency layers on which only legacy L1 measurements configured by CSI-ResourceConfig under csi-MeasConfig, are to be performed.

For RAN4 FG 39-3-3, it considers both serving cell and neighbour cells. Following the logic of observation 1, for CA scenario, SSB frequency layers of all serving cells, including the SSB frequency layer without LTM L1 measurements, are counted.

Proposal 2  RAN4 to clarify that in RAN4 FG 39-3-3 supportedMaxCellsWithoutGapsL1-Meas-r18, ‘total cells of serving cells and neighbouring cells’ counts both LTM candidate cell(s) and all serving cells, i.e. the serving cells that are not configured with intra-f LTM L1 measurements or inter-f LTM L1 measurements without gaps are also counted.

For RAN4 FG 39-3-5, the same logic shall also be followed.

Proposal 3  RAN4 to clarify that in RAN4 FG 39-3-5 maxSSB-PerFreqLayerL1-Meas-r18, ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes both LTM candidate cell(s) and some SSBs from serving cells.

Based on above 3 proposal, it would be good if RAN4 clarifies this understanding with LS to RAN2. In our view, to avoid confusion, RAN2 shall take actions in revising the corresponding descriptions in TS 38.306.

Proposal 4  RAN4 informs RAN2 about the clarifications of RAN4 FG 39-3-1, 39-3-3, 39-3-5, 39-3-6, by sending one LS to RAN2.

A draft of the LS is provided in the annex.

For R18 MIMO, RAN1 has introduced the following UE feature.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-2-6 | Rx timing difference larger than CP length | 1. Support of the Rx timing difference between the two DL reference timings is larger than CP length |  | yes | Rx timing difference larger than CP is not supported | Per FSPC | n/a | n/a | n/a |  | Optional with capability signaling |

All requirements captured under 9.5 and 9.13 shall follows the requirement applicability rules defined in 9.5.3 and 9.13.3, respectively, and RTD rule is defined based on whether UE supports above UE capability or not. The motivation behind this FG 40-2-6 rxTimingDiff-r18 is to allow UE to indicate the support of L1 measurement for RTD>CP case, before 2-TA based m-TRP is configured by gNB. This L1 measurement is supposed to be configured by CSI-ResourceConfig under csi-MeasConfig. RAN4 have also introduced maximum RTD UE may assume for this case, under clause 7.6.8 of TS 38.133. From RRM requirement perspective, although there is no yet pre-requisite UE capability defined for this FG in RAN1 feature list, UE supporting either multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 is the pre-requisite for MRTD and MTTD requirements, as defined in 7.6.8 an 7.5.7.

Note that this FG is per-FSPC, which means it only applies to one CC of a given band in a BC.

On the other hand, for R18 Mobility, RAN4 has introduced the following UE FG.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-1 | SSB based L1-RSRP measurements for multiple cells with RTD > CP | Capability of simultaneous L1-RSRP measurements for more than one cell when the max RTD among the cells on the same frequency layer or in the same active BWP is larger than CP length of the cell on the frequency layer or in the same active BWP. | 45-1 from RAN1 Rel-18 feature list or 39-2 or 39-2a | Yes | The corresponding RAN4 requirements may not be satisfied when the max RTD among the cells on the same frequency layer or in the same active BWP is larger than CP length of the cell on the frequency layer or in the same active BWP. | Per BC | No | No | N/A |  | Optional with capability signaling |

The motivation of this UE feature multiCellL1-measRTD-greaterThan-CP-r18 is to indicate the support of LTM L1 measurement on candidate cells when the RTD>CP between cells, including serving cell if possible, are in the same frequency layer or in the same active BWP. Actually, the RTD here can be either for the cell on serving cell CC, or for cells that not covered by any active BWP at all. UE supporting the capability shall support one of intra-frequency LTM L1 measurement, inter-frequency LTM L1 measurement within gaps, or inter-frequency LTM L1 measurement outside gaps as pre-requisite.

Note that this FG is per-BC capability.

Hence, we have the following observations.

Observation 2  For R18 MIMO, rxTimingDiff-r18 is introduced as a UE capability for indicating support of RTD>CP between any two downlink timing reference signals within a CC, based on which UE will be able to perform L1-RSRP measurement so as to enable 2-TA m-TRP operation. The capability is per-FSPC, and needs pre-requisite of either multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 from RRM requirement perspective.

Observation 3  For R18 Mobility, multiCellL1-measRTD-greaterThan-CP-r18 is introduced as a UE capability for indicating support of RTD>CP between any two cells on the same frequency layer or within the same active BWP, on which UE will be able to perform L1-RSRP measurement so as to enable LTM operations including early DL/UL sync. The capability is per-BC, and needs pre-requisite of one of intraFreqL1-MeasConfig-r18, ltm-InterFreqMeasGap-r18 or interFreqSSB-L1-MeasWithoutGaps-r18.

RAN4 has never discussed the scenario where R17 ICBM and R18 LTM are configured simultaneously. Although we see the potential benefit of this scenario, based on RAN4 discussion the requirement applicability may not be applicable to this case according to existing TS 38.133.

On the other hand, R18 rxTimingDiff-r18 is also applicable for the intra-cell Multi-TRP case. However, multiCellL1-measRTD-greaterThan-CP-r18 is only applicable when RTD between cells is larger than CP, i.e. multiCellL1-measRTD-greaterThan-CP-r18 is not applicable for the intra-cell case. Therefore, in this case

Proposal 5  RAN4 to agree on one of the following options:

- Option 1: Specify RRM requirements in the maintenance phase for the case when R17/R18 L1 measurement on non-serving cell and R18 LTM L1 measurement on neighbor cell are both configured simultaneously, and clarify the applicability rules of 2 RTD capabilities during maintenance phase.

- Option 2: Clarify in TS 38.133 that RRM requirements are not applicable to the case when L1 measurement on non-serving cell and LTM L1 measurement are both configured simultaneously.

In existing clause 9.5.4.1 of TS 38.133, the RRM requirements are defined as follows.

TS 38.133 v18.8.0 9.5.4.1 SSB based L1-RSRP Reporting … When there is no intra-frequency L1-RSRP measurement on LTM neighbor cell(s) to measure, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-1 for FR1, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is not configured, and defined in Table 9.5.4.1-2A for FR2-1 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is configured, and defined in Table 9.5.4.1-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where - M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise - The value of N in Table 9.5.4.1-2 is [reducedRxBeamNum] in FR2-1 for UE supporting [faster beam switching capability], according to the conditions described in clause 3.6.x, and activated with multi-Rx operation, otherwise the value of N in Table 9.5.4.1-2 is 8. The UE is activated with multi-Rx operation when the following conditions are met: - UE is configured with group-based beam reporting (GBBR) report - The value of N in Table 9.5.4.1-2A is 8. - The value of N in Table 9.5.4.1-3 is 8. … The value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-4 for UE incapable of [capability of measurement with RTD>CP] in FR1, Table 9.5.4.1-5 for UE capable of [capability of measurement with RTD>CP] in FR1, Table 9.5.4.1-6 for UE incapable of [capability of measurement with RTD>CP] in FR2 and Table 9.5.4.1-7 for UE capable of [capability of measurement with RTD>CP] in FR2 when there is intra-frequency L1-RSRP measurement on neighbor cell(s) to measure, where - M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise - N= 8 in Table 9.5.4.1-6 and Table 9.5.4.1-7. … Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17. … Table 9.5.4.1-4: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE incapable of [capability of measurement with RTD>CP] Configuration; TL1-RSRP_Measurement_Period_SSB_intra (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: The requirements apply if the actual RTD between serving cell and neighbor cell is not larger than CP.. Table 9.5.4.1-5: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE capable of [capability of measurement with RTD>CP] Configuration; TL1-RSRP_Measurement_Period_SSB_intra (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5..

The highlighted part is somehow unclear, because we see no reason why only intra-frequency L1-RSRP measurement on LTM neighbor cell, but no inter-frequency without gap L1-RSRP measurement on LTM neighbour cell, is considered here. As discussed above, the capability multiCellL1-measRTD-greaterThan-CP-r18 also applies between serving cell and an inter-frequency LTM neighbour cell in the same active BWP. To better clarify this issue, it is proposed to revise the wording to ‘when there is L1-RSRP measurement on LTM neighbour cell to measure within active BWP’

Proposal 6  In clause 9.5.4.1 of TS 38.133, change the wording from ‘when there is intra-frequency L1-RSRP measurement on neighbor cell(s) to measure’ to ‘when there is L1-RSRP measurement on LTM neighbour cell to measure within active BWP’ to include the scenario of ‘inter-frequency without gap L1-RSRP measurement’ on LTM neighbour cells.

Discussion on perf part maintenance

<On TCI related test case coverage and Test configurations>

In existing TS 38.133, the following is captured for LTM cell switch test cases including FR1 and FR2.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B. - Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18 - Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18. - Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18. - Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18 - Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18. - Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

As in RAN1 feature list, 45-3a and 45-4a are generally not correlated to each other. However, the R17 pre-requisite for 45-3a is FG 23-1-1, while 45-4a is FG 23-10-1. In R17 RAN1 feature list, it is clearly state that FG 23-1-1 is the pre-requisite of FG 23-10-1.

Observation 4  Since R17, supporting joint TCI configuration is the pre-requisite for supporting separate TCI.

Another issue is for FR1, how joint TCI or separate TCI would be configured. Actually, there was no ever test cases in FR1 that refers to R17 joint/separate TCI configuration. In TS 38.214, we see the following restrictions for UL TCI.

TS 38.214 v18.1.0 6.1 UE procedure for transmitting the physical uplink shared channel … The UE can be configured with a list of up to 64 TCI-UL-State configurations within the higher layer parameter BWP-UplinkDedicated. Each TCI-UL-State configuration contains a parameter for configuring one reference signal, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a CC, and SRS. … When the UE is configured dl-OrJointTCI-StateList or ul-TCI-StateList, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. …

In RAN1 spec, the reference RS configured in UL TCI would be effective only if it is applicable. Normally, the common understanding is that in FR2 the ‘QCL-D’ and UL TCI would be applicable, since the spatial Rx/Tx information, i.e. UE beam information, are necessary. For FR1, the meaning of UL TCI and QCL-D is ambiguous. Different UE implementation might have different interpretation. From test case design perspective, they should not be configured in FR1 test cases.

Observation 5  According to RAN1 spec, the meaning of configuring ‘QCL-D’ and UL TCI to UE would be ambiguous in FR1.

Note that ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18 are per-band UE capabilities and normally UE report supporting of ltm-MAC-CE-SeparateTCI-r18 in a FR1 band does not expect network to configure UL TCI state. Hence allow test coverage to this case would be redundant.

Proposal 7  In RRM test case design for LTM, RAN4 avoid TCI configuration with ‘QCL-D’ or ‘UL TCI’ to UE in FR1, which means only DL or Joint TCI with QCL-A/C configuration, and pathloss RS configuration if necessary, shall be provided to UE in FR1, even if UE supports ltm-MAC-CE-SeparateTCI-r18.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  Based on RAN4 #113 agreements on RAN4 UE FG 39-3-6, most UE vendors prefer to perform LTM L1 measurement, of which the measurement resources are configured under ltm-Config-r18, in a unified manner with legacy L1 measurement, of which the measurement resources are configured under csi-MeasConfig.

Proposal 1  RAN4 to clarify that in RAN4 FG 39-3-1 maxFreqLayersL1-Meas-r18, serving SSB frequency layers configured without LTM L1 measurements are also counted, i.e. it also counts serving SSB frequency layers on which only legacy L1 measurements configured by CSI-ResourceConfig under csi-MeasConfig, are to be performed.

Proposal 2  RAN4 to clarify that in RAN4 FG 39-3-3 supportedMaxCellsWithoutGapsL1-Meas-r18, ‘total cells of serving cells and neighbouring cells’ counts both LTM candidate cell(s) and all serving cells, i.e. the serving cells that are not configured with intra-f LTM L1 measurements or inter-f LTM L1 measurements without gaps are also counted.

Proposal 3  RAN4 to clarify that in RAN4 FG 39-3-5 maxSSB-PerFreqLayerL1-Meas-r18, ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes both LTM candidate cell(s) and some SSBs from serving cells.

Proposal 4  RAN4 informs RAN2 about the clarifications of RAN4 FG 39-3-1, 39-3-3, 39-3-5, 39-3-6, by sending one LS to RAN2.

Observation 2  For R18 MIMO, rxTimingDiff-r18 is introduced as a UE capability for indicating support of RTD>CP between any two downlink timing reference signals within a CC, based on which UE will be able to perform L1-RSRP measurement so as to enable 2-TA m-TRP operation. The capability is per-FSPC, and needs pre-requisite of either multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 from RRM requirement perspective.

Observation 3  For R18 Mobility, multiCellL1-measRTD-greaterThan-CP-r18 is introduced as a UE capability for indicating support of RTD>CP between any two cells on the same frequency layer or within the same active BWP, on which UE will be able to perform L1-RSRP measurement so as to enable LTM operations including early DL/UL sync. The capability is per-BC, and needs pre-requisite of one of intraFreqL1-MeasConfig-r18, ltm-InterFreqMeasGap-r18 or interFreqSSB-L1-MeasWithoutGaps-r18.

Proposal 5  RAN4 to agree on one of the following options:

- Option 1: Specify RRM requirements in the maintenance phase for the case when R17/R18 L1 measurement on non-serving cell and R18 LTM L1 measurement on neighbor cell are both configured simultaneously, and clarify the applicability rules of 2 RTD capabilities during maintenance phase.

- Option 2: Clarify in TS 38.133 that RRM requirements are not applicable to the case when L1 measurement on non-serving cell and LTM L1 measurement are both configured simultaneously.

Proposal 6  In clause 9.5.4.1 of TS 38.133, change the wording from ‘when there is intra-frequency L1-RSRP measurement on neighbor cell(s) to measure’ to ‘when there is L1-RSRP measurement on LTM neighbour cell to measure within active BWP’ to include the scenario of ‘inter-frequency without gap L1-RSRP measurement’ on LTM neighbour cells.

Observation 4  Since R17, supporting joint TCI configuration is the pre-requisite for supporting separate TCI.

Observation 5  According to RAN1 spec, the meaning of configuring ‘QCL-D’ and UL TCI to UE would be ambiguous in FR1.

Proposal 7  In RRM test case design for LTM, RAN4 avoid TCI configuration with ‘QCL-D’ or ‘UL TCI’ to UE in FR1, which means only DL or Joint TCI with QCL-A/C configuration, and pathloss RS configuration if necessary, shall be provided to UE in FR1, even if UE supports ltm-MAC-CE-SeparateTCI-r18.

- References

[1]  R4-2420234  WF on Further NR mobility enhancements, MediaTek.  RAN4 #113

[2]  R4-2420218  Reply LS on the fast RRC processing for LTM, Ericsson.  RAN4 #113

[3]  R4-2420125  LS on UE capability for fast RRC processing for LTM, MediaTek.  RAN4 #113

[4]  R4-2500008  LS on collision between SSB and RA occasion for LTM, RAN1.  RAN4 #114

[5]  R4-2419096  Discussion on maintenance of RRM requirements for R18 NR_Mob_enh2, vivo. RAN4 #113

- Annex Draft LS

Title:	LS on RAN4 UE capabilities for LTM L1-RSRP measurements

Response to:

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- 1	Overall description

[TBD…]

RAN4 discussed the FG 39-3-6 “supportedMaxSSB-L1-Meas-r18” further and agreed that ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all SSBs from serving cells, including the serving cells that are not configured with LTM L1 measurements, i.e. SSB resources configured by CSI-MeasConfig are also counted.

- 2	Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take into account above information and revise the TS 38.306 accordingly.

- 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #114bis	            7 – 11 April 2025	Wuhan, China

TSG RAN WG4 Meeting #115	            19 – 23 May 2025	Malta