3GPP TSG-RAN4 Meeting #114	R4-2501606

# Athens, Greece, Feb 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2682 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (HPUE_NR_FR1_FDD_R18) CR for TS 38.101-1 to clarify A-MPR for NS_43 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_FR1_FDD_R18-Core |  |  |  |  |  | Date: |  |  | 2025-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In some overlapping region, PC2 A-MPR requirement is not clear. For example, for Region A and 10MHz, Full RB configuration LCRB=24 for 30kHzSCS DFT-s-OFDM 64QAM belongs to both LCRB>7.2 MHz/12/SCS(red part) and LCRB>8.1 MHz/12/SCS(bule part) as shown below. Therefore, it is not clear that whether the PC2 A-MPR requirement is A2(≤ 4.5) or A3(≤ 5) for this example. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarfied that the maximum requirement among applicable AMPR requirements is applied. For the above example, A3(≤ 5) is used for full RB configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR for NS_43 is not clear, which affects the AMPR testing in RAN5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | ? |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2.3	UE additional maximum output power reduction

<Unchanged Skipped>

6.2.3.6	A-MPR for NS_43 and NS_43U

Table 6.2.3.6-1: A-MPR regions for NS_43 (Power class 3 and 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 2.7 MHz/12/SCS | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 8.1 MHz/12/SCS | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | ≤ 1.8 MHz /12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
|  |  | ≥ 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | < 7.2 MHz/12/SCS | A7 |
|  |  |  |  |  | ≥ 6.12 MHz/12/SCS ≤ 7.2 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.6-2 and Table 6.2.3.6-2a. NOTE 2: Void NOTE 3: Void NOTE 4: For the regions where multiple AMPR requirements are defined, the maximum requirement among applicable AMPR requirements is applied. |  |  |  |  |  |  |  |

Table 6.2.3.6-2: A-MPR for NS_43 (Power class 3)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A | ≤ 9 |
|  | QPSK | ≤ 2 |  |  |  |  |  | ≤ 2.5 |  |  |  | ≤ 9 |
|  | 16 QAM |  |  |  |  |  |  |  |  | ≤ 2.5 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 2.5 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |
| CP-OFDM | QPSK | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 4 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |

Table 6.2.3.6-2a: A-MPR for NS_43 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) | A7 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A | ≤ 12 | ≤ 4 | 3 |
|  | QPSK | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 4 | 3 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 5 | 4 |
|  | 64 QAM | ≤ 3.5 |  | ≤ 4.5 |  | ≤ 5 |  |  |  |  |  | ≤ 12 | ≤ 5.5 | 5.5 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 5.5 |
| CP-OFDM | QPSK | ≤ 4 |  |  |  |  |  | ≤ 4 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 4.5 |
|  | 16 QAM | ≤ 4 |  |  |  |  |  | ≤ 4.5 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 5 |
|  | 64 QAM | ≤ 4 |  | ≤ 5 |  | ≤ 6.5 |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 6 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 7 | 7 |

<End of Changes>