3GPP TSG-RAN WG4 Meeting #114	R4-2501619

Athens, Greece, 17-21 February, 2024

Source:	Ericsson

Title:	UE Capability/UE behavior and network signaling for Type 4 EN-DC/NR-CA

Agenda item:	7.5.2.2

Document for:	Approval

# 1	Introduction

In RAN4#113 the following WF was agreed [1]:

… ## Sub-topic 2-2 : UE Capability/UE behavior and network signaling for Type 4 EN-DC/NR-CA … ### <Issue 2-2-3: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b> Way forward: - When maxMIMO-layers = 2 - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1. - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2. NOTE: This is specified Rel-18 behavior. No need to further discuss. - When maxMIMO-layers ＞4 - FFS in the next meeting. - When maxMIMO-layers = 4 - FFS in the next meeting. NOTE: maxMIMO-layers means the existing BS signaling. …

In this contribution we discuss proposals related to the issues above.

# 2	Discussion

## 2.2 	Issue 2-2-3:  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b

The issue is stated as

- When maxMIMO-layers ＞4

  - FFS in the next meeting.

- When maxMIMO-layers = 4

  - FFS in the next meeting.

There has already been some deliberations regarding this in [2], Topic summary for [113][123] NonCol_intraB_ENDC_NR_CA, which offers some options.

… - Recommended WF - Discuss the following aspects step by step. - maxMIMO-layers=2 Either Type 1(collocated) or Type 2(non-collocated), pending on whether Rel-18 BS signaling (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) is present. - Note: This is specified Rel-18 behavior. No need to further discuss. - maxMIMO-layers ＞4 Type-1(collocated) - maxMIMO-layers=4 - Option 1 - “Rel-19 New BS signaling is present with value 0” is Type 4. - “Rel-19 New BS signaling is present with value 1” is Type 1. - “Rel-19 New BS signaling is absent” is that UE follows Rel-18 specification. - Option 2 - UE is Type 1 or Type 4 depending on always present Rel-19 BS signaling element distinction - NOTE: RAN2 has the authority to modify this, as long as case of Rel-18 NW vs Rel-19 UE remains resolved - UE is Type 1 when Rel-19 BS signaling is absent i.e. UE follows Rel-18 BS signaling - Option 3 - The UE behavior during initial connection among type 1, 2 and 4a/4b depends directly on the issues 2.3 and 2.4 and the outcome of those discussions clarifies the UE behavior for switching from type 4a/4b to type1. On the other hand switching between type 4 to type 2 can happen with the RRC maxMIMO-Layers=2, as type 4 and type 2 both use the same UE architecture. …

In our view, the solution from [2] with option 1, as originally proposed by KDDI and Samsung, describes a working solution.

Proposal 1: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b

  - maxMIMO-layers ＞4

Type-1(collocated)

  - maxMIMO-layers=4

- Option 1

  - “Rel-19 New BS signaling is present with value 0” is Type 4.

  - “Rel-19 New BS signaling is present with value 1” is Type 1.

  - “Rel-19 New BS signaling is absent” is that UE follows Rel-18 specification

# 3 	Summary

Proposal 1: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b

  - maxMIMO-layers ＞4

Type-1(collocated)

  - maxMIMO-layers=4

- Option 1

  - “Rel-19 New BS signalling is present with value 0” is Type 4.

  - “Rel-19 New BS signalling is present with value 1” is Type 1.

  - “Rel-19 New BS signalling is absent” is that UE follows Rel-18 specification.

# References

[1] 		R4-2420399, WF on non-collocated intra-band EN-DC and NR-CA, KDDI

[2]		R4-2418145, Topic summary for [113][123] NonCol_intraB_ENDC_NR_CA, Moderator (KDDI)