3GPP RAN4 Meeting #114		R4-2501623

Athens, Greece, 17th – 21st February, 2025

Agenda item:	7.24.3

Source:	Ericsson

Title:	Discussion on OTA testing for A-IoT device 1

WI/SI:	Ambient_IoT_Solutions-Core

Release:	Rel-19

Document for:	Approval

# Introduction

During RAN #106 meeting, a new work item was approved on Ambient IoT Solutions with the WID given in [1]. In this WID RAN4 scope is defined as follows:

This contribution provides our views related to OTA test methodology for A-IoT device 1.

# Discussion

## Test System Reuse

TR 38.870 details OTA test methods for NR1 TRP and TRS using both Anechoic Chamber (AC) and Reverberation Chamber (RC). These specifications were developed mainly for smartphones, wearable redcap UEs, tablets and Laptops. However, A-IoT device 1 is significantly different from these types of devices. Indeed, A-IoT device 1 does not generate the transmitted signal, it only backscatters an unmodulated incoming CW signal transmitted from a CW Transmitter (CWT). Hence, test methods given in TR 38.870 have to be modified to take into account this aspect. Furthermore, it is our view that A-IoT device 1 should be tested using realistic deployment scenarios, and this includes testing this device when backscattering a CW signal from an external CWT. Additionally, A-IoT device 1 has a very small Energy Storage Unit (ESU) that is charged using RF Energy Harvesting (EH) exploiting the CW signal. Hence, this ESU should be fully charged before the beginning of testing.

Observation 1: A-IoT device 1 does not generate the transmitted signal, it only backscatters an unmodulated incoming CW signal transmitted from a CWT, hence it should be tested using realistic deployment scenarios while backscattering a CW signal from an external CWT.

Proposal 1: A-IoT device 1 should be tested while backscattering a CW signal from an external CWT.

Observation 2: A-IoT device 1 has a very small Energy Storage Unit (ESU) that is charged using RF Energy Harvesting (EH) exploiting the incoming CW signal.

Proposal 2: A-IoT device 1 Energy Storage Unit (ESU) should be fully charged before begging testing for TRP and TRS.

Moreover, since device 1 only backscatters a CW signal, the value of TRP will depend on the received CW signal level. Hence, CWT -transmit power, -distance from device and -angle have to be specified. Alternatively, received (at the device antenna port) CW signal level has to be specified.

Proposal 3: For testing TRP, specify CWT power, distance, and angle. Alternatively specify received (at the device antenna port) CW signal level.

Due to A-IoT device 1 small ESU and likely omnidirectional antennas, to reduce the measurement time, a coarse measurement grid can be used for both TRP and TRS measurement. Given the table below from TR 38.870, we propose using 30 degrees for TRP and 45 degrees for TRS.

Observation 3: Due to A-IoT device 1 very small Energy Storage Unit (ESU) and likely omnidirectional antennas, a coarse measurement grid can be used for both TRP and TRS measurement.

Proposal 4: Use a coarse measurement grid of 30 degrees for TRP and 45 degrees for TRS measurement.

## Measurement Uncertainty (MU) assessment

The MU assessment will depend on the used test system and test procedure. So, it is worth agreeing on a test system and test procedure before performing the MU assessment.

# Conclusions

This contribution provides our views related to OTA test methodology for A-IoT device 1. The contribution makes the following proposals.

Observation 1: A-IoT device 1 does not generate the transmitted signal, it only backscatters an unmodulated incoming CW signal transmitted from a CWT, hence it should be tested using realistic deployment scenarios while backscattering a CW signal from an external CWT.

Proposal 1: A-IoT device 1 should be tested while backscattering a CW signal from an external CWT.

Observation 2: A-IoT device 1 has a very small Energy Storage Unit (ESU) that is charged using RF Energy Harvesting (EH) exploiting the incoming CW signal.

Proposal 2: A-IoT device 1 Energy Storage Unit (ESU) should be fully charged before begging testing for TRP and TRS.

Proposal 3: For testing TRP, specify CWT power, distance, and angle. Alternatively specify received (at the device antenna port) CW signal level.

Observation 3: Due to A-IoT device 1 very small Energy Storage Unit (ESU) and likely omnidirectional antennas, a coarse measurement grid can be used for both TRP and TRS measurement.

Proposal 4: Use a coarse measurement grid of 30 degrees for TRP and 45 degrees for TRS measurement.

# References

RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, 3GPP RAN #106.