3GPP TSG-RAN WG4 Meeting # 114											R4-2501662

Athens, Greece, Feb. 17th ‒ Feb. 21st, 2025

Agenda item:			7.6.3

Source:	Nokia

Title:	Discussion on requirements for Fragmented CA.

Document for:	Approval

# Introduction

The latest WF for NR FR1 DL fragmented carriers study [1] presents the open issues as found below relevant for in-gap requirements:

#### Issue 3-4: Discussion on in-gap requirements

RAN4 evaluate performance impact on the cases:

- image of in-gap interference overlaps with wanted carrier

- For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC

- image of in-gap blocking interference does not overlap with wanted carrier

#### Issue 3-5: Discussion on DL fragmented carriers in TDD bands

Background: To consider transition time if there’s one LO for the TDD band and study the feasibility

Discuss following options for the number LO’s needed for fragmented carrier operation in TDD with one of the carriers having UL allocation:

- Option 1: One LO will be enough for both UL and DL.

- Option 2: Separate LOs will be needed for UL and DL

In this contribution we present aspects that needs to be agreed to discuss requirements for the fragmented carrier aggregation operation. Recapturing the objective, we highlight the notable considerations for the study that matters if we discuss RF requirements.

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx RF chains (e.g. from separate Rx RF chains per CC to one Rx RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements for study purpose are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx RF chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4] ;

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. One Rx RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

Discussion

Impacts on DL Rx requirements

The study item is currently active at identifying the DL Rx requirements of the UE. In [2] we presented the Rx requirements mapped onto the example bands for further discussion, and we would like to bring the highlighted parts of the introduction closer to the discussion of the DL Rx requirements.

ΔRIBNC relaxation

Starting at the discussion related to ΔRIBNC relaxation, this requirement is related to the duplexer Tx to Rx isolation in the UE front-end, and Tx interference suppression for avoidance of Rx RF chain saturation. In our opinion this problem is not directly related to fragmented carrier operation as the scenario with an UL transmission of the UE scheduled here is already captured. This is captured by the UE requirement, that it shall be able to cope with up to 6 dB DL received power spectral density imbalance between two CCs. The only new aspect with fragmented carrier operation is the reduction in isolation from using a broader analog filter. Dependent on whether or not the CC is PCC or SCC and placement of the CC this may have different implications.

UEs are already required to cope with up to 6 dB DL received power spectral density imbalance between two CCs.

In case the fragmented carrier configuration is for PCC operation, the downlink primary carrier shall be configured closer to the uplink band. This as there then will be two fragmented DL CCs and only a single UL CC. This would reduce the potential interference of the secondary DL CC hence removing the need for any relaxation of REFSENS. This can be implemented with a restriction of the PCC placement, as captured in Notes of the ΔRIBNC forcing the PCC assignment to link the DL carrier assignment closest to the uplink band.

RAN4 should not consider relaxation of ΔRIBNC for fragmented carrier operation.

Whether or not RAN4 shall consider restriction of the PCC placement can be further discussed.

ACS and IBB relaxations

As also discussed in [2], the adjacent channel and in-band blocking interference is of a much higher power level than the wanted carrier in the RF requirements. 2 cases of ACS tests that the UE is compliant in the entire receiver dynamic range, and the requirements are recaptured in the figure to help the readers recall the test including the 2 IBB cases of 5MHz. As can be seen the analogue channel filter attenuates the interference, but in relation to this study all companies have identified that such suppression of the analogue filter is missing when the UE operates with a single Rx RF chain.

Fig. 1: ACS and IBB illustration

While these tests secure the Rx operation considering only that the UE will be subject to input level from REFSENS to -25dBm and at various interference levels, worst at the IBB2 case, the Rx requirements are considering any and all aspects of deployment. For that reason, they do not fit into this study for requirement considerations, since the study clearly outlines inter-operator co-located scenario.

For a new UE capability for fragmented carrier support to make sense, in our opinion, the feature should provide at least at low and moderate in-gap interference the same user experience as required for non-contiguous intra-band CA. To prevent end user dissatisfaction, the user experience should always be at least on par with UEs not supporting fragmented carriers. Strong in-gap interference may require changing the UE's configuration, as a fallback, from receiving both CCs with all antennas by fully shared RF chains (cf. the reference architecture for fragmented carriers [1]) e.g.

- to receiving only the PCELL using all antennas or,

- if 4 antennas are available in a band where only 2 antennas are mandated, to receiving each CC with only 2 antennas, respectively.

Study how, depending on the PSD of the in-gap interference, a UE may transition between carrier aggregation configurations that support fragmented carrier operation and carrier configurations that do not.

In the SI phase of the work, RAN4 can only study how this transition can be implemented. In our opinion the UE needs to “fallback” to another configuration when it can’t maintain performance due to an interferer. We see no point in introducing a worse performing UE CA configuration with corresponding relaxed requirements for a UEs with one available Rx RF chain less than the number of available CCs as compared to what can be achieved with the current non-contiguous CA framework.

RAN4 shall not consider requirements for fragmented carriers which will result in a performance degradation compared with legacy non-contiguous intra-band CA or – only in case of a strong in-gap interferer – single CC operation.

A key aspect of the RF performance requirements for UEs supporting fragmented carriers will be the level of in-gap interference up to which these UEs must fulfil the throughput requirement for non-contiguous intra-band CA. The robustness against in-gap interference is not the only challenge for a fully shared RF chain. However, other challenges, e.g. related to sensitivity or out-of-gap interference (see section 2.2), may be solved by a smaller UE design implementation margin or by a performance improvement of the Rx RF chain.

In fragmented carrier operation, a UE should not need relaxations for sensitivity or out-of-gap interference, but may need to achieve the throughput required for non-contiguous intra-band CA only in a limited range of the in-gap interference level, subject to discussion in RAN4.

Impacts of image interference to Rx requirements

In [2] we presented aspects of image interference, which occurs when the channel bandwidths of the carriers are of unequal widths. The larger of the two fragmented carriers will potentially be subject to larger levels of image interference. In RAN4#112-bis an agreement of re-using the Rx chain image rejection of 25dB was made as captured in R4-2417201, but that agreement seems to have missed that it indirectly implies a limitation to the power spectral density of the in-gap interference. If the in-gap interference exceeds the power level of the fragmented carriers, then also the image of the in-gap interference increases in power. This will decrease the C/I of the larger fragmented carrier, meaning that the 25dB image rejection ratio targeted for the study gets exceeded by the image-interference. This will cause a degradation in performance of the larger fragmented carrier than in the traditional non-contiguous intra band case, where the 25dB image rejection is focused. Hence the in-gap interference gets bound by the Rx chain image interference of 25dB.

Based on previous agreements of the image rejection ratio the unequal carrier bandwidth scenario may be subject to lesser C/I for the frequency range of the image of the in-gap interferer of higher power spectral density within the widest fragmented carrier.

Below we show how the Rx images surrounding the LO has 25dB (in blue) for the fragmented carriers, 25dB for the in-gap interference (in grey) and how the 25dB in red will not be achievable unless the in-gap interference equals the fragmented carriers in PSD.

25dB

25dB

25dB

Fig. 2: Image interference dependent on LO placement

Since there are limitations to the operational mode based on the power spectral density difference of the in-gap interferer, it would be beneficial to quantify and report this from the UE side.

RAN4 shall consider measurements of the in-gap interference for the sake of signaling and scheduling to determine when the UE can operate in fragmented carrier mode.

TDD operation

Based on Fig. 2 showing the LO placement of the downlink for symmetric/asymmetric fragmented carrier reception, we would also like to draw attention towards the LO placement in the uplink for TDD operation. One further RF architecture issue is the number of LOs used when the fragmented carriers are TDD carriers and one of these carriers has uplink. In this case, it shall be considered whether the assumption of using a single LO, in a similar way as captured in TS 38.101-1 table 5.3.6-2, is still in place or whether a second LO is needed for the uplink, as shown in Fig. 3.

Tx operation in the fragmented band may pose an issue for TDD operation, so we would like to further clarify the scope of the study – if uplink operation in the fragmented band is expected, and if uplink operation in the fragmented band is included in the study. To our understanding UL has to be considered since the SID (RP-243002) lists 2 cases with just 2 CCs and its note 3 suggests a single DL band. Also, the WF [1] states that TDD bands are included in the study, meaning UL has to occur at least in one of the fragments. In relation to Issue 3-5 of the WF [1] RAN4 needs to clarify the number of LOs needed for the use case of the fragmented carriers in TDD with one of the carriers having UL allocation

RAN4 to clarify the number of LOs needed for the use case of the fragmented carriers in TDD with one of the carriers having UL allocation.

Fig. 3: Multiple LO placements in the uplink

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: 	UEs are already required to cope with up to 6 dB DL received power spectral density imbalance between two CCs.

Proposal 1: 	RAN4 should not consider relaxation of ΔRIBNC for fragmented carrier operation.

Proposal 2: 	Study how, depending on the PSD of the in-gap interference, a UE may transition between carrier aggregation configurations that support fragmented carrier operation and carrier configurations that do not.

Proposal 3: 	RAN4 shall not consider requirements for fragmented carriers which will result in a performance degradation compared with legacy non-contiguous intra-band CA or – only in case of a strong in-gap interferer – single CC operation.

Proposal 4: 	In fragmented carrier operation, UEs should not need relaxations for sensitivity or out-of-gap interference, but they may need to achieve the throughput required for non-contiguous intra-band CA only in a limited range of the in-gap interference level, subject to discussion in RAN4.

Observation 2: 		Based on previous agreements the unequal carrier bandwidth can not handle an in-gap interferer of higher power spectral density than the widest fragmented carrier.

Proposal 5: 	RAN4 shall consider measurements of the in-gap interference for the sake of signaling and scheduling to determine when the UE can operate in fragmented carrier mode

Proposal 6: 	RAN4 to clarify the number of LOs needed for the use case of the fragmented carriers in TDD with one of the carriers having UL allocation.

- References

- R4-2420519, WF on NR FR1 DL fragmented carriers study, MediaTek Inc.

- R4-2419455, Discussion on Fragmented Carrier impacts on UE RF requirements, Nokia

- R4-2418146, Topic summary for [113][124] FS_NR_DL_Frag_Carrier, MediaTek Inc.