3GPP TSG-RAN WG4 Meeting # 114											R4-2501663

Athens, Greece, Feb. 17th ‒ Feb. 21st, 2025

Agenda item:			6.2.6

Source:	Nokia

Title:	TP to PRD01 for Guidelines on delta TIB and RIB

Document for:	Approval

# Introduction

This is a TP to PRD01 version 0.2.0, Rules guidelines and ways of working for introduction of band combinations for including guidance on delta TIB and RIB .

History of the delta values

The delta Rib/Tib values were first introduced in LTE with the first CA combinations. In the LTE specification, 36.101 we now have the following notes in Table 6.2.5-2 :

NOTE 3:   In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then:

-     When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances above, truncated to one decimal place for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied

-     When the E-UTRA operating band frequency range is >1GHz, the applicable additional 2DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 2DL CA configurations

NOTE 5:    Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then:

-     When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied

-     When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations.

These we introduced based on the WF “Additional framework for Interband CA relaxations” in R4-146833 and “Framework for MID band relaxations” in R4-148105 which list the following rules:

Proposal for the new Low-Low and High-High band combination frame work is as follows

- Applies to FDD-FDD and FDD-TDD band combinations

- Single and dual UL assumed

- Low band < 1 GHz

- 1.7 GHz < High band < 2.7 GHz

- Low-Low band combination dTib = 0.5 dB for both bands and dRib = 0 dB for both bands

- High-High band combination dTib = 0.5 dB for both bands and dRib = 0 dB for both bands

- Framework is valid for band combination that can be implemented at least with quadplexer, triplexer or a diplexer combining two duplex filters

Low-Mid and Mid-High band combination frame work

∙             Applies to FDD-FDD band combinations

∙             Single and dual UL assumed

∙             1427.9 MHz <= MID band < 1700 MHz

∙             Low band < 1 GHz

∙             1.7 GHz < High band < 2.7 GHz

∙             Mid - High band combination dTib = 0.5 / dRib = 0

∙             Low - Mid dTib = 0.3 / dRib = 0

∙             Framework is valid for band combination that can be implemented at least with quadplexer or a diplexer combining two duplex filters

The delta values were again discussed in Rel-17 in relation to NR and the band combination simplification as captured in the WF R4-2107888:

Rule-set outlined below

  - H-L dTib=0.3 and dRib=0

  - L-L and H-H dTib=0.5 and dRib=0

  - L-L-L and H-H-H maybe needs case by case study

  - Carriers in bands with a harmonic falling onto one of the DLs: default dTIB = 0.6 dB

  - 42,77,78 gets dTib=0.8 and dRib=0.5

  - n79 dTib = 0 and dRib=0 unless is combined with 3.5 GHz bands then dTib=0.8 and dRib=0.5

  - dRib = dTib-0.5 dB

  - Combinations with notes are handled case by case

Low bands in above are below 1GHz and high bands above 1 GHz but bands around 3.5 GHz and above are handled separately.

This was during the E-meeting phase of RAN4 so this was hard to conclude. Nevertheless, a new WF was agreed in R4-2120019:

- Following rules.

  - L-bands in above are below 1GHz

  - H-bands above 1 GHz and below 3GHz

  - VH-bands are above 3 GHz

  - H-L dTib=0.3 and dRib=0

  - L-L and H-H dTib=0.5 and dRib=0 (exception 1+3 dTib=0.3 and dRib=0)

  - VH bands dTib=0.8 and dRib=0.5

  - L-VH: L dTib=0.3 and dRib=0 and VH band dTib=0.8 and dRib=0.5

  - H-VH: H dTib=0.5 and dRib=0 and VH band dTib=0.8 and dRib=0.5

  - Carriers in bands with a harmonic falling onto one of the DLs: dTIB = 0.6 dB

  - n79 with L and/or H bands dTib=0

  - if not specified above then dRib = dTib-0.5 dB

  - L-L-L and H-H-H case by case study

The above are the last agreed rules to our understanding. Nokia presented in R4-2200705/R4-2112792 statistics of dTib and dRib to show that they did not follow any set rules as there were a number of outliers and not all of these could be explained by exceptions. It was proposed to remove all the delta tables, but this was not concluded in Rel-17. In Rel-18, the discussion continued but there has been no agreement to remove the delta values

This means a set of rules or “rule set” should be captured in the PRD. With the following TP we summarize all the rules previously agreed by RAN4 for inclusion to the PRD.

************************************* Start of TP*****************************************

## 5.4			Guidelines on delta TIB and RIB

The delta Rib/Tib values are introduced to allow relaxation to the UEs transmit power (delta TIB) and reference sensitivity (delta RIB) to compensate for hardware complexity of supporting a given band combination.

### 5.4.1		2 band CA/DC delta TIB and RIB

To define delta TIB and RIB when introducing new two band CA/DC band combinations the following guidelines can be followed.

Grouping of bands:

- L-bands are below 1 GHz.

- H-bands are above 1 GHz and below 3GHz

- VH-bands are above 3 GHz

Delta values apply as follows for two band CA/DC:

- VH bands dTib =0.8 and dRib=0.5

- H bands dTib=0.5 and dRib=0

  - Except for n1 and n3 which have dTib=0.3

- H-L: Both gets dTib=0.3 and dRib=0

- L-L,H-H: Both gets dTib=0.5 and dRib=0

  - Except for n1 and n3 which have dTib=0.3

- L-VH: L dTib=0.3 and dRib=0.2

- H-VH: H dTib=0.5 and dRib=0.2

- Carriers with any uplink harmonic falling onto the DL:

  - L-H: L dTib=0.6 dB H dTib=0.3 dB

  - L-VH: L dTib=0.6 dB VH dTib=0.8 dB

  - H-VH: H dTib=0.6 dB VH dTib=0.8 dB

- For SDL combinations SDL band gets “N/A” and  L,H dTib=0.3 and VH dTib=0.8

### 5.4.2			More than 2 band CA/DC delta TIB and RIB

Delta values for more than two band CA/DC are found as the MAX of the included delta values defined for the two bands CA/DC combinations.

### 5.4.3			Exceptions of delta TIB and RIB for specific band combinations

Due to multiple reasons the delta values captured together with the band combinations in the specifications may differ from the presented guidelines in Clause 5.4.1 and Clause 5.4.2. These values are found on a case-by-case basis when they are introduced.

************************************* End of TP*****************************************