3GPP TSG-RAN4 Meeting #114	R4-2501687

# Athens, Greece, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on MPR for SL intra-band contiguous CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_intraB_CA_ITS-Core |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For PC2 SL intra-band contiguous CA, MPR requirements are specified for PSSCH/PSCCH, PSFCH, and SSSB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add PC2 in Table 6.2E.1A-1. - Add MPR requirements of PSSCH/PSCCH, PSFCH, and SSSB for PC2 SL intra-band contiguous CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For SL intra-band contiguous CA, PC2 MPR requirements for PSSCH/PSCCH, PSFCH, and SSSB are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.1A, 6.2E.2.1A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

6.2E.1A	UE maximum output power for Sidelink CA

For the intra-band SL CA operation, the following NR SL CA UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1A-1: NR SL CA UE Power Class

| NR SL CA band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SL _n47B |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: For intra-band SL CA UE, the maximum power requirement apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

6.2E.2.1A	MPR for sidelink CA

6.2E.2.1A.1	MPR for sidelink intra-band contiguous CA

For SL intra-band contiguous CA of PSCCH and PSSCH simultaneous transmission with contiguous RB allocation, the allowed MPR for the maximum output power are specified in Table 6.2E.2.1A.1-1 for UE power class 3. The MPR with contiguous RB allocation is specified in Table 6.2E.2.1A.1-2 for power class 2 when the signalling is absent for dualPA-Architecture IE. The MPR with contiguous RB allocation is specified in Table 6.2E.2.1A.1-3 for power class 2 with TxD supported.

Table 6.2E.2.1A.1-1: MPR for power class 3 SL CA with contiguous RB allocation

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 6.5 | ≤ 7.0 |

Table 6.2E.2.1A.1-2: MPR for power class 2 SL CA with contiguous RB allocation

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 2.5 | ≤ 5.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.5 |
|  | 64QAM | ≤ 4.0 | ≤ 5.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Table 6.2E.2.1A.1-3: MPR for power class 2 SL CA with contiguous RB allocation with 2Tx

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 2.5 | ≤ 5.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.5 |
|  | 64QAM | ≤ 4.0 | ≤ 5.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

The contiguous allocation rule of inner and outer for SL intra-band contiguous CA refers to that for NR intra-band contiguous CA in 6.2A.2.1 in TS38.101-1.

For SL intra-band contiguous CA of PSCCH and PSSCH simultaneous transmission with non-contiguous RB allocation, the allowed MPR for the maximum output power are specified in Table 6.2E.2A.1-4 for UE power class 3. The MPR with non-contiguous RB allocation is specified in Table 6.2E.2.1A.1-5 for power class 2 when the signalling is absent for dualPA-Architecture IE. The MPR with non-contiguous RB allocation is specified in Table 6.2E.2.1A.1-6 for power class 2 with TxD supported.

Table 6.2E.2A.1-4: MPR for power class 3 SL CA with non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 | ≤ 9.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 | ≤ 9.5 |

Table 6.2E.2A.1-5: MPR for power class 2 SL CA with non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 7.0 | ≤ 10.0 |
|  | 16QAM | ≤ 4.0 | ≤ 7.0 | ≤ 10.0 |
|  | 64QAM | ≤ 5.5 | ≤ 7.0 | ≤ 10.0 |
|  | 256QAM | ≤ 8.0 | ≤ 8.0 | ≤ 10.0 |

Table 6.2E.2A.1-6: MPR for power class 2 SL CA with non-contiguous RB allocation with 2Tx

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 8.5 | ≤ 11.5 |
|  | 16QAM | ≤ 4.5 | ≤ 8.5 | ≤ 11.5 |
|  | 64QAM | ≤ 7.0 | ≤ 8.5 | ≤ 11.5 |
|  | 256QAM | ≤ 9.5 | ≤ 9.5 | ≤ 11.5 |

The non-contiguous allocation rule of inner, outer1, and outer2 for SL intra-band contiguous CA refers to that for NR intra-band contiguous CA in 6.2A.2.1 in TS38.101-1.

For SL intra-band CA of PSFCH with single RB transmission on each carrier, the required MPR are specified as Table 6.2E.2.1A.1-7 for UE power class 3. The MPR is specified in Table 6.2E.2.1A.1-8 for power class 2 when the signalling is absent for dualPA-Architecture IE. The MPR is specified in Table 6.2E.2.1A.1-9 for power class 2 with TxD supported.

Table 6.2E.2.1A.1-7: PSFCH MPR for intra-band SL CA for power class 3

| MPR (dB) | R (Ratio of gap bandwidth) |
| --- | --- |
| ≤ 2.5 | 0< R ≤ 0.3 |
| ≤ 7.5 | 0.3< R ≤ 0.5 |
| ≤ 12 | 0.5< R ≤ 1 |

Table 6.2E.2.1A.1-8: PSFCH MPR for intra-band SL CA for power class 2

| MPR (dB) | R (Ratio of gap bandwidth) |
| --- | --- |
| ≤ 5.0 | 0< R ≤ 0.3 |
| ≤ 10.0 | 0.3< R ≤ 0.5 |
| ≤ 14.0 | 0.5< R ≤ 1 |

Table 6.2E.2.1A.1-9: PSFCH MPR for intra-band SL CA for power class 2 with 2Tx

| MPR (dB) | R (Ratio of gap bandwidth) |
| --- | --- |
| ≤ 6.0 | 0< R ≤ 0.3 |
| ≤ 11.0 | 0.3< R ≤ 0.5 |
| ≤ 15.0 | 0.5< R ≤ 1 |

Where,

R is the ratio of the gap bandwidth between the two PSFCH transmitted on the two intra-band carrier by the total bandwidth of the two carrier.

When single S-SSB is transmitted on intra-band contiguous carriers, required MPR for single cell V2X in Table 6.2E.2.2-1 and Table 6.2E.2.2-2 is reused for UE power class 3 and for UE power class 2 respectively. For two S-SSB transmissions on intra-band contiguous carriers, the required MPR are specified as Table 6.2E.2.1A.1-10 for UE power class 3. The MPR is specified in Table 6.2E.2.1A.1-11 for power class 2 when the signalling is absent for dualPA-Architecture IE. The MPR is specified in Table 6.2E.2.1A.1-12 for power class 2 with TxD supported

Table 6.2E.2.1A.1-10: MPR for two S-SSB transmissions on intra-band contiguous carriers for power class 3

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 3.5 | ≤ 9.0 | ≤ 13.0 |

Table 6.2E.2.1A.1-11: MPR for two S-SSB transmissions on intra-band contiguous carriers for power class 2

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 4.0 | ≤ 9.5 | ≤ 14.0 |

Table 6.2E.2.1A.1-12: MPR for two S-SSB transmissions on intra-band contiguous carriers for power class 2 with 2Tx

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 4.5 | ≤ 10.0 | ≤ 14.5 |

< END OF CHANGE #2 >