3GPP TSG-RAN WG4 Meeting #114	R4-2501692

Athens, Greece Feb 17 – Feb 21, 2025

Source:	Tejas Networks

Title:	RRM core requirement for AI/ML based beam management

Agenda item:	7.19.3

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#113 in the scope of Rel-19 WI. Agreements achieved during this meeting are summarized in the WF [1] and the ad-hoc minutes [2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion, in this paper, we provide our views on issues related to requirements for beam prediction.

Discussion

Relative RSRP accuracy

In the previous meeting (RAN4 #113) RAN4 discussed about the definition of relative RSRP accuracy and following options were discussed.

Issue 3-1: Relative RSRP Accuracy - Proposals - Option 1: The definition of relative RSRP accuracy shall be: The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i - the ground truth of the L1-RSRP of the Top-1 ideal beam index j - Option 2: The definition of relative RSRP accuracy shall be: predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams. - Option 3: The definition of relative RSRP accuracy shall be: the RSRP difference between the predicted L1-RSRP of different beams. - Option 4: others

RAN4 agreed on the definition of absolute RSRP accuracy as follows:

The absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i. Here, the index i may correspond to any beam index within the top-K beams based on the ground truth L1-RSRP.

The absolute RSRP accuracy metric provides insight into how much the predicted RSRP values deviate from the actual ground truth RSRP values. It is a critical parameter for evaluating the performance of AI/ML-based beam prediction.

Regarding the relative RSRP accuracy, several options have been proposed:

- Option 1: The ground truth of L1-RSRP of the predicted beam index i – the ground truth of L1-RSRP of the Top-1 ideal beam index j.

This option compares the ground truth RSRP of the predicted beam with the ground truth RSRP of the best beam. However, this approach does not directly evaluate the predicted results in a relative manner. Additionally, if the gap between the best beam and the second-best beam is large, the UE might fail the test, even if the prediction is reasonably accurate.

- Option 2: Predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where beam index n has the largest RSRP among the predicted beams.

This option compares the predicted RSRP values of different beams, focusing on the deviation from the best-predicted beam. However, the predicted RSRP values can be influenced by factors such as beam direction, which may introduce variability.

- Option 3: The RSRP difference between the predicted L1-RSRP of different beams.

This option is similar to Option 2 but is more general in nature, as it does not explicitly reference the best-predicted beam.

Observation 1: If the gap between the best beam and the second-best beam is large, the UE may fail to meet any of the requirements in the test.

Instead of relying solely on the ground truth or predicted RSRP values, a more comprehensive approach would be to evaluate the absolute RSRP accuracy for each predicted beam and then compare these values across the top-K predicted beams. This approach provides a clearer understanding of how accurately the beams are predicted relative to the best-predicted beam. The definition could be as follows:

The relative RSRP accuracy for AI/ML-based beam prediction = Absolute L1-RSRP Accuracy of the predicted beam index i – Absolute L1-RSRP Accuracy of the best-predicted beam.

This method ensures that the relative accuracy is evaluated based on the deviation of predicted values from the ground truth, while also considering the performance of the best-predicted beam. It provides a more robust and meaningful metric for assessing AI/ML-based beam prediction performance.

Proposal 1: The definition should capture the idea of relative L1-RSRP accuracy, not relative L1-RSRP values.

Proposal 2: The relative RSRP accuracy for AI/ML-based beam prediction = Absolute L1-RSRP Accuracy of the predicted beam index i – Absolute L1-RSRP Accuracy of the best-predicted beam.

UE Rx beam knowledge

To accurately predict the L1-RSRP of the Tx beams, it would be beneficial for the UE to know the Rx beam paired with each predicted Tx beam. However, if the UE does not have prior knowledge of which Rx beam to use for a new Tx beam direction (due to a change in the TCI state), the UE would need to detect and adapt its Rx beam to align with the new Tx beam. While having knowledge of the Rx beam paired with the predicted Tx beam would help improve prediction accuracy, this knowledge should be internal to the UE and its implementation.

Observation 2: An unknown TCI state change can result in increased delay during the state transition.

Proposal 3: TCI state associated with the predicted beam should be known.

Beam prediction KPIs

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered. For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options The proposals discussed in the RAN4 #113 meeting are as follows: - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use combined Option 2 and 3: - Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB” - FFS: K = 2 and x = 3 as baseline - Option 4: Add new metric: - Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". FFS how N is derived. - Option 5: Add a requirement based on relative difference between predictions under different conditions/inputs (for example varying signal levels by N dBs but maintain same relative difference). - consider defining multiple tests under different conditions but with the same outcome(same predicted beam) - Option 6: both RSRP accuracy and beam prediction accuracy are used as KPIs for beam prediction - RSRP absolute accuracy applies to all predicted beams

For beam prediction evaluation, Top-K/1(%) accuracy is a critical KPI alongside RSRP accuracy. This metric measures the likelihood that the actual strongest beam is correctly identified within the Top-K predictions, ensuring that the model’s beam selection aligns closely with the strongest available signal. Combined with RSRP accuracy, which validates the predicted signal strength accuracy, Top-K/1 accuracy confirms that the model prioritizes optimal beams, helping to maintain effective connectivity even in rapidly changing network conditions. Together, these KPIs offer a balanced assessment of the model’s ability to predict both beam selection and strength.

Observation 3: Accurate L1-RSRP and Top-K/1 metrics ensure reliable identification of the strongest beams.

It is important to emphasize that, for the top K predicted beams, consistency should be maintained alongside RSRP accuracy and beam prediction accuracy. Consistency ensures that the predicted beams remain stable and reliable over time, reflecting the correct order and selection of the strongest beams without significant fluctuations. This helps in maintaining the integrity of the predictions and ensures that the system operates effectively in real-world scenarios.

Proposal 4: Option 6 should be adopted in conjunction with consistency in beam prediction to ensure both accuracy and reliability in the predictions.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: If the gap between the best beam and the second-best beam is large, the UE may fail to meet any of the requirements in the test.

Observation 2: An unknown TCI state change can result in increased delay during the state transition.

Observation 3: Accurate L1-RSRP and Top-K/1 metrics ensure reliable identification of the strongest beams.

Proposal 1: The definition should capture the idea of relative L1-RSRP accuracy, not relative L1-RSRP values.

Proposal 2: The relative RSRP accuracy for AI/ML-based beam prediction = Absolute L1-RSRP Accuracy of the predicted beam index i – Absolute L1-RSRP Accuracy of the best-predicted beam.

Proposal 3: TCI state associated with the predicted beam should be known.

Proposal 4: Option 6 should be adopted in conjunction with consistency in beam prediction to ensure both accuracy and reliability in the predictions.

- References

- R4-2420334, WF on AI/ML air interface part 1, Qualcomm Incorporated, Orlando, USA, 18th – 22nd November, 2024.

- R4-2420335, AI/ML ad-hoc meeting minutes, Qualcomm Incorporated, Orlando, USA, 18th – 22nd November, 2024.

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Rel-18, ver. 18.0.0.