# 3GPP TSG-RAN WG4 Meeting #114	R4-2501697

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.23.2.2

Source: 	Qualcomm Incorporated

Title: 	Views on existing BS RF requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#113, a WF was agreed in [3] based on some of the topics listed in the moderator summary, that is summarized in [2]. In this contribution, we share our views on existing BS RF requirements for Rel-19 SBFD.

# Discussion

## Unwanted emissions

In [2], the following was agreed:

- FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

In Section 11 in TR 38.858, RAN4 has concluded that outcome of the SBFD and legacy TDD adjacent channel coexistence. The SBFD-to-SBFD coexistence case was not investigated since there are no SBFD deployments prior to Rel-19. However, from coexistence point of view, the impact of the inter-BS CLI on the UL subband at the SBFD capable BS that coexists with another SBFD BS will be similar to the impact of the legacy TDD DL on UL subband of an SBFD-capable BS. To address the inter-BS CLI, RAN4 has studied only ACIR enhancements as a solution to ensure feasible coexistence.

On the other hand, for the coexistence case with BS-capable SBFD BS in the adjacent channel, RAN4 has identified coexistence challenges due to inter-BS CLI on the UL subband. Within the SI phase, RAN4 has considered only ACIR enhancements as a solution to ensure feasible coexistence.

Observation 1: When coexisting with SBFD-capable BS, legacy ACLR requirement might not be sufficient to ensure feasible coexistence. Additionally, RAN4 has only considered ACIR enhancement as a possible solution during the SI phase.

As a way forward, it is viable to consider urban macro deployment since it is the most challenging deployment scenario. RAN4 then should consider a case where additional new ACLR requirement is considered to allow feasible coexistence with new SBFD-capable BS while assuming 1 dB desensitization.

Proposal 1: RAN4 to consider additional ACLR requirement assuming 1 dB desensitization to ensure feasible coexistence with SBFD-capable gNB. As a starting point, urban macro deployments should be considered as representing the challenging scenario.

Relying on proprietary solutions to address the SBFD-to-SBFD coexistence is questionable at this stage, since RAN4 has agreed in RAN4#113 to introduce new in-channel inter-subband leakage and blocking, which are new requirements intra-operator to address inter-gNB CLI.

Proposal 2: Given that RAN4 agreed to introduce new in-channel inter-subband leakage and blocking requirements, it should not be excluded to introduce new additional OBUE requirements for inter-SBFD inter-operator coexistence. Relying on a proprietary solution for such a deployment scenario might not be feasible.

# In-band blocking

In [2], the following was agreed:

- For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements. - For FR1 WA BS, FFS following options. - Option 1: a unified blocking requirement to consider both scenarios - Option 2: separate requirements are defined - One set of requirements for coexisting with legacy TDD system in adjacent channel - One set of requirements for coexisting with new SBFD system in adjacent channel - Option 3: In-band blocking requirement is defined by manufacture declared level of interference in the specified range, which comes from the system-level evaluation (from 100% grid-shifting to 10% grid-shifting). - For FR1 WA BS co-existence study, FFS what grid-shift value(s) should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - Option 5: 5% - Option 6: 20% ~ 50% - For FR2-1 WA SBFD BS, FFS following two options. - Option 1: The current FR2-1 WA BS in-band blocking requirements can be re-used Option 2: Define in-band blocking interferer signal power to -23 dBm

In our previous contribution (R4-2415468), we have provided our in-band blocking requirement for 50% GS around -26 dB, considering AAS configuration considering two panels with 46 dBm conducted power per polarization. Given that there are difference in the underlying assumptions among companies within RAN4 and given that the main source of higher values for the in-band blocking stem from the inter-gNB CLI. It is preferable that RAN4 decouples the in-band requirements for coexistence with legacy TDD and coexistence with SBFD deployments. Such a decoupling of requirements can be implemented via defining different in-band blocking requirements and different declarations for the different levels. In this regard, we support option 2. In addition, due to the dependency of the in-band blocking value on the underlying assumptions, and especially the grid shift and Tx power assumptions, option 3 is also a valid way forward. The rationale there that, similar to the study phase, converging on a single set of parameters among the companies within RAN4 might be difficult.

Proposal 3: RAN4 to consider option 2 for FR1 WA BS, where separate requirements are defined, such that One set of requirements for coexisting with legacy TDD system in adjacent channel and another set of requirements for coexisting with new SBFD system in adjacent channel.

# Conclusion

In this contribution, we provided our views on BS RF requirements for SBFD with the following observations and proposals:

Observation 1: When coexisting with SBFD-capable BS, legacy ACLR requirement might not be sufficient to ensure feasible coexistence. Additionally, RAN4 has only considered ACIR enhancement as a possible solution during the SI phase.

Proposal 1: RAN4 to consider additional ACLR requirement assuming 1 dB desensitization to ensure feasible coexistence with SBFD-capable gNB. As a starting point, urban macro deployments should be considered as representing the challenging scenario.

Proposal 2: Given that RAN4 agreed to introduce new in-channel inter-subband leakage and blocking requirements, it should not be excluded to introduce new additional OBUE requirements for inter-SBFD inter-operator coexistence. Relying on a proprietary solution for such a deployment scenario might not be feasible.

Proposal 3: RAN4 to consider option 2 for FR1 WA BS, where separate requirements are defined, such that One set of requirements for coexisting with legacy TDD system in adjacent channel and another set of requirements for coexisting with new SBFD system in adjacent channel.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2416585, “Way Forward for [112][308] NR_duplex_evo_General”, RAN4#112bis, Huawei, HiSilicon.