3GPP TSG RAN WG4 Meeting #114		R4-2501726

Athens, Greece, 17th – 21st February 2025

Source: 		Rohde & Schwarz

Title: 			Discussion on test setup for AI/ML based beam management

Agenda Item: 	7.19.3

Document for: 	Discussion

# Introduction

From the WF in [7], the following analysis is required with regards to the test setup for AI/ML based beam management:

UE Rx beam knowledge

- Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS

- RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not.

  - Other way to know UE Rx beams is not precluded

    - UE capability is one of the solutions

Test setup for Beam prediction testing

  - Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs.

In this contribution, we elaborate further our views on single-AoA and multi-AoA test setups.

# Discussion

As initially introduced in our contribution in [4] and considering the discussions in the past few meetings, the different aspects impacting the end-to-end UE performance for AI/ML based BM can be described as below in Figure 1 (top down: from BTS towards the UE)

Figure 1: AI/ML BM Layers Model

Observation 1: There are several factors to be considered in the end-to-end AI/ML BM performance.

With respect of the test setup configuration, both alternatives, single-AoA and multi-AoA, are still being discussed. The challenges to conclude to a system seem to be mainly:

- Key factors which impact the AI/ML BM performance

- Test system complexity and feasibility

Observation 2: The main challenges to conclude to a baseline setup are the identification of factors impacting the AI/ML BM performance, as well as the test system complexity and feasibility.

One of the key factors considered relevant for the AI/ML BM performance so far, was the need for a CDL channel. Ideally, such a channel is emulated OTA i.e. spatially from multi AoAs, which was the main driver for a multi-AoA test setup.

In addition, from several contributions and discussions online, it was made clear that there mostly two different assumptions and reference implementations of the AI/ML L1-RSRP model:

- Considering as input only L1-RSRP values (“L1-RSRP Scalars”)

- Considering as input L1-RSRP associated UE Rx Beam index / directional information (i.e. “L1-RSRP Vectors”)

Observation 3: There seems to be two different assumptions and reference implementation for the AI/ML L1-RSRP model, with and without directional information for the L1-RSRP measurement.

This directional information required by the AI/ML L1-RSRP model as described above, seems at a first glance also to require a multi-AoA test setup.

Observation 4: Main drivers for a multi-AoA setup are spatial emulation of the CDL channel and the usage of directional information for the measured L1-RSRP in the prediction model.

In the following sections we asses again the single/multi-AoA test setups, considering the observations above.

## Single-AoA testing

### Options’ analysis

The main challenge for the single-AoA setup is how to consider sufficiently the CDL channel and the UE directional performance, which (as mentioned in section 2) ideally require a multi-AoA emulation.  Figure 2 shows in a simplistic diagram how in a multi-AoA environment, at a certain point of time, the UE measures with one of its beams (e.g. UE Rx B1) the signals (e.g. S1 to S4) seen through the CDL channel model (composed by clusters C1 to C4) illuminated by one BS beam (e.g. Tx B1).

Elaborating further the example, the vectors S1 to S4 can be understood as the AoA/ZoA from the channel model, and thus the power received by the UE can be expressed as follows (using a similar formulation as presented in [5]):

Pr,UERx B1=i=14GRxRx B1,AoASi×Pt,BS×GTxTx B1,AODCi×Gpath,iAODCi,AOASi

So, the power received by the UE using Rx B1 Pr,UERx B1 is the combination of:

- The UE Rx Beam gain in the AoA from the respective cluster GRxRx B1,AoASi

- The gain of the path (or path loss) between the BS, the cluster and the UE, the UE Gpath,iAODCi,AOASi

- The gain of the BS Tx Beam in the direction of the respective cluster GTxTx B1,AODCi

- The BS transmitted power.

Figure 2: Example of multi-AoA emulation

When extending this combination to each UE Rx Beam and BS Tx Beam in their respective codebooks, we can calculate the power received by the UE for each BS Tx Beam.

The multi-AoA approach is designed to emulate all the above, except the UE Rx beam, while, as explained in [4] and other contributions, there are possibilities to transform such multi-AoA emulation into a single-AoA. Therefore, the TE must include the UE Rx Beam/Antenna pattern in the channel transmitted from the single-AoA.

Observation 5: (Single-AoA) The multi-AoA environment can be transformed into a single-AoA test for AI/ML BM testing, by including the UE Rx beam/antenna pattern in the combination of BS Tx beams with the CDL channel model to be emulated, without impacting the number of BS Tx beams or the parameters of the CDL channel model.

Depending on the assumptions and approaches how this is done, different coverage of the UE spatial / directional performance aspects is achieved.

We can differentiate 4 options how for AI/ML BM, a multi-AoA test can be transformed into a single-AoA test:

- Option 1: RAN4 defines a Reference Fixed UE Rx Beam/Antenna pattern. In this case, since the testing becomes primarily a “baseband test”, a second test would be required to address the UE spatial/directional performance.

- Option 2: RAN4 defines a Reference Complex UE Rx Beam/Antenna pattern (Codebook). Also in this case, testing becomes primarily a “baseband test”, thus a second test would be required to address the UE spatial/directional performance.

- Option 3: UE manufacturer provides the Complex UE Rx Beam/Antenna pattern (Codebook) to the TE prior to testing. Assuming that the pattern is realistic, the UE spatial/directional performance is covered in the test.

- Option 4: TE measures the Complex UE Rx Beam/Antenna pattern (Codebook) before starting test. This approach would rely on the basics of Radiated Two Stage Method (RTS) defined in [7]. In this case, the UE spatial/directional performance is covered in the test.

Observation 6: (Single-AoA) There are 4 options for a multi-AoA to single-AoA test transformation and inclusion of the UE directional performance: Option 1 use a reference fixed UE pattern for the baseband test, and a second separate test for the UE directional performance; Option 2 use a reference complex UE pattern a the baseband test, and a second separate test for the UE directional performance; Option 3 use a complex UE pattern provided by the UE manufacturer, no additional test required; Option 4 use complex UE pattern measured by the TE, no additional test required;

A candidate for the additional test required in Options 1 and 2, to verify the UE Rx spatial/directional performance, is described in section 2.1.2.

The Complex UE Rx Beam/Antenna pattern (Codebook) required in Options 2, 3 and 4 is discussed in section 2.1.3.

### Additional test to verify the UE Rx spatial/directional performance

As agreed in TR 38.843 [9], the UE must measure and use the L1-RSRP for a reduced set of BS Tx beams (Set B) to predict the remaining BS Tx beams (Set A). Therefore, in addition to the AI/ML BM baseband performance tested in Option 1 and 2 when presented to the combination of the BS Tx beams and the CDL channel model, the most relevant metric driving the UE performance is the measurement accuracy on different directions of the sphere (AoAs).

Currently, RF and RRM requirements do not address this spherical measurement accuracy because they either rely on actual EIS / Spherical Coverage for RF testing, or on individual RSRP accuracy measurements on a single particular AoA, which in most test cases is the found UE Rx beam peak.

Observation 7: (Single-AoA) L1-RSRP accuracy is currently measured only on a single particular AoA, which in most test cases is the found UE Rx beam peak.

In fact, the intended L1-RSRP accuracy in a spatial CDL, includes also the ability of the UE to choose the proper UE Rx beam/direction to measure the L1-RSRP. In case of a reference UE Rx beam/antenna pattern this is ignored (in case of fixed pattern) or assumed, but not verified (in case of a complex pattern). This important aspect can be addressed and verified by a L1-RSRP Spherical Coverage requirement and test.

Following this rationale, a new test case addressing the L1-RSRP spherical coverage accuracy will be required to complement the AI/ML BM test based on single direction baseband testing with reference UE Rx beam/antenna pattern.

Observation 8: (Single-AoA) A new requirement and test case addressing the L1-RSRP spherical coverage accuracy, will be required to complement the AI/ML BM test based on single direction baseband testing, with reference UE Rx beam/antenna pattern.

This certainly requires some more dedicated studies from RAN4, given that similar metric and test methodology were developed only for RF EIS, and it is new to RRM.

### Complex UE Rx beam/antenna pattern

Referring again to section 2.1.1, the Complex UE Rx beam/antenna pattern used for Option 2 to 4 for single-AoA testing, is provided by either:

- Common definition captured in the specification (for Option 2)

- Manufacturer declaration (for Option 3)

- TE Measurements (for Option 4)

In all the cases, the test system complexity is shifted to the computational effort required to combine all possible BS Tx beams (Set A or Set B) and the CDL-C channel model, with each of the UE Rx beams. This is visualized in Figure 3 and Figure 4 .

Figure 3: UE using 4 Rx beams to measure BS Tx beam 1

Figure 4: UE using 4 Rx beams to measure BS Tx beam 2

Some clarifications on Figure 3 and Figure 4:

- Tx B1 and Tx B2 are part of Set B BS Tx beams

- C1-C4 are channel model clusters

- Rx B1 - Rx B4 are UE Rx beams used to measure L1-RSRP for each BS Tx beam

Each BS Tx beam “illuminates” all channel clusters in a different way, and thus the L1-RSRP measured on each UE Rx beam as a combination of all clusters is different. How each BS Tx beam “illuminates” the channel model can be seen in the power angular spectrum (PAS) figures from [2] and [4]. Therefore, the analysis of the single-AoA approach must consider the computational load to obtain all the resulting channel models when combining each BS Tx beam, the CDL channel model and each UE Rx Beam. E.g. assuming 128 BS Tx Beams (Set A) and 64 UE Rx Beams, it will result in 8192 “resulting CDL” channels to handle.

Observation 9: (Single-AoA) The analysis of the single-AoA approach must consider the computational load to obtain all the resulting channel models when combining each BS Tx beam, the CDL channel model and each UE Rx beam.

Following this logic, the Complex UE Rx beam/antenna pattern must contain the complex beam pattern (magnitude and phase) with sufficient resolution for the complete sphere around the UE, and for each UE Rx beam in its codebook. In case of Option 4 (TE measures the Complex UE Rx Beam/Antenna pattern), the measurement of all these patterns is very time consuming and requires the concrete methods to acquire both magnitude and phase. E.g. using an Antenna Test Function for the UE to report magnitude and phase.

Observation 10: (Single-AoA) In case of Option 2 and 3 (Reference or Manufacturer Complex UE Rx Beam/Antenna pattern), the UE Rx beam/antenna pattern must include magnitude and phase, as well as beam id information (i.e. codebook).

Observation 11: (Single-AoA) In case of Option 4 (TE measures the Complex UE Rx Beam/Antenna pattern), the UE Rx beam/antenna pattern measurement is very time consuming and must include magnitude and phase information, e.g. using an Antenna Test Function.

The complexity and challenges to assess and use the complex pattern is in the several details to be considered and are listed in the following as direct observations.

Observation 12: (Single-AoA) The definition of the format and granularity of the Complex UE Rx beam pattern is key to ensure consistent results among different TE vendors for the single-AoA methodology.

Observation 13: (Single-AoA) The use of fine or rough beams for AI/ML BM prediction may affect the complexity of the data to be considered for the single-AoA methodology, as well as the number of iterations to be calculated.

Observation 14: (Single-AoA) If several UE orientations towards the channel model must be assessed for the single-AoA methodology, the number of combinations and the computational effort rises exponentially.

Observation 15: (Single-AoA) The measurement timing perspective is also a critical aspect for the single-AoA methodology, because not all BS Tx beams might be measured with all UE Rx beams.

### Options’ comparison

In summary, there are pros and cons for the single-AoA solution options described in section 2.1.1:

- All options allow to use the CDL channel model without further simplifications.

- All options allow in theory unlimited number of BS Tx beams. In practice the number will depend on the overall compromised computational effort derived by the combination of BS Tx Beams / CDL Clusters / UE Rx beams and time granularity.

- Options 1 and 2 allow the CDL model presented to the UE baseband to consider a reference pattern (to be agreed) and focus on the AI/ML BM prediction based only on L1-RSRP. A second non-AI/ML related test (e.g. L1-RSRP Accuracy Spherical coverage as described in section 2.1.2) is needed to cover the UE spatial/directional performance.

- Options 2, 3 and 4 require that the Complex UE Rx beam/antenna pattern (Codebook) is either common, provided by the UE manufacturer or measured prior to test. In all cases, the computational effort to combine this information with the BS Tx beam and CDL-C channel model is quite high. This would be alleviated in case of Option 2 since the Complex UE Rx beam/antenna pattern (Codebook) would be common to all UEs.

- Option 3 requires a very concrete definition of the data format so it can be used by the TE.

- Option 4 imposes a very high test-time effort in the first stage where each and every beam pattern has to measured.

Given the challenging cons for Option 3 and 4, we recommend focusing on Options 1 and 2.

Proposal 1: (Single-AoA) UE AI/ML BM performance can be tested in single-AoA test setup, by emulating the combination of BS Tx beams, CDL channel and UE Rx beams in the baseband, based on a Reference UE Rx beam/antenna pattern, which is either fixed or complex (codebook). The UE Rx spatial/directional performance can be covered complementarily in a separate test which verifies the UE L1-RSRP spherical coverage accuracy.

As mentioned in Observation 8, this will require studies from RAN4 for the new L1-RSRP spherical coverage requirement.

Proposal 2: (Single-AoA) Study the definition of a L1-RSRP spherical coverage accuracy requirement as well as the respective test methodology, which can be used to verify the UE spatial/directional performance, complementarily to single-AoA baseband testing based on reference UE Rx beam/antenna pattern.

## Multi-AoA Testing

### Approaches for assessing a multi-AoA setup

Multi-AoA setups are discussed for the AI/ML BM testability, mainly due to the spatial emulation of the CDL channel and the usage of direction information for the measured L1-RSRP in the prediction model (as mentioned in Observation 4).  In case of a multi-AoA setup, the crucial question is the test equipment complexity required to properly address the intended requirement, as well as the respective feasibility.

In the previous meetings the main approach which was followed to conclude to a multi-AoA baseline setup was:

Current Approach: CDL Channel Centric reusing the MIMO-OTA / 3D MPAC methodology, i.e.:

- Focus on the CDL channel emulation with a given number of probes per cluster when considering a limited number of Tx beams.

- The analysis showed that, for the required number of Set B Tx beams, the number of probes is unfeasible (>=100).

- On the other side, in a feasible system with a limited number of probes, the number of BS Tx beams resulted very low, down scoping considerably the AI/ML BM performance testing.

Observation 16: (Multi-AoA) Current approach to conclude to a multi-AoA baseline system for AI/ML BM testing, which is CDL channel centric, focuses on the spatial CDL emulation as per MIMO-OTA / 3D MPAC methodology, i.e. it considers a given number of probes per channel cluster and only a limited number of Tx beams.

We think that following the current approach will not conclude to a feasible test system which fulfills the scope of AI/ML BM testing. The reason is that when following this approach, the system required to test the requirement is not feasible, and a feasible system does not have capability to test the intended requirement.

Observation 17: (Multi-AoA) The CDL centric approach will probably not lead to a baseline system for AI/ML BM, since the system required to test the AI/ML BL requirement is not feasible, and a feasible system does not have capability to test the intended AI/ML BM requirement.

Also, the above approach is very focused on the CDL channel emulation, as done for the purposes of MIMO OTA performance. However, it is not clear what is the final impact of such a CDL emulation metric on the end-to-end UE AI/ML BM performance, which is in fact the real scope of the test.

Observation 18: (Multi-AoA) The CDL centric approach is also questionable, since it is not clear what is the quantified impact of such a spatial CDL emulation on the actual UE AI/ML BM performance, which is the real scope of the test.

For these reasons, we propose to focus our analysis more on the AI/ML BM performance aspects and change the approach of the definition of the baseline system, to a more AI/ML BM centric one. Since at this stage not all the factors impacting the AI/ML BM performance are known, we propose to consider systems which exists already (including potential and feasible extensions) as baseline candidates and try to assess their suitability for the UE AI/ML BM performance testing. With this approach, we have at least the system feasibility clarified and ensured, which will then probably facilitate the requirement discussion drastically.

In the following we summarize the proposed new approach:

New Approach: AI/ML BM Performance Centric based on feasible systems i.e.:

- Focus on existing multi-AoA systems, including potential and feasible extensions.

- Analyze by simulations (e.g. as described in [4]) the UE AI/ML BM performance based on these systems, for different combinations of Set A and Set B and channel models.

- Key in this analysis, are simplifications to the channel model, resulting from the concrete system implementation, and the assessment of their impact on the UE AI/ML BM performance.

We think that following this new approach, is more realistic and will ensure a faster and more solid progress towards the finalization of the testability assessment for the multi-AoA setups.

Observation 19: (Multi-AoA) A new AI/ML BM performance centric approach, intending to study the suitability of already feasible multi-AoA systems for the end-to-end UE AI/ML BM performance, is more realistic and leads to more solid progress towards testability assessment for multi-AoA setups.

Proposal 3: (Multi-AoA) Change the approach for the multi-AoA baseline system analysis, from CDL channel centric to UE AI/ML BM performance centric.

Proposal 4: (Multi-AoA) Following the new UE AI/ML BM performance centric approach, analyze the AI/ML BM testing suitability for already existing multi-AoA setups (including potential feasible extensions), by assessing through simulations the UE AI/ML BM performance impact with different Set B / Set A combinations and channel model simplifications as per the concrete test setup implementation.

### Candidate system for the new approach

Since there exist different types of multi-AoA systems, we need first to define candidates which seem to be suitable and worth for the suitability study as per the new approach.

Current commercial implementations of eIFF test system described in TS 38.508-1 [14], Annex B.2.6, deploy mostly 4x CATR with layout shown in Figure 5.

Figure 5: Commercial implementation of eIFF system with 4 CATRs / AoAs

With this implementation, these systems – including feasible upgrades - achieve following technical capabilities:

- 4 Active 2x2 MIMO probes / AoAs, with distinctive angular separation (max 150°)  UE Rx directional testing

- High performing positioner  full positioning / movement flexibility

- Big quiet zone of at least 30cm for all AoAs  real black box testing

- Best far field conditions with lowest MU  highest (L1-RSRP) accuracy measurements

- Reference for legacy requirements  side conditions (RF-RRM /Cross-Feature) ensured

Observation 20: (Multi-AoA) Current implementations of eIFF legacy systems, support with feasible upgrades: 4 Active 2x2 MIMO probes / AoAs with distinctive angular separation up to150°, overall big quiet zone of at least 30cm (black-box testing), lowest MUs (best L1-RSRP measurement accuracy) and ensure preservation of side conditions (RF-RRM, cross-feature).

In addition, these systems have also practical advantages as listed below:

- Available and matured as already broadly used for FR2 testing

- All RF/ Demod / RRM FR2 tests up to release 19 (incl. Multi-Rx, eMIMO) supported

- If suitable for AI/ML  one single system for full conformance testing coverage

- Continuity in the cost & complexity-sensitive FR2 testing ecosystem

Observation 21: (Multi-AoA) These system, have also practical advantages, since they are already matured and broadly used, support conformance testing for all features up to Rel-19, and if suitable for AI/ML BM testing, they avoid the need of an additional system, ensuring continuity in the cost sensitive FR2 testing ecosystem.

Considering all the listed capabilities and advantages, we think that the above system is currently an appropriate candidate to be considered for the AI/ML BM baseline suitability study.

Proposal 5: (Multi-AoA) Define the implementation of eIFF test setup with 4x active 2x2 MIMO probes / AoAs as a candidate for the AI/ML BM baseline system and analyze its suitability (as per Proposal 4).

# Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: There are several factors to be considered in the end-to-end AI/ML BM performance.

Observation 2: The main challenges to conclude to a baseline setup are the identification of factors impacting the AI/ML BM performance, as well as the test system complexity and feasibility.

Observation 3: There seems to be two different assumptions and reference implementation for the AI/ML L1-RSRP model, with and without directional information for the L1-RSRP measurement.

Observation 4: Main drivers for a multi-AoA setup are spatial emulation of the CDL channel and the usage of directional information for the measured L1-RSRP in the prediction model.

Observation 5: (Single-AoA) The multi-AoA environment can be transformed into a single-AoA test for AI/ML BM testing, by including the UE Rx beam/antenna pattern in the combination of BS Tx beams with the CDL channel model to be emulated, without impacting the number of BS Tx beams or the parameters of the CDL channel model.

Observation 6: (Single-AoA) There are 4 options for a multi-AoA to single-AoA test transformation and inclusion of the UE directional performance: Option 1 use a reference fixed UE pattern for the baseband test, and a second separate test for the UE directional performance; Option 2 use a reference complex UE pattern a the baseband test, and a second separate test for the UE directional performance; Option 3 use a complex UE pattern provided by the UE manufacturer, no additional test required; Option 4 use complex UE pattern measured by the TE, no additional test required;

Observation 7: (Single-AoA) L1-RSRP accuracy is currently measured only on a single particular AoA, which in most test cases is the found UE Rx beam peak.

Observation 8: (Single-AoA) A new requirement and test case addressing the L1-RSRP spherical coverage accuracy, will be required to complement the AI/ML BM test based on single direction baseband testing, with reference UE Rx beam/antenna pattern.

Observation 9: (Single-AoA) The analysis of the single-AoA approach must consider the computational load to obtain all the resulting channel models when combining each BS Tx beam, the CDL channel model and each UE Rx beam.

Observation 10: (Single-AoA) In case of Option 2 and 3 (Reference or Manufacturer Complex UE Rx Beam/Antenna pattern), the UE Rx beam/antenna pattern must include magnitude and phase, as well as beam id information (i.e. codebook).

Observation 11: (Single-AoA) In case of Option 4 (TE measures the Complex UE Rx Beam/Antenna pattern), the UE Rx beam/antenna pattern measurement is very time consuming and must include magnitude and phase information, e.g. using an Antenna Test Function.

Observation 12: (Single-AoA) The definition of the format and granularity of the Complex UE Rx beam pattern is key to ensure consistent results among different TE vendors for the single-AoA methodology.

Observation 13: (Single-AoA) The use of fine or rough beams for AI/ML BM prediction may affect the complexity of the data to be considered for the single-AoA methodology, as well as the number of iterations to be calculated.

Observation 14: (Single-AoA) If several UE orientations towards the channel model must be assessed for the single-AoA methodology, the number of combinations and the computational effort rises exponentially.

Observation 15: (Single-AoA) The measurement timing perspective is also a critical aspect for the single-AoA methodology, because not all BS Tx beams might be measured with all UE Rx beams.

Observation 16: (Multi-AoA) Current approach to conclude to a multi-AoA baseline system for AI/ML BM testing, which is CDL channel centric, focuses on the spatial CDL emulation as per MIMO-OTA / 3D MPAC methodology, i.e. it considers a given number of probes per channel cluster and only a limited number of Tx beams.

Observation 17: (Multi-AoA) The CDL centric approach will probably not lead to a baseline system for AI/ML BM, since the system required to test the AI/ML BL requirement is not feasible, and a feasible system does not have capability to test the intended AI/ML BM requirement.

Observation 18: (Multi-AoA) The CDL centric approach is also questionable, since it is not clear what is the quantified impact of such a spatial CDL emulation on the actual UE AI/ML BM performance, which is the real scope of the test.

Observation 19: (Multi-AoA) A new AI/ML BM performance centric approach, intending to study the suitability of already feasible multi-AoA systems for the end-to-end UE AI/ML BM performance, is more realistic and leads to more solid progress towards testability assessment for multi-AoA setups.

Observation 20: (Multi-AoA) Current implementations of eIFF legacy systems, support with feasible upgrades: 4 Active 2x2 MIMO probes / AoAs with distinctive angular separation up to150°, overall big quiet zone of at least 30cm (black-box testing), lowest MUs (best L1-RSRP measurement accuracy) and ensure preservation of side conditions (RF-RRM, cross-feature).

Observation 21: (Multi-AoA) These system, have also practical advantages, since they are already matured and broadly used, support conformance testing for all features up to Rel-19, and if suitable for AI/ML BM testing, they avoid the need of an additional system, ensuring continuity in the cost sensitive FR2 testing ecosystem.

Proposal 1: (Single-AoA) UE AI/ML BM performance can be tested in single-AoA test setup, by emulating the combination of BS Tx beams, CDL channel and UE Rx beams in the baseband, based on a Reference UE Rx beam/antenna pattern, which is either fixed or complex (codebook). The UE Rx spatial/directional performance can be covered complementarily in a separate test which verifies the UE L1-RSRP spherical coverage accuracy.

Proposal 2: (Single-AoA) Study the definition of a L1-RSRP spherical coverage accuracy requirement as well as the respective test methodology, which can be used to verify the UE spatial/directional performance, complementarily to single-AoA baseband testing based on reference UE Rx beam/antenna pattern.

Proposal 3: (Multi-AoA) Change the approach for the multi-AoA baseline system analysis, from CDL channel centric to UE AI/ML BM performance centric.

Proposal 4: (Multi-AoA) Following the new UE AI/ML BM performance centric approach, analyze the AI/ML BM testing suitability for already existing multi-AoA setups (including potential feasible extensions), by assessing through simulations the UE AI/ML BM performance impact with different Set B / Set A combinations and channel model simplifications as per the concrete test setup implementation.

Proposal 5: (Multi-AoA) Define the implementation of eIFF test setup with 4x active 2x2 MIMO probes / AoAs as a candidate for the AI/ML BM baseline system and analyze its suitability (as per Proposal 4).

# References

- R4-2409739, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2411587, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112, August 2024

- R4-2416391, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112bis, October 2024

- R4-2418941, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #113, November 2024

- R4-2407333, “On AI/ML RAN4 Use Cases: Beam Management”, Qualcomm, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, 3GPP TSG RAN WG4 Meeting #112bis, October 2024

- R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm, 3GPP TSG RAN WG4 Meeting #113, November 2024

- 3GPP TR 38.827 V16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”

- 3GPP TR 38.843 V18.0.0, “Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”

- 3GPP TR 38.901 V18.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”

- 3GPP TS 38.101-4 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”

- 3GPP TS 38.133 V18.8.0, “NR; Requirements for support of radio resource management”

- 3GPP TS 38.151 V18.3.0, “NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements”

- 3GPP TS 38.508-1 V18.5.0, “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”