# 3GPP TSG-RAN WG4 Meeting #114		  R4-2501730

# Athens, Greece, 17– 21 February, 2025

Agenda item:			7.15.2

Source:	Qualcomm Incorporated

Title:	TP to TR 38.771 on test methodology for STxMP

Document for:	Approval

# 1  Introduction

In this paper, we provide text proposal to TR 38.771 on test methodology for STxMP

# 2  Text propsoal

## <<Start of Change>>

## 5.1	General

<Editor’s note: general aspects related to the UE RF testing methodology for STxMP can be captured in this clause>

The test methodology defied in this document only applies for STxMP testing with two-layer transmission (i.e., one layer per TCI). The methodology for single layer transmission needs to be further studied with the corresponding core requirements.

## 5.2	Measurement setup

<Editor’s note: outcome of measurement setup for STxMP UE RF testing can be captured in this clause>

Through the analysis of system complexity, feasibility of guaranteeing the same relative angular orientations between specific UE test directions and probes, etc, it was confirmed that multi-Rx system is suitable for STxMP testing. Therefore, the measurement setup with full degrees of freedom for AoA1 with fixed angular Offset(s) between AoA1 and AoA2 and with all probes located on the xz plane specified in [TR 38.871] can be reused for STxMP testing.

The example multi-AoA measurement setups are similar to baseline setups for 2 AoA RRM testing [TS 38.508-1], see Clause B.2, as illustrated in Figure 5.2-1 or FR2 MIMO testing [TS 38.151], see Clause B.2, as illustrated in Figure 5.2-2.

Figure 5.2-1: Example RRM baseline system with two simultaneously active AoA using top: DFF setup, centre: Enhanced IFF setup, bottom: IFF+DFF setup.

Figure 5.2-2: Example FR2 MIMO OTA 3D-MPAC system.

## 5.3	Test System Aspects

<Editor’s note: outcome of  test system aspects for STxMP UE RF testing can be captured in this clause>

### 5.3.1	Absolute Probe Locations

Given the rotational symmetry, two systems shown in Figure 5.2-1 and Figure 5.2-2 with different absolute probe locations shall still be able to guarantee the same relative angular orientations between specific UE test directions and probes provided the baseline system with probes in the xz plane and angular sweeps of 0≤q<360° and 0≤f<180° are supported.

### 5.3.2	AoA1-AoA2 DL Orientation Vectors

It was agreed that the directionality of the AoA1-AoA2 DL orientation vectors matters. Given RAN4 has confirmed that the baseline multi-RX/STxMP system is suitable to guarantee the same relative angular separations between specific UE test orientations and probes, similar as multi-Rx test system specified in [TR 38.871], the impact of directionality of the AoA1-AoA2 orientation vectors shall also be accommodated in the measurement to ensure no testing bias is introduced.

### 5.3.3	Calibration measurement procedure

The calibration measurement per DFF probe and IFF probe specified in [TR 38.871] can be reused for STxMP testing.

## <<End of Change>>