3GPP TSG-RAN WG4 Meeting #114	R4-2501742

Athens, Greece, 17th – 21st February, 2025

Source:	Qualcomm Incorporated

Title:	MPR for Intra-band contiguous UL CA for PC1.5

Agenda item:	7.1.1.1.2

Document for:	Approval

# 1.	Introduction

For this WI companies were requested to measure/simulate MPR/AMPR numbers for several intra-band contiguous and non-contiguous scenarios. In this paper we present details on the MPR measurements that were done on contiguous CC with contiguous RBs.

# 2. 	Discussion

In RAN4#111 a WF was created that stated the following [1]:

- Evaluate MPR/A-MPR numbers for contiguous CA first and then work on non-contiguous CA MPR/A-MPR afterwards.

- The intra-band PC1.5 ULCA R19 work focusses on UE implementing with two 26dBm PAs. Architectures requiring a 29dBm PA are not specified but can be implemented by fulfilling the requirements based on two 26dBm PAs.

- For intra-band contiguous ULCA w/ and w/o UL MIMO, the Architecture#2 is prioritized for MPR/A-MPR evaluation.

  - If time is allowed, architecture #1 can be evaluated.

| Architecture | Description | Indicated capability | Whether can support UL MIMO | Applicable cases |
| --- | --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | Contiguous CA |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD | Yes | Contiguous CA |

In this paper we describe our measurements for architecture#2 given in the above table for contiguous CA with contiguous RBs. The MPR for band n41C was measured using two 26 dBm PAs setup with an antenna-to-antenna isolation of 10 dB. The contiguous CCs with contiguous RBs scenario have RBs allocated in one CC exclusively, or in both CCs with the RB allocation being contiguous across the CC1 and CC2 RB boundary. Numerous RB allocations belonging to this scenario were measured for MPR performance. For the cases where RBs were present in both CCs their PSDs were set to be equal. Based on these measurements the following MPRs are suggested for intra-band contiguous CC with contiguous RBs for bandwidth class C operation.

| Modulation |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 4.0 | 8.5 |
|  | QPSK | 4.0 | 8.5 |
|  | 16QAM | 4.0 | 8.5 |
|  | 64QAM | 6.5 | 8.5 |
|  | 256QAM | 8.5 | 9.0 |
| CP-OFDM | QPSK | 4.7 | 9.0 |
|  | 16QAM | 4.7 | 9.0 |
|  | 64QAM | 6.0 | 9.0 |
|  | 256QAM | 8.0 | 9.0 |

Table 1 – MPR for contiguous CC and contiguous RBs

It is proposed that the above MPRs be used for contiguous CC with contiguous RBs.

Proposal 1: For bandwidth class C for contiguous CC with contiguous RBs use the following MPR table

| Modulation |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 4.0 | 8.5 |
|  | QPSK | 4.0 | 8.5 |
|  | 16QAM | 4.0 | 8.5 |
|  | 64QAM | 6.5 | 8.5 |
|  | 256QAM | 8.5 | 9.0 |
| CP-OFDM | QPSK | 4.7 | 9.0 |
|  | 16QAM | 4.7 | 9.0 |
|  | 64QAM | 6.0 | 9.0 |
|  | 256QAM | 8.0 | 9.0 |

Currently we are also measuring A-MPR values for CA_NS_04. In the spec we see that this NS case has both a SEM requirement captured in section 6.5A.2.3.1.1 and a spurious emissions requirement captured in section 6.5A.3.3.1.1 [2].

Table 3: SEM requirement for CA_NS_04

Table 3: Spurious emissions requirement for CA_NS_04

Further requirements for CA_NS_04 are given in 6.2.3.1.1.1 for contiguous and non-contiguous allocations [2]. An inspection of these sections shows the A-MPR is based on several factors such as inner/outer allocations, RBstart, BWChannel_CA, Fedge, low and BW class. For example, for certain inner allocations the A-MPR=MPR while for certain outer allocations A-MPR=MPR+1.5dB. We anticipate having to create additional relationships between A-MPR and MPR for PC1.5 ULCA. The excerpt below for intra-band CA contiguous RB allocations shows the current A-MPR formulation for CA_NS_04 [2] for single carrier PC3 and PC2 for BW class B and BW class C.

6.2A.3.1.1.1.1	Contiguous allocations

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in clause 6.2A.2.1,  then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in clause 6.2A.2.1,

then A-MPR = MPR+1.5dB for BW Class B A-MPR = MPR for BW class C.

Where

-	MPR is the MPR as defined in Table 6.2A.2.1-1, Table 6.2A.2.1-1a and Table 6.2A.2.1-1b for PC3 and PC2 respectively and the respective CA bandwidth class

-	 AMPRcc is defined as the PC3_A2 or PC2_A4 AMPR in Table 6.2.3.2-2 for PC3 and PC2 respectively

Observation 1: Many relationships exist between A-MPR and MPR for CA_NS_04 in the current standard based on several variables such as RB allocation, BW class, CA bandwidth and frequency location. It is anticipated that additional relationships may need to be developed for PC1.5 ULCA A-MPR.

The intra-band noncontiguous RB A-MPR formulation is more complex and consists of an A-MPR = AMPRIM3 requirement to meet the -25dBm/MHz specification when the IM3 created by the 2 CCs falls in the -25dBm/MHz regions of  table 2 or table 3. The detailed set of criteria for evaluating a given RB allocation is given in the following excerpt [2]. It is anticipated that these rules may be modified for PC1.5 ULCA

6.2A.3.1.1.1.2	Non-contiguous allocations

For intra-band contiguous CA_n41B and CA_n41C and it receives IE CA_ NS_04, the UE determines the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified by AMPRIM3 to meet -25dBm/MHz when IM3 falls in -25dBm/MHz region of Table 6.5A.2.3.1.1-1 or Table 6.5A.3.3.1.1-1. And uses MPR for all other cases.

The UE determines the AMPR type as follows:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA ≥ 2490.5 MHz,

-	if allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

-	if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.1-2, Table 6.2A.2.1-3 and Table 6.2A.2.1-4 for PC3 and PC2 respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.1.1

# Conclusion

In this paper we present our measurements for contiguous CC with contiguous RB MPRs for various modulations and waveform types.

Proposal 1: For bandwidth class C for contiguous CC with contiguous RBs use the following MPR table

| Modulation |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 4.0 | 8.5 |
|  | QPSK | 4.0 | 8.5 |
|  | 16QAM | 4.0 | 8.5 |
|  | 64QAM | 6.5 | 8.5 |
|  | 256QAM | 8.5 | 9.0 |
| CP-OFDM | QPSK | 4.7 | 9.0 |
|  | 16QAM | 4.7 | 9.0 |
|  | 64QAM | 6.0 | 9.0 |
|  | 256QAM | 8.0 | 9.0 |

Observation 1: Many relationships exist between A-MPR and MPR for CA_NS_04 in the current standard based on several variables such as RB allocation, BW class, CA bandwidth and frequency location. It is anticipated that additional relationships may need to be developed for PC1.5 ULCA A-MPR.

# References

[1] R4-2410565, WF on higher power UE, May 20th – May 24th, 2024

[2] TS38.101-1, User Equipment (UE) radio transmission and reception;Part 1: Range 1 Standalone, V18.7.0