3GPP TSG-RAN WG4 Meeting # 114 	    R4-2501758

Athens, Greece, 17  – 21 February 2025

Agenda Item:	7.19.4

Source: 	Ericsson

Title:	On requirements for AI/ML based positioning

Document for:	Approval

# Introduction

In this contribution requirements for AI/ML based positioning are discussed. Issues related to the positioning use case 1 from RAN4 point of view are highlighted.

# Discussion

In RAN4#112bis meeting it was agreed to bring feasibility of defining requirements for positioning use case 1. The agreement reached is generic, and is valid for both core requirement, and performance requirement discussions. Since, defining the performance requirement is more challenging than defining the core requirement for use case 1, in this contribution feasibility issues related to defining the performance requirement for use case 1 are addressed.

Agreement from RAN4#112bis [1]

- Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible.

Accuracy of the estimated UE position, among others, highly depends on the accuracy of each measurement used to estimate the UE position, and the scenario in which the UE is located while performing the measurements to estimate its location. To give a better overview, simulation results showing the accuracy of UE positioning in urban macro (UMa), urban micro (UMi), indoor open office (IOO) and indoor factory (InF) scenarios are presented. Parameters specific to each of these scenarios are taken from TR 38.857. As the accuracy of the positioning measurements depend on the PRS processing capability of the UE, PRS bandwidth in all simulations is fixed to 100MHz. Further to this, the simulation results discussed in this paper do not use AI/ML models to infer UE location and the results are presented for DL-TDoA based method in FR1.

Figure 1 shows the comparison of positioning accuracies in UMa, UMi, and IOO scenarios. As can be observed the positioning accuracy is different in different scenarios, where the positioning accuracy is highest in IOO and lowest in UMa. Based on this evaluation, it is fair to conclude that the achievable positioning accuracy is scenario specific, and the same level of positioning accuracy cannot be observed in different scenarios.

Figure 1.  Positioning accuracy in urban macro, urban micro, and indoor open office scenarios in FR1.

Figure 2 and Figure 3 show positioning accuracies in indoor factory (InF) scenarios. Deployment considered for InF simulations is shown in Figure 4. Results in Figure 2 are for InF dense high (InF-DH) scenario that characterizes presence of dense clutters in InF environment. Results in Figure 3 are for InF sparse high (InF-SH) scenario that characterizes an InF environment with significantly less dense clutters compared to InF-DH scenario. Therefore, the number of LoS links between the UE performing measurements on PRS resources transmitted by 18 TRPs that are deployed as shown in Figure 4 is significantly high in InF-SH in comparison to InF-DH scenario. Due to the availability of large number of LoS links most of the positioning measurements are not impacted by the NLoS excess delay and therefore higher positioning accuracy is achieved in InF-SH scenario in comparison to the InF-DH scenario. Another important observation that can be made in the results for InF scenarios is the impact of geometric dilution of precision. To highlight the impact of geometric dilution of precision, positioning accuracy of UEs that are at the center of the InF deployments while performing the measurements are denoted by the curve for UE(s) in convex hull in Figure 2 and Figure 3.

Observation 1: Achievable positioning accuracy is scenario specific, and the same level of positioning accuracy cannot be observed in different scenarios.

Figure 2. Positioning accuracy in indoor factory-dense high scenario in FR1.

Figure 3.  Positioning accuracy in indoor factory-sparse high scenario in FR1.

InF-DH: W = 120m, L = 60m, and D = 20m.

InF-SH: W = 300m, L = 150m, and D = 50m.

InF-DH: W = 120m, L = 60m, and D = 20m.

InF-SH: W = 300m, L = 150m, and D = 50m.

Figure 4.  Indoor factory (InF) deployment. Black dots denote location of deployed TRPs.

During the Rel. 18 study item, achievable positioning accuracy of direct AI/ML based methods were studied in InF scenarios. Based on the results submitted by the participating companies, it was observed that the direct AI/ML positioning can significantly improve the positioning accuracy in comparison to the existing RAT-based positioning methods when the generalization aspects are not considered [3]. Generalization capability of direct AI/ML positioning was evaluated when the model is trained in one InF scenario and tested in different InF scenario, where InF scenarios considered for evaluation were different in terms of the clutter parameters and the density of the clutters present in the InF environment [3]. This signifies that the observation 1 also holds true for the direct AI/ML positioning. We therefore do not see a feasibility of defining a general requirement in terms of the accuracy of estimated UE position to be met by UE in any scenario for case 1. Furthermore, it is also our observation that for UE based positioning, UE can also use measurements from sensors such as WLAN, Bluetooth, A-GNSS etc. to derive its location. UE can in principle combine measurements from different sensors and achieve positioning accuracy requirements mandated by TS 22.261 to support use cases requiring different service levels [4]. Defining requirements for this type of scenario is however outside of RAN4 scope.

Proposal 1: RAN4 to conclude that defining requirement in terms of the accuracy of estimated UE position for case 1 is not feasible.

For the legacy methods RAN4 defined performance requirement for positioning measurements depends on the propagation condition, SNR/SINR value and the PRS processing capability of the UE. Thus, defined accuracy requirements are generic and are not scenario dependent. The same approach can be considered to define the accuracy requirement for case 1. Based on this approach, the requirement is however defined for the model input rather than the model output.

Proposal 2: RAN4 to define requirements for model input for case 1. Details are FFS.

# Summary

In this contribution, feasibility of defining requirements for case 1 is discussed. The observations and proposal below summarize the discussion presented in this paper.

Observation 1: Achievable positioning accuracy is scenario specific, and the same level of positioning accuracy cannot be observed in different scenarios.

Proposal 1: RAN4 to conclude that defining requirement in terms of the accuracy of estimated UE position for case 1 is not feasible.

Proposal 2: RAN4 to define requirements for model input for case 1. Details are FFS.

# References

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated.

- TR 38.857, Study on NR Positioning Enhancements (Release 17), 3GPP.

- TR 38.847, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 3GPP.

- TS 22.261, Service requirements for the 5G system (Release 18), 3GPP.