3GPP TSG-RAN WG4 Meeting # 114	R4-2501774

Athens, Greece, 17th – 21st February 2025

Title:	TP TR 36.719-01-01 adding CA_48A-106A, CA_48C-106A and CA_48A-48A-106A

Source:	Ericsson, Anterix

Agenda item:	6.4.2

Document for:	Approval

# 1	Introduction

This contribution is a text proposal for TR 36.719-01-01[1] to add CA_48A-106A, CA_48C-106A and CA_48A-48A-106A.

# 2. Reference

[1] TR36.719-01-01, Rel-19 LTE Advanced CA for x bands DL (1<=x<=6) with y bands UL (y=1, 2) V0.0.2

# Text Proposal

# ---Start of changes---

### 5.4.x	CA_48-106

#### 5.4.x.1	Channel bandwidths per operating band for CA

Table 5.4.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| 106 | 896 MHz | – | 901 MHz | 935 MHz | – | 940 MHz | FDD |

Table 5.4.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_48A-106A | CA_48A-106A | 48 |  |  | Yes | Yes | Yes | Yes | 23 | 0 |
|  |  | 106 | Yes | Yes |  |  |  |  |  |  |
| CA_48C-106A | CA_48A-106A | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 43 | 0 |
|  |  | 106 | Yes | Yes |  |  |  |  |  |  |
| CA_48A-48A-106A | CA_48A-106A | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 43 | 0 |
|  |  | 106 | Yes | Yes |  |  |  |  |  |  |

#### 5.4.x.2	Co-existence studies

Table 5.4.x.2-1 summarizes frequency ranges where IMD components due to  CA_48A-106A could fall.

Table 5.4.x.2-1: IMD analysis CA_48A-106A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 896 | 901 | 3550 | 3700 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2804 | 2649 | 4446 | 4601 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1908 | 1748 | 6199 | 6504 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5342 | 5502 | 7996 | 8301 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5608 | 5298 | 8892 | 9202 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1012 | 847 | 9749 | 10204 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6238 | 6403 | 11546 | 12001 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13904 | 13299 | 54 | 116 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15096 | 15701 | 7134 | 7304 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9308 | 8848 | 4397 | 4712 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12442 | 12902 | 9788 | 10103 |

Based on Table 5.4.x.2-1, own Rx impact is the followings:

- 4th order IMD products generated by band 48 and band 106 uplink may fall into own Rx of band 106.

#### 5.4.x.3	∆TIB and ∆RIB values

∆TIB and ∆RIB values for CA_8-48 are reused.

Table 5.3.2.3-1: IB,c

| CA_48-106 | 48 | 0.8 |
| --- | --- | --- |
|  | 106 | 0.6 |

Table 5.3.2.3-2: R IB,c

| CA_48-106 | 48 | 0.5 |
| --- | --- | --- |
|  | 106 | 0.2 |

#### 5.4.x.4	REFSENS requirements

MSD values for band 106 is reused from CA_n8-n78.

Table 5.4.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | Uplink CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_48A-106A CA_48C-106A CA_48A-48A-106A | CA_48A-106A | 48 | 3633 | 10 | 50 | N/A | N/A | TDD | N/A |
|  |  | 106 | 898.5 | 5 | 25 | 937.5 | 8.3 | FDD | IMD4 |

---End of changes---