3GPP TSG-RAN WG4 Meeting # 114							R4-2501792

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.24.1

Source: 	OPPO

Title: 	General aspects

Document for:	Approval

- Introduction

The new WID [1] for ambient IOT has been approved in the RAN#106 meeting. In this paper we try to give discussion on the RF requirements for device 1 as per the WID request and also based on the outcome of the study item as captured in the TR 38.769.

- Discussion

As per the WID general scope is captured below. The deployment scenario 1 with topology 1 and also R2D in DL spectrum and D2R and CW in UL spectrum. The spectrum deployment is in-band to NR and standalone with A-IOT BS located indoor.

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- The overall objective shall be to standardize the following Ambient IoT device:

Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Deployment scenario 1 with Topology 1, according to D1T1-B.

- FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

- Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

- Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command).

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769:

  - Case 1-4 for D1T1-B

- Proximity determination via Solution 1 in TR 38.769 only.

- Device (un)availability via Direction 1 in TR 38.769 only.

With the above scope, as per the RAN4 co-existence study, the scenario 2-1 and 2-2 as captured in TR 38.769 subclause 6.6.5.3 and 6.6.5.4 is the exact case for the WID scope.

Observation 1: The coexistence scenario 2-1 and 2-2 is the WID scope.

The summary of these two scenarios has been captured below from TR 38.769. It can be found that both NR UE indoor and NR UE outdoor has been simulated. Firstly, for the NR UE only outdoor case, the co-existence conclusion is quite good that for all the cases the degradation is negligible. However, for NR UE indoor scenario 2-2, the SINR degradation of reader and also the throughput degradation of NR DL is observed.

Observation 2: for NR UE indoor scenario 2-2, the SINR degradation of reader and also the throughput degradation of NR DL is observed.

.6.5.3	Scenario 2-1: D1T1-B- NR UE only outdoor (R2D: DL, CW2D/ D2R: UL)

Scenario 2-1 considers D1T1 topology, all the NR UE outdoors and Readers/CW non-collocated, with DL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.3-1.

Table 6.6.5.3-1: Scenario 2-1 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
| f | NR UL | reader | SINR degradation is less than 1dB. Inter-system interference is negligible. |
| c | Reader | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

6.6.5.4	Scenario 2-2: D1T1-B- NR UE indoor (R2D: DL, CW2D/ D2R: UL)

Scenario 2-2 considers D1T1 topology, 10% of the NR UE indoors and Readers/CW non-collocated, with DL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.4-1.

Table 6.6.5.4-1: Scenario 2-2 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
| f | NR UL | reader | Under the assumption of 10% and 100% NR UE indoor, significant SINR degradation is observed |
| c | Reader | NR DL | Under the assumption of 10% and 100% NR UE indoor, NR DL throughput degradation is observed. |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

In such case, it is encouraged to further study possible solutions for scenario 2-2 when NR UE indoor to protect the reader and also NR DL.

Proposal: It is encouraged to further study possible solutions for scenario 2-2 when NR UE indoor to protect the reader and also NR D.

3	Conclusions

In this paper, we give discussion on the general aspect of A-IOT, the observations and proposals are as below:

Observation 1: The coexistence scenario 2-1 and 2-2 is the WID scope.

Observation 2: for NR UE indoor scenario 2-2, the SINR degradation of reader and also the throughput degradation of NR DL is observed.

Proposal: It is encouraged to further study possible solutions for scenario 2-2 when NR UE indoor to protect the reader and also NR D.

4 References

[1] RP-243326	New Work Item: Solutions for Ambient IoT (Internet of Things) in NR