3GPP TSG-RAN4 Meeting #114	R4-2501815

# Athens, GR, 17th – 21st February, 2025

Title:	Discussion on Ku-band UE RF requirements

Source:	Samsung

Agenda item:	7.11.4

Document for:	Approval

1	Introduction

In the RAN#106 meeting, the WID of introduction of Ku bands for NR NTN was revised and approved in [2], where it further incorporate the RF requirements for mobile VSAT served by NGSO and GSO into the scope. The changes are as follows:

Given the co-existence work of Ku-band is still on-going, in this paper, we will focus on discussing the UE RF impacts of introducing NGSO mobile VSAT in NR-NTN.

2	Mobile VSAT for NGSO

The revised WID [2] suggested the following to be considered to introduce the new VSAT type for mobile VSAT served by both NGSO and GSO:

The potential impact of a VSAT testability study on how Ku band VSAT core and performance requirements might be defined is not in this WID scope. Note: The Release 18 NTN scope for operation above 10 GHz, does not support Mobile VSATs (ESIMs) connected to NGSO Satellites. This may be considered at a later date. The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 4 and 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability Carrier aggregation is not currently in scope for this spectrum only WI but will be necessary to enable supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Note: Since only VSAT Type 4 and 5 are in scope, any implementation of type 2 (Fixed VSAT communicating with GSO and LEO with electronic steering antenna) can be based on the same RF requirements as VSAT type 5.

There’s some ambiguity in the current WID, with regards to the introduction of this new mobile VSAT for NGSO. The current WID covers Type 4 and Type 5 in the scope, where both of these two types are not for NGSO. And the additional ‘Note’ above refer to the type 2, where Type 2 is for fixed VSAT. None of these Type 2, 4 and 5 matches the additional objectives to specify mobile VSAT served by NGSO.

The first and most important thing is to clarify this new objective, which is “Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements”.

Proposal 1: To clarify how to start the work for the new objective of “Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements”. And whether a new VSAT type is expected, or to update the definition of existing Type 5?

Regardless of how to incorporate the new VSAT type for this new objective, we believe the folowing impacts may need to be considered for mobile VSAT by NGSO.

Observation 1: The potential impacted requirements

- The ACLR/ACS can re-use the co-ex results with current framwork;

- The MOP of such type may need to consider the previous Type 2 and Type 3, if NGSO only mobile VSAT is separately introduced;

- The off axis EIRP emission requirement of such type may be divided to two sets of requirement, if the mobile VSAT off-axis regulatory requirements differs from GSO and NGSO.

- Similar as above to spurious emission requirement for both on-axis and off-axis.

- The antenna pointing accuracy requirements may have similar impacts when considering GSO and NGSO.

3	Conclusion

In this contribution, we provide views on UE RF by introducing mobile VSAT for NGSO:

Proposal 1: To clarify how to start the work for the new objective of “Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements”. And whether a new VSAT type is expected, or to update the definition of existing Type 5?

Observation 1: The potential impacted requirements

- The ACLR/ACS can re-use the co-ex results with current framwork;

- The MOP of such type may need to consider the previous Type 2 and Type 3, if NGSO only mobile VSAT is separately introduced;

- The off axis EIRP emission requirement of such type may be divided to two sets of requirement, if the mobile VSAT off-axis regulatory requirements differs from GSO and NGSO.

- Similar as above to spurious emission requirement for both on-axis and off-axis.

- The antenna pointing accuracy requirements may have similar impacts when considering GSO and NGSO.

4	Reference

[1] R4-2419790, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

[2] RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT