3GPP TSG-RAN WG4 Meeting #114                                         R4-2501837

Athens, Greece, February 17 - 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On 6Rx SRS antenna switching requirements

Agenda Item:	7.1.1.3.3

Document for:	Approval

Introduction

During RAN4#113, FR1 6Rx UE RF requirements have been discussed. Following conclusion regarding ΔTRxSRS can be found in [1].

- Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 1-1-1: Whether to consider a subset of available patterns for 3T6R AS-SRS Way Forward: RAN4 to further discuss the following options. - Option 1: Consider a subset of available patterns for 3T6R AS-SRS. FFS on the subset details. - Option 2: Consider all available patterns for 3T6R AS-SRS. - Sub-topic 2-2: ΔTRxSRS values Issue 1-2-1: Proposed ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 Way Forward: RAN4 to check whether the ∆TRxSRS values in the table below could be agreed. The average values from company proposals, as well as the existing values for other AS-SRS configurations, were considered as the starting point and were rounded to align with 0.5dB increments to keep consistency with the existing specification approach. Table: ΔTRxSRS values Operating Bands; ΔTRxSRS t3r6 (dB); ΔTRxSRS t1r6-t3r6 (dB); ΔTRxSRS t2r6-t3r6 (dB); ΔTRxSRS t1r6-t2r6-t3r6 (dB). Band n41, n77, n78; 3.0; 4.5; 4.0; 5.5. Band n79; 4.5; 6.0; 5.5; 7.0. Band n104; 5.0; 7.0; 6.0; 8.0.

In this contribution, we would like to share our views regarding this topic.

Discussion

In last meeting, we proposed to consider a subset of patterns for 3T6R for requirements definition. This proposal is motivated by an observation that the corresponding values for xT6R which definitely come from valid analysis based on implementation assumptions are close to some of the specified requirements e.g. 2t4r versus 3t6r. We can understand that only specify a subset to make the current definition of ΔTRxSRS less lengthy may not seem fair comparing to completeness. Also, whether to test such requirement still needs more discussion so another argument regarding test burden alleviation cannot be used here. Anyway, we can live with all available patterns for 3T6R AS-SRS for the time being.

Proposal 1: Consider all available patterns for 3T6R AS-SRS.

As for the proposed values as captured in the WF, we think the whole table is acceptable especially considering that band n104 has been taken care of.

Proposal 2: Support to specify the 3T6R ΔTRxSRS as the values captured in the WF.

Conclusion

In this contribution we discussed on the 6Rx SRS antenna switching requirements. We have the following proposals:

Proposal 1: Consider all available patterns for 3T6R AS-SRS.

Proposal 2: Support to specify the 3T6R ΔTRxSRS as the values captured in the WF.

- References

- R4-2420376, “WF on 6Rx UE requirements”, AT&T, 3GPP TSG-RAN4 Meeting #113, AT&T, Orlando, USA, November 18-22, 2024.