3GPP TSG-RAN WG4 Meeting #114 	R4-2501850

Athens, Greece, 17 – 21 February, 2024

Source: 	Huawei, HiSilicon

Title: 	Further discussion on Expected EIRP test

Agenda Item:	7.13.4.1

Document for:	Approval

1 Introduction

In the last meeting RAN4#113 the WF [1] had a number of open issues regarding the conformance testing of the EEIRP requirement, including :

Issue 2-3: Test beams for EEIRP mask

There have been a number of offline meetings arranged by the rapporteur since the last meeting to discuss how this can be done. A number of key issues have become apparent

- The amount of data when comparing many potential test direction sets over 7 angular bins and potentially a number of different antenna configurations and steering range definitions make comparison difficult. Various methods of forming an ”average” metric have been used with different contributors using different methods often making direct comparisons difficult.

Issue 2-5: Conformance testing for different mechanical tilts

Issue 2-8: Measurement uncertainty

Issue 2-9: CI

2 Discussion

2.1	Issue 2-3: Test beams for EEIRP mask

2.1.2	Average Metric

The average metric used should be appropriate to the data, the values themselves are not as important as how they compare between test beam direction sets. So far 4 different metrics have been used:

RMS average of the error ins dB

RMSdB=E0-52+E5-102+E10-152+E15-202+E20-302+E30-602+E60-90270.5

ABSAVdB=E0-5+E5-10+E10-15+E15-20+E20-30+E30-60+E60-907

Average of the absolute dB values

RealAVdB=10*log1010E0-510+10E5-1010+10E10-1510+10E15-2010+10E20-3010+10E30-6010+10E60-90107

RMS average of the absolute error in linear units (displayed in dB)

Average of the error in W

RealRMSdB=10*log10∆0-52+ ∆5-102+ ∆10-152+ ∆15-202+ ∆20-302+ ∆30-602+ ∆60-90270.5

Ebin=EEIRP_TBSbin-EEIRP_REFbin

Where

∆bin=10EEIRP_TBSbin10-10EEIRP_REFbin10

The 1st 3 method all use the absolute error in dB and form slightly different averages, the last method uses the error in W, as such high power angular bins (such as the 0-5deg) bin dominate excessively. As such we do not believe that is suitable.

Considering the other 3 metrics, in the range of dB errors e are experiencing (approx. 0 to 2dB), the difference in the resulting average is not huge as such its probably not very important which is used.

In addition to the average metric the over estimate and underestimate values are also compared to ensure that no excessive error in individual angular bins are experienced.

Observation 1: Any of the dB averages are suitable for the analysis, over estimate and under estimate values are also considered.

2.1.2	Test cases from offline meetings

The test cases agreed so far are to use what we refer to as ANT4 (see table 2.1.2-1) with 2 different steering range configurations.

Table 2.1.2-1: Antenna parameters

The steering range configurations are:

Steering range 1

Mechanical tilt = 0deg

Electrical steering range = 0 to -12deg (0deg is perpendicular to the antenna, negative direction is towards the ground)

Electrical pre-set = -6deg

Steering range 2

Mechanical tilt = 0deg

Electrical steering range = 0 to -16deg (0deg is perpendicular to the antenna, negative direction is towards the ground)

Electrical pre-set = -6deg

It has been discussed that in practice Steering range 2 may need some additional mechanical tilt applied as such we have derived Steering range 3 which has 4 deg mechanical tilt.

Steering range 3

Mechanical tilt = 4deg

Electrical steering range = 0 to -16deg (0deg is perpendicular to the antenna, negative direction is towards the ground)

Electrical pre-set = -6deg

The test direction sets under discussion have evolved based on the previous discussions, test directions are defined in a grid arrangement (Mvertical , NHorizontal). Each Grid square can have a 5x5 sub grid applied with pseudo random locations.

(Please note in previous papers I have used the opposite way to refer to the grid i.e. (NHorizontal, Mvertical), I have changed to try to match the way the grids are represented in by other contributors)

The test beam direction sets currently under consideration are shown in figure 2.2-1:

Figure 2.1.2-1. Test beam direction sets

Grids shown are between -1 to +1 in x and y dimensions.

The (x,y) value (of the centre of the marked square) is multiplied by the electrical steering value.

The mid point in the vertical direction is equivalent to the mechanical steering direction.

Table 2.1.2-2: Results for ANT4, Steering range 1

Table 2.1.2-3: Results for ANT 4, Steering range 2

Table 2.1.2-4: Results for ANT 4, Steering range 3

Table 2.1.2-5: Results for ANT 1, Steering range 1

Table 2.1.2-6: Results for ANT 1, Steering range 2

Already there is too much data to take in easily, to try reduce this down further and summarize the more interesting test beam direction sets in table 2.1.2-7.

Table 2.1.2-7: Summary of EEIRP results

In general it is difficult to notice any big difference between the test beam directions sets.

The pseudo random profile ZTE1b is noticeably better for the rectangular steering range but is spoor for the ellipse.

Given these results the main conclusion is that it is very difficult to identify any test beam direction set as being significantly better than any other. As has been seen previously it seems if there are at least 4 points in elevation then the errors are small.

Observation 2: Using the agreed antenna definition and steering ranges none of the test beam direction sets are significantly better than any other.

As the results for the test beam direction sets are all very similar we think it is important to keep the number of test directions reasonable and simple to define. As such we still recommend the 16 point fixed (4x4) test beam direction set

Proposal 1: Use the 16 point fixed (4x4) test beam direction set.

2.1.3	Extended test cases

Over the course of the work item we have looked at a number of different antenna configurations and steering ranges which are different to those RAN4 agreed t compare. By using this larger data set we took a different approach to both analysing the data and forming a usable CI value to see if this would yield more insight.

Taking antennas 1,4 and 5 in table 2.1-1and studying a number of other steering ranges.

Steering range h1

Mechanical tilt = 0deg

Electrical steering range = 0 to -20deg

Electrical pre-set = -6deg

Steering range h2

Mechanical tilt = 8deg

Electrical steering range = 5 to -5deg

Electrical pre-set = 0deg

Electrical steering 0deg is perpendicular to the antenna, negative direction is towards the ground

Mechanical tilt is positive in direction of ground – opposite to electrical steering – confusing I know but convention.

Considering the error for each angular bin independently we are now gathering hundreds of data points for may different potential BS deployments as such the statistics of the results are starting to become usable.

Looking for example at the results for the 20 point 4x5 test beam directions set we have the following:

| 20 point fixed (4x5) |  |  |
| --- | --- | --- |
| Max | 2.15 | dB |
| Min | -3.70 | dB |
| Average | -0.03 | dB |
| Std dev | 0.74 | dB |

Figure 2.3-1. EEIRP errors for 20 point (4x5) test beam direction set

The average in this case is a true average and is almost zero, implying that the errors are evenly spaced around zero, as such they can be considered fair, neither favouring underestimate or over estimate.

Observation 3: The average error is zero as such the test beam direction set is fair.

From the histogram we can see that the data is now looking more normal and as such the standard deviation can be treated using our usual techniques.

In this case the standard deviation of the data is 0.77dB (which would give a CI of 1.5dB)

The maximum overshoot and undershoot value are quite high but they can be considered outliers as the standard deviation gives a better statistical analysis of the data.

Observation 4: The data is normal and the standard deviation is a reasonable value which can be used to form a more useful CI

Observation 5. The max and min values can be seen as outliers.

Looking at all the test beam direction sets under consideration we have the following:

|  | max (dB) | min (dB) | average (dB) | Std Dev (dB) | RMS average (dB) | abs average (dB) | linear rms (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20 point fixed (4x5) | 2.15 | -3.70 | -0.03 | 0.74 | 0.78 | 0.90 | 0.53 |
| 20 point fixed (5x4) | 2.00 | -2.26 | 0.00 | 0.65 | 0.71 | 0.93 | 0.51 |
| 16 point(4x4) fixed | 1.83 | -3.70 | -0.08 | 0.75 | 0.79 | 0.97 | 0.57 |
| 12 point fixed (4x3) | 1.66 | -3.71 | -0.08 | 0.73 | 0.77 | 0.84 | 0.55 |
| ZTE1b | 2.17 | -2.29 | 0.12 | 0.66 | 0.73 | 0.79 | 0.49 |
| 25 point fixed (5x5) nc | 2.23 | -3.63 | -0.19 | 0.79 | 0.82 | 0.92 | 0.54 |
| 25 point fixed (5x5) | 2.34 | -2.12 | 0.05 | 0.64 | 0.72 | 0.89 | 0.52 |
| 36 point fixed (6x6) | 2.69 | -1.12 | 0.10 | 0.64 | 0.72 | 0.82 | 0.51 |
| 64 point fixed (8x8) | 3.06 | -0.90 | 0.16 | 0.64 | 0.75 | 0.75 | 0.47 |
| 100 point fixed (10x10) | 3.23 | -0.91 | 0.17 | 0.66 | 0.77 | 0.77 | 0.48 |

Figure 2.3-1. EEIRP errors for test beam direction sets under consideration

I also added a 5x5, 6x6, 8x8, and 10x10 test beam direction set to compare what happens when more points are added, the standard deviation is not reduced.

In all cases the standard deviation is about 0.7-0.8dB and the average metrics are also very consistent.

Adding more test directions does not seem to greatly decrease the standard deviation.

These results back up the more limited results gathered in section 2.2 for ANT 4 and the 2 agreed steering ranges. None of the test patterns under consideration stand out significantly.

In addition we can see that adding more points at this level does not significantly improve performance.

Observation 6: None of the test beam direction sets stand out as significantly better considering the addition data sets.

Observation 7: Increasing the number of directions (in the range studied) does not seem to improve the accuracy.

Observation 7 of course has a corollary that reducing the number of points doesn’t seem to reduce accuracy but we have seen previously that to few points in the vertical dimension will start to reduce accuracy. This was previously seen to happen with 3 vertical points, as all our test beam direction sets have at least 4 then we are not seeing this effect.

This analysis has 2 uses:

- It backs up the results shown in section 2.1.2 and proposal 1.

- It generates a set of sampled data were the BS is the variable and the errors are normally distributed. This can be used to estimate the sampling error in the MU calculations.

  - Issue 2-5: Conformance testing for different mechanical tilts

The WF still has an open issue on the conformance testing:

- Further discuss the conformance testing for different mechanical tilts and whether the conformance testing for the worst case is sufficient

We presented in the last meeting our findings on identifying a worst case mechanical tilt and steering range, as the issue was not resolved we repeat the same information here to be considered when discussing the issue.

The metric used in this case was the minimum margin over the specification.

Figure 2.2-1. left: Margin vs mechanical down tilt, right margin vs electrical steering range

In figure 2.4-1 in the left hand graph the mechanical tilt is varied for various electrical steering ranges. In all cases it can be seen that as mechanical tilt increases the margin gets larger.

Observation 8: Worst case margin is when mechanical tilt in minimum

In figure 2.1-1 in the right hand graph the electrical steering range is varied (for a fixed mechanical tilt). It is clear that as the steering range increases the margin reduces

Observation 9: Worst case margin is when electrical steering range is maximum.

It seems clear that the worst-case condition is when mechanical tilt is minimum (closest to horizon) and the electrical steering range is maximum. As such this condition should be declared and tested.

Proposal 2: The minimum mechanical down tilt and the maximum electrical steering range shall be declared.

In real products we should allow for that fact that these 2 conditions may not be declared simultaneously as such the following caveat should be added.

Proposal 3: If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

  - Minimum mechanical down tilt and the reduced electrical steering range

  - Maximum electrical steering range and increased mechanical down tilt

2.3	Issue 2-8: Measurement uncertainty

Measurement uncertainty has 2 parts that need to be discussed:

- The normal MU of the OTA measurement, the chamber accuracy etc

- The uncertainty of the EEIRP averaging assumptions – measurement grid sizes, beam steering test patterns etc.

The 1st part should be relatively simple as we have done this a number of times, the background and many of the error definitions and uncertainty assumptions are documented in the TR 39.941. We can use this as a good starting point to estimate the MU for each of the directional power measurements associated with the EEIRP measurement.

The 2nd part is more difficult. We had a similar issue when looking at TRP measurements and grid sizes, the solution was to define an uncertainty called TRP summation error which accounted for errors between using an infinite set of spherical measurement grid points and a reduced set which provided a reasonable measurement time.

For EEIRP this issue is extended somewhat as we have both a measurement grid (split into sections above the horizon) and also a set of random beam directions which must be averaged.

The requirement from Resolution 220 which covers both of the EEIRP averaging steps is:

The averaging processes in steps 1 and 2 shall allow for accurate averaging of the expected e.i.r.p. (e.g. to the confidence interval of 95%).

Unfortunately “accurate averaging” is not a clear requirement. The practical step where inaccuracy occurs in the averaging process is going from effectively an infinite number of beam directions (as an integral) to a practical number as discussed in section 2.1. As this is part of the test description then it is reasonable to both include any inaccuracy in the MU calculations for the test and also to try to use methodologies we have previously used when considering how to interpret the text in the resolution.

When forming a distribution of errors from which we can extract a standard deviation and a 95% confidence interval, it has been shown in section 2.1.3 that if sufficient BS types (antenna geometries) and steering range definitions are examined then the errors form a normal distribution. From this distribution we can extract an accuracy value for a 95% confidence. That value will represent the accuracy achieved when using the test beam direction set selected compared to an infinite (or very large) set of test beam directions with the BS effectively being the unknown element. This is broadly in line with how we calcite measurement uncertainty currently.

To ensure the validity of this process it is important we consider all possible antennas geometries and steering range definitions.

Currently this has been done for a relatively large number of configurations in section 2.1.3, the standard deviation of the error is approximately 0.75dB. This figure has been used in the measurement uncertainty calculations below.

2.1.3	Measurement uncertainty calculations

The EEIRP measurement is very similar to the in-band TRP measurements, as such the in-band emission MU budgets can be used as a starting point for the EEIRP budgets.

Additional sources of uncertainty are:

Dynamic range uncertainty

One major difference with EEIRP however is that it is necessary to measure and add power on the same frequency as the wanted signal when the wanted signal is present and steered in another direction. Existing measurements either are measuring on adjacent channels (where frequency isolation can be used) or where the wanted signal is also being measured and is dominant. As such we are relying on the chamber spatial isolation to remove the effect of the wanted signal. Reflections and refractions of the main beam will limit the minimum power level that can be accurately measured.

This error can be referred to as dynamic range uncertainty.

The lowest EEIRP requirement is 15dBm/MHz

Currently the dynamic range contribution is 0.51dB, in order to maintain that level of accuracy in the above scenario an isolation of approximately 58dB would be required:

Pint=10*log101063.7-5810+101510=15.5dBm/MHz i.e. 0.5dB error

EEIRP sampling Error

As is being discussed the ideal definition and a reasonably sampled test case will result in potential uncertainties.

These will occur due to the differences between the test beam directions set and infinite set of random beam directions and also due to errors caused by the sampling grid.

Currently the analysis of the test beam direction sets we are using various different metrics to compare the accuracy of the beam sets. Its not clear what is the exact metric (or on whose data it will be derived) would be suitable to add into the MU budget (its distribution etc, its standard deviation etc), however this can be further discussed.

EEIRP sampling error seems like an appropriate name for this parameter a potential definition is provided below:

EEIRP sample error – Error due to the sampling of the 2 averaging processes used to define EEIRP. The sampling error covers errors due to difference in power distribution due to the test beam direction set and an infinite set of random beam directions as well as errors due to the sampling grid. It is based on the statistical error from a range of BS antenna architectures and BS steering range definitions.

Based on the analysis done so far in section 2.3 we estimate a EEIRP sampling error of 0.75dB (Equivalent to a 95% CI of 1.5dB)

These 2 errors have been added to the MU budget as follows:

| Table 11.2.2.3-1: IAC MU value derivation for EIRP measurement of OTA BS output power |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-1 | Positioning misalignment between the BS and the reference antenna |  |  | 0.03 | Rectangular | 1.73 | 1 |  |  |  |  | 0.02 |  |
| A1-2 | Pointing misalignment between the BS and the receiving antenna |  |  | 0.3 | Rectangular | 1.73 | 1 |  |  |  |  | 0.17 |  |
| A1-3 | Quality of quiet zone |  |  | 0.1 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4a | Polarization mismatch between the BS and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5a | Mutual coupling between the BS and the receiving antenna |  |  | 0 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.26 | Gaussian | 1.00 | 1 |  |  |  |  | 0.26 |  |
| A1-7 | Impedance mismatch in the receiving chain |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A1-8 | Random uncertainty |  |  | 0.1 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| A2-21 | Measurement system dynamic range uncertainty |  |  | 0.51 | Gaussian | 1 | 1 |  |  |  |  | 0.51 |  |
| *new | [EEIRP sampling error] |  |  | [0.75] | [Gaussian] | 1 | 1 |  |  |  |  | [0.75] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-9 | Impedance mismatch between the receiving antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-10 | Positioning and pointing misalignment between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-11 | Impedance mismatch between the reference antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-3 | Quality of quiet zone (normal test conditions) |  |  | 0.10 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4b | Polarization mismatch between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5b | Mutual coupling between the reference antenna and the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A1-12 | Influence of the reference antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| A1-13 | Reference antenna feed cable loss measurement uncertainty |  |  | 0.06 | Gaussian | 1.00 | 1 |  |  |  |  | 0.06 |  |
| A1-14 | Influence of the receiving antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A1-15 | Uncertainty of the absolute gain of the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [1.06] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [2.07] |
| Table 11.2.3.3-1: CATR MU value derivation for OTA BS output power measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) |  |  | 0.30 | Rectangular | 1.73 | 1 |  |  |  |  | 0.173 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.26 | Gaussian | 1.00 | 1 |  |  |  |  | 0.260 |  |
| A2-2a | Standing wave between BS and test range antenna |  |  | 0.21 | U-shaped | 1.41 | 1 |  |  |  |  | 0.148 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.001 |  |
| A2-4a | QZ ripple experienced by BS |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.093 |  |
| A2-12 | Frequency flatness of test system |  |  | 0.25 | Gaussian | 1.00 | 1 |  |  |  |  | 0.250 |  |
| A2-17 | Measurement system dynamic range uncertainty |  |  | 0.51 | Gaussian | 1 | 1 |  |  |  |  | 0.51 |  |
| *new | [EEIRP sampling error] |  |  | [0.75] | [Gaussian] | 1 | 1 |  |  |  |  | [0.75] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A2-6 | Insertion loss of receiver chain |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.00 |  |
| A2-7 | Influence of the calibration antenna feed cable |  |  | 0.02 | U-shaped | 1.41 | 1 |  |  |  |  | 0.02 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A2-8 | Misalignment positioning system |  |  | 0.00 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.00 |  |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) |  |  | 0.50 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.25 |  |
| A2-9 | Rotary joints |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.03 |  |
| A2-2b | Standing wave between calibration antenna and test range antenna |  |  | 0.09 | U-shaped | 1.41 | 1 |  |  |  |  | 0.06 |  |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) |  |  | 0.01 | Gaussian | 1.00 | 1 |  |  |  |  | 0.01 |  |
| A2-11 | Switching uncertainty |  |  | 0.26 | Rectangular | 1.73 | 1 |  |  |  |  | 0.15 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [1.13] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [2.21] |

As with the TRP cases the CATR is slightly higher error so we can concentrate on this when considering the final MU value.

  - Issue 2-9: CI

The issue of the CI for the data has been a difficult discussion which is somewhat due to the lack of clarity in the statement in the recommendation:

The averaging processes in steps 1 and 2 shall allow for accurate averaging of the expected e.i.r.p. (e.g. to the confidence interval of 95%).

The error (or accuracy) in the averaging process is due to the limited number of sampled spatial points in each of the defined angular bins and limiting the number of beam directions included in the EEIRP average.

Inaccuracy due to stepwise sampling of the sphere

Inaccuracy due to the limited number of test beam directions

We have managed to find a solution to the sampling of the sphere, however as yet no agreement on how to estimate the statistical inaccuracy due to the test beam direction set selected.

We have a process by which we can estimate the error between a reference power distribution (we use 10k random beam directions) and a test beam direction set. These results are shown in section 2.1.2. However for each BS configuration and steering range we get only a single number (per bin or some metric representing all bins).

In section 2.1.3 we have shown how by using multiple BS configurations and steering rage definitions we can form a distribution of errors which will cover all BS cases.

This approach seems to respect the request for accuracy in the recommendation as well as the request confidence in this accuracy.

The exact range of BS configurations and steering ranges we should consider can be further discussed, noting that forming the reference cases for each BS configuration is a lengthy process.

3 Summary

In this paper we have presented our view on the open issues, as well as our results on the agreed upon simulation cases and some addition simulation information we believe will be useful in the measurement uncertainty discussion.

We have made the following observations and proposals:

Observation 1: Any of the dB averages are suitable for the analysis, over estimate and under estimate values are also considered.

Observation 2: Using the agreed antenna definition and steering ranges none of the test beam direction sets are significantly better than any other.

Proposal 1: Use the 16 point fixed (4x4) test beam direction set.

When looking at the statistics of the accuracy over a number of BS configurations and steering range definitions we make the following observations:

Observation 3: The average error is zero as such the test beam direction set is fair.

Observation 4: The data is normal and the standard deviation is a reasonable value which can be used to form a more useful CI

Observation 5. The max and min values can be seen as outliers.

Observation 6: None of the test beam direction sets stand out as significantly better considering the addition data sets.

Observation 7: Increasing the number of directions (in the range studied) does not seem to improve the accuracy.

On the issue of conformance testing conditions we make the following observations and proposals

Observation 8: Worst case margin is when mechanical tilt in minimum

Observation 9: Worst case margin is when electrical steering range is maximum.

Proposal 2: The minimum mechanical down tilt and the maximum electrical steering range shall be declared.

Proposal 3: If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

  - Minimum mechanical down tilt and the reduced electrical steering range

  - Maximum electrical steering range and increased mechanical down tilt

In addition we have provided information for discussion to use the BS configurations statistics as a means to estimate the measurement uncertainty at the defined confidence interval.

References

[1]	R4-2419827 Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP – ZTE