3GPP TSG-RAN WG4 Meeting #114 	R4-2501863

Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	RF requirements for CW

Agenda Item:	7.24.2.3

Document for:	Approval

1 Introduction

In this paper we provide discussion and proposals for CW RF requirements.

2 Discussion

2.1 Which specification?

In the WID [1], one of the open issue is which specification will contain the RF requirements for CW.

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| New TS | TS 38.xxx | Requirements for Ambient IoT device |  | RAN#109 | RAN4 Core part Note: including RF requirements and RRM requirements (if any) |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) radio transmission and reception |  | RAN#109 | RAN4 Core part |
| New TS | TS 38.xxx | Requirements for Ambient IoT device |  | RAN#111 | RAN4 Performance part Note: including RF requirements, demodulation and RRM requirements (if any) |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) radio transmission and reception |  | RAN#111 | RAN4 Performance part |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) conformance testing |  | RAN#111 | RAN4 Performance part |

Note: which specification will contain the RF requirements for CW is to be further discussed at RAN#107 based on RAN4 feedback.

There are two candidates: Option 1 Ambient IoT Base Station spec; Option 2 Ambient IoT device spec.

Option 1: TS 38.xxx Ambient IoT Base Station (BS) radio transmission and reception

  - Both BS and CW are adopting the same conductive requirements

  - Normally CW will be supplied by BS vendors

Option 2: TS 38.xxx Requirements for Ambient IoT device

  - Both CW and device transmit at UL spectrum

We slightly prefer option 1. The BS transmits at downlink band while the CW node transmits at the Uplink band. If option 1 is adopted, the title needs to be updated to include CW node.

Proposal 1: The Ambient IoT BS spec will contain the RF requirements and the Title is updated to “TS 38.xxx Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception”

2.2 RF requirements

Per Study Item discussion, output power, frequency error, unwanted emission including spurious emission are potentially needed. We investigate them one by one in the following text.

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| RF Requirement for CW for D1T1 | Requirements potentially needed or not |
| --- | --- |
| Output power | YES |
| ON/OFF power | TBD in WI phase |
| Frequency error | YES |
| Phase noise | No for inside topology TBD in WI phase for outside topology |
| Unwanted emission | YES |
| Spurious emissions | YES |
| Intermodulation | TBD in WI phase |

Output power

In the release, CW node is deployed separately from the base station reader, using UL spectrum. Different to that for the UE different power classes are defined based on UE capability, single out power limit is preferred for the CW node. From TS 45.005, we can find that up to 33 dBm is supported for GSM 700, GSM 850 and GSM 900. And for RFID, the max ERP limit is 33 dBm for 800M and 36 dBm for 900M (ETSI EN 302 208). Hence it is proposed to consider 33 dBm as the up bound for CW node.

Table 4.1-2 in 45.005 MS maximum power at other modulations

| Power | GSM 400 and GSM 900 & ER-GSM 900 & GSM 850 & GSM 700 | GSM 400 & GSM 900 & ER-GSM 900 & GSM 850 & GSM 700 |  | DCS 1 800 | PCS 1 900 | DCS 1 800 & PCS 1 900 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| class | Nominal Maximum output | Tolerance (dB) for conditions |  | Nominal Maximum output | Nominal Maximum output | Tolerance (dB) for conditions |  |
|  | Power | normal | extreme | power | power | normal | extreme |
| E1 | 33 dBm | ±2 | ±2,5 | 30 dBm | 30 dBm | ±2 | ±2,5 |
| E2 | 27 dBm | ±3 | ±4 | 26 dBm | 26 dBm | -4/+3 | -4,5/+4 |
| E3 | 23 dBm | 3 | ±4 | 22 dBm | 22 dBm | 3 | 4 |
| E4 | 19 dBm1) | 2 | ±2,5 | 18 dBm | 18 dBm | 3 | ±3,5 |
| NOTE 1: Power class E4 applies only to GSM 850 and E-GSM 900. |  |  |  |  |  |  |  |

The CW conducted output power requirement is at antenna connector. The rated output power of the CW node shall be less than or equal to 33 dBm.

Proposal 2: The rated output power of the CW node shall be less than or equal to 33 dBm.

Frequency error

To reduce the interference to outdoor Macro NR BS, the single tone must be aligned with the uplink subcarriers of NR. It is proposed to define the same frequency error requirement as MR and LA BS.

Table 6.5.1.2-1 in 38.104: Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

Proposal 3: the frequency error for CW node is defined as ±0.1 ppm.

Unwanted emission

CW node transmits singe tone signal. The emission in the frequency range immediately outside the CW is phase noise dominate. Table x-y show one example phase noise performance with carrier frequency 900 MHz. Assuming the CW occupies one PRB, the integrate noise phase at the adjacent PRB is less than -25 dBm/30 kHz, which is already close to the GSM MS emission level (-27 dBm/30 kHz) at 400 kHz offset.

Table 2.2-1: example phase noise performance @ 900 MHz

| Offset | Phase Noise |
| --- | --- |
| 300Hz | -78.5dBc/Hz |
| 500Hz | -83.2dBc/Hz |
| 1kHz | -91dBc/Hz |
| 10kHz | -101dBc/Hz |
| 100kHz | -103dBc/Hz |
| 1MHz | -115dBc/Hz |

Referring to boundary definition of BS, the spectrum emission mask for CW are defined from 10 MHz below the lowest frequency of the supported uplink operating band up to 10 MHz above the highest frequency of the supported uplink operating band.

For narrow band signal in sub-1 GHz, NB-IOT and GSM share similar case, and can be used as a reference. Assuming one PRB resource is reserved for CW, the emission mask starts from adjacent PRB, using GSM MS mask as reference, the spectrum emission mask for CW node is as defined in Table 2.2-2.

Table 2.2-2: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

The spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from 10 MHz below the lowest frequency of the supported uplink operating band, up to 10 MHz above the highest frequency of the supported uplink operating band. Spurious emission limits are given in ITU recommendation ITU-R M.1581 and are captured in Table 2.2-3.

Table 2.2-3: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz |

Proposal 3：The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as FOBUE = 10 MHz

Proposal 4: CW node spectrum emission mask is defined as in Table 2.2-2.

Proposal 5: Spurious emissions limits is defined as in Table 2.2-3

As discussed above, the phase noise performance is captured in the unwanted emission limits, hence it is not needed to be defined alone for CW. And for intermodulation, since CW node transmits singe tone signal in the Release, the requirement is not needed.

| RF Requirement for CW for D1T1 | Requirements potentially needed or not |
| --- | --- |
| Output power | YES |
| ON/OFF power | TBD in WI phase |
| Frequency error | YES |
| Phase noise | No for inside topology TBD in WI phase for outside topology |
| Unwanted emission | YES |
| Spurious emissions | YES |
| Intermodulation | TBD in WI phase |

Proposal 6: Phase noise and intermodulation are not needed for CW node

Proposal 7: otherwise justification is agreed, ON/OFF power is not defined for CW node

3 Conclusion

In this paper we provide discussion and proposals for CW RF requirements.

Proposal 1: The Ambient IoT BS spec will contain the RF requirements and the Title is updated to “TS 38.xxx Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception”

Proposal 2: The rated output power of the CW node shall be less than or equal to 33 dBm.

Proposal 3：The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as FOBUE = 10 MHz

Proposal 4: CW node spectrum emission mask is defined as in Table 2.2-2.

Proposal 5: Spurious emissions limits is defined as in Table 2.2-3

Proposal 6: Phase noise and intermodulation are not needed for CW node

Proposal 7: otherwise justification is agreed, ON/OFF power is not defined for CW node

References

[1]	RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2]	R4-2420470,