3GPP TSG-RAN WG4 Meeting#114                                R4-2501870

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:	7.24.2.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement for CW node

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

Discussion

- In the previous RAN4 meeting and RAN task, there were some discussion on the feasibility study for A-IoT BS especially from CW cancellation similar as SBFD DL sub-band to UL sub-band interference analysis. As we know that A-IoT BS will receive the CW signal from the transmitter side. CW interference could be mainly dominated by phase noise model. In the following figure 2.3-1, we proposed the phase noise model according to FR2 phase model and update it according to its operating frequency according to the Lesson theory. Based on the initial analysis from phase noise model according to the following figure 2.3-1, the CW leakage would be roughly around 50dB (0dB-[-91+10*log10(15*10^3)]) which is much better than 28dBc assumption at UE side.

- In addition, the spatial isolation for D1T1-B could be considered.

- Figure 2.3-1. Phase noise model at 1GHz [7.5kHz: -117dBc/Hz，90kHz: -128dBc/Hz]

- Table 2.3-1. the feasibility study for A-IoT BS

- From our understanding, the RF requirement for CW node should be focused on the following issues:

- Maximum output power for CW node;

- Power accuracy requirement for CW node;

- Spurious emission requirement for CW node outside of carrier;

- Potential DC leakage related requirements..

Conclusions

In this contribution, we shared some initial views on the RF requirement for CW node in D1T1-B and proposals are made as following:

Proposal 1: propose to consider the following RF requirement for CW node:

- Maximum output power for CW node;

- Power accuracy requirement for CW node;

- Spurious emission requirement for CW node outside of carrier;

- Potential DC leakage related requirements.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2413282	RF requirements for Ambient IoT BS, Huawei,