3GPP TSG-RAN WG4 Meeting#114                                R4-2501876

Athens, Greece, 17th – 18th Feb, 2025

Agenda item:	7.13.4.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on conformance testing for U6GHz BS EEIRP mask

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we reached lots of consensus for U6GHz EIRP mask as captured in [5], however there are still lots of open issues left for further discussions. In this contribution, we would like to share further understandings on these remaining issues.

Agreement: For any number of the testing beams, to use the alternative 1: Step 1: to calculate the average EIRP from different beams for certain direction: EIRPθn,φm=k=1KwkEIRPkθn,φm,αk,βk , where wk is a beam weighting and EIRPkθn,φm,αk,βk is the EIRP pattern per measured test beam. Step 2: to calculate the EEIRP for different elevation bins: EEIRPi=θHHi−θHLi∙π/180sinθHHi−sinθHLiMN∙n=1Nm=1MEIRPθn,φmcosθn Note: the order of step 1 and step 2 can be swapped. NOTE : To average the results in the linear units; For CI calculation, if EEIRP per beam is needed, then corresponding formula should be also defined. Issue 2-2: CAR and mechanical tilt related issue Agreement: - To use the 4 values Φmax, Φmin, θmax, θmin to describe testing coverage range for the testing beams; - To describe the reference coordinate system with the mechanical downtilt; D.10; OTA peak directions set maximum steering direction(s); The beam direction pair(s) corresponding to the following points: 1) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3).. Issue 2-3: Test beams for EEIRP mask Agreement: - To minimize the EEIRP difference between candidate testing beams and reference testing beam set; - [To ensure the confidence interval tight within candidate testing beams]; - For the weighting factor for test beams: - Equally weighting factor To split the CAR into the several square and the number of square starts with 20; FFS how to generate the beam within each square; - Alternative 1: to generate the beams randomly within each square; - Alternative 2: to generate the beam per squared as much as close to the center of square; Issue 2-4: Measurement grid over the azimuth angle - Agreement - For azimuth angle - use the same approach as TRP captured in Annex I in TS 38.141-2. - Alternative 1: Δϕref= 180∘πλDcyl - Alternative 2:effectively half the beam width of narrowest beam Issue 2-5: Conformance testing for different mechanical tilts - Agreement - Further discuss the conformance testing for different mechanical tilts and whether the conformance testing for the worst case is sufficient Issue 2-6: RF channels for the conformance testing - Agreement - M channel for conformance testing Issue 2-8: Measurement uncertainty - Agreement - Further discuss the MU Issue 2-9: CI - Agreement - Further discuss the CIs

Discussion

  - .Testing beams for EEIRP conformance testing

### 2.1.1.Procedures for the EEIRP evaluations

In the following section, for the convenience of the understanding of EEIRP evaluations, the evaluation procedures are provided as following:

- Step 1: to generate the testing beams with the electrical beam steering range , e.g. electrical down-tilt and scanning direction per beam;

- Step 2: use the antenna modelling in TR38.803 to create the radiation pattern per beam;

- Step 3: to calculate the EEIRP according to the EEIRP formula per beam;

- Step 4: to repeat Step 2/3 to get the EEIRP results for all testing beams;

- Step 5: to calculate the EEIRP results for BS among testing beams;

- Step 6: to calculate the mean of EEIRP according to the weighting factor in step 5.

- Step 7: to calculate the standard deviation from N testing beams in step 5 and mean in step 6.

- Step 8: to calculate the CI according to standard deviation in step 7 and also the number of testing beams.

For comparison between candidate testing beam (e.g. 16 testing beams or 20 testing beams within the steering range) and also reference testing beams (e.g. 10000 testing beams or 8000 testing within steering range), just to repeat the above procedures and to calculate the difference between EEIRP of candidate testing beams and that of reference testing beams.

### 2.1.2．Simulation assumptions for antenna modelling

For antenna modelling with sub-array considered, then following antenna modelling as shown in table 2.1.2-1 is considered in the EEIRP simulation in this contribution.

Table 2.1.2-1: Extended AAS model [extracted from TR38.803]

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=−min−−min12φφ3dB2,Am−min12θ−90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm−1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm−1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm−1dvλsinθetilt−n−1dℎλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm−1dvλcosθ+n−1dℎλsinθsinφ |

The detailed parameters for antenna configuration 4 are listed in the following table 2.1.2-2.

Table 2.1.2-2: Antenna array parameters

| Parameter | Macro Rural |
| --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna sub-array configuration (Row × Column) (Note 4) | 16 × 8 elements |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | 6 |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 26.76 |
| Mechanical down-tilt (degrees) | 0 |

For antenna modelling without sub-array considered, then following antenna modelling as shown in table 2.1.2-3 is considered in the EEIRP simulation in this contribution.

Table 2.1.2-3: Composite antenna pattern

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the super position vector is given by: the weighting is given by: |

The detailed parameters for antenna configuration 1 are listed in the following table 2.1.2-4.

Table 2.1.2-4: Antenna array parameters

| Parameter | Macro Rural |
| --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 4) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.7 of wavelength for V |
| Number of element rows in sub-array | 1 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 26.76 |
| Mechanical down-tilt (degrees) | 0 |

For the EEIRP simulation, two different electrical down-tilt (0-12 degree and 0-16 degree) are considered. For electrical beam steering range from elevation angle perspective, considering the boresight direction as 0 degree, then NR BS could steer downwards from 0 degree to 12 degree or 16 degree. For electrical beam scanning range from azimuth angle perspective, it is ranging from -60 degree to 60 degree as the legacy BS.

### 2.1.3.  Methodology on how to create the reference testing beams

In order to get the ideal EEIRP simulation results as much as possible, we tested the following extreme number of testing number e.g. 10K testing beams within rectangular steering range and 8k testing beams within ellipse steering range. In the following section, the methodology on how to generate the beams within rectangular and ellipse steering range would be clarified in details.

#### 2.1.3.1. Rectangular reference beam sets (10k testing beams)

Figure 2.1.3.1-1. the illustration of rectangular steering range

The methodology on how to generate the beams within rectangular steering range as shown in Figure 2.1.3.1-1. is described as following:

- To generate the 10k uniform distributed values between 0 degree to 12 degree, say X

- To generate the 10k uniform distributed values between -60 degree to 60 degree, say Y.

- To create rectangular reference beam sets as [X, Y] where Xi is electrical down-tilt of beam i and Yi is electrical scanning of beam i.

#### 2.1.3.2. Ellipse reference beam sets (8k testing beams)

Figure 2.1.3.2-1. the illustration of rectangular steering range

Figure 2.1.3.2-2. the illustration of rectangular steering range

The methodology on how to generate the beams within ellipse steering range as shown in Figure 2.1.3.2-2 is described as following:

- To generate the 8k random values for the angle: theta=2 * pi * rand(N, 1) ;

- To generate the 8k random values for r: r=sqrt(rand(N, 1)) ;

- To create the 8k azimuth angle as following

- Y= 0+60 * r .* cos(theta);

- To create the 8k elevation angle as following

- X= 6 + 6 * r .* sin(theta)

- To create ellipse reference beam sets as [X, Y] where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i.

In fact, considering the practical measurement in the OTA chamber, it is not possible to conduct such kind of the measurement from both measurement equipment capability and also extremely long testing time to finis the measurement for all testing beams. In order to simplify the EEIRP testing in more realistic approach, in the following section, we try to find the candidate testing beams to achieve similar EEIRP measurement compared with the ideal EEIRP measurement.

### 2.1.4.  Candidate testing beams for EEIRP conformance testing

#### 2.1.4.1. Option 1a: random pattern with 20 testing beams

As shown in the following figure 2.1.4-1, to use the random approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/20=0.6 degree

- To calculate the step size for azimuth angle as 120/25=4.8 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 2.1.4.1-1. random approach to create the testing beams for EEIRP conformance testing

Based on the above testing beam pattern and also EEIRP equation as agreed in the last RAN4 meeting, EEIRP results per beam per bin and also mean EEIRP results per bin across different beams are provided in the following table. Meanwhile in order to check the difference between the simplified testing beams (e.g. option 1a) and reference/ideal testing beams (e.g. rectangular reference beam sets or ellipse reference beam sets), both EEIRP results per bin for 10k testing beams within the rectangular steering range and also 8k testing beams within the ellipse steering range are provided as basis.

Regarding the metric to check the measurement of simplified testing beam compared with reference testing beams, the following metrics are provided in the table to show its measurement performance.

- Mean error= 10*log10((10^(abs(error_1))/10+10^(abs(error_2))/10+10^(abs(error_3))/10...++10^(abs(error_7))/10 )/7)

- OverEstimate =maximum positive value from errors in the 7 bin

- underEstimate= minimum negative value from errors in the 7 bin

Regarding the mean errors to estimate the overall errors across different bin, the evaluation results would also be quite similar as RMS errors if errors from different bin is calculated in dB unit directly considering the maximum error or overestimate/underestimate is controlled within 2dB.

CIs from different beams within the candidate test beam pattern:

- Step 1:  Covert the EEIRP results per beam from dB unit to linear unit;

- Step 2:  Calculate the std per bin across the testing beams;

- Step 3:  Calculate the CIs according to std per bin

- Step 4:  Covert the CIs in step 3 into dB;

|  | 20 testing beams [random] 12 degree electrical downtilt, Ant4 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |  |  |  |  |  |  |
| Beam index | 1 |  | 2 | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  |
| 1 | 24.47 |  | 17.29 | 11.74 |  | 5.65 |  | 17.63 |  | 5.00 |  | -3.76 |  |
| 2 | 32.35 |  | 17.40 | 13.87 |  | 6.11 |  | 20.48 |  | 8.28 |  | 2.32 |  |
| 3 | 26.19 |  | 19.05 | 13.58 |  | 7.60 |  | 19.71 |  | 8.37 |  | 14.77 |  |
| 4 | 34.96 |  | 19.59 | 13.22 |  | 7.53 |  | 21.24 |  | 11.09 |  | 11.63 |  |
| 5 | 28.61 |  | 16.72 | 13.17 |  | 4.88 |  | 18.97 |  | 6.17 |  | -2.21 |  |
| 6 | 21.09 |  | 15.49 | 13.46 |  | 8.43 |  | 11.04 |  | 3.59 |  | -9.74 |  |
| 7 | 24.15 |  | 16.78 | 13.65 |  | 9.51 |  | 15.59 |  | 6.53 |  | 7.62 |  |
| 8 | 22.63 |  | 17.05 | 15.06 |  | 10.11 |  | 12.98 |  | 7.08 |  | 7.73 |  |
| 9 | 23.54 |  | 18.26 | 12.27 |  | 9.59 |  | 16.90 |  | 7.19 |  | 8.59 |  |
| 10 | 22.61 |  | 14.74 | 13.46 |  | 7.87 |  | 12.70 |  | 3.63 |  | -7.80 |  |
| 11 | 20.16 |  | 15.34 | 16.40 |  | 18.97 |  | 7.36 |  | 7.29 |  | -15.75 |  |
| 12 | 20.74 |  | 17.59 | 14.56 |  | 16.25 |  | 9.42 |  | 8.25 |  | 0.33 |  |
| 13 | 21.14 |  | 16.34 | 17.43 |  | 20.06 |  | 8.58 |  | 10.19 |  | 0.39 |  |
| 14 | 19.21 |  | 17.61 | 12.84 |  | 13.46 |  | 9.13 |  | 6.31 |  | -9.73 |  |
| 15 | 19.67 |  | 14.65 | 14.39 |  | 16.18 |  | 7.17 |  | 0.37 |  | -17.51 |  |
| 16 | 19.41 |  | 16.43 | 21.27 |  | 25.01 |  | 2.21 |  | 11.12 |  | -18.99 |  |
| 17 | 18.58 |  | 18.81 | 17.18 |  | 24.74 |  | 3.46 |  | 12.87 |  | -4.43 |  |
| 18 | 20.64 |  | 17.69 | 22.60 |  | 26.39 |  | 3.83 |  | 15.18 |  | -0.65 |  |
| 19 | 18.07 |  | 18.18 | 16.57 |  | 23.01 |  | 4.74 |  | 11.05 |  | -10.23 |  |
| 20 | 18.30 |  | 16.75 | 17.86 |  | 24.09 |  | 1.29 |  | 4.76 |  | -19.60 |  |
| Mean EEIRP | 26.12 |  | 17.29 | 16.38 |  | 19.68 |  | 15.16 |  | 9.09 |  | 5.45 |  |
| CIs | 25.13 |  | 8.60 | 12.67 |  | 17.64 |  | 12.54 |  | 5.14 |  | 5.01 |  |
| Rect Ref | 25.82 |  | 17.41 | 16.96 |  | 19.75 |  | 15.01 |  | 9.22 |  | 6.09 |  |
| Error | 0.30 |  | -0.12 | -0.58 |  | -0.07 |  | 0.16 |  | -0.14 |  | -0.64 |  |
| Mean error | 0.29 |  |  |  |  |  |  |  |  |  |  |  |  |
| OverEstimate | 0.30 |  |  |  |  |  |  |  |  |  |  |  |  |
| UnderEstimate | -0.64 |  |  |  |  |  |  |  |  |  |  |  |  |
| Ellipse Ref | 24.59 | 17.46 |  |  | 16.28 |  | 19.39 |  | 14.50 |  | 9.37 |  | 6.85 |
| Error | 1.53 | -0.17 |  |  | 0.10 |  | 0.29 |  | 0.66 |  | -0.29 |  | -1.40 |
| Mean error | 0.67 |  |  |  |  |  |  |  |  |  |  |  |  |
| OverEstimate | 1.53 |  |  |  |  |  |  |  |  |  |  |  |  |
| UnderEstimate | -1.40 |  |  |  |  |  |  |  |  |  |  |  |  |

|  | 20 testing beams [random] 16 degree electrical downtilt,Ant4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |
| Beam index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 21.86 | 17.61 | 10.52 | 7.48 | 16.35 | 4.87 | -4.66 |
| 2 | 30.92 | 17.14 | 14.02 | 5.69 | 20.20 | 7.63 | 2.55 |
| 3 | 23.58 | 19.37 | 12.36 | 9.43 | 18.43 | 8.28 | 13.65 |
| 4 | 34.63 | 19.23 | 13.43 | 7.39 | 21.22 | 10.66 | 11.77 |
| 5 | 25.52 | 17.59 | 12.42 | 5.87 | 18.23 | 6.18 | -2.60 |
| 6 | 19.20 | 16.37 | 12.87 | 14.42 | 7.80 | 3.91 | -15.65 |
| 7 | 21.86 | 16.81 | 14.51 | 9.88 | 12.15 | 6.32 | 3.58 |
| 8 | 20.74 | 17.93 | 14.47 | 16.11 | 9.67 | 8.48 | 3.40 |
| 9 | 23.87 | 15.89 | 14.51 | 9.11 | 14.36 | 6.17 | 6.38 |
| 10 | 18.56 | 16.87 | 12.24 | 11.58 | 8.76 | 3.93 | -13.10 |
| 11 | 19.40 | 16.97 | 20.06 | 25.14 | 2.24 | 12.42 | -17.96 |
| 12 | 18.66 | 18.50 | 17.13 | 23.25 | 5.64 | 12.00 | -1.98 |
| 13 | 20.37 | 17.96 | 21.08 | 26.18 | 3.52 | 14.66 | -1.30 |
| 14 | 19.85 | 16.10 | 16.83 | 20.14 | 7.09 | 8.98 | -14.67 |
| 15 | 17.17 | 17.14 | 15.70 | 23.13 | 1.53 | 2.67 | -19.61 |
| 16 | 20.17 | 20.02 | 30.95 | 12.44 | 1.75 | 16.65 | -19.67 |
| 17 | 18.44 | 21.41 | 29.35 | 22.90 | 1.61 | 17.04 | -5.85 |
| 18 | 21.37 | 21.25 | 32.22 | 13.76 | 3.34 | 18.89 | -1.26 |
| 19 | 18.49 | 19.59 | 26.96 | 24.74 | 2.42 | 15.47 | -12.31 |
| 20 | 18.73 | 19.85 | 29.41 | 17.62 | 0.25 | 10.29 | -20.41 |
| Mean EEIRP | 25.07 | 18.49 | 24.56 | 19.96 | 14.02 | 12.36 | 4.18 |
| CIs | 24.62 | 11.07 | 23.28 | 17.51 | 12.27 | 9.52 | 4.13 |
| Rect Ref | 25.07 | 18.84 | 24.97 | 19.97 | 13.76 | 12.58 | 4.67 |
| Error | 0.00 | -0.35 | -0.40 | -0.01 | 0.26 | -0.22 | -0.49 |
| Mean error | 0.25 |  |  |  |  |  |  |
| OverEstimate | 0.26 |  |  |  |  |  |  |
| UnderEstimate | -0.49 |  |  |  |  |  |  |
| Ellipse Ref | 23.65 | 18.43 | 23.81 | 20.64 | 13.15 | 12.42 | 5.75 |
| Error | 1.42 | 0.06 | 0.75 | -0.68 | 0.87 | -0.06 | -1.57 |
| Mean error | 0.81 |  |  |  |  |  |  |
| OverEstimate | 1.42 |  |  |  |  |  |  |
| UnderEstimate | -1.57 |  |  |  |  |  |  |

|  | 20 testing beams [random] 12 degree electrical downtilt, Ant1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |
| Beam index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 29.19 | 21.00 | 17.05 | 14.43 | 11.83 | 4.35 | -9.64 |
| 2 | 34.11 | 23.29 | 18.86 | 16.02 | 13.07 | 7.83 | -4.29 |
| 3 | 30.80 | 22.63 | 18.71 | 16.16 | 13.78 | 9.24 | 5.40 |
| 4 | 35.90 | 23.74 | 19.22 | 16.30 | 13.16 | 8.65 | 0.74 |
| 5 | 31.70 | 22.21 | 17.95 | 15.17 | 12.40 | 6.12 | -9.62 |
| 6 | 22.61 | 18.16 | 15.32 | 13.21 | 10.58 | 5.18 | -10.21 |
| 7 | 24.21 | 19.45 | 16.47 | 14.24 | 11.81 | 8.67 | 0.96 |
| 8 | 24.07 | 19.64 | 16.85 | 14.79 | 12.33 | 9.26 | 4.89 |
| 9 | 25.78 | 20.16 | 16.92 | 14.60 | 12.31 | 8.71 | 1.44 |
| 10 | 22.60 | 18.13 | 15.25 | 13.07 | 10.43 | 5.50 | -9.75 |
| 11 | 20.08 | 16.82 | 14.51 | 12.74 | 11.00 | 7.39 | -5.35 |
| 12 | 22.88 | 18.93 | 16.36 | 14.55 | 12.77 | 8.94 | 4.57 |
| 13 | 20.99 | 17.75 | 15.49 | 13.79 | 12.18 | 9.44 | 6.63 |
| 14 | 22.92 | 18.69 | 15.99 | 14.05 | 11.93 | 7.36 | -5.94 |
| 15 | 20.47 | 16.81 | 14.30 | 12.40 | 10.35 | 3.24 | -8.60 |
| 16 | 18.49 | 15.82 | 13.91 | 12.45 | 10.69 | 6.33 | -7.47 |
| 17 | 19.44 | 16.73 | 14.77 | 13.22 | 11.21 | 9.08 | 3.65 |
| 18 | 19.65 | 17.01 | 15.15 | 13.78 | 12.19 | 9.55 | 6.78 |
| 19 | 19.19 | 16.42 | 14.39 | 12.76 | 10.78 | 8.45 | -0.20 |
| 20 | 18.14 | 15.43 | 13.49 | 11.98 | 9.90 | 4.27 | -9.01 |
| Mean EEIRP | 27.91 | 19.69 | 16.37 | 14.17 | 11.86 | 7.76 | 1.41 |
| CIs | 26.41 | 14.25 | 9.01 | 5.48 | 2.21 | 0.03 | -1.62 |
| Rect Ref | 27.71 | 19.59 | 16.31 | 14.12 | 11.83 | 7.77 | 1.46 |
| Error | 0.21 | 0.10 | 0.06 | 0.05 | 0.03 | -0.01 | -0.05 |
| Mean error | 0.07 |  |  |  |  |  |  |
| OverEstimate | 0.21 |  |  |  |  |  |  |
| UnderEstimate | -0.05 |  |  |  |  |  |  |
| Ellipse Ref | 26.59 | 19.44 | 16.31 | 14.18 | 11.96 | 8.19 | 2.31 |
| Error | 1.32 | 0.25 | 0.06 | -0.01 | -0.10 | -0.43 | -0.90 |
| Mean error | 0.46 |  |  |  |  |  |  |
| OverEstimate | 1.32 |  |  |  |  |  |  |
| UnderEstimate | -0.90 |  |  |  |  |  |  |

|  | 20 testing beams [random]16 degree electrical downtilt, Ant1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |
| Beam index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 26.44 | 19.78 | 16.24 | 13.78 | 11.35 | 4.71 | -9.05 |
| 2 | 33.26 | 23.11 | 18.77 | 15.99 | 13.17 | 7.84 | -3.63 |
| 3 | 28.05 | 21.41 | 17.91 | 15.51 | 13.29 | 9.42 | 5.91 |
| 4 | 35.68 | 23.74 | 19.23 | 16.32 | 13.21 | 8.62 | 0.76 |
| 5 | 29.96 | 21.55 | 17.54 | 14.88 | 12.32 | 6.33 | -8.98 |
| 6 | 21.72 | 17.70 | 15.09 | 13.21 | 11.26 | 5.23 | -8.48 |
| 7 | 23.66 | 19.23 | 16.41 | 14.34 | 11.84 | 8.36 | -0.47 |
| 8 | 23.17 | 19.17 | 16.59 | 14.77 | 12.97 | 9.23 | 6.44 |
| 9 | 23.69 | 19.21 | 16.33 | 14.16 | 11.63 | 8.57 | 0.32 |
| 10 | 22.47 | 18.13 | 15.38 | 13.39 | 11.09 | 4.97 | -9.71 |
| 11 | 18.76 | 16.07 | 14.14 | 12.65 | 10.76 | 7.09 | -6.92 |
| 12 | 19.67 | 16.89 | 14.87 | 13.25 | 11.35 | 9.38 | 5.51 |
| 13 | 19.66 | 16.99 | 15.10 | 13.69 | 11.96 | 9.44 | 5.95 |
| 14 | 19.81 | 16.72 | 14.52 | 12.80 | 11.06 | 7.99 | -2.23 |
| 15 | 18.08 | 15.32 | 13.27 | 11.59 | 9.15 | 3.21 | -9.04 |
| 16 | 15.95 | 14.07 | 12.60 | 11.35 | 9.68 | 7.28 | -4.87 |
| 17 | 17.67 | 15.52 | 13.91 | 12.65 | 11.54 | 9.52 | 6.26 |
| 18 | 17.08 | 15.23 | 13.82 | 12.66 | 11.15 | 10.34 | 9.15 |
| 19 | 18.18 | 15.81 | 14.07 | 12.79 | 11.63 | 8.60 | 1.65 |
| 20 | 15.72 | 13.75 | 12.22 | 10.95 | 9.47 | 5.49 | -5.60 |
| Mean EEIRP | 26.79 | 18.94 | 15.83 | 13.78 | 11.64 | 7.96 | 2.47 |
| CIs | 25.97 | 14.16 | 9.11 | 5.64 | 2.25 | 0.31 | -0.15 |
| Rect Ref | 26.54 | 18.82 | 15.75 | 13.72 | 11.60 | 7.98 | 2.46 |
| Error | 0.25 | 0.12 | 0.08 | 0.06 | 0.04 | -0.01 | 0.01 |
| Mean error | 0.08 |  |  |  |  |  |  |
| OverEstimate | 0.25 |  |  |  |  |  |  |
| UnderEstimate | -0.01 |  |  |  |  |  |  |
| Ellipse Ref | 25.27 | 18.63 | 15.75 | 13.79 | 11.76 | 8.36 | 3.31 |
| Error | 1.52 | 0.32 | 0.09 | -0.02 | -0.12 | -0.40 | -0.83 |
| Mean error | 0.50 |  |  |  |  |  |  |
| OverEstimate | 1.52 |  |  |  |  |  |  |
| UnderEstimate | -0.83 |  |  |  |  |  |  |

#### 2.1.4.2. Option 2: 20 fixed testing beams for the following pattern

As shown in the following figure 2.1.4.2-1, to use the equally distributed approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/25=0.48 degree

- To calculate the step size for azimuth angle as 120/20=4.8 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 2.1.4.2-1. equally distributed approach to create the 20 testing beams for EEIRP conformance testing

Based on the above testing beam pattern and also EEIRP equation as agreed in the last RAN4 meeting, EEIRP results per beam per bin and also mean EEIRP results per bin across different beams are provided in the following table. The metrics calculations are the same as previous.

|  | 20 testing beams [fixed] 12degree electrical downtilt, Ant4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Beam index | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  |  |
| 1 | 30.16 |  | 16.36 |  | 13.20 |  | 4.81 |  | 19.19 |  | 6.29 |  | -2.28 |  |  |
| 2 | 31.37 |  | 17.60 |  | 14.49 |  | 6.19 |  | 20.79 |  | 8.37 |  | 14.30 |  |  |
| 3 | 31.37 |  | 17.60 |  | 14.49 |  | 6.19 |  | 20.79 |  | 8.37 |  | 14.30 |  |  |
| 4 | 30.16 |  | 16.36 |  | 13.20 |  | 4.81 |  | 19.19 |  | 6.29 |  | -2.28 |  |  |
| 5 | 23.00 |  | 16.60 |  | 11.92 |  | 8.58 |  | 15.07 |  | 5.02 |  | -5.20 |  |  |
| 6 | 24.20 |  | 17.83 |  | 13.21 |  | 9.94 |  | 16.59 |  | 7.31 |  | 10.68 |  |  |
| 7 | 24.20 |  | 17.83 |  | 13.21 |  | 9.94 |  | 16.59 |  | 7.31 |  | 10.68 |  |  |
| 8 | 23.00 |  | 16.60 |  | 11.92 |  | 8.58 |  | 15.07 |  | 5.02 |  | -5.20 |  |  |
| 9 | 18.72 |  | 17.03 |  | 12.38 |  | 11.71 |  | 8.95 |  | 4.46 |  | -12.85 |  |  |
| 10 | 19.92 |  | 18.25 |  | 13.67 |  | 13.07 |  | 10.44 |  | 7.43 |  | 2.98 |  |  |
| 11 | 19.92 |  | 18.25 |  | 13.67 |  | 13.07 |  | 10.44 |  | 7.43 |  | 2.98 |  |  |
| 12 | 18.72 |  | 17.03 |  | 12.38 |  | 11.71 |  | 8.95 |  | 4.46 |  | -12.85 |  |  |
| 13 | 19.31 |  | 15.54 |  | 16.24 |  | 19.52 |  | 6.41 |  | 5.18 |  | -17.39 |  |  |
| 14 | 20.50 |  | 16.76 |  | 17.52 |  | 20.88 |  | 7.89 |  | 10.50 |  | -1.23 |  |  |
| 15 | 20.50 |  | 16.76 |  | 17.52 |  | 20.88 |  | 7.89 |  | 10.50 |  | -1.23 |  |  |
| 16 | 19.31 |  | 15.54 |  | 16.24 |  | 19.52 |  | 6.41 |  | 5.18 |  | -17.39 |  |  |
| 17 | 19.10 |  | 16.66 |  | 19.71 |  | 24.76 |  | 1.85 |  | 9.34 |  | -18.98 |  |  |
| 18 | 20.27 |  | 17.85 |  | 20.97 |  | 26.08 |  | 3.39 |  | 14.43 |  | -3.51 |  |  |
| 19 | 20.27 |  | 17.85 |  | 20.97 |  | 26.08 |  | 3.39 |  | 14.43 |  | -3.51 |  |  |
| 20 | 19.10 |  | 16.66 |  | 19.71 |  | 24.76 |  | 1.85 |  | 9.34 |  | -18.98 |  |  |
| Mean EEIRP | 25.35 |  | 17.12 |  | 16.47 |  | 19.89 |  | 14.97 |  | 8.97 |  | 6.32 |  |  |
| CIs | 22.97 |  | 6.15 |  | 12.01 |  | 17.83 |  | 12.38 |  | 5.02 |  | 5.71 |  |  |
| Rect Ref | 25.82 |  | 17.41 |  | 16.96 |  | 19.75 |  | 15.01 |  | 9.22 |  | 6.09 |  |  |
| Error | -0.47 |  | -0.29 |  | -0.49 |  | 0.15 |  | -0.04 |  | -0.26 |  | 0.23 |  |  |
| Mean error | 0.28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OverEstimate | 0.23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UnderEstimate | -0.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ellipse Ref |  | 24.59 |  | 17.46 |  | 16.28 |  | 19.39 |  | 14.50 |  | 9.37 |  | 6.85 |
| Error |  | 0.76 |  | -0.34 |  | 0.18 |  | 0.50 |  | 0.46 |  | -0.41 |  | -0.54 |
| Mean error |  | 0.46 |  |  |  |  |  |  |  |  |  |  |  |  |
| OverEstimate |  | 0.76 |  |  |  |  |  |  |  |  |  |  |  |  |
| UnderEstimate |  | -0.54 |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 20 testing beams [fixed] 16degree electrical downtilt, Ant4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bin index |  |  |  |  |  |  |  |  |
| Beam index |  | 1 | 2 | 3 | 4 |  | 5 | 6 | 7 |  |
| 1 |  | 27.92 | 16.79 | 12.98 | 4.90 |  | 18.70 | 5.96 | -2.53 |  |
| 2 |  | 29.13 | 18.03 | 14.27 | 6.27 |  | 20.28 | 8.07 | 14.01 |  |
| 3 |  | 29.13 | 18.03 | 14.27 | 6.27 |  | 20.28 | 8.07 | 14.01 |  |
| 4 |  | 27.92 | 16.79 | 12.98 | 4.90 |  | 18.70 | 5.96 | -2.53 |  |
| 5 |  | 22.13 | 15.10 | 13.72 | 8.06 |  | 12.10 | 4.02 | -8.50 |  |
| 6 |  | 23.33 | 16.33 | 15.01 | 9.43 |  | 13.61 | 6.64 | 7.36 |  |
| 7 |  | 23.33 | 16.33 | 15.01 | 9.43 |  | 13.61 | 6.64 | 7.36 |  |
| 8 |  | 22.13 | 15.10 | 13.72 | 8.06 |  | 12.10 | 4.02 | -8.50 |  |
| 9 |  | 19.98 | 14.92 | 15.93 | 18.27 |  | 7.12 | 4.78 | -16.98 |  |
| 10 |  | 21.17 | 16.14 | 17.20 | 19.64 |  | 8.60 | 9.77 | -0.98 |  |
| 11 |  | 21.17 | 16.14 | 17.20 | 19.64 |  | 8.60 | 9.77 | -0.98 |  |
| 12 |  | 19.98 | 14.92 | 15.93 | 18.27 |  | 7.12 | 4.78 | -16.98 |  |
| 13 |  | 19.34 | 16.30 | 21.64 | 24.93 |  | 2.16 | 10.19 | -19.24 |  |
| 14 |  | 20.50 | 17.50 | 22.91 | 26.25 |  | 3.70 | 14.96 | -3.45 |  |
| 15 |  | 20.50 | 17.50 | 22.91 | 26.25 |  | 3.70 | 14.96 | -3.45 |  |
| 16 |  | 19.34 | 16.30 | 21.64 | 24.93 |  | 2.16 | 10.19 | -19.24 |  |
| 17 |  | 19.79 | 19.86 | 30.36 | 15.35 |  | 1.24 | 15.89 | -19.68 |  |
| 18 |  | 20.93 | 21.02 | 31.59 | 16.62 |  | 2.74 | 18.30 | -4.24 |  |
| 19 |  | 20.93 | 21.02 | 31.59 | 16.62 |  | 2.74 | 18.30 | -4.24 |  |
| 20 |  | 19.79 | 19.86 | 30.36 | 15.35 |  | 1.24 | 15.89 | -19.68 |  |
| Mean EEIRP |  | 23.92 | 17.64 | 24.86 | 19.98 |  | 13.80 | 12.51 | 5.18 |  |
| CIs |  | 20.39 | 11.16 | 23.42 | 17.95 |  | 11.93 | 9.63 | 5.25 |  |
| Rect Ref |  | 25.07 | 18.84 | 24.97 | 19.97 |  | 13.76 | 12.58 | 4.67 |  |
| Error |  | -1.15 | -1.20 | -0.11 | 0.01 |  | 0.03 | -0.07 | 0.52 |  |
| Mean error |  | 0.47 |  |  |  |  |  |  |  |  |
| OverEstimate |  | 0.52 |  |  |  |  |  |  |  |  |
| UnderEstimate |  | -1.20 |  |  |  |  |  |  |  |  |
| Ellipse Ref | 23.65 | 18.43 | 23.81 | 20.64 | 13.15 |  | 12.42 | 5.75 |
| Error | 0.27 | -0.79 | 1.05 | -0.67 | 0.65 |  | 0.09 | -0.56 |
| Mean error | 0.59 |  |  |  |  |  |  |  |
| OverEstimate | 1.05 |  |  |  |  |  |  |  |
| UnderEstimate | -0.79 |  |  |  |  |  |  |  |

|  |  | 20 testing beams [fixed] 12degree electrical downtilt, Ant1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bin index |  |  |  |  |  |  |
| Beam index |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  | 32.54 | 22.39 | 18.02 | 15.19 | 12.29 | 5.71 | -10.14 |
| 2 |  | 33.69 | 23.55 | 19.23 | 16.47 | 13.70 | 8.76 | 2.59 |
| 3 |  | 33.69 | 23.55 | 19.23 | 16.47 | 13.70 | 8.76 | 2.59 |
| 4 |  | 32.54 | 22.39 | 18.02 | 15.19 | 12.29 | 5.71 | -10.14 |
| 5 |  | 23.96 | 18.83 | 15.71 | 13.40 | 10.97 | 6.40 | -8.65 |
| 6 |  | 25.11 | 20.00 | 16.93 | 14.68 | 12.37 | 9.20 | 4.10 |
| 7 |  | 25.11 | 20.00 | 16.93 | 14.68 | 12.37 | 9.20 | 4.10 |
| 8 |  | 23.96 | 18.83 | 15.71 | 13.40 | 10.97 | 6.40 | -8.65 |
| 9 |  | 22.62 | 18.28 | 15.51 | 13.50 | 11.20 | 5.61 | -9.37 |
| 10 |  | 23.75 | 19.44 | 16.71 | 14.77 | 12.61 | 8.79 | 3.45 |
| 11 |  | 23.75 | 19.44 | 16.71 | 14.77 | 12.61 | 8.79 | 3.45 |
| 12 |  | 22.62 | 18.28 | 15.51 | 13.50 | 11.20 | 5.61 | -9.37 |
| 13 |  | 19.30 | 16.20 | 13.98 | 12.23 | 10.44 | 6.47 | -7.14 |
| 14 |  | 20.42 | 17.35 | 15.17 | 13.49 | 11.83 | 9.30 | 5.51 |
| 15 |  | 20.42 | 17.35 | 15.17 | 13.49 | 11.83 | 9.30 | 5.51 |
| 16 |  | 19.30 | 16.20 | 13.98 | 12.23 | 10.44 | 6.47 | -7.14 |
| 17 |  | 18.45 | 15.76 | 13.83 | 12.34 | 10.45 | 5.98 | -7.92 |
| 18 |  | 19.56 | 16.88 | 15.00 | 13.59 | 11.85 | 9.22 | 4.96 |
| 19 |  | 19.56 | 16.88 | 15.00 | 13.59 | 11.85 | 9.22 | 4.96 |
| 20 |  | 18.45 | 15.76 | 13.83 | 12.34 | 10.45 | 5.98 | -7.92 |
| Mean EEIRP |  | 27.39 | 19.61 | 16.33 | 14.15 | 11.88 | 7.81 | 1.46 |
| CIs |  | 25.41 | 14.21 | 9.00 | 5.53 | 2.09 | -0.35 | -2.22 |
| Rect Ref |  | 27.71 | 19.59 | 16.31 | 14.12 | 11.83 | 7.77 | 1.46 |
| Error |  | -0.31 | 0.02 | 0.02 | 0.03 | 0.05 | 0.04 | 0.00 |
| Mean error |  | 0.07 |  |  |  |  |  |  |
| OverEstimate |  | 0.05 |  |  |  |  |  |  |
| UnderEstimate |  | -0.31 |  |  |  |  |  |  |
| Ellipse Ref | 26.59 | 19.44 | 16.31 | 14.18 | 11.96 | 8.19 | 2.31 |  |
| Error | 0.80 | 0.17 | 0.02 | -0.03 | -0.08 | -0.38 | -0.85 |  |
| Mean error | 0.35 |  |  |  |  |  |  |  |
| OverEstimate | 0.80 |  |  |  |  |  |  |  |
| UnderEstimate | -0.85 |  |  |  |  |  |  |  |

|  |  | 20 testing beams [fixed],16degree electrical downtilt, Ant1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bin index |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Beam index |  | 1 | 2 |  |  | 3 |  | 4 |  | 5 | 6 |  |  | 7 |  |
| 1 |  | 31.28 | 22.00 |  |  | 17.79 |  | 15.05 |  | 12.33 | 5.84 |  |  | -9.58 |  |
| 2 |  | 32.42 | 23.17 |  |  | 19.00 |  | 16.32 |  | 13.74 | 8.82 |  |  | 3.05 |  |
| 3 |  | 32.42 | 23.17 |  |  | 19.00 |  | 16.32 |  | 13.74 | 8.82 |  |  | 3.05 |  |
| 4 |  | 31.28 | 22.00 |  |  | 17.79 |  | 15.05 |  | 12.33 | 5.84 |  |  | -9.58 |  |
| 5 |  | 22.73 | 18.27 |  |  | 15.40 |  | 13.23 |  | 10.58 | 5.93 |  |  | -9.65 |  |
| 6 |  | 23.87 | 19.44 |  |  | 16.62 |  | 14.53 |  | 12.00 | 8.98 |  |  | 3.36 |  |
| 7 |  | 23.87 | 19.44 |  |  | 16.62 |  | 14.53 |  | 12.00 | 8.98 |  |  | 3.36 |  |
| 8 |  | 22.73 | 18.27 |  |  | 15.40 |  | 13.23 |  | 10.58 | 5.93 |  |  | -9.65 |  |
| 9 |  | 19.92 | 16.60 |  |  | 14.28 |  | 12.50 |  | 10.78 | 6.38 |  |  | -7.18 |  |
| 10 |  | 21.05 | 17.75 |  |  | 15.47 |  | 13.76 |  | 12.16 | 9.22 |  |  | 5.41 |  |
| 11 |  | 21.05 | 17.75 |  |  | 15.47 |  | 13.76 |  | 12.16 | 9.22 |  |  | 5.41 |  |
| 12 |  | 19.92 | 16.60 |  |  | 14.28 |  | 12.50 |  | 10.78 | 6.38 |  |  | -7.18 |  |
| 13 |  | 18.36 | 15.71 |  |  | 13.82 |  | 12.38 |  | 10.67 | 6.01 |  |  | -7.53 |  |
| 14 |  | 19.46 | 16.83 |  |  | 14.98 |  | 13.62 |  | 12.07 | 9.23 |  |  | 5.23 |  |
| 15 |  | 19.46 | 16.83 |  |  | 14.98 |  | 13.62 |  | 12.07 | 9.23 |  |  | 5.23 |  |
| 16 |  | 18.36 | 15.71 |  |  | 13.82 |  | 12.38 |  | 10.67 | 6.01 |  |  | -7.53 |  |
| 17 |  | 15.91 | 13.99 |  |  | 12.49 |  | 11.20 |  | 9.62 | 7.02 |  |  | -5.12 |  |
| 18 |  | 16.98 | 15.09 |  |  | 13.64 |  | 12.44 |  | 10.99 | 9.98 |  |  | 7.62 |  |
| 19 |  | 16.98 | 15.09 |  |  | 13.64 |  | 12.44 |  | 10.99 | 9.98 |  |  | 7.62 |  |
| 20 |  | 15.91 | 13.99 |  |  | 12.49 |  | 11.20 |  | 9.62 | 7.02 |  |  | -5.12 |  |
| Mean EEIRP |  | 26.01 | 18.86 |  |  | 15.78 |  | 13.75 |  | 11.64 | 8.02 |  |  | 2.47 |  |
| CIs |  | 24.18 | 14.07 |  |  | 9.11 |  | 5.73 |  | 2.48 | -0.03 |  |  | -0.79 |  |
| Rect Ref |  | 26.54 | 18.82 |  |  | 15.75 |  | 13.72 |  | 11.60 | 7.98 |  |  | 2.46 |  |
| Error |  | -0.53 | 0.04 |  |  | 0.03 |  | 0.03 |  | 0.04 | 0.04 |  |  | 0.01 |  |
| Mean error |  | 0.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OverEstimate |  | 0.04 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UnderEstimate |  | -0.53 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ellipse Ref | 25.27 |  |  | 18.63 | 15.75 |  | 13.79 |  | 11.76 |  |  | 8.36 | 3.31 |  |
| Error | 0.74 |  |  | 0.23 | 0.03 |  | -0.05 |  | -0.12 |  |  | -0.35 | -0.83 |  |
| Mean error | 0.35 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OverEstimate | 0.74 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UnderEstimate | -0.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 2.1.4.3. Option 3: 16 fixed testing beams for the following pattern

As shown in the following figure 2.1.4.3-1, to use the equally distributed approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/20=0.6 degree

- To calculate the step size for azimuth angle as 120/20=6 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 2.1.4.3-1. equally distributed approach to create the 16 testing beams for EEIRP conformance testing

|  | 16 testing beams [equally distributed] 12 degree electrical downtilt, Ant4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |
| Beam index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 28.52 | 16.63 | 13.08 | 4.79 | 18.84 | 5.99 | -2.47 |
| 2 | 29.73 | 17.87 | 14.37 | 6.16 | 20.42 | 8.09 | 14.10 |
| 3 | 29.73 | 17.87 | 14.37 | 6.16 | 20.42 | 8.09 | 14.10 |
| 4 | 28.52 | 16.63 | 13.08 | 4.79 | 18.84 | 5.99 | -2.47 |
| 5 | 22.78 | 14.89 | 13.60 | 7.99 | 12.91 | 4.08 | -7.54 |
| 6 | 23.98 | 16.13 | 14.89 | 9.36 | 14.43 | 6.63 | 8.29 |
| 7 | 23.98 | 16.13 | 14.89 | 9.36 | 14.43 | 6.63 | 8.29 |
| 8 | 22.78 | 14.89 | 13.60 | 7.99 | 12.91 | 4.08 | -7.54 |
| 9 | 20.19 | 15.19 | 14.95 | 16.72 | 7.62 | 4.53 | -16.40 |
| 10 | 21.38 | 16.41 | 16.23 | 18.09 | 9.10 | 8.96 | -0.70 |
| 11 | 21.38 | 16.41 | 16.23 | 18.09 | 9.10 | 8.96 | -0.70 |
| 12 | 20.19 | 15.19 | 14.95 | 16.72 | 7.62 | 4.53 | -16.40 |
| 13 | 18.66 | 17.11 | 18.23 | 24.47 | 1.65 | 8.72 | -18.80 |
| 14 | 19.83 | 18.31 | 19.49 | 25.80 | 3.18 | 14.01 | -3.49 |
| 15 | 19.83 | 18.31 | 19.49 | 25.80 | 3.18 | 14.01 | -3.49 |
| 16 | 18.66 | 17.11 | 18.23 | 24.47 | 1.65 | 8.72 | -18.80 |
| Mean EEIRP | 24.95 | 16.71 | 16.16 | 19.96 | 14.97 | 8.82 | 6.35 |
| CIs | 21.90 | 7.84 | 10.59 | 18.42 | 12.80 | 5.43 | 6.26 |
| Rect Ref | 25.82 | 17.41 | 16.96 | 19.75 | 15.01 | 9.22 | 6.09 |
| Error | -0.87 | -0.70 | -0.80 | 0.22 | -0.03 | -0.41 | 0.26 |
| Mean error | 0.48 |  |  |  |  |  |  |
| OverEstimate | 0.26 |  |  |  |  |  |  |
| UnderEstimate | -0.87 |  |  |  |  |  |  |
| Ellipse Ref | 24.59 | 17.46 | 16.28 | 19.39 | 14.50 | 9.37 | 6.85 |
| Error | 0.36 | -0.75 | -0.13 | 0.57 | 0.47 | -0.56 | -0.50 |
| Mean error | 0.48 |  |  |  |  |  |  |
| OverEstimate | 0.57 |  |  |  |  |  |  |
| UnderEstimate | -0.75 |  |  |  |  |  |  |

|  | 16 testing beams [equally distributed] 16 degree electrical downtilt, Ant4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |
| Beam index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 25.43 | 17.50 | 12.33 | 5.78 | 18.10 | 6.00 | -2.86 |
| 2 | 26.64 | 18.73 | 13.62 | 7.15 | 19.67 | 8.10 | 13.58 |
| 3 | 26.64 | 18.73 | 13.62 | 7.15 | 19.67 | 8.10 | 13.58 |
| 4 | 25.43 | 17.50 | 12.33 | 5.78 | 18.10 | 6.00 | -2.86 |
| 5 | 18.72 | 17.03 | 12.38 | 11.71 | 8.95 | 4.46 | -12.85 |
| 6 | 19.92 | 18.25 | 13.67 | 13.07 | 10.44 | 7.43 | 2.98 |
| 7 | 19.92 | 18.25 | 13.67 | 13.07 | 10.44 | 7.43 | 2.98 |
| 8 | 18.72 | 17.03 | 12.38 | 11.71 | 8.95 | 4.46 | -12.85 |
| 9 | 17.68 | 17.67 | 16.27 | 23.72 | 1.91 | 7.76 | -18.55 |
| 10 | 18.86 | 18.87 | 17.53 | 25.05 | 3.42 | 13.28 | -3.18 |
| 11 | 18.86 | 18.87 | 17.53 | 25.05 | 3.42 | 13.28 | -3.18 |
| 12 | 17.68 | 17.67 | 16.27 | 23.72 | 1.91 | 7.76 | -18.55 |
| 13 | 19.10 | 20.22 | 29.76 | 17.97 | 0.66 | 15.38 | -19.58 |
| 14 | 20.24 | 21.38 | 30.99 | 19.25 | 2.16 | 17.98 | -4.48 |
| 15 | 20.24 | 21.38 | 30.99 | 19.25 | 2.16 | 17.98 | -4.48 |
| 16 | 19.10 | 20.22 | 29.76 | 17.97 | 0.66 | 15.38 | -19.58 |
| Mean EEIRP | 22.13 | 18.94 | 24.74 | 19.70 | 13.59 | 12.53 | 5.15 |
| CIs | 18.60 | 11.43 | 23.77 | 17.53 | 12.25 | 10.02 | 5.73 |
| Rect Ref | 25.07 | 18.84 | 24.97 | 19.97 | 13.76 | 12.58 | 4.67 |
| Error | -2.94 | 0.10 | -0.23 | -0.27 | -0.18 | -0.04 | 0.49 |
| Mean error | 0.73 |  |  |  |  |  |  |
| OverEstimate | 0.49 |  |  |  |  |  |  |
| UnderEstimate | -2.94 |  |  |  |  |  |  |
| Ellipse Ref | 23.65 | 18.43 | 23.81 | 20.64 | 13.15 | 12.42 | 5.75 |
| Error | -1.52 | 0.50 | 0.93 | -0.94 | 0.44 | 0.11 | -0.59 |
| Mean error | 0.74 |  |  |  |  |  |  |
| OverEstimate | 0.93 |  |  |  |  |  |  |
| UnderEstimate | -1.52 |  |  |  |  |  |  |

|  | 16 testing beams [equally distributed],12 degree electrical downtilt, Ant1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |
| Beam index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 31.61 | 22.11 | 17.86 | 15.10 | 12.34 | 5.80 | -9.72 |
| 2 | 32.75 | 23.28 | 19.07 | 16.37 | 13.74 | 8.80 | 2.93 |
| 3 | 32.75 | 23.28 | 19.07 | 16.37 | 13.74 | 8.80 | 2.93 |
| 4 | 31.61 | 22.11 | 17.86 | 15.10 | 12.34 | 5.80 | -9.72 |
| 5 | 22.75 | 18.28 | 15.38 | 13.18 | 10.54 | 6.08 | -9.40 |
| 6 | 23.89 | 19.45 | 16.60 | 14.47 | 11.96 | 9.06 | 3.56 |
| 7 | 23.89 | 19.45 | 16.60 | 14.47 | 11.96 | 9.06 | 3.56 |
| 8 | 22.75 | 18.28 | 15.38 | 13.18 | 10.54 | 6.08 | -9.40 |
| 9 | 20.79 | 17.16 | 14.70 | 12.89 | 11.14 | 6.19 | -7.47 |
| 10 | 21.92 | 18.31 | 15.89 | 14.14 | 12.52 | 9.09 | 5.07 |
| 11 | 21.92 | 18.31 | 15.89 | 14.14 | 12.52 | 9.09 | 5.07 |
| 12 | 20.79 | 17.16 | 14.70 | 12.89 | 11.14 | 6.19 | -7.47 |
| 13 | 18.47 | 15.75 | 13.79 | 12.26 | 10.26 | 6.02 | -8.06 |
| 14 | 19.57 | 16.88 | 14.98 | 13.52 | 11.67 | 9.24 | 4.88 |
| 15 | 19.57 | 16.88 | 14.98 | 13.52 | 11.67 | 9.24 | 4.88 |
| 16 | 18.47 | 15.75 | 13.79 | 12.26 | 10.26 | 6.02 | -8.06 |
| Mean EEIRP | 27.20 | 19.64 | 16.36 | 14.17 | 11.90 | 7.79 | 1.41 |
| CIs | 25.30 | 14.70 | 9.53 | 6.07 | 2.98 | 0.11 | -1.82 |
| Rect Ref | 27.71 | 19.59 | 16.31 | 14.12 | 11.83 | 7.77 | 1.46 |
| Error | -0.50 | 0.05 | 0.04 | 0.05 | 0.07 | 0.02 | -0.04 |
| Mean error | 0.12 |  |  |  |  |  |  |
| OverEstimate | 0.07 |  |  |  |  |  |  |
| UnderEstimate | -0.50 |  |  |  |  |  |  |
| Ellipse Ref | 26.59 | 19.44 | 16.31 | 14.18 | 11.96 | 8.19 | 2.31 |
| Error | 0.61 | 0.21 | 0.05 | -0.01 | -0.06 | -0.39 | -0.90 |
| Mean error | 0.33 |  |  |  |  |  |  |
| OverEstimate | 0.61 |  |  |  |  |  |  |
| UnderEstimate | -0.90 |  |  |  |  |  |  |

|  | 16 testing beams [equally distributed]16 degree electrical downtilt, Ant1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Beam index | 1 | 2 |  | 3 |  |  | 4 | 5 |  | 6 |  |  | 7 |  |
| 1 | 29.86 | 21.46 |  | 17.45 |  |  | 14.80 | 12.25 |  | 6.02 |  |  | -9.08 |  |
| 2 | 31.01 | 22.62 |  | 18.65 |  |  | 16.07 | 13.65 |  | 8.92 |  |  | 3.49 |  |
| 3 | 31.01 | 22.62 |  | 18.65 |  |  | 16.07 | 13.65 |  | 8.92 |  |  | 3.49 |  |
| 4 | 29.86 | 21.46 |  | 17.45 |  |  | 14.80 | 12.25 |  | 6.02 |  |  | -9.08 |  |
| 5 | 22.62 | 18.28 |  | 15.51 |  |  | 13.50 | 11.20 |  | 5.61 |  |  | -9.37 |  |
| 6 | 23.75 | 19.44 |  | 16.71 |  |  | 14.77 | 12.61 |  | 8.79 |  |  | 3.45 |  |
| 7 | 23.75 | 19.44 |  | 16.71 |  |  | 14.77 | 12.61 |  | 8.79 |  |  | 3.45 |  |
| 8 | 22.62 | 18.28 |  | 15.51 |  |  | 13.50 | 11.20 |  | 5.61 |  |  | -9.37 |  |
| 9 | 18.43 | 15.70 |  | 13.70 |  |  | 12.10 | 9.99 |  | 6.16 |  |  | -8.00 |  |
| 10 | 19.54 | 16.83 |  | 14.90 |  |  | 13.37 | 11.39 |  | 9.29 |  |  | 4.97 |  |
| 11 | 19.54 | 16.83 |  | 14.90 |  |  | 13.37 | 11.39 |  | 9.29 |  |  | 4.97 |  |
| 12 | 18.43 | 15.70 |  | 13.70 |  |  | 12.10 | 9.99 |  | 6.16 |  |  | -8.00 |  |
| 13 | 16.06 | 14.08 |  | 12.53 |  |  | 11.22 | 9.77 |  | 7.03 |  |  | -5.02 |  |
| 14 | 17.13 | 15.19 |  | 13.69 |  |  | 12.46 | 11.13 |  | 9.92 |  |  | 7.65 |  |
| 15 | 17.13 | 15.19 |  | 13.69 |  |  | 12.46 | 11.13 |  | 9.92 |  |  | 7.65 |  |
| 16 | 16.06 | 14.08 |  | 12.53 |  |  | 11.22 | 9.77 |  | 7.03 |  |  | -5.02 |  |
| Mean EEIRP | 25.60 | 18.91 |  | 15.84 |  |  | 13.80 | 11.67 |  | 8.00 |  |  | 2.47 |  |
| CIs | 23.53 | 14.28 |  | 9.54 |  |  | 6.31 | 3.33 |  | 0.52 |  |  | -0.17 |  |
| Rect Ref | 26.54 | 18.82 |  | 15.75 |  |  | 13.72 | 11.60 |  | 7.98 |  |  | 2.46 |  |
| Error | -0.94 | 0.09 |  | 0.09 |  |  | 0.08 | 0.07 |  | 0.02 |  |  | 0.00 |  |
| Mean error | 0.20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OverEstimate | 0.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UnderEstimate | -0.94 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ellipse Ref | 25.27 |  | 18.63 |  | 15.75 | 13.79 |  |  | 11.76 |  | 8.36 | 3.31 |  |
| Error | 0.32 |  | 0.28 |  | 0.09 | 0.01 |  |  | -0.08 |  | -0.37 | -0.84 |  |
| Mean error | 0.29 |  |  |  |  |  |  |  |  |  |  |  |  |
| OverEstimate | 0.32 |  |  |  |  |  |  |  |  |  |  |  |  |
| UnderEstimate | -0.84 |  |  |  |  |  |  |  |  |  |  |  |  |

#### 2.1.4.4. Option 4: 25 fixed testing beams for the following pattern

As shown in the following figure 2.1.4.4-1, to use the equally distributed approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/25=0.48 degree

- To calculate the step size for azimuth angle as 120/25=4.8 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 2.1.4.4-1. equally distributed approach to create the 25 testing beams for EEIRP conformance testing

Based on the above testing beam pattern and also EEIRP equation as agreed in the last RAN4 meeting, EEIRP results per beam per bin and also mean EEIRP results per bin across different beams are provided in the following table. The metrics calculations are the same as previous.

|  | 25 testing beams [fixed]12 degree electrical downtilt, Ant4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |
| Beam index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 30.00 | 16.21 | 13.06 | 4.69 | 18.97 | 6.03 | -2.64 |
| 2 | 31.11 | 17.33 | 14.21 | 5.89 | 20.45 | 7.93 | 8.10 |
| 3 | 31.55 | 17.78 | 14.68 | 6.38 | 21.02 | 8.65 | 16.15 |
| 4 | 31.11 | 17.33 | 14.21 | 5.89 | 20.45 | 7.93 | 8.10 |
| 5 | 30.00 | 16.21 | 13.06 | 4.69 | 18.97 | 6.03 | -2.64 |
| 6 | 22.83 | 16.44 | 11.78 | 8.46 | 14.85 | 4.65 | -5.57 |
| 7 | 23.94 | 17.56 | 12.93 | 9.65 | 16.26 | 6.86 | 8.17 |
| 8 | 24.38 | 18.01 | 13.39 | 10.14 | 16.81 | 7.61 | 12.16 |
| 9 | 23.94 | 17.56 | 12.93 | 9.65 | 16.26 | 6.86 | 8.17 |
| 10 | 22.83 | 16.44 | 11.78 | 8.46 | 14.85 | 4.65 | -5.57 |
| 11 | 18.56 | 16.87 | 12.24 | 11.58 | 8.76 | 3.93 | -13.10 |
| 12 | 19.66 | 17.99 | 13.39 | 12.77 | 10.12 | 6.91 | -0.05 |
| 13 | 20.10 | 18.43 | 13.85 | 13.26 | 10.65 | 7.77 | 5.15 |
| 14 | 19.66 | 17.99 | 13.39 | 12.77 | 10.12 | 6.91 | -0.05 |
| 15 | 18.56 | 16.87 | 12.24 | 11.58 | 8.76 | 3.93 | -13.10 |
| 16 | 19.15 | 15.39 | 16.11 | 19.39 | 6.22 | 4.28 | -17.65 |
| 17 | 20.24 | 16.49 | 17.24 | 20.59 | 7.57 | 9.87 | -5.60 |
| 18 | 20.67 | 16.93 | 17.70 | 21.08 | 8.10 | 10.91 | 0.99 |
| 19 | 20.24 | 16.49 | 17.24 | 20.59 | 7.57 | 9.87 | -5.60 |
| 20 | 19.15 | 15.39 | 16.11 | 19.39 | 6.22 | 4.28 | -17.65 |
| 21 | 18.94 | 16.50 | 19.57 | 24.63 | 1.63 | 7.27 | -19.43 |
| 22 | 20.01 | 17.59 | 20.70 | 25.79 | 3.05 | 13.82 | -7.97 |
| 23 | 20.44 | 18.03 | 21.15 | 26.26 | 3.61 | 14.82 | 0.00 |
| 24 | 20.01 | 17.59 | 20.70 | 25.79 | 3.05 | 13.82 | -7.97 |
| 25 | 18.94 | 16.50 | 19.57 | 24.63 | 1.63 | 7.27 | -19.43 |
| Mean EEIRP | 25.34 | 17.11 | 16.47 | 19.90 | 14.95 | 8.95 | 5.75 |
| CIs | 22.46 | 5.72 | 11.50 | 17.33 | 11.87 | 4.62 | 5.31 |
| Rect Ref | 25.82 | 17.41 | 16.96 | 19.75 | 15.01 | 9.22 | 6.09 |
| Error | -0.48 | -0.30 | -0.49 | 0.16 | -0.05 | -0.28 | -0.34 |
| Mean error | 0.30 |  |  |  |  |  |  |
| OverEstimate | 0.16 |  |  |  |  |  |  |
| UnderEstimate | -0.49 |  |  |  |  |  |  |
| Ellipse Ref | 24.59 | 17.46 | 16.28 | 19.39 | 14.50 | 9.37 | 6.85 |
| Error | 0.75 | -0.35 | 0.18 | 0.51 | 0.45 | -0.43 | -1.11 |
| Mean error | 0.55 |  |  |  |  |  |  |
| OverEstimate | 0.75 |  |  |  |  |  |  |
| UnderEstimate | -1.11 |  |  |  |  |  |  |

|  | 25 testing beams [fixed] 16 degree electrical downtilt, Ant4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |
| Beam index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 27.76 | 16.63 | 12.84 | 4.77 | 18.47 | 5.69 | -2.91 |
| 2 | 28.87 | 17.76 | 13.99 | 5.97 | 19.94 | 7.63 | 8.61 |
| 3 | 29.31 | 18.21 | 14.45 | 6.47 | 20.51 | 8.35 | 15.65 |
| 4 | 28.87 | 17.76 | 13.99 | 5.97 | 19.94 | 7.63 | 8.61 |
| 5 | 27.76 | 16.63 | 12.84 | 4.77 | 18.47 | 5.69 | -2.91 |
| 6 | 21.97 | 14.95 | 13.59 | 7.94 | 11.89 | 3.54 | -8.73 |
| 7 | 23.07 | 16.07 | 14.73 | 9.14 | 13.28 | 6.14 | 5.10 |
| 8 | 23.51 | 16.51 | 15.19 | 9.63 | 13.83 | 6.95 | 8.69 |
| 9 | 23.07 | 16.07 | 14.73 | 9.14 | 13.28 | 6.14 | 5.10 |
| 10 | 21.97 | 14.95 | 13.59 | 7.94 | 11.89 | 3.54 | -8.73 |
| 11 | 19.82 | 14.77 | 15.79 | 18.14 | 6.93 | 3.98 | -17.34 |
| 12 | 20.91 | 15.87 | 16.93 | 19.34 | 8.28 | 9.15 | -5.17 |
| 13 | 21.34 | 16.32 | 17.39 | 19.83 | 8.81 | 10.17 | 1.34 |
| 14 | 20.91 | 15.87 | 16.93 | 19.34 | 8.28 | 9.15 | -5.17 |
| 15 | 19.82 | 14.77 | 15.79 | 18.14 | 6.93 | 3.98 | -17.34 |
| 16 | 19.18 | 16.15 | 21.51 | 24.81 | 1.95 | 8.01 | -19.63 |
| 17 | 20.25 | 17.24 | 22.64 | 25.97 | 3.37 | 14.36 | -8.15 |
| 18 | 20.67 | 17.67 | 23.09 | 26.43 | 3.92 | 15.34 | -0.53 |
| 19 | 20.25 | 17.24 | 22.64 | 25.97 | 3.37 | 14.36 | -8.15 |
| 20 | 19.18 | 16.15 | 21.51 | 24.81 | 1.95 | 8.01 | -19.63 |
| 21 | 19.64 | 19.71 | 30.22 | 15.22 | 1.03 | 14.66 | -20.08 |
| 22 | 20.68 | 20.76 | 31.32 | 16.35 | 2.41 | 17.79 | -9.08 |
| 23 | 21.09 | 21.19 | 31.76 | 16.80 | 2.95 | 18.63 | -0.57 |
| 24 | 20.68 | 20.76 | 31.32 | 16.35 | 2.41 | 17.79 | -9.08 |
| 25 | 19.64 | 19.71 | 30.22 | 15.22 | 1.03 | 14.66 | -20.08 |
| Mean EEIRP | 23.91 | 17.63 | 24.86 | 19.98 | 13.78 | 12.34 | 4.52 |
| CIs | 19.88 | 10.66 | 22.91 | 17.45 | 11.42 | 9.06 | 4.65 |
| Rect Ref | 25.07 | 18.84 | 24.97 | 19.97 | 13.76 | 12.58 | 4.67 |
| Error | -1.16 | -1.21 | -0.11 | 0.02 | 0.02 | -0.24 | -0.15 |
| Mean error | 0.44 |  |  |  |  |  |  |
| OverEstimate | 0.02 |  |  |  |  |  |  |
| UnderEstimate | -1.21 |  |  |  |  |  |  |
| Ellipse Ref | 23.65 | 18.43 | 23.81 | 20.64 | 13.15 | 12.42 | 5.75 |
| Error | 0.26 | -0.80 | 1.05 | -0.66 | 0.63 | -0.08 | -1.23 |
| Mean error | 0.69 |  |  |  |  |  |  |
| OverEstimate | 1.05 |  |  |  |  |  |  |
| UnderEstimate | -1.23 |  |  |  |  |  |  |

|  | 25 testing beams [fixed] 12 degree electrical downtilt, Ant1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |
| Beam index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 32.39 | 22.24 | 17.88 | 15.08 | 12.18 | 5.08 | -10.42 |
| 2 | 33.45 | 23.30 | 18.96 | 16.17 | 13.37 | 8.24 | -0.58 |
| 3 | 33.86 | 23.73 | 19.41 | 16.65 | 13.91 | 9.10 | 4.52 |
| 4 | 33.45 | 23.30 | 18.96 | 16.17 | 13.37 | 8.24 | -0.58 |
| 5 | 32.39 | 22.24 | 17.88 | 15.08 | 12.18 | 5.08 | -10.42 |
| 6 | 23.81 | 18.69 | 15.58 | 13.29 | 10.86 | 5.85 | -8.85 |
| 7 | 24.87 | 19.75 | 16.65 | 14.38 | 12.04 | 8.71 | 1.25 |
| 8 | 25.28 | 20.18 | 17.11 | 14.87 | 12.58 | 9.53 | 6.04 |
| 9 | 24.87 | 19.75 | 16.65 | 14.38 | 12.04 | 8.71 | 1.25 |
| 10 | 23.81 | 18.69 | 15.58 | 13.29 | 10.86 | 5.85 | -8.85 |
| 11 | 22.47 | 18.13 | 15.38 | 13.39 | 11.09 | 4.97 | -9.71 |
| 12 | 23.51 | 19.19 | 16.45 | 14.48 | 12.28 | 8.26 | 0.07 |
| 13 | 23.92 | 19.61 | 16.89 | 14.96 | 12.82 | 9.14 | 5.32 |
| 14 | 23.51 | 19.19 | 16.45 | 14.48 | 12.28 | 8.26 | 0.07 |
| 15 | 22.47 | 18.13 | 15.38 | 13.39 | 11.09 | 4.97 | -9.71 |
| 16 | 19.15 | 16.06 | 13.85 | 12.12 | 10.34 | 5.91 | -7.33 |
| 17 | 20.18 | 17.10 | 14.91 | 13.20 | 11.50 | 8.81 | 2.76 |
| 18 | 20.59 | 17.52 | 15.35 | 13.68 | 12.03 | 9.63 | 7.60 |
| 19 | 20.18 | 17.10 | 14.91 | 13.20 | 11.50 | 8.81 | 2.76 |
| 20 | 19.15 | 16.06 | 13.85 | 12.12 | 10.34 | 5.91 | -7.33 |
| 21 | 18.30 | 15.61 | 13.69 | 12.23 | 10.34 | 5.36 | -8.31 |
| 22 | 19.32 | 16.64 | 14.74 | 13.30 | 11.52 | 8.69 | 1.47 |
| 23 | 19.72 | 17.05 | 15.18 | 13.77 | 12.06 | 9.58 | 6.73 |
| 24 | 19.32 | 16.64 | 14.74 | 13.30 | 11.52 | 8.69 | 1.47 |
| 25 | 18.30 | 15.61 | 13.69 | 12.23 | 10.34 | 5.36 | -8.31 |
| Mean EEIRP | 27.39 | 19.60 | 16.33 | 14.15 | 11.89 | 7.79 | 1.47 |
| CIs | 24.91 | 13.71 | 8.50 | 5.02 | 1.54 | -0.65 | -2.07 |
| Rect Ref | 27.71 | 19.59 | 16.31 | 14.12 | 11.83 | 7.77 | 1.46 |
| Error | -0.32 | 0.01 | 0.02 | 0.03 | 0.05 | 0.02 | 0.01 |
| Mean error | 0.07 |  |  |  |  |  |  |
| OverEstimate | 0.05 |  |  |  |  |  |  |
| UnderEstimate | -0.32 |  |  |  |  |  |  |
| Ellipse Ref | 26.59 | 19.44 | 16.31 | 14.18 | 11.96 | 8.19 | 2.31 |
| Error | 0.79 | 0.17 | 0.02 | -0.03 | -0.07 | -0.40 | -0.84 |
| Mean error | 0.34 |  |  |  |  |  |  |
| OverEstimate | 0.79 |  |  |  |  |  |  |
| UnderEstimate | -0.84 |  |  |  |  |  |  |

|  | 25 testing beams [fixed] 16 degree electrical downtilt, Ant1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |  |
| Beam index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |
| 1 | 31.12 | 21.85 | 17.66 | 14.94 | 12.22 | 5.22 | -9.80 |  |
| 2 | 32.18 | 22.91 | 18.73 | 16.03 | 13.41 | 8.31 | 0.14 |  |
| 3 | 32.60 | 23.34 | 19.18 | 16.51 | 13.95 | 9.15 | 5.07 |  |
| 4 | 32.18 | 22.91 | 18.73 | 16.03 | 13.41 | 8.31 | 0.14 |  |
| 5 | 31.12 | 21.85 | 17.66 | 14.94 | 12.22 | 5.22 | -9.80 |  |
| 6 | 22.58 | 18.13 | 15.28 | 13.13 | 10.47 | 5.34 | -10.04 |  |
| 7 | 23.63 | 19.19 | 16.35 | 14.22 | 11.67 | 8.46 | -0.19 |  |
| 8 | 24.04 | 19.61 | 16.80 | 14.71 | 12.21 | 9.33 | 5.04 |  |
| 9 | 23.63 | 19.19 | 16.35 | 14.22 | 11.67 | 8.46 | -0.19 |  |
| 10 | 22.58 | 18.13 | 15.28 | 13.13 | 10.47 | 5.34 | -10.04 |  |
| 11 | 19.77 | 16.46 | 14.14 | 12.39 | 10.68 | 5.82 | -7.34 |  |
| 12 | 20.81 | 17.50 | 15.20 | 13.47 | 11.83 | 8.73 | 2.76 |  |
| 13 | 21.22 | 17.92 | 15.65 | 13.94 | 12.36 | 9.55 | 7.56 |  |
| 14 | 20.81 | 17.50 | 15.20 | 13.47 | 11.83 | 8.73 | 2.76 |  |
| 15 | 19.77 | 16.46 | 14.14 | 12.39 | 10.68 | 5.82 | -7.34 |  |
| 16 | 18.21 | 15.56 | 13.68 | 12.27 | 10.57 | 5.38 | -7.84 |  |
| 17 | 19.23 | 16.59 | 14.72 | 13.34 | 11.74 | 8.70 | 1.99 |  |
| 18 | 19.63 | 17.00 | 15.16 | 13.80 | 12.27 | 9.58 | 7.17 |  |
| 19 | 19.23 | 16.59 | 14.72 | 13.34 | 11.74 | 8.70 | 1.99 |  |
| 20 | 18.21 | 15.56 | 13.68 | 12.27 | 10.57 | 5.38 | -7.84 |  |
| 21 | 15.76 | 13.85 | 12.36 | 11.09 | 9.52 | 6.48 | -5.52 |  |
| 22 | 16.75 | 14.85 | 13.38 | 12.16 | 10.67 | 9.47 | 4.45 |  |
| 23 | 17.14 | 15.26 | 13.81 | 12.62 | 11.18 | 10.32 | 9.54 |  |
| 24 | 16.75 | 14.85 | 13.38 | 12.16 | 10.67 | 9.47 | 4.45 |  |
| 25 | 15.76 | 13.85 | 12.36 | 11.09 | 9.52 | 6.48 | -5.52 |  |
| Mean EEIRP | 26.01 | 18.85 | 15.78 | 13.75 | 11.65 | 8.00 | 2.50 |  |
| CIs | 23.68 | 13.56 | 8.61 | 5.21 | 1.94 | -0.34 | -0.69 |  |
| Rect Ref | 26.54 | 18.82 | 15.75 | 13.72 | 11.60 | 7.98 | 2.46 |  |
| Error | -0.53 | 0.04 | 0.03 | 0.03 | 0.04 | 0.02 | 0.04 |  |
| Mean error | 0.11 |  |  |  |  |  |  |  |
| OverEstimate | 0.04 |  |  |  |  |  |  |  |
| UnderEstimate | -0.53 |  |  |  |  |  |  |  |
| Ellipse Ref | 25.27 | 18.63 | 15.75 | 13.79 | 11.76 | 8.36 | 3.31 |
| Error | 0.73 | 0.23 | 0.03 | -0.04 | -0.11 | -0.37 | -0.81 |
| Mean error | 0.34 |  |  |  |  |  |  |
| OverEstimate | 0.73 |  |  |  |  |  |  |
| UnderEstimate | -0.81 |  |  |  |  |  |  |

#### 2.1.4.5. Option 5: scaling testing beams for the following pattern

As shown in the following figure 2.1.4.5-1, considering BS capable with different steering range in the elevation angle. According the the previous evaluation results, for electrical downtilt as 12 degree, then 16 testing beams should be sufficient to get the accurate results compared with ideal EEIRP results, however 16 degree electrical downtilt, more testing beams in the elevation angle are expected to achieve the dense testing beam sampling to achieve better accuracy.

Scaling testing beams is proposed as following with fixed step size in the elevation angle as following. If testing beams within the steering range could be select as conformance testing. e.g. if NR BS is capable of downtilt as 12 degree, then 16 test beams should be tested; if NR BS is capable of downtilt as 16 degree, then 20 test beams should be considered.

Figure 2.1.4.5-1. equally distributed approach to create the 16 testing beams for EEIRP conformance testing

Figure 2.1.4.5-2. equally distributed approach to create the 20 testing beams for EEIRP conformance testing

Figure 2.1.4.5-2. equally distributed approach to create the 24 testing beams for EEIRP conformance testing

Based on the above testing beam pattern and also EEIRP equation as agreed in the last RAN4 meeting, EEIRP results per beam per bin and also mean EEIRP results per bin across different beams are provided in the following table. The metrics calculations are the same as previous.

|  | 16 testing beams [equally distributed] 12 degree electrical downtilt, Ant4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |
| Beam index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 28.52 | 16.63 | 13.08 | 4.79 | 18.84 | 5.99 | -2.47 |
| 2 | 29.73 | 17.87 | 14.37 | 6.16 | 20.42 | 8.09 | 14.10 |
| 3 | 29.73 | 17.87 | 14.37 | 6.16 | 20.42 | 8.09 | 14.10 |
| 4 | 28.52 | 16.63 | 13.08 | 4.79 | 18.84 | 5.99 | -2.47 |
| 5 | 22.78 | 14.89 | 13.60 | 7.99 | 12.91 | 4.08 | -7.54 |
| 6 | 23.98 | 16.13 | 14.89 | 9.36 | 14.43 | 6.63 | 8.29 |
| 7 | 23.98 | 16.13 | 14.89 | 9.36 | 14.43 | 6.63 | 8.29 |
| 8 | 22.78 | 14.89 | 13.60 | 7.99 | 12.91 | 4.08 | -7.54 |
| 9 | 20.19 | 15.19 | 14.95 | 16.72 | 7.62 | 4.53 | -16.40 |
| 10 | 21.38 | 16.41 | 16.23 | 18.09 | 9.10 | 8.96 | -0.70 |
| 11 | 21.38 | 16.41 | 16.23 | 18.09 | 9.10 | 8.96 | -0.70 |
| 12 | 20.19 | 15.19 | 14.95 | 16.72 | 7.62 | 4.53 | -16.40 |
| 13 | 18.66 | 17.11 | 18.23 | 24.47 | 1.65 | 8.72 | -18.80 |
| 14 | 19.83 | 18.31 | 19.49 | 25.80 | 3.18 | 14.01 | -3.49 |
| 15 | 19.83 | 18.31 | 19.49 | 25.80 | 3.18 | 14.01 | -3.49 |
| 16 | 18.66 | 17.11 | 18.23 | 24.47 | 1.65 | 8.72 | -18.80 |
| Mean EEIRP | 24.95 | 16.71 | 16.16 | 19.96 | 14.97 | 8.82 | 6.35 |
| CIs | 21.90 | 7.84 | 10.59 | 18.42 | 12.80 | 5.43 | 6.26 |
| Rect Ref | 25.82 | 17.41 | 16.96 | 19.75 | 15.01 | 9.22 | 6.09 |
| Error | -0.87 | -0.70 | -0.80 | 0.22 | -0.03 | -0.41 | 0.26 |
| Mean error | 0.48 |  |  |  |  |  |  |
| OverEstimate | 0.26 |  |  |  |  |  |  |
| UnderEstimate | -0.87 |  |  |  |  |  |  |
| Ellipse Ref | 24.59 | 17.46 | 16.28 | 19.39 | 14.50 | 9.37 | 6.85 |
| Error | 0.36 | -0.75 | -0.13 | 0.57 | 0.47 | -0.56 | -0.50 |
| Mean error | 0.48 |  |  |  |  |  |  |
| OverEstimate | 0.57 |  |  |  |  |  |  |
| UnderEstimate | -0.75 |  |  |  |  |  |  |

|  | 20 testing beams [equally distributed] 16 degree electrical downtilt, Ant4 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |  |  |  |  |  |  |
| Beam index | 1 |  | 2 |  | 3 |  | 4 |  | 5 | 6 | 7 |  |  |
| 1 | 28.52 |  | 16.63 |  | 13.08 |  | 4.79 |  | 18.84 | 5.99 | -2.47 |  |  |
| 2 | 29.73 |  | 17.87 |  | 14.37 |  | 6.16 |  | 20.42 | 8.09 | 14.10 |  |  |
| 3 | 29.73 |  | 17.87 |  | 14.37 |  | 6.16 |  | 20.42 | 8.09 | 14.10 |  |  |
| 4 | 28.52 |  | 16.63 |  | 13.08 |  | 4.79 |  | 18.84 | 5.99 | -2.47 |  |  |
| 5 | 22.78 |  | 14.89 |  | 13.60 |  | 7.99 |  | 12.91 | 4.08 | -7.54 |  |  |
| 6 | 23.98 |  | 16.13 |  | 14.89 |  | 9.36 |  | 14.43 | 6.63 | 8.29 |  |  |
| 7 | 23.98 |  | 16.13 |  | 14.89 |  | 9.36 |  | 14.43 | 6.63 | 8.29 |  |  |
| 8 | 22.78 |  | 14.89 |  | 13.60 |  | 7.99 |  | 12.91 | 4.08 | -7.54 |  |  |
| 9 | 20.19 |  | 15.19 |  | 14.95 |  | 16.72 |  | 7.62 | 4.53 | -16.40 |  |  |
| 10 | 21.38 |  | 16.41 |  | 16.23 |  | 18.09 |  | 9.10 | 8.96 | -0.70 |  |  |
| 11 | 21.38 |  | 16.41 |  | 16.23 |  | 18.09 |  | 9.10 | 8.96 | -0.70 |  |  |
| 12 | 20.19 |  | 15.19 |  | 14.95 |  | 16.72 |  | 7.62 | 4.53 | -16.40 |  |  |
| 13 | 18.66 |  | 17.11 |  | 18.23 |  | 24.47 |  | 1.65 | 8.72 | -18.80 |  |  |
| 14 | 19.67 |  | 18.15 |  | 19.32 |  | 25.62 |  | 2.97 | 13.63 | -5.93 |  |  |
| 15 | 19.99 |  | 18.48 |  | 19.66 |  | 25.97 |  | 3.39 | 14.39 | 0.21 |  |  |
| 16 | 19.46 |  | 17.94 |  | 19.10 |  | 25.39 |  | 2.71 | 13.14 | -10.85 |  |  |
| 17 | 17.95 |  | 20.47 |  | 28.85 |  | 20.54 |  | 0.19 | 14.67 | -19.59 |  |  |
| 18 | 18.93 |  | 21.49 |  | 29.92 |  | 21.65 |  | 1.47 | 17.22 | -6.93 |  |  |
| 19 | 19.25 |  | 21.81 |  | 30.26 |  | 21.99 |  | 1.88 | 17.87 | -0.16 |  |  |
| 20 | 18.73 |  | 21.28 |  | 29.70 |  | 21.42 |  | 1.22 | 16.78 | -12.00 |  |  |
| Mean EEIRP | 24.25 |  | 18.13 |  | 23.44 |  | 20.43 |  | 14.05 | 12.12 | 5.49 |  |  |
| CIs | 18.08 |  | 10.93 |  | 23.26 |  | 17.02 |  | 11.72 | 9.44 | 5.08 |  |  |
| Rect Ref | 25.07 |  | 18.84 |  | 24.97 |  | 19.97 |  | 13.76 | 12.58 | 4.67 |  |  |
| Error | -0.82 |  | -0.71 |  | -1.52 |  | 0.46 |  | 0.29 | -0.45 | 0.82 |  |  |
| Mean error | 0.74 |  |  |  |  |  |  |  |  |  |  |  |  |
| OverEstimate | 0.82 |  |  |  |  |  |  |  |  |  |  |  |  |
| UnderEstimate | -1.52 |  |  |  |  |  |  |  |  |  |  |  |  |
| Ellipse Ref | 23.65 |  | 18.43 | 23.81 |  | 20.64 |  | 13.15 |  | 12.42 |  | 5.75 |
| Error | 0.59 |  | -0.31 | -0.37 |  | -0.21 |  | 0.90 |  | -0.30 |  | -0.26 |
| Mean error | 0.43 |  |  |  |  |  |  |  |  |  |  |  |
| OverEstimate | 0.90 |  |  |  |  |  |  |  |  |  |  |  |
| UnderEstimate |  | -0.37 |  |  |  |  |  |  |  |  |  |  |

#### 2.1.4.6. Comparison between different candidate testing beams

The simulation results for above candidate testing beam are summarized in the following table.

| Candidate test beam | Ant 4 with electrical downtilt as 12 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.29 | 0.30 | -0.64 | 0.67 | 1.53 | -1.40 |
| 5x4 | 0.28 | 0.23 | -0.49 | 0.46 | 0.76 | -0.54 |
| 4x4 | 0.48 | 0.26 | -0.87 | 0.48 | 0.57 | -0.75 |
| 5x5 | 0.30 | 0.16 | -0.49 | 0.55 | 0.75 | -1.11 |
| Scaling test beam | 0.48 | 0.26 | -0.87 | 0.48 | 0.57 | -0.75 |

| Candidate test beam | Ant 4 with electrical downtilt as 16 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.25 | 0.26 | -0.49 | 0.81 | 1.42 | -1.57 |
| 5x4 | 0.47 | 0.52 | -1.20 | 0.59 | 1.05 | -0.79 |
| 4x4 | 0.73 | 0.49 | -2.94 | 0.74 | 0.93 | -1.52 |
| 5x5 | 0.44 | 0.02 | -1.21 | 0.69 | 1.05 | -1.23 |
| Scaling test beam | 0.74 | 0.82 | -1.52 | 0.43 | 0.90 | -0.37 |

| Candidate test beam | Ant 1 with electrical downtilt as 12 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.07 | 0.21 | -0.05 | 0.46 | 1.32 | -0.90 |
| 5x4 | 0.07 | 0.05 | -0.31 | 0.35 | 0.80 | -0.85 |
| 4x4 | 0.12 | 0.07 | -0.50 | 0.33 | 0.61 | -0.90 |
| 5x5 | 0.07 | 0.05 | -0.32 | 0.34 | 0.79 | -0.84 |

| Candidate test beam | Ant 1 with electrical downtilt as 16 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.08 | 0.25 | -0.01 | 0.50 | 1.52 | -0.83 |
| 5x4 | 0.11 | 0.04 | -0.53 | 0.35 | 0.74 | -0.83 |
| 4x4 | 0.20 | 0.09 | -0.94 | 0.29 | 0.32 | -0.84 |
| 5x5 | 0.11 | 0.04 | -0.53 | 0.34 | 0.73 | -0.81 |

Observation 1: the performance metric for 5x4 and 5x5 is quite close. The underestimate performance for 5x5 is somehow around 0.5dB worse than 5x4 case if ellipse reference is considered.

Observation 2: performance metric for 5x4 and 4x4 is also quite close when electrical downtilt as 12 degree.

Based on the above observation and also previous agreement reached in Orlando meeting, we propose to use the fixed test beam pattern 5x4 as the final solution for U6GHz BS EEIRP mask conformance testing.

Proposal 1: propose to consider the 5x4 with equally distributed testing beam within the beam steering range.

| Beam index | Elevation angle | Azimuth angle | Weighting factor |
| --- | --- | --- | --- |
| 1 | Θmin+2.5*(Θmax-Θmin)/20 | Φmin+2.5*(Φmax-Φmin)/20 | 1/20 |
| 2 | Θmin+2.5*(Θmax-Θmin)/20 | Φmin+7.5*(Φmax-Φmin)/20 | 1/20 |
| 3 | Θmin+2.5*(Θmax-Θmin)/20 | Φmin+12.5*(Φmax-Φmin)/20 | 1/20 |
| 4 | Θmin+2.5*(Θmax-Θmin)/20 | Φmin+17.5*(Φmax-Φmin)/20 | 1/20 |
| 5 | Θmin+7.5*(Θmax-Θmin)/20 | Φmin+2.5*(Φmax-Φmin)/20 | 1/20 |
| 6 | Θmin+7.5*(Θmax-Θmin)/20 | Φmin+7.5*(Φmax-Φmin)/20 | 1/20 |
| 7 | Θmin+7.5*(Θmax-Θmin)/20 | Φmin+12.5*(Φmax-Φmin)/20 | 1/20 |
| 8 | Θmin+7.5*(Θmax-Θmin)/20 | Φmin+17.5*(Φmax-Φmin)/20 | 1/20 |
| 9 | Θmin+12.5*(Θmax-Θmin)/20 | Φmin+2.5*(Φmax-Φmin)/20 | 1/20 |
| 10 | Θmin+12.5*(Θmax-Θmin)/20 | Φmin+7.5*(Φmax-Φmin)/20 | 1/20 |
| 11 | Θmin+12.5*(Θmax-Θmin)/20 | Φmin+12.5*(Φmax-Φmin)/20 | 1/20 |
| 12 | Θmin+12.5*(Θmax-Θmin)/20 | Φmin+17.5*(Φmax-Φmin)/20 | 1/20 |
| 13 | Θmin+17.5*(Θmax-Θmin)/20 | Φmin+2.5*(Φmax-Φmin)/20 | 1/20 |
| 14 | Θmin+17.5*(Θmax-Θmin)/20 | Φmin+7.5*(Φmax-Φmin)/20 | 1/20 |
| 15 | Θmin+17.5*(Θmax-Θmin)/20 | Φmin+12.5*(Φmax-Φmin)/20 | 1/20 |
| 16 | Θmin+17.5*(Θmax-Θmin)/20 | Φmin+17.5*(Φmax-Φmin)/20 | 1/20 |
| 17 | Θmin+22.5*(Θmax-Θmin)/20 | Φmin+2.5*(Φmax-Φmin)/20 | 1/20 |
| 18 | Θmin+22.5*(Θmax-Θmin)/20 | Φmin+7.5*(Φmax-Φmin)/20 | 1/20 |
| 19 | Θmin+22.5*(Θmax-Θmin)/20 | Φmin+12.5*(Φmax-Φmin)/20 | 1/20 |
| 20 | Θmin+22.5*(Θmax-Θmin)/20 | Φmin+17.5*(Φmax-Φmin)/20 | 1/20 |

  - . Measurement uncertainty

- In the MU uncertainty table, the sampling error and dynamic range error are still TBD. Based on the offline discussions, sampling error for agreed test beams could be calculated in the following way:

- calculate the errors between agreeable testing beams and reference testing beams of different antenna configuration and also different steering range;

- To specify the standard deviations and also confirm its distribution for MU;

- To calculate the MU from the above errors.

|  | Error for Rect Ref | -0.47 |  | -0.29 |  |  |  |  | -0.49 |  |  |  | 0.15 |  | -0.04 |  |  | -0.26 |  |  | 0.23 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ant 4, 12degree | Mean error | 0.28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Error for Ellipse Ref |  | 0.76 |  |  | -0.34 |  |  |  | 0.18 |  |  | 0.50 |  |  | 0.46 |  |  | -0.41 |  |  | -0.54 |  |
|  | Mean error |  | 0.46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Error for Rect Ref | -1.15 |  | -1.20 |  |  |  |  | -0.11 |  |  |  | 0.01 |  | 0.03 |  |  | -0.07 |  |  | 0.52 |  |  |
| Ant 4, 16degree | Mean error | 0.47 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Error for Ellipse Ref |  | 0.27 |  |  | -0.79 |  |  |  | 1.05 |  |  | -0.67 |  |  | 0.65 |  |  | 0.09 |  |  | -0.56 |  |
|  | Mean error |  | 0.59 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Error for Rect Ref | -0.31 |  | 0.02 |  |  |  |  | 0.02 |  |  |  | 0.03 |  | 0.05 |  |  | 0.04 |  |  | 0.00 |  |  |  |
| Ant 1, 12degree | Mean error | 0.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Error for Ellipse Ref |  | 0.80 |  |  |  | 0.17 |  |  |  | 0.02 |  | -0.03 |  |  | -0.08 |  |  | -0.38 |  |  | -0.85 |  |  |
|  | Mean error |  | 0.35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Error for Rect Ref | -0.53 |  |  | 0.04 |  |  |  |  | 0.03 |  |  | 0.03 |  | 0.04 |  |  | 0.04 |  |  |  | 0.01 |  |  |
| Ant 1, 16degree | Mean error | 0.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Error for Ellipse Ref |  | 0.74 |  |  |  |  | 0.23 |  |  |  | 0.03 |  | -0.05 |  |  | -0.12 |  |  | -0.35 |  |  | -0.83 |  |
|  | Mean error |  | 0.35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

According to the errors for option 2 with 5x4 test beams compared with rectangular reference beam sets and ellipse reference beam sets, Kernel density and probability distribution for errors are demonstrated in the following table. Based on the following figure, sampling errors seems to be the normal distribution on top of two different antenna configuration and also two different beam steering range. Based on the above errors, the standard deviation is 0.4573dB which is close to 0.5dB.

Figure 2.2-1. the distribution of error and Kernel density

Proposal 2: propose to define the sampling error as 0.5dB with normal distribution.

Regarding the dynamic range error requirements, we think that the existing dynamic range contribution 0.51dB could be reused since the exact same OTA chamber will be used.

Proposal 3: propose to reuse the dynamic range error 0.51dB for EEIRP measurement with normal distribution.

The detailed examples for CATR measurement could be found as following with exact value to be further discussed.

Table 6.6-1: EEIRP in CATR

| UID | Uncertainty source |  | Uncertainty value (dB) | Distribution of the | Divisor based on | ci |  | Standard uncertainty ui (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6425 < f ≤ 7125 MHz | probability | distribution shape |  |  | 6425 < f ≤ 7125 MHz |
|  |  | Stage 2: BS measurement |  |  |  |  |  |  |
| TBD | Misalignment and pointing error of BS |  | 0.30 | Rectangular | 1.73 | 1 |  | 0.173 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  | 0.26 | Normal | 1.00 | 1 |  | 0.260 |
| A2-2a | Standing wave between BS and test range antenna |  | 0.21 | U-shaped | 1.41 | 1 |  | 0.148 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  | 0.00 | Normal | 1.00 | 1 |  | 0.001 |
| A2-4a | QZ ripple experienced by BS |  | 0.09 | Normal | 1.00 | 1 |  | 0.093 |
| A2-12 | Frequency flatness of test system |  | 0.25 | Normal | 1.00 | 1 |  | 0.250 |
|  | [Dynamic range error] |  | 0.51 | Normal | 1.00 | 1 |  | 0.51 |
|  | [Sampling error] |  | 0.50 | Normal | 1.00 | 1.00 |  | 0.50 |
|  |  | Stage 1: Calibration measurement |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer |  | 0.20 | Normal | 1.00 | 1 |  | 0.20 |
| A2-5a | Mismatch or receiver chain between receiving antenna and measurement receiver |  | 0.33 | U-shaped | 1.41 | 1 |  | 0.23 |
| A2-6 | Insertion loss of receiver chain |  | 0.18 | Rectangular | 1.73 | 1 |  | 0.10 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  | 0.00 | Normal | 1.00 | 1 |  | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable |  | 0.02 | U-shaped | 1.41 | 1 |  | 0.02 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  | 0.43 | Rectangular | 1.73 | 1 |  | 0.25 |
| A2-8 | Misalignment positioning system |  | 0.00 | Exp. normal | 2.00 | 1 |  | 0.00 |
| TBD | Misalignment and pointing error of calibration antenna |  | 0.50 | Exp. normal | 2.00 | 1 |  | 0.25 |
| A2-9 | Rotary joints |  | 0.05 | U-shaped | 1.41 | 1 |  | 0.03 |
| A2-2b | Standing wave between calibration antenna and test range antenna |  | 0.09 | U-shaped | 1.41 | 1 |  | 0.06 |
| A2-4b | QZ ripple experienced by calibration antenna |  | 0.01 | Normal | 1.00 | 1 |  | 0.01 |
| A2-11 | Switching uncertainty |  | 0.26 | Rectangular | 1.73 | 1 |  | 0.15 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 0.98 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 1.92 |

  - . Confidence interval

In the previous RAN4 meetings, there were extensive discussions on CIs, however there are no consensus been reached so far. There were some proposals to calculate the CIs within testing beams, e.g. 20 testing beam with CI stepwise procedure as clarified in 2.1.1. Based on the following simulation results, CIs for different testing beams for single measurement is very large especially for the first bin and 2nd bin. The main reasons that EEIRP per beam per bin vary very big due to the different beamforming directions.

Observation 3: the CIs within the testing beams are very large which also cannot guide the antenna array design.

From our understanding, indeed to calculate the confidence interval of errors which can well represent the accuracy average should be collected among different antenna configurations and also different beam steering range

|  | 20 testing beams [fixed] 12degree electrical downtilt, Ant4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bin index |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Beam index | 1 | 2 |  |  | 3 |  |  | 4 |  |  | 5 | 6 |  |  | 7 |  |  |
| CIs | 22.97 | 6.15 |  |  | 12.01 |  |  | 17.83 |  |  | 12.38 | 5.02 |  |  | 5.71 |  |  |
|  | 20 testing beams [fixed] 16degree electrical downtilt, Ant4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bin index |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Beam index | 1 |  | 2 | 3 |  |  | 4 |  |  |  | 5 |  |  | 6 |  | 7 |  |  |
| CIs | 20.39 |  | 11.16 | 23.42 |  |  | 17.95 |  |  |  | 11.93 |  |  | 9.63 |  | 5.25 |  |  |
|  | 20 testing beams [fixed] 12degree electrical downtilt, Ant1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bin index |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Beam index | 1 | 2 |  |  |  | 3 |  |  | 4 | 5 |  |  | 6 |  |  |  | 7 |  |
| CIs | 25.41 | 14.21 |  |  |  | 9.00 |  |  | 5.53 | 2.09 |  |  | -0.35 |  |  |  | -2.22 |  |
|  | 20 testing beams [fixed],16degree electrical downtilt, Ant1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bin index |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Beam index | 1 | 2 |  |  | 3 |  |  |  | 4 |  | 5 |  |  | 6 |  |  | 7 |
| CIs | 24.18 | 14.07 |  |  | 9.11 |  |  |  | 5.73 |  | 2.48 |  |  | -0.03 |  |  | -0.79 |

Proposal 4: for the confidence interval, if necessary, propose to calculate the confidence interval of errors among different antenna configurations and different beam steering range e.g. 0.5dB*1.96.

Conclusions

In this contribution, we shared some further views on the EIRP mask requirement for band n104 and observations / proposals are made as following:

Observation 1: the performance metric for 5x4 and 5x5 is quite close. The underestimate performance for 5x5 is somehow around 0.5dB worse than 5x4 case if ellipse reference is considered.

Observation 2: performance metric for 5x4 and 4x4 is also quite close when electrical downtilt as 12 degree.

Proposal 1: propose to consider the 5x4 with equally distributed testing beam within the beam steering range.

Proposal 2: propose to define the sampling error as 0.5dB with normal distribution.

Regarding the dynamic range error requirements, we think that the existing dynamic range contribution 0.51dB could be reused since the exact same OTA chamber will be used.

Proposal 3: propose to reuse the dynamic range error 0.51dB for EEIRP measurement with normal distribution.

Observation 3: the CIs within the testing beams are very large which also cannot guide the antenna array design.

Proposal 4: for the confidence interval, if necessary, propose to calculate the confidence interval of errors among different antenna configurations and different beam steering range e.g. 0.5dB*1.96.

References

- R4-2419207	TR 38.908 v1.2.0 Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz	ZTE Corporation, Sanechips

- R4-2419817	Draft CR for TS 38.141-2: Declarations	CATT

- R4-2419818	TP to TR38.908: clause 3 and 5	ZTE Corporation, Sanechips

- R4-2419819	TP to TR38.908: Clause 6.4 spatial sampling grid	ZTE Corporation, Sanechips

- R4-2419820	Draft CR to 38.141-2 –OTA spatial emissions	Huawei, HiSilicon

- R4-2419821	TP to TR 38.908 – section 6.5	Huawei, HiSilicon

- R4-2419822	TP to TR 38.908 – section 6.6	Huawei, HiSilicon

- R4-2419823	Draft CR to TS 38.104 for introduction of expected EIRP mask core requirement	Nokia, ZTE, Ericsson, Huawei, Samsung, Qualcomm, CATT, Spark NZ

- R4-2419824	TP to TR 38.908 clause 6 – Conformance testing	Nokia

- R4-2419825	Draft CR to TS 38.141-2 with Annex N Expected EIRP calculation	Nokia

- R4-2419826 	Discussion on conformance test aspects for EEIRP and TP to TR 38.908	Ericsson

- R4-2419827	Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP	ZTE

- R4-2419911	Draft Big CR to 38.141-2 Introduction of U6GHz BS EEIRP mask	ZTE

Annex

<Start of TP>

6.3	Test beams

6.3.1	General

Considering the practical measurement in the OTA chamber, it is not possible to conduct the excessive measurement from both measurement equipment capability and also extremely long testing time to finis the measurement for all testing beams (e.g. 10k or 8k testing beam). In order to simplify the EEIRP testing in more realistic approach, in the following section, the EEIRP evaluation are conducted to identify the candidate testing beams to achieve similar EEIRP measurement compared with the ideal EEIRP measurement.

6.3.1	Procedures for the EEIRP evaluations

In the following section, for the convenience of the understanding of EEIRP evaluations, the evaluation procedures are provided as following:

- Step 1: to generate the testing beams with the electrical beam steering range , e.g. electrical down-tilt and scanning direction per beam;

- Step 2: use the antenna modelling in TR38.803 to create the radiation pattern per beam;

- Step 3: to calculate the EEIRP according to the EEIRP formula per beam;

- Step 4: to repeat Step 2/3 to get the EEIRP results for all testing beams;

- Step 5: to calculate the EEIRP results for BS among testing beams;

- Step 6: to calculate the mean of EEIRP according to the weighting factor in step 5.

- Step 7: to calculate the standard deviation from N testing beams in step 5 and mean in step 6.

6.3.2	Simulation assumptions for antenna modelling

For antenna modelling with sub-array considered, then following antenna modelling as shown in table 6.3.2-1 is considered in the EEIRP simulation in this contribution.

Table 6.3.2-1: Extended AAS model [extracted from TR38.803]

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=−min−−min12φφ3dB2,Am−min12θ−90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm−1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm−1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm−1dvλsinθetilt−n−1dℎλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm−1dvλcosθ+n−1dℎλsinθsinφ |

The detailed parameters for antenna configuration 4 are listed in the following table 6.3.2-2.

Table 6.3.2-2: Antenna array parameters

| Parameter | Macro Rural |
| --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna sub-array configuration (Row × Column) (Note 4) | 16 × 8 elements |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | 6 |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 26.76 |
| Mechanical down-tilt (degrees) | 0 |

For antenna modelling without sub-array considered, then following antenna modelling as shown in table 6.3.2-3 is considered in the EEIRP simulation in this contribution.

Table 6.3.2-3: Composite antenna pattern

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the super position vector is given by: the weighting is given by: |

The detailed parameters for antenna configuration 1 are listed in the following table 6.3.2-4.

Table 6.3.2-4: Antenna array parameters

| Parameter | Macro Rural |
| --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 4) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.7 of wavelength for V |
| Number of element rows in sub-array | 1 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 26.76 |
| Mechanical down-tilt (degrees) | 0 |

For the EEIRP simulation, two different electrical down-tilt (0-12 degree and 0-16 degree) are considered.

6.3.2.  Methodology on how to create the reference testing beams

In order to get the ideal EEIRP simulation results as much as possible, the following extreme number of testing number e.g. 10K testing beams within rectangular steering range and 8k testing beams within ellipse steering range are evaluated. In the following section, the methodology on how to generate the beams within rectangular and ellipse steering range is clarified in details.

6.3.2.1. Rectangular reference beam sets

Figure 6.3.2.1-1. the illustration of rectangular steering range

The methodology on how to generate the beams within rectangular steering range as shown in Figure 6.3.2.1-1 is described as following:

- To generate the 10k uniform distributed values between 0 degree to 12 degree, say X

- To generate the 10k uniform distributed values between -60 degree to 60 degree, say Y.

- To create rectangular reference beam sets as [X, Y] where Xi is electrical down-tilt of beam i and Yi is electrical scanning of beam i.

6.3.2.2. Ellipse reference beam sets

Figure 6.3.2.2-1. the illustration of ellipse steering range

The methodology on how to generate the beams within ellipse steering range as shown in Figure 6.3.2.2-1is described as following:

- To generate the 8k random values for the angle: theta=2 * pi * rand(N, 1) ;

- To generate the 8k random values for r: r=sqrt(rand(N, 1)) ;

- To create the 8k azimuth angle as following

- Y= 0+60 * r .* cos(theta);

- To create the 8k elevation angle as following

- X= 6 + 6 * r .* sin(theta)

- To create ellipse reference beam sets as [X, Y] where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i.

6.3.3.  Candidate testing beams for EEIRP conformance testing

6.3.3.1. Option 1a: random pattern with 20 testing beams

As shown in the following Figure 6.3.3.1-1, to use the random approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/20=0.6 degree

- To calculate the step size for azimuth angle as 120/25=4.8 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 6.3.3.1-1. random approach to create the testing beams for EEIRP conformance testing

Regarding the metric to check the measurement of simplified testing beam compared with reference testing beams, the following metrics are provided in the table to show its measurement performance.

- Mean error= 10*log10((10^(abs(error_1))/10+10^(abs(error_2))/10+10^(abs(error_3))/10...++10^(abs(error_7))/10 )/7)

- OverEstimate =maximum positive value from errors in the 7 bin

- underEstimate= minimum negative value from errors in the 7 bin

6.3.3.2. Option 2: 20 fixed testing beams for the following pattern

As shown in the following figure 6.3.3.2-1, to use the equally distributed approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/25=0.48 degree

- To calculate the step size for azimuth angle as 120/20=6 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 6.3.3.2-1. equally distributed approach to create the 20 testing beams for EEIRP conformance testing

6.3.3.3. Option 3: 16 fixed testing beams for the following pattern

As shown in the following figure 6.3.3.3-1, to use the equally distributed approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/20=0.6 degree

- To calculate the step size for azimuth angle as 120/20=6 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 6.3.3.3-1. equally distributed approach to create the 16 testing beams for EEIRP conformance testing

6.3.3.4. Option 4: 25 fixed testing beams for the following pattern

As shown in the following figure 6.3.3.4-1, to use the equally distributed approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/25=0.48 degree

- To calculate the step size for azimuth angle as 120/25=4.8 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 6.3.3.4-1. equally distributed approach to create the 25 testing beams for EEIRP conformance testing

6.3.3.5. Option 5: scaling testing beams for the following pattern

As shown in the following figure 6.3.3.5-1/2/3, considering BS capable with different steering range in the elevation angle. According the the previous evaluation results, for electrical downtilt as 12 degree, then 16 testing beams should be sufficient to get the accurate results compared with ideal EEIRP results, however 16 degree electrical downtilt, more testing beams in the elevation angle are expected to achieve the dense testing beam sampling to achieve better accuracy.

Scaling testing beams is proposed as following with fixed step size in the elevation angle as following. If testing beams within the steering range could be select as conformance testing. e.g. if NR BS is capable of downtilt as 12 degree, then 16 test beams should be tested; if NR BS is capable of downtilt as 16 degree, then 20 test beams should be considered.

Figure 6.3.3.5-1. equally distributed approach to create the 16 testing beams for EEIRP conformance testing

Figure 6.3.3.5-2. equally distributed approach to create the 20 testing beams for EEIRP conformance testing

Figure 6.3.3.5-3. equally distributed approach to create the 24 testing beams for EEIRP conformance testing

6.3.4.  Simulation results for EEIRP evaluation

The simulation results for above candidate testing beam are summarized in the following table.

Table 6.3.4-1. simulation results for Antenna configuration 4 with electrical downtilt as 12 degree

| Candidate test beam | Ant 4 with electrical downtilt as 12 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.29 | 0.30 | -0.64 | 0.67 | 1.53 | -1.40 |
| 5x4 | 0.28 | 0.23 | -0.49 | 0.46 | 0.76 | -0.54 |
| 4x4 | 0.48 | 0.26 | -0.87 | 0.48 | 0.57 | -0.75 |
| 5x5 | 0.30 | 0.16 | -0.49 | 0.55 | 0.75 | -1.11 |
| Scaling test beam | 0.48 | 0.26 | -0.87 | 0.48 | 0.57 | -0.75 |

Table 6.3.4-2. simulation results for Antenna configuration 4 with electrical downtilt as 16 degree

| Candidate test beam | Ant 4 with electrical downtilt as 16 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.25 | 0.26 | -0.49 | 0.81 | 1.42 | -1.57 |
| 5x4 | 0.47 | 0.52 | -1.20 | 0.59 | 1.05 | -0.79 |
| 4x4 | 0.73 | 0.49 | -2.94 | 0.74 | 0.93 | -1.52 |
| 5x5 | 0.44 | 0.02 | -1.21 | 0.69 | 1.05 | -1.23 |
| Scaling test beam | 0.74 | 0.82 | -1.52 | 0.43 | 0.90 | -0.37 |

Table 6.3.4-3. simulation results for Antenna configuration 1 with electrical downtilt as 12 degree

| Candidate test beam | Ant 1 with electrical downtilt as 12 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.07 | 0.21 | -0.05 | 0.46 | 1.32 | -0.90 |
| 5x4 | 0.07 | 0.05 | -0.31 | 0.35 | 0.80 | -0.85 |
| 4x4 | 0.12 | 0.07 | -0.50 | 0.33 | 0.61 | -0.90 |
| 5x5 | 0.07 | 0.05 | -0.32 | 0.34 | 0.79 | -0.84 |

Table 6.3.4-4. simulation results for Antenna configuration 1 with electrical downtilt as 12 degree

| Candidate test beam | Ant 1 with electrical downtilt as 16 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.08 | 0.25 | -0.01 | 0.50 | 1.52 | -0.83 |
| 5x4 | 0.11 | 0.04 | -0.53 | 0.35 | 0.74 | -0.83 |
| 4x4 | 0.20 | 0.09 | -0.94 | 0.29 | 0.32 | -0.84 |
| 5x5 | 0.11 | 0.04 | -0.53 | 0.34 | 0.73 | -0.81 |

Based on the above simulation results, the performance metric for 5x4 and 5x5 is quite close. The underestimate performance for 5x5 is even around 0.5dB worse than 5x4 case if ellipse reference is considered. And performance metric for 5x4 and 4x4 is also quite close when electrical downtilt as 12 degree.

6.3.5.  Conclusion

Based on the above observation and the simplicity of conformance testing, 5x4 testing beams with equally distributed method is agreed to defined for conformance testing of U6GHz BS EEIRP mask.

Table 6.3.5-1. simulation results for Antenna configuration 1 with electrical downtilt as 12 degree.

| Beam index | Elevation angle | Azimuth angle | Weighting factor |
| --- | --- | --- | --- |
| 1 | Θmin+2.5(Θmax-Θmin)/20 | Φmin+2.5(Φmax-Φmin)/20 | 1/20 |
| 2 | Θmin+2.5(Θmax-Θmin)/20 | Φmin+7.5(Φmax-Φmin)/20 | 1/20 |
| 3 | Θmin+2.5(Θmax-Θmin)/20 | Φmin+12.5(Φmax-Φmin)/20 | 1/20 |
| 4 | Θmin+2.5(Θmax-Θmin)/20 | Φmin+17.5(Φmax-Φmin)/20 | 1/20 |
| 5 | Θmin+7.5(Θmax-Θmin)/20 | Φmin+2.5(Φmax-Φmin)/20 | 1/20 |
| 6 | Θmin+7.5(Θmax-Θmin)/20 | Φmin+7.5(Φmax-Φmin)/20 | 1/20 |
| 7 | Θmin+7.5(Θmax-Θmin)/20 | Φmin+12.5(Φmax-Φmin)/20 | 1/20 |
| 8 | Θmin+7.5(Θmax-Θmin)/20 | Φmin+17.5(Φmax-Φmin)/20 | 1/20 |
| 9 | Θmin+12.5(Θmax-Θmin)/20 | Φmin+2.5(Φmax-Φmin)/20 | 1/20 |
| 10 | Θmin+12.5(Θmax-Θmin)/20 | Φmin+7.5(Φmax-Φmin)/20 | 1/20 |
| 11 | Θmin+12.5(Θmax-Θmin)/20 | Φmin+12.5(Φmax-Φmin)/20 | 1/20 |
| 12 | Θmin+12.5(Θmax-Θmin)/20 | Φmin+17.5(Φmax-Φmin)/20 | 1/20 |
| 13 | Θmin+17.5(Θmax-Θmin)/20 | Φmin+2.5(Φmax-Φmin)/20 | 1/20 |
| 14 | Θmin+17.5(Θmax-Θmin)/20 | Φmin+7.5(Φmax-Φmin)/20 | 1/20 |
| 15 | Θmin+17.5(Θmax-Θmin)/20 | Φmin+12.5(Φmax-Φmin)/20 | 1/20 |
| 16 | Θmin+17.5(Θmax-Θmin)/20 | Φmin+17.5(Φmax-Φmin)/20 | 1/20 |
| 17 | Θmin+22.5(Θmax-Θmin)/20 | Φmin+2.5(Φmax-Φmin)/20 | 1/20 |
| 18 | Θmin+22.5(Θmax-Θmin)/20 | Φmin+7.5(Φmax-Φmin)/20 | 1/20 |
| 19 | Θmin+22.5(Θmax-Θmin)/20 | Φmin+12.5(Φmax-Φmin)/20 | 1/20 |
| 20 | Θmin+22.5(Θmax-Θmin)/20 | Φmin+17.5(Φmax-Φmin)/20 | 1/20 |

<End of TP>