3GPP TSG-RAN WG4 Meeting #114	R4-2501907

Athens, GR, 17th – 21st February, 2025

Agenda Item:	8.2

Source:	Ericsson

Title:	On UE capability for NTN FR2 200MHz channel bandwidth

Document for:	Discussion

# 1	Introduction

There is an incoming LS from RAN2 asking RAN4 ‘s feedback on whether UE capabilities for 200MHz channel bandwidth and 2 MIMO layers are mandatory for FR2-NTN-bands, for which this contribution gives our views and corresponding observations.

# 2	Discussion

For Rel-18 FR2 NTN, following three new NTN satellite bands have been introduced:

- n512

- n511

- n510

together with SCS of 60kHz and 120kHz. According to the table 5.3.5-2 in TS38.101-4 referred below, the supported channel BWs are:

- 50MHz, 100MHz and 200MHz channel bandwidths for 60 kHz SCS.

- 50MHz, 100MHz, 200MHz and 400MHz channel bandwidths for 120 kHz SCS.

Table 5.3.5-2: Channel bandwidths for each NTN satellite band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |

Based on the table it is obvious that 200MHz channel bandwidth is mandatory to be supported for all SAN operating band for FR2-NTN. On top of that, RAN4 Demod session agreed to introduce performance requirements and test cases assuming 200MHz channel bandwidth. Corresponding content can be found in chapter xxx in TS38.101-4.

Observation 1: 200MHz channel bandwidth is mandatory to be supported without capability for FR2-NTN bands

During the meetings of Rel-18 in RAN4, there have been discussions towards the antenna configuration and MIMO layers. Given the consideration of satellites’ capability and flexibility, it’s agreed to only consider 1Tx for the SAN so that to consider only 1 MIMO layer for the demodulation requirements without any discussion on the UE capability of MIMO layer happened in RAN4. We believe if the 2 MIMO layer is mandatory then there should be corresponding requirements defined in RAN4.

Observation 2: There are no demodulation requirements regarding 2 MIMO layers

Based on the above, we can conclude that UE support for a 200 MHz bandwidth for FR2-NTN bands is mandatory, whereas support for 2 MIMO layers is not.

# 5	Summary

In this contribution, we shared our views regarding the UE capability for 200MHz bandwidth and MIMO layers for NTN-FR2 bands. We summarized our observations in the following:

Observation 1: 200MHz channel bandwidth is mandatory to be supported without capability for FR2-NTN bands

Observation 2: There are no demodulation requirements regarding 2 MIMO layers

# 6	Annex

Draft reply LS attached.

3GPP TSG-RAN WG2 Meeting #114                                                                         R2-250xxxx

Athens, GR, 17th – 21st February, 2025

Title:	Reply LS on UE Capabilities for FR2-NTN

Response to:

Release:	Release 18

Work Item:	NR_NTN_enh-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	xxx

#### E-mail Address:	xxx

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

# 1. Overall Description:

RAN4 would like to thank RAN2 for their LS R2-2411195.

RAN4 would like to reply RAN2 that from RAN4 perspective 200MHz channel bandwidth is mandatory to be supported without capability for FR2-NTN bands, whereas support for 2 MIMO layers is not.

# 2. Actions:

To RAN2 group:

RAN4 respectfully asks RAN2 to take above into account.

# 3. Date of Next RAN4 Meetings:

RAN4#114bis	from 2025-04-07	to 2025-04-11		China, CN

RAN4#115	from 2025-05-19	to 2025-05-23		Malta, MT

# References

[1]	R2-2411195: LS on UE Capabilities for FR2-NTN, RAN2, RAN2 #128, Orlando, USA