3GPP TSG-RAN WG4 Meeting #114	R4-2501924

# Athens, Greece, 17th – 21st  February 2025

Source: 	KDDI Corporation, LGE, Samsung

Title:	TP for adding MSD result for CA_n74-n77 to TR38.719-02-01

Agenda item:	6.3.3

Document for:	Approval

- Introduction

CA_n74-n77_BCS0 with power class 3 (PC3) has been included in TS 38.101-1 [1]. Power class 2 (PC2) for this band combination has been collected in the revised R19 basket WID [2]. However, the MSD analysis for the PC3 case is not available. Therefore, we provide this TP or adding test points and MSD result for CA_n74-n77 to TR38.719-02-01 [3].

- References

[1] 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[2] RP-242865_Revised WID for HPUE_NR_CADC_SUL_R19 (Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL), China Telecom, China Unicom).

[3] TR38.719-02-01, Rel-19 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)

- Text proposal

----- Start of Text Proposal -----

  - 5.X		CA_n74-n77

    - 5.X.1	Common for 1 band UL and 2 bands UL CA

      - 5.X.1.1	Operating bands for CA

Table 5.2.1.1-1:  CA band combination of band n74+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low | - | FUL_high | FDL_low | - | FDL_high |  |
| n74 | 1427 MHz | - | 1470 MHz | 1475 MHz | - | 1518 MHz | FDD |
| n77 | 3300 MHz | - | 4200 MHz | 3300 MHz | - | 4200 MHz | TDD |

      - 5.X.1.2	Channel bandwidths per operating band for CA

Table 5.X.1.2-1: Supported bandwidths per CA band combination of band n74+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n74A-n77A | CA_n74A-n77A | n74 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n74A-n77A | 23 | +2/-3 |

      - 5.x.2.2	UE co-existence studies for 2 bands UL

Table 5.x.2.2-1 lists the 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis result for the UL CA_n74-n77 (2CC). Based on Table 5.X.2.2-1, IMD5 of UL CA_n74-n77 fall into Rx frequencies of bands n74.

Table 5.x.2.2-1: UL CA_n74-n77 IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427 | 1470 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2854 | 2940 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4281 | 4410 | 9900 | 12600 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1830 | 2773 | 4727 | 5670 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1346 | 360 | 5130 | 6973 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6154 | 7140 | 8027 | 9870 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 81 | 1110 | 8430 | 11173 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7581 | 8610 | 11327 | 14070 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5546 | 3660 | 9454 | 11340 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15373 | 11730 | 2580 | 1508 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14627 | 18270 | 9008 | 10080 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9746 | 6960 | 2190 | 4119 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12754 | 15540 | 10881 | 12810 |

      - 5.X.2.3	REFSENS requirements

Test points and MSD value are as shown in Table 5.X.2.3-1.

Table 5.X.2.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n74-n77 | n74 | 1427 | 5 | 25 | 1508 | 5.5 | FDD | IMD516,X |
|  | n77 | 4200 | 10 | 50 | 4200 | N/A | TDD | N/A |
| NOTE 16: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range. NOTE X: This band is also subject to IMD6, for which the MSD is not specified. |  |  |  |  |  |  |  |  |

----- End of Text Proposal -----