3GPP TSG-RAN WG4 Meeting #114									                        R4- 2501936

Athens, Greece, Feb. 17 – 21, 2025

Agenda item:			7.6.2

Source:	Ericsson

Title:	Discussion on Signaling of Fragmented Carriers

Document for:	Discussion

# Introduction

Fragmented carriers in the same band requiring multiple Rx chains may limit the UE CA capabilities if part of complex band combinations with many DL CCs combined with DL MIMO in fragmented operator spectrum holdings. A new RAN4 study item was approved in the RAN #104 meeting to investigate solutions to address this.

The objectives of the SI are as follows [1]:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4];

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

The following sections will discuss some signaling-related issues of fragmented carriers.

# Discussions

## Signaling related

#### Issue 2-4: Signaling related [2]

WF:

Companies are encouraged to provide analysis with pros and cons on preferred signalling solutions so that they can be discussed in next meeting.

During the last meeting, various signaling-related proposals were presented by companies. It is mainly focused on how the UE should indicate the fragmented carrier operation. In our opinion, the necessity of whether relaxation is needed, or to what extent the relaxation would be, needs to be decided first, taking into account the risk of proliferation of combinations with these relaxed requirements using a single Rx chain, and afterward, decide how to indicate these particular band combinations. RAN4 can further discuss whether additional capabilities are needed. For instance, a slight relaxation of a few dB (if necessary) is unlikely to affect a network’s decision to configure or release a cell for a supported band combination.

- The necessity and extent of relaxation should first be determined, considering the risk of an increased proliferation of band combinations arising from these relaxed requirements using a single Rx chain.

The options on how the UE would indicate the fragmented carriers’ operation that were not captured in the WF are listed below:

Option 1: The new CA configuration supported only by fully shared RX chains could be reported to the NW together with other default supported CA configuration

Option 2: Several alternatives can be considered for band combination reporting

- Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

- Alt. 2: Band combination capability update with the supported number of CCs

- Alt. 3: New band combination definition e.g. CA_F_n25(2A) + new set of signaling

Option 3: One-bit per-band capability signaling to indicate the supported CA configuration within a band combination

- the capability signalling only for the testing purpose

Option 4: Legacy band combination reporting scheme + Rx sharing capability indication

- UE shall indicate whether is applies fully-shared Rx for intra-band DL NCCA per band in the highest order configuration reporting

Option 5: The currently undefined NR CA bandwidth class ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation

Among the proposed alternatives, we prefer those that re-use the existing capability reporting framework to the maximum extent and minimize the amount of signaling to be exchanged. Both aspects would considerably expedite the adoption of the feature if it is decided to be standardized. However, the selected option also needs to be backward compatible with networks that may not support the new signaling. For instance, options 1, 3, and 4 lack the possibility for a legacy network to differentiate between shared and non-shared RX chain. Regarding option 2, both Alt. 1 and Alt. 3 suffer from the same issue as the previously mentioned options. Option 5 seems to be a way forward provided it is backward compatible and does not significantly increase the amount of capability signaling. Nonetheless, there may be more options that RAN2 should consider when making the signaling decision. Therefore, before consulting RAN2 about the signaling means, the expected UE and network behavior upon the configuration of fragmented carriers need first to be clarified.

- The signalling method should re-use the existing capability reporting framework to the maximum extent and minimize the amount of signaling to be exchanged.

- It is necessary to clarify the expected network and UE behavior before deciding on the signaling means.

### PCell DL performance degradation

The analysis in [3] indicated that the UE receiver gain setting must be changed due to the sensitivity being degraded by 13.1 dB on PCC and 13.5 dB on SCC respectively, which would affect both the DL PCell and DL SCell. If these are the actual reduction of the UE OTA/conducted sensitivity, the DL PCell coverage would be significantly reduced when this band combination is configured. The PCell coverage may be lost even if the actual DL PCell coverage reduction is not as bad as a degradation of the RAN4 minimum requirement. The network must ensure there is a sufficient DL margin before configuring this band combination to maintain PCell coverage.

- Degradation of PCell performance is a problem due to the lack of a mechanism for the UE to indicate the order of the band entries of an NCCA BC (i.e., the PCell position)

- A band combination implemented by a single Rx chain with significant degradation on PCell REFSENS is not considered feasible.

## Triggering condition

#### Issue 2-5: Triggering condition to enable ”One RF Rx chain”

RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled

- Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled

  - Other determining factors are not precluded to be studied

- Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2

The SID clarified that the PSD level between two co-located CCs should be within 6 dB, it is important to note that the 6 dB PSD imbalance for intra-band collocated CA is only the minimum requirement for UE conformance testing. However, in real-world deployment, a 6 dB power imbalance between two non-contiguous CCs cannot always be guaranteed in real deployment, the network(s) can’t maintain a 6 dB PSD imbalance in the field at all times and UEs may still be able to receive the wanted carriers with adequate performance at a PSD difference greater than the minimum performance.

It was agreed that the PSD difference under the study indicates the difference between the unwanted signal (in-gap interference) and the wanted signal. This represents a realistic situation, as the in-gap interference is from a different operator (Operator B) than the two carriers (Operator A). One can assume that the operators A and B are co-located. Therefore, there may be a high imbalance between the power received by the UE from these two operators. Given the uncertain condition of the blocker, a maximum PSD difference of 25 dB could be considered. Besides, the PSD level varies a lot depending on the antenna sector arrangement, beamforming gain, and other fading factors, etc. In scenarios where the interference is spread out over the entire gap between two CCs, PSD matters as it affects the amount of power present in the gap. The absolute power of in-gap interference will be critical when the interference is narrowband with a strong signal.

Issue 2-5 “triggering condition to enable one RF Rx chain” seems a bit ambiguous. Is the UE going to configure its Rx chain based on the received in-gap blocker level and is the network supposed to reconfigure the UE like other capabilities?, e.g. MIMO layer support? Normally, the UE declares its capability to support “one RF Rx chain” operation via RRC signaling, the network uses this information to configure UE with certain BC. Will the next step be that the UE continuously measures in-gap interference, and if the interference exceeds a threshold, then the UE sends a report back to the network? What the network should do after receiving such information remains unclear.

Figure 1. Visual representation of CCs and blocker.

- Clarify the definition and procedure of “triggering one RF Rx chain”.

## Support of 2Rx in the 4Rx mandatory band

There were some discussions on the support of 2Rx in the 4Rx mandatory bands in the previous meetings which provided another possibility to free up the Rx path to allow more CCs, however, due to the time limit, the debate did not continue [4][5]. In our opinion, it is worth noting that the current implementation already supports the solution, and the network may need the least action/effort to configure it.

It is noted that the number of layers supported per CC is a capability, the UE needs to only support 4 layers for non-CA transmission (w.r.t single CC), as shown in TS 38.306 and excerpted below. Currently, there is no requirement to support 4 layers per CC for CA for any type of band combination.

Table 1 Excerpt from TS 38.306

| maxNumberMIMO-LayersPDSCH Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE does not support MIMO on this carrier. For the bands where pdsch-1024QAM-2MIMO-FR1-r17 is indicated, MIMO layers for 1024 QAM is the smaller value between 2 and maxNumberMIMO-LayersPDSCH. | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

- UE can reduce the number of supported layers for NCCA, as there is only a requirement on 4 layers for bands with 4Rx in single CC.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	The signalling method should re-use the existing capability reporting framework to the maximum extent and minimize the amount of signaling to be exchanged.

Observation 2	It is necessary to clarify the expected network and UE behavior before deciding on the signaling means.

Observation 3	Degradation of PCell performance is a problem due to the lack of a mechanism for the UE to indicate the order of the band entries of an NCCA BC (i.e., the PCell position)

Observation 4	UE can reduce the number of supported layers for NCCA, as there is only a requirement on 4 layers for bands with 4Rx in single CC.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	The necessity and extent of relaxation should first be determined, considering the risk of an increased proliferation of band combinations arising from these relaxed requirements using a single Rx chain.

Proposal 2	A band combination implemented by a single Rx chain with significant degradation on PCell REFSENS is not considered feasible.

Proposal 3	Clarify the definition and procedure of “triggering one RF Rx chain”.

# References

- RP-241669, Study on NR FR1 DL Fragmented Carriers

- R4-2420337, WF on NR FR1 DL fragmented carriers study, MediaTek Inc.

- R4-2418862, Discussion of impacts on UE RF requirements and DL performance impacts, MediaTek Inc.

- R4-2415533, On in-gap and adjacent blockers level for fragmented spectrum reception in a single Rx path, Skyworks Solutions Inc.

- R4-2418301, Discussion on Fragmented carriers, Samsung.