3GPP TSG-RAN WG4 Meeting #114									                        R4- 2501938

Athens, Greece, Feb. 17 – 21, 2025

Agenda item:			7.12.1.1

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE inter-band CA

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements addressed the general concerns [2]. Part of the agreements regarding the inter-band are shown below:

Issue 3-1: Assumed parameters when supporting n39 UL for CA_n3A-n39A with common antenna.

WF:

- Further check these two sets of parameters

  - Option 1

| Parameters | Values |
| --- | --- |
| Antenna Isolation | 20dB |
| The rejection for band n3 Rx filter at 1885~1915MHz | 50dB |
| n39 Tx filter rejection at 1805~1880MHz | 50dB |

  - Option 2

| Parameters | Values |
| --- | --- |
| Antenna Isolation | 20dB |
| The rejection for band n3 Rx filter at 1885~1915MHz | 20dB |
| n39 Tx filter rejection at 1805~1880MHz | 40dB |

- Consider the assumed parameters with separate antenna.

Issue 3-3: MSD for CA_n3A-n39A with n39 UL

Agreement:

- Define the MSD value with 40dBm output power

- FFS on how to scale MSD for other output power level.

- Following the parameters in the table below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | [TBD] | ACLR2 |

Issue 1-1-2: UE antenna type for inter-band CA

Agreement:

- Refine the agreement from last meeting as:

  - For intra-band contiguous CA, same antenna type should be applied on a band.

  - For inter-band CA, either different or same antenna type can be applied on different bands.

  - For inter-band CA, consider the following cases:

    - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC.

    - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC.

    - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC.

    - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.]

    - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed.

Further check which understanding is correct:

- Understanding #1: Case 4 is excluded in RF session.

Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

  - Inter-band CA

### Whether to allow n39 UL for CA_n3A-n39A

The bands n3 and n39 are defined in the table below.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |

The CA_n3-n39 is an FDD and TDD inter-band combination, specifically, the upper bound of the n3 DL is next to the lower bound of the n39 without any gap in between. Therefore, the actual deployment scenarios/configuration would impact the potential interference.

Essentially, for the inter-band CA, PCell can operate either in FDD band or TDD band. To mitigate the potential interference, if only one UL transmission is assumed in band n3 for DL CA_n3-n39, then FDD band n3 could serve as the PCell and TDD band n39 serve as SCell while n39 is configured as DL to avoid interference with the n3 DL. It would be easier to adopt the following proposal to mitigate the potential interference:

However, it is worth noting that the above discussion was based on the handheld UE architecture. ATG UE is a different type of UE with a larger form factor. Therefore, the isolation is larger than that of handheld UE, which could make the n39 UL feasible for CA_n3A-n39A. This approach needs to be further verified by thoroughly studying the ATG UE architecture.

- The n39 UL may be feasible for CA_n3A-n39A due to its larger form factor and isolation than handheld UE.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	The n39 UL may be feasible for CA_n3A-n39A due to its larger form factor and isolation than handheld UE.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2420338, WF on Rel-19 ATG UE RF requirements

- TS 38.101-1, V18.5.0