3GPP TSG-RAN WG4 Meeting #114	R4-2501956

Athens, Greece, 17th – 21st February 2025

Source:	Nokia

Title:	On General Aspects of AI/ML for NR Air Interface

Agenda item:	7.19.1

Document for:	Discussion

Introduction

The objectives of WI NR_AIML_air RP-243244 ‎[1] were updated in RAN#106 and following scope was agreed for RAN4:

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In the above scope, mentioned three use cases are:

- Beam management

- Positioning accuracy enhancements

- CSI prediction (UE-sided model):

In this paper, we share our views on the following topics:

- LCM related aspects

Generalization and post deployment related aspects are treated in our accompanying paper ‎[2] for RP-243245 New SID for NR_AIML_air_Phase2.

Discussion

LCM related aspects

Following agreements were reached in RAN4#113 meeting:

Agreement: - The discussion should consider use-cases specific aspects

Agreement: - Whether transition from AI to non-AI would be UE initiated or network initiated - This will be a decision for other groups - RAN4 to assess what RRM requirements are needed for each case.

General

In RAN2 the discussions on LCM aspects (definitions and procedures) have been ongoing since meeting RAN2 #127. Most of the agreements reached are use case specific, with notable exception of the definitions on supported, applicable and activated functionalities. These definitions are to be used in both UE-sided beam management and UE-sided positioning use case specific discussions and design of LCM procedures.

Agreements related to LCM from RAN2#127:

Agreements on definitions from #127: - Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) - Applicable functionalities refers to functionalities that the UE is ready to apply for inference - Activated functionalities refers to functionalities already enabled for performing inference

In response to the earlier LS sent from RAN2 to RAN1, RAN2 received the clarifications from RAN1 on the use of ‘functionality’ terminology (mostly based on the beam management use case work) [R1-2410898]:

RAN1 reply to LS from RAN2: - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “ functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

In RAN#4 the definitions related to LCM agreed by RAN1 and RAN2 can be adopted when developing core requirements for LCM.

We quote below the most LCM relevant RAN1 and RAN2 agreements available so far from the work on beam management and positioning use cases.

For the UE-sided beam management use case, RAN2 agreed as working assumption on a 5 step procedure, intended to cover the main signalling steps from UE (ML) capability enquire up to NW provisioning and activation of the ML functionality/configuration.

Agreements related to LCM from RAN2#127, #127-bis and #128 meetings:

Agreements on procedures from #127: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig. - 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. - 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: - 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI - 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: - 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). - 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality Agreements from #127-bis: - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content - FFS if inference configuration can be signalled in step3. - UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality) Agreements from #128: - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content. - When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable. - FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability. Verify this aligns with RAN1 configuration design. - Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover. - Source cell UAI (as is) can be sent from source cell to target cell using existing signaling. No further optimizations will be considered in RAN2 related to UAI.

Related to the Steps described above, in response to the earlier LS sent from RAN2 to RAN1 [R2-2407848], RAN2 received the following clarifications from RAN1 [R1-2410898]:

RAN 1 made the following agreements related to the Questions from RAN 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 RAN1 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. RAN1 Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig Any specification impact is a separate discussion

Furthermore, the noted open issue after the RAN #106 was: Related to the topic of LCM for UE-sided model for Beam Management use case, RAN2 can continue analyzing the details of LCM-related signalling procedures, focusing on the FFS agreed during RAN2#128. Additionally, RAN2 should discuss the RAN1 reply to the RAN2 LS (R2-2407848) about the applicability functionality reporting, and evaluate the impacts in RAN2 protocols.

Use case related aspects for the beam management (BM) and positioning use case are discussed in our accompanying papers on BM ‎[3] and Positioning ‎[4] respectively.

Agreements related to LCM from RAN2#127-bis meeting:

Agreements from #127-bis: - The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: - Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. - Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. - Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). - Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. - Step 5: The LMF requests the inferred location information using the LPP request location information message. - Step 6: UE reports the inferred location using LPP provide location information message. Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

The noted open issue after the RAN #106 was: Related to the topic of LCM for Positioning use case, RAN2 can continue the discussion on how to handle dynamic capabilities considering the FFFs from RAN2#128, such as the configurations details and the fallback options when the AIML based positioning method becomes non-applicable. Further, RAN2 should identify issues (if any) from which RAN1 feedback is needed.

As starting point, RAN4 to analyze the core requirements for LCM based on the 5-step procedure agreed in RAN2 and take in consideration the use-case specific details provided by RAN1.

One of the main LCM related aspects for which RAN4 requirements might be required are the delay requirements for the 5-step procedure agreed in RAN2. Out of these steps, the signalling and execution of Step 4 and 5  seem to be the most critical ones for correct AI/ML operations at the UE-side.

As starting point, RAN4 to analyze the delay requirements as part of core requirements for LCM Step 4 and Step 5, from the agreed 5-step procedure in RAN2.

Conclusion

In this contribution we elaborated further our view on LCM related aspects. Following Observation and Proposals are made in this paper:

Observation 1: In RAN#4 the definitions related to LCM agreed by RAN1 and RAN2 can be adopted when developing core requirements for LCM.

Proposal 1: As starting point, RAN4 to analyze the core requirements for LCM based on the 5-step procedure agreed in RAN2 and take in consideration the use-case specific details provided by RAN1.

Proposal 2: As starting point, RAN4 to analyze the delay requirements as part of core requirements for LCM Step 4 and Step 5, from the agreed 5-step procedure in RAN2.

- References

- RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#106, December 9-12, 2024, Madrid, Spain.

- R4-2501958, “General aspects of AI/ML for NR air interface Phase 2”, Nokia, RAN4#114, 17 – 21 February 2025, Athens, Greece.

- R4-2501957, “RRM core requirements for AI/ML Based Beam Management”, Nokia, RAN4#114, 17 – 21 February 2025, Athens, Greece.

- R4-2502071, “RRM core requirements for AI/ML Positioning accuracy enhancement”, Nokia, RAN4#114, 17 – 21 February 2025, Athens, Greece.