3GPP TSG-RAN WG4 Meeting #114	R4-2501957

Athens, Greece, 17th – 21st February 2025

Source:	Nokia

Title:	RRM core requirements for AI/ML Based Beam Management

Agenda item:	7.19.3

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#113 in the scope of Rel-19 WI based. Agreements achieved during this meeting are summarized in the WF ‎[1].

The objectives of WI NR_AIML_air RP-243244 ‎[2] were updated in RAN#106 and following scope was agreed for RAN4:

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In the above scope, mentioned three use cases are:

- Beam management

- Positioning accuracy enhancements

- CSI prediction (UE-sided model):

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion still in the scope of RP-243244 [2], In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Measurement error impact and the ground truth

- KPIs for beam prediction

- Indicated/activated TCI state requirements

- Measurement delay requirements

- Impacts on requirements related to candidate beam detection

- LCM and performance monitoring related requirements

- Applicable functionality reporting

Generalization, test coverage and testing setup related aspects are discussed in our accompanying paper ‎[3] for RP-243245 New SID for NR_AIML_air_Phase2 ‎[4].

Discussion

Measurement error impact and the ground truth Agreed simulation assumptions are available here.

During RAN4#112 and RAN4112bis meetings, RAN4 agreed to study the impact of measurement errors to AI/ML BM functionality on the performance of prediction of AI/ML BM. The agreements of RAN4 #113 are provided below:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Agreement: ​ RAN4 to perform study to assess the impact of measurement errors on prediction accuracy​ Perform LLS to evaluate the RSRP prediction accuracy and other KPIs​ - UE Model trained using ideal measurements​ Simulation assumptions in R4-2417211 can be used as starting point.

Due to the nature of analog RF design, the RF power will vary every time it is used. Therefore, there are calibration tables created from the manufactory for each single RF circuit assembly into the UE devices. The main contribution of RF error is the power amplifier (PA) on the RF circuit.  Each single PA is unique in the world, it has its own parameters that are different from any other PA’s even from the same factory and produced at the same time. Normally the manufactory will provide a statistical table to show that the PAs produced on the same day from the same production line follow a certain distribution. Such distribution should meet the RF design specification of the PA. When those PAs are used in RF circuits, there are also various algorithms for Tx/Rx RF chains, such as automatic gain control (AGC) and feedback loop control, to correct the power level of Tx/Rx RF circuits in real-time.

The stability of RF input/output can also be significantly impacted for other reasons. The temperature of PA, battery levels, the human body and other surroundings of UE. Therefore, each single use of RF circuit will generate different results, this is so called RF power uncertainty. Though there are calibration tables and various algorithms for Tx/Rx RF chains to compensate the RF circuits inaccuracy, those can hardly compensate the RF errors    in the field. This is the reason that we still keep some tolerance in the standard to limit the PA power variations within a certain range in 38.101 series, and the close loop between UE and gNB can still see the uncertainty of UE power. There is no clear specification for Rx beam gain in FR2 in 3GPP, however, due to the complexity of designing PAs for Rx and the factors to impact PA power, we should expect similar tolerance of Rx beam gain compared to that of the Tx beam gain.

Due to the nature of analog circuit design of RF, the RF uncertainty will be presenting all the time, even there are calibration tables, and algorithm, such as AGC and feedback-loop, to compensate Tx/Rx uncertainty of RF circuits, the RF error will still be presenting in the field.

RF error will be presenting as independent error for every time RF circuit is used for Tx/Rx. The error will however follow statistics distribution.

RAN4 should consider RF error model with independent error on each Tx beam.

In this section, we evaluate the impact of measurement errors for an AI/ML model in inference operation. We consider spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement as a baseline. The AI/ML model’s output is the Top-K predicted beam IDs.

Methodology

We followed the procedure described in Section 7 of R4-2420480, where we trained an AI/ML model with a subset of samples from an ideal L1-RSRP dataset generated from the system level simulation (SLS) following the assumptions defined in TR 38.843 Table 6.3.1-1.

Next, we followed the “Approach A” of R4-2420480, and we used the LLS simulation assumptions defined in Table 3 of Annex I to generate the Baseband (BB) errors. We also modelled the RF errors, and we added both BB and RF errors to the subset of ideal L1-RSRP data to derive the dataset with measurement errors as follows,

RSRP with measurement errors = Ideal RSRP + Baseband (BB) error + Radio frequency (RF) error

Given the data obtained by this approach, we evaluate the performance metrics considering as the input of the AI/ML model the RSRP with measurement errors and we use as ground-truth for evaluating the predictions using the ideal RSRP.

The results are compared to the performance metrics without considering measurement errors, for which we used another subset of samples from the ideal L1-RSRP dataset generated from the SLS and without measurement errors.

We evaluated the performance of the AI/ML model with Top-K accuracy for K =1, and 3, and we evaluated the 90%-tile of L1-RSRP difference between the maximum RSRP of the Top-K predicted beam(s) and the RSRP of the genie aided strongest beam.

For BB error modeling, we consider following options:

- No error: This is the baseline case where BB errors are not considered in AI/ML model input.

- LLS (# samples = 3, SINR = -4 dB): The BB error distribution obtained by LLS simulations based on the LLS assumption in Table 3 of Annex I for side conditions where number of samples in L1 averaging=3 and SINR = -4 dB. Gaussian distribution: 95% of users are distributed within ±2.5 dB error margin with zero mean.

For RF error modeling, we consider following options:

- No error: This is the baseline case where RF errors are not considered as input of the AI/ML model.

- Gaussian (Independent for each TX beam): 95% of users are distributed within ±4.5 dB or ±3 dB error margin with mean zero. The RF error is generated independently for each RSRP measurement of TX beam in a single inference instance so that different TX beams are measured with different RF errors.

- Uniform distribution (Independent for each TX beam): All users are distributed within ±4.5 dB error margin with mean zero. The RF error is added independently for each RSRP measurement of TX beam in a single inference instance so that different TX beams are measured with different RF errors.

- Gaussian distribution (Same for all TX beams): 95% of users are distributed within ±4.5 dB error margin with mean zero. The same RF error is added to all the RSRP measurements of the different TX beams for a single inference instance.

In addition to the above-mentioned measurement error configurations, we separate low SNR UEs and high SNR UEs among the subset of ideal L1-RSRP dataset as follows:

- Low SNR UEs: 5% of UEs who are experiencing lowest SNR among all UEs, and SNR of UE is calculated for the highest quality beam using SLS results.

- High SNR UEs: 5% of UEs who are experiencing highest SNR among all UEs, and SNR of UE is calculated for the highest quality beam using SLS results.

Then the impact of measurement error is considered for low SNR UEs, high SNR UEs and all UEs (including high SNR and low SNR UEs) as shown in Table 1.

Table 1. Evaluation of BB and RF measurement error for UEs in high and SNR range.

| Measurement error |  | L1-RSRP difference (Low SNR UEs) | L1-RSRP difference (All UEs) | L1-RSRP difference (high SNR UEs) | Top1 (Low SNR UEs) | Top1 (All UEs) | Top1 (High SNR UEs) | Top3 (Low SNR UEs) | Top3 (All UEs) | Top3 (High SNR UEs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BB | RF |  |  |  |  |  |  |  |  |  |
| No BB error | No RF error | 7.84 | 5.63 | 0.55 | 66.35 | 83.78 | 92.53 | 94.17 | 96.48 | 99.04 |
| LLS (# samples = 3, SINR = -4 dB). | Gaussian with ±4.5 dB error margin (Independent for each TX beam) | 10.93 | 13.04 | 8.85 | 55.46 | 70.55 | 75.71 | 88.47 | 92.36 | 95.26 |
|  |  | 3.09 | 7.41 | 8.30 | 10.89 | 13.23 | 16.82 | 5.7 | 4.12 | 3.78 |
|  | Gaussian with ±3 dB error margin (Independent for each TX beam) | 9.76 | 10.35 | 3.19 | 59.47 | 76.05 | 82.71 | 90.91 | 94.48 | 97.29 |
|  |  | 1.92 | 4.7 | 2.64 | 6.88 | 7.73 | 9.8 | 3.26 | 2.00 | 1.75 |
|  | Uniform with ±4.5 dB error margin (Independent for each TX beam) | 10.68 | 13.99 | 12.62 | 52.93 | 67.75 | 71.88 | 86.42 | 90.97 | 93.69 |
|  |  | 2.84 | 8.36 | 12.07 | 13.42 | 16.03 | 20.65 | 7.75 | 5.51 | 5.35 |
|  | Gaussian with ±4.5 dB error margin (Same for all TX beams) | 8.51 | 5.84 | 0.54 | 67.19 | 83.63 | 92.98 | 93.76 | 96.51 | 99.02 |
|  |  | 0.67 | 0.21 | 0 | 0.84 | 0.15 | 0.45 | 0.41 | 0.03 | 0.02 |
| Gaussian (95% of UEs in ±2.5 dB). | Gaussian with ±4.5 dB error margin (Independent for each TX beam) | 10.94 | 13.84 | 12.44 | 53.09 | 68.77 | 73.16 | 86.74 | 91.36 | 94.44 |
|  |  | 3.10 | 8.21 | 11.89 | 13.26 | 15.01 | 19.37 | 7.43 | 5.12 | 4.60 |
|  | Gaussian with ±3 dB error margin (Independent for each TX beam) | 9.65 | 11.86 | 5.17 | 58.21 | 73.74 | 80.54 | 90 | 93.59 | 96.61 |
|  |  | 1.81 | 6.23 | 4.62 | 8.14 | 10.04 | 11.99 | 4.17 | 2.89 | 2.43 |
|  | Uniform with ±4.5 dB error margin (Independent for each TX beam) | 10.61 | 14.60 | 13.85 | 53.41 | 66.30 | 69.67 | 86.47 | 90.26 | 93.16 |
|  |  | 2.77 | 8.97 | 13.30 | 12.94 | 17.48 | 22.86 | 7.70 | 6.22 | 5.88 |
|  | Gaussian with ±4.5 dB error margin (Same for all TX beams) | 8.86 | 5.72 | 0.63 | 68.15 | 83.66 | 91.8 | 94.24 | 96.54 | 98.95 |
|  |  | 1.02 | 0.09 | 0.08 | 1.80 | 0.12 | 0.73 | 0.07 | 0.06 | 0.09 |

Note: L1-RSRP difference = max(abs(95%-tile L1-RSRP), abs(5%-tile L1-RSRP)) and gray colored field highlight the difference of KPI with respect to the scenario with no errors (row 1).

Impact of SNR (without considering measurement error):

Results show that the 90%-tile L1-RSRP difference for low SNR UEs is 7.84 dB in contrast to 0.55 dB of high SNR UEs in the case where no measurement error is considered. The same trend is observed in terms of Top-K accuracy. The main reason for this observation is that the AI/ML model is trained with a mixed dataset containing low and high SNR UEs to get better generalization, and as a result, the low SNR UEs show low prediction accuracy even before considering measurement errors.

When no measurement error, low SNR UEs experience a high 90%-tile L1-RSRP difference and high SNR UEs experience a low 90%-tile L1-RSRP difference. The same trend is observed in Top-K accuracy.

Impact of SNR (considering measurement error):

When the measurement error is added to RSRP measurements, both low SNR and high SNR UEs are experiencing performance degradation in beam prediction related KPIs. However, the impact of measurement error is higher for low SNR UEs compared to high SNR UEs based on the performance degradation which is observed comparatively to the AI/ML model performance without measurement error. For high SNR UEs, there can be a strong beam and rest of the beams may have relatively low strength in terms of L1-RSRP. Addition of measurement error tends to make higher number of prediction errors. Hence, RSRP error drops significantly. Contrary, low SNR UEs may have a set of weak beams in terms of L1-RSRP and relative L1-RSRP difference between beams could be very close to each other beam which cause beam prediction challenging even without measurement error. Addition of measurement error does not decrease the beam prediction drastically as in high SNR UEs because already UEs with low SNR suffering from low prediction accuracy.

We considered the same BB error derived based on LLS (# samples = 3, SINR = -4 dB) conditions for all UEs regardless of the SNR of the UE. Similarly, Table 3 in Annex 1 considers BB error derived based on LLS (# samples = 5, SINR = 0 dB) with a higher SINR target, and the result verifies the afore-mentioned trend for a BB error model derived using a higher SINR target. Hence, we show that regardless of the BB error model, high SNR UEs experience a higher performance degradation compared to low SNR UEs in the presence of measurement error.

With measurement error, all UEs experience degradation in beam prediction-related KPIs. However, a severe impact is observed with high SNR UEs compared to low SNR UEs.

Evaluation of Gaussian with ±4.5 dB error margin vs. Uniform with ±4.5 dB error margin RF error:

If we consider the Gaussian BB error model, results shows a Top-1 accuracy reduction of 15.01% with Gaussian RF error (Independent for each TX beam) compared to the scenario without measurement error.  For the same BB error model, the Uniform distribution (Independent for each TX beam) shows the highest degradation of 17.48% in terms of Top-1 accuracy compared to the scenario with no measurement error. The Top-1 accuracy degradation of Gaussian is lower than the Uniform distribution as most of the RF errors are concentrated near the mean value zero, and few RF errors appear further away from the mean.

The performance degradation of AI/ML model in presence of Gaussian RF error is lower than that of Uniform distribution as most of the RF errors are concentrated near the mean value zero, and few RF errors appear further away from the mean.

Evaluation of independent RF errors vs. the same RF error for different TX beams:

For Gaussian RF error independent for each TX beam, results show that with RF+BB errors for both LLS and Gaussian, the impact on performance KPIs is significant. On the other hand, when using the same RF error, the Top-K accuracy reduction is less than 2% for all BB error models compared to no error case. Similarly, the 90%-tile L1-RSRP difference shows less than 1 dB degradation when the same RF error is applied for all TX beams. Additionally, same trend is observed for high SNR UEs as well as low SNR UEs. Therefore, using the same RF error for all TX beams does not significantly impact the beam prediction accuracy. This is similar to apply a RSRP offset for AI/ML model’s input and therefore, since normalization is applied, the AI/ML model is able to predict the strongest beam IDs without impacting the accuracy.

The performance degradation of AI/ML model is high when RF error is added independently. Contrary, performance degradation is minimal when RF error is same for all TX beams. Hence, the performance degradation difference between two cases is significant.

RAN4 to clarify the impact of measurement error is higher for low SNR UEs or high SNR UEs, given the significant degradation in prediction performance observed with high SNR UEs.

Given the significant degradation in prediction performance observed with independent RF error,    RAN4 shall discuss: 
          - Whether such degradation of prediction performance metrics (Top-1 decreases by 13% and 90%-tile L1-RSRP difference decreases by 7 dB) with respect to the case without errors is acceptable or not.

RAN4 to discuss the RF error model since a large difference of the prediction accuracy was observed with different RF error models. Discuss whether the RF error model that assumes the same RF error for all TX beams is feasible or not.

Furthermore, in RAN4#112bis, in general aspects agenda item, following agreement was achieved:

Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

Regarding, FFS on target performance of beam prediction and RSRP prediction for BM use case, based on the output of ongoing study on the measurement error impacts on RSRP prediction, RAN4 will be in better position to decide if at the advantage of measurement reduction, worse than legacy performance for RSRP prediction can be acceptable or not.

RAN4 should consider, based on the study on the measurement error impacts on RSRP prediction, if worse than legacy performance for RSRP prediction can be acceptable if there is significant reduction in the measurements.

If independent error is assumed on each beam, then worse than legacy performance is observed otherwise, the performance is comparable to legacy.

KPIs for beam prediction

RAN4 identified different test metrics to assess and establish performance requirements of AI/ML Beam Prediction. The metrics were categorized into different options, see below, in 3GPP TR 38.843 V18.0.0. And the targeted use cases for the metrics include beam prediction for BM-Case1 and BM-Case2. Here, we are discussing the merits of these options and providing our view on which of these option(s) might be relevant.

During RAN4#112, four options of the test metrics/KPIs for beam prediction are proposed as follows:

The following test metrics/KPIs for beam prediction are considered:

-	Option 1: RSRP accuracy

-	Option 2: Beam prediction accuracy

-	Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam"

-	Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams", where K >1 and values can be reported.

-	Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams"

-	Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – xdB,

-	Option 4: combinations of above options

During RAN4#112bis, the following definition of absolute RSRP accuracy as a test metric in option 1 has been agreed.

Agreement​ If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on ground truth L1-RSRP].

In addition to the agreed metric/KPI in option 1, we introduced new metrics to assist testing framework and cover the testing when the ground truth is not known. The proposed metrics/KPIs are follows.

Relative RSRP accuracy

The definition of relative RSRP accuracy for AI/ML BM is still under discussion. The definition of relative L1-RSRP accuracy in legacy is provided for SSB and CSI-RS beams in TS 38.133 Clause 10.1.20.1.2 and Clause 10.1.20.2.2, respectively. For SSB, it is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell (serving cell or cell with different PCI from serving cell) on which UE performs L1-RSRP measurements. Similarly for CSI-RS, it is defined as the L1-RSRP measured from one CSI-RS compared to the largest measured value of L1-RSRP among all CSI-RS resources of the serving cell. Based on the definition, the relative accuracy for SSB/CSI-RS beam bk, denoted by rRSRP(bk), can be formulated as

rRSRPbk=maxbk'∈K0RSRPbk'-RSRPbk

Where K0∈{b1,…,bK0 } is the SSB/CSI-RS beam set, K0 are the total number of beams in the beam set and RSRPbk is the measured L1-RSRP of beam bk.

Utilizing the similar definition in the context of AI/ML beam management, relative accuracy of one of the top-K predicted beams bk is given as

rRSRPbk=maxbk'∈K0RSRPbk'-RSRPbk

Now Option 3, which discusses the successful rate of correct prediction evaluates the condition that the maximum L1-RSRP among top-K predicted beams is larger than the L1-RSRP of the strongest beam - X dB. The condition, Coption3, can then be expressed as

Coption3=maxbk∈K0RSRPbk>maxbk'∈K0RSRPbk'-X

Here, K0∈{b1,..,bK} is the top-K predicted beam set.

Ranking the beams in the beam sets K0 and K0 based on their measured L1-RSRPs, denote their corresponding ranked beam index sets as Kr and Kr, respectively. Hence, the ranked beam sets are Kr∈{br=1,..,br=K}, Kr∈{br=1,…,br=K0}. One can clearly identify that br=1 is the beam with the maximum L1-RSRP among the top-K predicted beams and br=1 is the strongest beam.

Hence, 
Coption3= RSRPbr=1>RSRPbr=1-X

Rearranging the terms in Coption3, we obtain RSRPbr=1-RSRPbr=1<X↔rRSRPbr=1<X

Hence, we identify that Coption3is evaluated by comparing the relative RSRP of beam b1r with the X dB threshold

Coption3=rRSRPbr=1<X

So, the successful rate of correct prediction as defined by option 3 and denoted by STop-Kmax, is expressed as

STop-Kmax=1Nt n=1NtlrRSRPbr=1<X

Where Nt is the number of inferences and lCoption3 is the indicator function which is 1 if the condition Coption3 is true and 0 if the condition is false.

The issue with option 3 is that it only considers the beam with the maximum L1-RSRP among the Top-K predicted beams or alternatively the top-1 predicted beam br=1 in it’s evaluation of STop-Kmax. However, it is important to note that the evaluation should consider the correct prediction as the event of predicting Top-K beam set and NOT just the set containing Top-1 predicted beam. Additionally, network may indicate to the UE to use any of the top-K predicted beams in the field, and if the prediction accuracy of other beams is not verified then it may cause a degraded performance in the field. To address these drawbacks, we introduce a more useful and accurate indicator of the prediction performance in real field deployment, STop-K, to evaluate the successful rate of correct prediction as follows:

STop-K=1Nt n=1Nt1Wbr∈KrwbrlrRSRPbr<X

Where W=br∈Krwbr

Where the weights corresponding the beam br can be scenario/case specific. For example, when K=2, Kr consists of two predicted beams and a large weighing factor can be assigned to the beam with highest RSRP.

RAN4 to consider STop-K to accurately evaluate the successful rate of correct prediction.

Beam IDs prediction accuracy

In this option, when AI/ML functionality outputs Top-K beam IDs prediction, RAN4 discussed following metrics:

Top-1(%): This is the success rate, based on a number of occurrences, when Top-1 predicted beam is also the actual Top-1 strongest beam. The number of occurrences required for this metric needs yet to be discussed in RAN4. In our view, this is the strictest requirement, and the test may be failed due to even smallest prediction errors. Therefore, either RAN4 should have a relaxed performance target for this metric, or this metric should not be considered in RAN4.

Top-1(%) metric of Option2 is the strictest requirement and RAN4 should not consider it as a performance metric.

Top-K/1(%): This is the success rate, based on a number of occurrences, when Top-1 actual strongest beam is one of the Top-K predicted beams, where K>1. The number of occurrences required for this metric needs yet to be discussed in RAN4. This metric only guarantees that Top-1 actual strongest beam is one of the predicted beams. It doesn’t verify the accuracy of prediction by the AI/ML functionality for other beams. Therefore, this metric should not be considered alone. Although, one solution may be to use this metric in combination with the Top-1/K(%) metric.

Top-K/1(%) metric of Option2 only verifies if actual strongest beam is among Top-K predicted beams and it doesn’t verify the accuracy of prediction for other beams.

Top-K/1(%) metric of Option 2 should be considered in the combination of Top-1/K(%).

Top-1/K (%) : This is the success rate, based on a number of occurrences, when Top-1 predicted beam is one of the Top-K actual strongest beams, where K>1. The number of occurrences required for this metric needs yet to be discussed in RAN4. This metric only guarantees if the prediction of Top-1 predicted beam has a reasonable accuracy. It doesn’t verify the accuracy of other predictions. Since, network may indicate to the UE to use any of the predicted beams in the field, and if the prediction accuracy of other beams is not verified then it may cause a degraded performance in the field. Therefore, this metric should not be considered alone. Although, one solution may be to use this metric in combination with the Top-K/1(%) metric.

Top-1/K(%) metric of Option2 only verifies if Top-1 predicted beam is among Top-K actual strongest beams and it doesn’t verify the accuracy of other predictions.

Relative difference in prediction due to change in conditions/inputs:

We introduced Option 6 as another option, which is to define the requirements not based on UE measurement as a reference, but to verify the expected behaviour of AI/ML-enabled functionality based on the change in the configurations/inputs of the functionality. For example, the best predicted beam should not change, if the power of all measured beams is scaled in the same way or when one of the measured beams cannot be received anymore, etc. In other words, the AI/ML functionality can be tested without explicit definition of the ground truth.

The behaviour of beam prediction functionalities can be tested without explicit definition of the ground truth by comparing the predictions to each other.

RAN4 to introduce additional requirements based on relative difference in between the predictions due to change in the conditions/inputs, i.e., without dependency on the measurement accuracy errors and uncertainties.

Relative difference in prediction due to change in controlled conditions/input should be considered as an additional metric to test the robustness of prediction in combination of other approximated ground truth-based options.

Indicated/activated TCI state requirements

Following agreement was reached in RAN4#113 meeting:

Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

In Rel-19 AI/ML BM, only Top-1 or Top-K/1 DL Tx beams out of Set A is considered. The DL Rx beam paired with the predicted DL Tx beam using Set B DL Tx beams as input to the AI/ML functionality is out of scope of Rel-19 AI/ML BM following the approval in RAN#102 Rel-19 WI on AI/ML for NR Air Interface [RP-234039]. In RAN #106 the scope of the WI was further updated in the revised WID [RP-243244]. Here is an extract from the revised WID [2]:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

However, as it is assumed in legacy that UE Rx beam is known to the UE whenever UE reports any L1-RSRP measurement. This is a UE implementation issue in legacy and should remain a UE implementation in AI/ML based BM use case as well. Therefore, the legacy assumption of knowledge of Rx beam paired with the measured/predicted Tx beam should remain applicable for AI/ML BM.

Regarding TCI state associated with a predicted DL Tx beam, the NW can determine whether any changes are required in the set of active TCI states, and if yes, the NW can adapt the set of active TCI states based on the prediction report received from the UE. Here, the NW needs to ensure that the RS resource corresponding to the activated/indicated beams which can be measured by the UE prior any beam indication takes place corresponding to the predicted RS resource.

In the case of UE uses subset of beam measurements (Set B) to predict beam(s) from Set A, the UE may not have measured the RS resource included in the indicated/activated TCI state. Since the predicted beam(s) is not measured by the UE for longer period, the predicted beam(s) will be corresponding to unknown TCI state. The delay for switching the beam to an unknown TCI state is higher than switching to a known TCI state. Therefore, in our view, for a UE that supports beam prediction, there is a need to relax the known conditions for TCI state, specified in Clause 8.10.2 of TS 38.133:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

If this condition is relaxed, then the predicted beam (which may not be part of Set B beams) corresponding to a Set A/subset of set A, may be measured with a longer delay than 1280 ms and still fulfil TCI known state condition. In other words, a higher value of the delay requirements should be used for known TCI state conditions in Clause 8.10.2 of TS 38.133 for predicted beams.

In order to relax the known conditions of TCI state such that the predicted beam could be measured, e.g.,with a delay longer than 1280 ms, we need to further specify on how long the delay could be possible. Further analysis of this delay requirements or simulation results might be needed to define the exact delay requirements of known TCI states.

RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for BM-Case1 and BM-Case2. The TCI state switch command is received within a delay X*1280 ms. FFS: Further analysis on defining X values.

Measurement delay requirements

Following agreement was reached during RAN4#113:

Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS

The delay component related to CSI-RS measurement period could be addressed for measurement delay. This delay component can also be mapped into the delay due to CSI-RS measurement period component in Table 2, Section 2.6.

For NW-sided model, the UE would need to report the measured L1-RSRP for both BM-Case1 (spatial domain beam prediction) and BM-Case2 (temporal domain beam prediction) based on measurement delay condition. For UE-sided model, the UE also needs to follow the measurement delay condition when performing beam prediction.

In BM-Case2, multiple time instances of measurement reporting need to be reported as observation window. The CSI report configuration will associate to time requirement defined with measurement period, where the corresponding delay related to measurement should be within measurement period (TL1-RSRP_Measurement_Period_CSI-RS) defined in Clause 9.5.4.2 TS 38.133. However, the current measurement period for L1-RSRP measurement, TL1-RSRP_Measurement_Period_CSI-RS,  in TS 38.133 Clause 9.5.4.2 could not be applied for AI/ML beam prediction especially BM-Case2. Since the current measurement period in Clause 9.5.4.2 does not include when there are multiple time instances measurements corresponding to observation window for BM-Case2. The new measurement reporting requirements will require to be addressed with inference delay to allow multiple time instances measurements for BM-Case2.

The below table shows the measurement period for L1-RSRP reporting in legacy.

Table 9.5.4.2-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2 Configuration; TL1-RSRP_Measurement_Period_CSI-RS (ms). non-DRX; max( TReport, ceilM*P*N*TCSI-RS ). DRX cycle ≤ 320ms; max( TReport, ceil 1.5*M*P*N)*max⁡( TDRX, TCSI-RS). DRX cycle > 320ms; ceilM*P*N* TDRX. Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. Treport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3..

RAN4 should consider the impact on legacy measurement requirements for BM-Case2. It needs further analysis if legacy measurement requirements can be reused for BM-Case1.

Impacts on requirements related to candidate beam detection

In current RAN4 requirements, as mentioned in Clause 8.5.6.2 of TS 38.133, the UE is supposed to evaluate during a certain time period the L1-RSRP measured on the configured CSI-RS resource in the candidate beam set (called q1) containing CSI-RS resources configured for a serving cell. The objective is to verify if the L1-RSRP measured on the configured CSI-RS resource has not been degraded too much and is still better than a certain threshold (Qin_LR). The certain period used to evaluate L1-RSRP is known as TEvaluate_CBD_CSI-RS and is specified in Table 8.5.6.2-2 of clause 8.5.6.2 in TS 38.133.

Table 8.5.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2 Configuration; TEvaluate_CBD_CSI-RS (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(MCBD ´ P ´ N ´ PCBD) ´ TCSI-RS). DRX cycle > 320ms; Ceil(MCBD ´ P ´ N ´ PCBD) ´ TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length..

- However, with the addition of AI/ML BM feature, the UE is supposed to predict best beams from the Set A beams and the UE is configured to measure only a few beams (i.e. Set B). Therefore, the network will often configure only CSI-RS of the Set B beams, therefore the CSI-RS periodicity of Set A beams may be increased. If the NW uses same CSI-RS periodicity also for candidate RS resources, it may lead to a higher value of TCSI-RS used for the calculation of TEvaluate_CBD_CSI-RS. This will eventually lead to a higher value of  TEvaluate_CBD_CSI-RS, which consequently may cause a delayed beam recovery operation and may cause frequent beam failures A higher value of TCSI-RS in Table 8.5.6.2-2 may lead to a higher value of TEvaluate_CBD_CSI-RS, which consequently may be the cause of frequent beam failures.

RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

LCM and performance monitoring related requirements

LCM related requirements

Agreement:

- The discussion should consider use-cases specific aspects

Following agreements was reached in RAN4#113 meeting:

Agreement:

- Whether transition from AI to non-AI would be UE initiated or network initiated

  - This will be a decision for other groups

  - RAN4 to assess what RRM requirements are needed for each case.

Agreements on definitions from RAN2#127:

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

In RAN2 the discussions on LCM aspects (definitions and procedures) have been ongoing since RAN2 #127. The LCM procedures and definitions that are used for UE-sided model beam management are defined in the following agreements.

RAN1 reply to LS from RAN2:

- The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs

- The concept/terminology “ functionality”  of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

In response to the earlier LS sent from RAN2 to RAN1, RAN2 received the clarifications from RAN1 on the use of ‘functionality’ terminology in [R1-2410898]:

The LCM operations for AI/ML BM, e.g., activation, deactivation, and fall-back (e.g., fall-back from AI/ML beam prediction to legacy mode) could be supported by CSI report configuration. These operations can be triggered, for example, based on the performance monitoring output. Reusing the existing capability framework, NW can configure CSI report configurations to enable BM-Case1 or BM-Case2 beam prediction. The CSI reports for both BM-Case1 and BM-Case2 can be based on periodic, semi-persistent, or aperiodic CSI-reporting. The latency of these operations will depend on multiple factors such as,

- Whether CSI configuration (ResourceSet) to be used is already available in the device or not,

- The type of the CSI reporting (periodic, semi-persistent, aperiodic),

- The availability of (previous) measurements that shall be used as an input to the model/functionality inference/prediction,

- Availability of the necessary model(s) in the device,

- The time it takes to substitute one model/functionality with another (the UE may have specialized AI-hardware accelerators to run the inference, which may require loading of the model),

However, it should be clearly specified how much time it takes for the UE to report the new predictions/inference results following the changes indicated by the LCM operation. Without such information NW will not be able to decide when to schedule grants for the reporting of measurements/predictions or data and taking too much time in these operations may also degrade the performance.

In LCM operation, the functionality activation operation is the first step when the UE receives from NW the necessary inference configuration needed for running the inference with the AI/ML functionality available at the UE. Then, other operations, e.g., deactivation, switching and fallback, come after functionality activation operation.

For AI/ML beam prediction reporting enabled by periodic CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

- delay related to RRC Reconfiguration,

- delay from UE processing time: This may be related to the size of the inference report, model-related pre-processing time or memory limitations on storing models (associated with beam prediction),

- delay related to associated ID: This delay will depend on if the associated ID is used or not used in the past, there might be delay corresponding to configure/indicate the associate ID to UE,

- delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related to RRC Reconfiguration,

  - delay from UE processing time

  - delay due to CSI-RS measurement period.

For AI/ML beam prediction reporting enabled by semi-persistent CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via semi-persistent CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay from activating semi-persistent CSI resource set and delay for selecting of semipersistent CSI resource set.

  - delay from UE processing time

  - delay related to associated ID

  - delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay from NW selecting of semi-persistent CSI resource set.

  - delay from UE processing time related

  - delay due to CSI-RS measurement period.

For AI/ML beam prediction reporting enabled by aperiodic CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related activating aperiodic CSI resource set,

  - delay from UE processing time

  - delay related to associated ID,

  - delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related NW selecting of aperiodic CSI resource set,

  - delay from UE processing time related

  - delay due to CSI-RS measurement.

In summary, the delay components related to LCM operations are provided in Table 2.

Table 2. Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Switching | Fall-back (From AI/ML BM to legacy) |
| --- | --- | --- | --- |
| Periodic CSI-reporting | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Semi-persistent CSI-reporting | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Aperiodic CSI reporting | Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report Delay due to CSI-RS measurement period |

The delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

RAN4 should consider Table 2 as a baseline to start discussions for LCM operations related RRM core requirements.

Performance monitoring related requirements

In RAN1#118 bis and RAN1#119, there are ongoing discussions [R1-2408544, #RAN1 112bis] on a need to determine the monitoring window for performance monitoring instances, where the total number of valid performance monitoring instances should be considered when UE calculates performance monitoring metric/KPI.

For UE-sided model, the delay requirements related to performance monitoring of AI/ML-enabled functionality (CSI-ReportConfig) depending on performance monitoring mechanism especially for UE-assisted performance monitoring option. The UE could calculate one of the performance metrics and report it to the NW or UE could report event to NW based on the performance metric.

Accordingly, RAN4 will need to define new delay requirements on the reporting of the performance metric(s) related to the duration of the monitoring window when the UE calculates performance monitoring metric/KPI or performs event-based performance monitoring. When the UE calculates and reports the performance monitoring metric, then NW would interpret those reports and, if necessary, makes a decision and indicates to UE about functionality (CSI-ReportConfig) selection/activation/ deactivation/switching/ fallback operation.

Following the RAN1 discussions, the performance information is calculated per sample (one-shot), or per set of samples (monitoring window). In the case, when a monitoring window is used, RAN 4 requirements will need to be defined.

RAN4 should consider the impact in RAN4 requirements for performance monitoring window when UE reports performance metric or event to the NW.

Applicable functionality reporting

RAN2 agreed as working assumption on a 5 steps procedure, intended to cover the main signalling steps from UE capability enquire up to NW provisioning and activation of the ML functionality.

Agreements related to LCM from RAN2#127, #127-bis and #128 meetings:

Agreements on procedures from RAN2#127:

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

- 1) UE is allowed to do UAI reporting via OtherConfig.

- 2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

- 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS  how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality in the following scenarios:

- 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

- 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

- Step 5:

- 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

- 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.  FFS the initial activation state.  FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.  FFS if multiple applicable functionalities can be activated at the same time.  FFS what is the granularity of functionality

Agreements from RAN2#128:

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design.

- Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

- Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

Agreements from RAN2#127-bis:

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

Related to the Steps described above, in response to the earlier LS sent from RAN2 to RAN1 [R2-2407848], RAN2 received the following clarifications from RAN1 [R1-2410898]:

RAN 1 made the following agreements related to the Questions from RAN 2:

- In Step 3, following configurations are provided from NW to UE:

- UE is allowed to do UAI reporting via OtherConfig,

- The applicability report is based on A) and/or B)

- It is up to RAN 2 to design the container

- A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

- Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

- B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

- It is up to RAN2 to design the container.

- The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

- the associated ID

- Note: this doesn’t imply the associated ID is mandatory

- Set A related information

- Set B related information

- Report content related information

- For BM-Case 2,

- Time instances related information for measurements

- Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

- FFS on whether/what other information along with the applicability is needed

- If A) is configured in Step 3,

- Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

- Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

- Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

RAN1 Agreement

For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

RAN1 Conclusion

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

Any specification impact is a separate discussion

Regarding five steps procedures to cover the signalling steps from UE capability enquire to NW. Within these steps, the step 4 and step 5 are most critical ones to ensure applicable functionality reporting procedure.

As starting point, RAN4 should analyse the delay requirements for applicable functionality reporting based on 5-step procedures agreed in RAN2.

Conclusion

In this paper we share our views on RAN4 impacts from issues related to RRM core requirements for AI/ML enabled BM-Case1 and BM-Case2. Specifically, we cover following aspects:

In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Measurement error impact and the ground truth

- KPIs for beam prediction

- Indicated/activated TCI state requirements

- Measurement delay requirements

- Impacts on requirements related to candidate beam detection

- LCM and performance monitoring related requirements

- Applicable functionality reporting

In the paper, the following Observations and Proposals were made:

Observation 1: Due to the nature of analog circuit design of RF, the RF uncertainty will be presenting all the time, even there are calibration tables, and algorithm, such as AGC and feedback-loop, to compensate Tx/Rx uncertainty of RF circuits, the RF error will still be presenting in the field.

Observation 2: RF error will be presenting as independent error for every time RF circuit is used for Tx/Rx. The error will however follow statistics distribution.

Proposal 1: RAN4 should consider RF error model with independent error on each Tx beam.

Observation 3: When no measurement error, low SNR UEs experience a high 90%-tile L1-RSRP difference and high SNR UEs experience a low 90%-tile L1-RSRP difference. The same trend is observed in Top-K accuracy.

Observation 4: With measurement error, all UEs experience degradation in beam prediction-related KPIs. However, a severe impact is observed with high SNR UEs compared to low SNR UEs.

Observation 5: The performance degradation of AI/ML model in presence of Gaussian RF error is lower than that of Uniform distribution as most of the RF errors are concentrated near the mean value zero, and few RF errors appear further away from the mean.

Observation 6: The performance degradation of AI/ML model is high when RF error is added independently. Contrary, performance degradation is minimal when RF error is same for all TX beams. Hence, the performance degradation difference between two cases is significant.

Proposal 2: RAN4 to clarify the impact of measurement error is higher for low SNR UEs or high SNR UEs, given the significant degradation in prediction performance observed with high SNR UEs.

Proposal 3: Given the significant degradation in prediction performance observed with independent RF error,    RAN4 shall discuss:

- Whether such degradation of prediction performance metrics (Top-1 decreases by 13% and 90%-tile L1-RSRP difference decreases by 7 dB) with respect to the case without errors is acceptable or not.

Proposal 4: RAN4 to discuss the RF error model since a large difference of the prediction accuracy was observed with different RF error models. Discuss whether the RF error model that assumes the same RF error for all TX beams is feasible or not.

Proposal 5: RAN4 should consider, based on the study on the measurement error impacts on RSRP prediction, if worse than legacy performance for RSRP prediction can be acceptable if there is significant reduction in the measurements.

Proposal 6: RAN4 to consider STop-K  to accurately evaluate the successful rate of correct prediction.

STop-K=1Nt n=1Nt1Wbr∈KrwbrlrRSRPbr<X

Where W=br∈Krwbr

Observation 7: Top-1(%) metric of Option2 is the strictest requirement and RAN4 should not consider it as a performance metric.

Observation 8: Top-K/1(%) metric of Option2 only verifies if actual strongest beam is among Top-K predicted beams and it doesn’t verify the accuracy of prediction for other beams.

Proposal 7: Top-K/1(%) metric of Option 2 should be considered in the combination of Top-1/K(%).

Observation 9: Top-1/K(%) metric of Option2 only verifies if Top-1 predicted beam is among Top-K actual strongest beams and it doesn’t verify the accuracy of other predictions.

Observation 10: The behaviour of beam prediction functionalities can be tested without explicit definition of the ground truth by comparing the predictions to each other.

Proposal 8: RAN4 to introduce additional requirements based on relative difference in between the predictions due to change in the conditions/inputs, i.e., without dependency on the measurement accuracy errors and uncertainties.

Proposal 9: Relative difference in prediction due to change in controlled conditions/input should be considered as an additional metric to test the robustness of prediction in combination of other approximated ground truth-based options.

Proposal 10: RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for BM-Case1 and BM-Case2. The TCI state switch command is received within a delay X*1280 ms. FFS: Further analysis on defining X values.

Proposal 11: RAN4 should consider the impact on legacy measurement requirements for BM-Case2. It needs further analysis if legacy measurement requirements can be reused for BM-Case1.

Proposal 12: RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

Proposal 13: The delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

Proposal 14: RAN4 should consider Table below as a baseline to start discussions for LCM operations related RRM core requirements.

Table. Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Switching | Fall-back (From AI/ML BM to legacy) |
| --- | --- | --- | --- |
| Periodic CSI-reporting | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Semi-persistent CSI-reporting | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Aperiodic CSI reporting | Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report Delay due to CSI-RS measurement period |

Proposal 15: RAN4 should consider the impact in RAN4 requirements for performance monitoring window when UE reports performance metric or event to the NW.

Proposal 16: As starting point, RAN4 should analyse the delay requirements for applicable functionality reporting based on 5-step procedures agreed in RAN2.

- References

- R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm Incorporated, RAN4#113 meeting, 18th – 22nd November 2024, Orlando, USA.

- RP-243244, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#106 meeting, 9 – 12 December 2024, Madrid, Spain.

- R4-2501959, “Testing framework and procedure for one-sided models”, Nokia, RAN4#114, 17 – 21 February 2025, Athens, Greece.

- RP-243245, “New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm Incorporated, RAN#106 meeting, 9 – 12 December 2024, Madrid, Spain.

- Annex I (LLS assumptions for BB error modeling)

Table 3. Baseline LLS assumptions for AI/ML in beam management evaluations.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Channel model | AWGN channel |
| BS antenna configurations | Option 2: 1 Tx antenna |
| BS antenna element radiation pattern | Option 2: N/A |
| BS antenna height and antenna array down-tilt angle. | Option 2: N/A |
| Beamforming characteristic of the BS pattern | Option 2: N/A |
| UE antenna configurations | Option 2: 2 Rx |
| UE antenna element radiation pattern | Option 2: N/A |
| UE moving speed | 3km/h |
| Reference signal | CSI-RS |
| DRX | No |
| Number of samples in L1 averaging | [3, 5] |

Table 4. Evaluation of BB and RF measurement error for UEs in high and SNR range with BB error model: LLS (# samples = 5, SINR = 0 dB).

| Measurement error |  | L1-RSRP difference (Low SNR UEs) | L1-RSRP difference (All UEs) | L1-RSRP difference (high SNR UEs) | Top1 (Low SNR UEs) | Top1 (All UEs) | Top1 (High SNR UEs) | Top3 (Low SNR UEs) | Top3 (All UEs) | Top3 (High SNR UEs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BB | RF |  |  |  |  |  |  |  |  |  |
| No BB error | No RF error | 7.84 | 5.63 | 0.55 | 66.35 | 83.78 | 92.53 | 94.17 | 96.48 | 99.04 |
| LLS (# samples = 5, SINR = 0 dB). | Gaussian with ±4.5 dB error margin (Independent for each TX beam) | 10.71 | 12.81 | 8.11 | 56.55 | 71.23 | 75.8 | 88.49 | 92.47 | 95.63 |
|  |  | 2.28 | 7.18 | 7.56 | 9.8 | 12.55 | 16.73 | 5.68 | 4.01 | 3.41 |
|  | Gaussian with ±3 dB error margin (Independent for each TX beam) | 9.97 | 9.98 | 2.68 | 60.47 | 76.94 | 83.85 | 91.48 | 94.78 | 97.65 |
|  |  | 2.13 | 4.35 | 2.13 | 5.88 | 6.84 | 8.68 | 2.69 | 1.7 | 1.39 |
|  | Uniform with ±4.5 dB error margin (Independent for each TX beam) | 10.53 | 13.73 | 12.18 | 52.4 | 68.34 | 72.82 | 86.08 | 91.21 | 94.49 |
|  |  | 2.69 | 8.10 | 11.63 | 13.95 | 15.44 | 19.71 | 8.09 | 5.27 | 4.55 |
|  | Gaussian with ±4.5 dB error margin (Same for all TX beams) | 8.68 | 5.79 | 0.54 | 67.53 | 83.71 | 92.69 | 94.14 | 96.6 | 98.95 |
|  |  | 0.84 | 0.16 | 0.01 | 1.18 | 0.07 | 0.16 | 0.03 | 0.12 | 0.09 |