3GPP TSG-RAN WG4 Meeting #114									R4-2501973

Athen, GR, 17th – 21nd February, 2025

Agenda item:	7.14.2.1

Source:	CAICT

Title:	Discussion on XR OTA

Document for:	Approval

1	Introduction

During the RAN4#113 meeting, an alternative test method to address the battery issue for XR devices was agreed, and CR R4-2419806 was approved [1]. However, some offline discussions took place during the last meeting regarding the definition and use of Peak EIS. The decision was made to hold off on resolving this issue until companies can collect measurement results for further verification.

In this contribution, we presents measurement results and provides further comments on this issue.

2	Low UL power solution for XR OTA

According to the previous WF [2], The draft CR in R4-2419806 for Low UL power solution is approved.

Issue 2-3-4: Low UL power solution Agreements: - The draft CR in R4-2419806 for Low UL power solution is approved

The detailed of this low UL power solution is presented as below.

During the offline discussion in RAN4#113 meeting, the key issue regarding the low uplink power testing method for XR devices was how to accurately define and use the peak value to calculate the offset and ensure the precision of TRS measurements based on SPOT approach. Currently, there are two ways to define the peak EIS:

- Total EIS method: Identify the peak total EIS direction from step 1, which means the peak search is conducted using the total EIS value, i.e., the sum of the results from both polarizations (θ and φ).

- Single Polarization Method: Identify the peak EIS direction and corresponding polarization from step 1, with the focus placed on the peak EIS of a single polarization (either θ or φ).

The SPOT method is not a new concept, it has been a well-established approach in CTIA certification for many years [3]. It has been primarily used to reduce the test time for TRP measurements in multi-protocol devices operating in the equivalent channels, such as between standalone LTE and LTE CA. The assumption is that the antenna patterns do not change significantly between these protocols, allowing the TRP measurement to be offset from one protocol to another.

In the context of SPOT, peak value is defined as the maximum EIRP measured in a single polarization direction for a given channel. Although the low uplink power SPOT testing method discussed in this paper differs from the CTIA TRP measurement scenario in its specific application, the fundamental principles remain similar. In both cases, the DUT should uses the same physical radiator and aperture tuning, ensuring that the antenna's working mode do not change significantly, and then derive the corrected TRP/TRS value based on offset calculation. Therefore, it is reasonable to adopt the aligned definition of peak value when defining the low uplink power test method for XR devices. Specifically, the peak value is defined as the maximum EIS measured in a single polarization direction for a given channel.

Observation 1: It is reasonable to adopt the aligned definition of peak value for the SPOT method in both low uplink power solutions for XR OTA and the CTIA SPOT method, as they are based on the same fundamental principles and considerations.

Given some company consider low UL power testing as similar to desense level testing, which differs from the CTIA testing scenario. Using peak values in a single polarization may affect the accuracy of offset evaluations. To better understand this, we conducted some measurements to further verify. We tested 5 commercial smartphones in B2, N1, and N41 frequency bands. The results were compared to traditional full-power TIS measurements (standard OTA method), which served as a reference. This helped us assess the deviation caused by the alternative methods.

Table 1. TRS Measurement results using low UL power SPOT method (Single Polarization Method)

| Band | DUT | TRS |  | EIS @peak direction |  | Results |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TRS_full power (Reference) | TRS_low power | EIS_full power | EIS_low power | TRS_SPOT | Deviation |
| B2 | DUT 1 | -97.70 | -99.84 | -100.29 | -102.29 | -97.84 | -0.14 |
| N1 | DUT 1 | -95.80 | -96.40 | -97.93 | -97.93 | -96.40 | -0.60 |
|  | DUT 2 | -96.11 | -96.45 | -98.43 | -98.93 | -95.95 | 0.16 |
|  | DUT 3 | -96.75 | -97.08 | -98.48 | -98.98 | -96.58 | 0.16 |
|  | DUT 4 | -95.63 | -95.99 | -98.33 | -98.83 | -95.49 | 0.14 |
| N41 | DUT 1 | -85.46 | -85.60 | -87.00 | -87.00 | -85.60 | -0.14 |
|  | DUT 2 | -86.76 | -86.95 | -89.42 | -89.42 | -86.95 | -0.19 |
|  | DUT 3 | -87.32 | -87.57 | -86.80 | -86.80 | -87.57 | -0.25 |

Observation 2: According to the measurement results, it can be seen that the deviation caused by the SPOT method using single polarization peak EIS is mostly less than 0.3dB, which is very small.

Proposal 1: The peak value is defined as the maximum EIS measured in a single polarization direction for a given channel. Further refine the draft CR R4-2419806 as in Annex.

3	Conclusion

This contribution makes the following observations and proposals.

Observation 1: It is reasonable to adopt the aligned definition of peak value for the SPOT method in both low uplink power solutions for XR OTA and the CTIA SPOT method, as they are based on the same fundamental principles and considerations.

Observation 2: According to the measurement results, it can be seen that the deviation caused by the SPOT method using single polarization peak EIS is mostly less than 0.3dB, which is very small.

Proposal 1: The peak value is defined as the maximum EIS measured in a single polarization direction for a given channel. Further refine the draft CR R4-2419806 as in Annex.

References

- R4-2419806, Draft Rel-19 CR for 38.870 on XR OTA alternative test method

- R4-2419808, WF for [113][325] TRP_TRS_Ph3, vivo

- CTIA 01.20 Test Methodology, SISO, Anechoic Chamber

Annex revision of Draft Rel-19 CR for 38.870 on XR OTA alternative test method

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

10	Alternative methodologies to resolve battery issue

10.1	General

This Clause define alternative test methodologies to resolve the battery issue of devices during TRP/TRS testing.

10.2	Low UL power solution

The low UL power solution for XR device is a Single Point Offset Test (SPOT) approach for TRS measurements. To reduce the power battery issue of XR devices under maximum transmission condition, SPOT with maximum transmit power can be performed and the end of the measurement to scale the TRS result measured under low UL power. The low transmit power is set as 10 dBm. The final TRS is achieved by offsetting the TRS from the delta at Peak EIS, using both maximum output power (single point EIS) and low transmit power (full EIS).

The detailed test procedure is as following:

1)	Step 1: perform the TRS measurement based on the procedure defined in clause 7.5, with one exception that the UL power of DUT is configured as 10dBm. TRS value is calculated using the equation and integration approaches outlined in TS 38.161 Clause A.3.5.2. This TRS is called TRSlow_power.

2)	Step 2: identify the peak EIS direction and polarization from step 1. The EIS obtained at the peak position and corresponding polarization is called  EISlow_power. Configure the UE to transmit at full power to measure the EISmax_power at the same peak position and polarization identified from Step 2.

3)	Step 3: calculate the delta value at peak EIS direction with delta= EISlow_power -EISmax_power. then get the final TRS at maximum output power based on the equation TRSMax_power = TRSlow_power - delta.

FFS whether above test procedure can also be applied to Wearable Redcap UE.

<<< END OF CHANGES >>>