3GPP TSG-RAN WG4 Meeting #114	R4-2502014

# Athens, Greece, 17 – 21 February 2025

Agenda item:		7.30.2

Source: 		Qualcomm Inc.

Title:	 	IoT NTN UL Capacity Enhancements

Document for:		Approval

## 1	Introduction

IoT NTN UL capacity enhancement work is on-going in RAN1. RAN4 has had initial discussion on whether any specified scheme results in RAN4 specification impact. In this contribution RAN4 specification impact is further discussed.

## 2	Discussion

#### Issue 1-4: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2

- Agreement:

  - Pending on RAN1 progress of slot-level OCC2 resource mapping.

#### Issue 1-5: Whether there would be UE RF requirement impact when OCC shemes would cross gap in UL transmission.

- Agreement:

  - Whether to study the case that gap between OCC slots may affect RF requirement pending on RAN1.

#### Issue 1-6: Whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

- Agreement:

  - Further discuss whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

In RAN4#113 it was agreed that potential specification impacts from 15 kHz single-tone NPUSCH with slot-level OCC2 and cases were OCC schemes cross a gap in UL transmission are to be further discussed:

RAN1 is still working on these topics. Most likely outcome is that RAN1 may change their specifications on transmission gaps and segmentation when these happen together with OCC. Given that RAN4 requirements only reference RAN1 specifications, there would be no direct impact to RAN4 specifications.

For NR NTN it was already concluded in RAN1, that CFO grouping is up to implementation and no new RAN4 frequency error requirements are assumed. Similarly as for NR, for IoT NTN scheduling is fully controlled by the network. Network decides which UEs it pairs together, how many repetitions are configured and therefore has control over transmission gaps and segmentation. This means also that network has means to avoid worst case configurations.

A phase continuity type of requirement was mentioned earlier, but it would not be helpful, as measurement of phase continuity is done only after frequency error has been compensated. A requirement to re-synchronize Tx frequency periodically would not help either, as the baseline frequency error requirement does not change and it would be sufficient to synchronize to just meet the frequency error.

Observation 1: Network controls which UEs are paired together for OCC, and should do the pairing so that potential negative performance impacts are minimized.

Observation 2: New requirements may not help to improve performance.

The work item for Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 was just updated in previous RAN meeting in December 2024 [1]. UE RF specification is not listed as an impacted specification despite RAN spending multiple meetings discussing potential RF impact. This has been visible also in status reports to RAN. Therefore, it can be concluded that RAN does not expect UE RF specification impact from IoT NTN capacity enhancements.

Observation 3: UE RF specification is not in scope of the impacted specifications in the work item.

Proposal 1: There is no RAN4 UE RF specification impact from IoT NTN UL capacity enhancements and further discussion on UL capacity enhancements is not needed.

## 3	Conclusion

In this contribution NTN IoT UL capacity enhancements were discussed and following observation and proposal was made:

Observation 1: Network controls which UEs are paired together for OCC, and should do the pairing so that potential negative performance impacts are minimized.

Observation 2: New requirements may not help to improve performance.

Observation 3: UE RF specification is not in scope of the impacted specifications in the work item.

Proposal 1: There is no RAN4 UE RF specification impact from IoT NTN UL capacity enhancements and further discussion on UL capacity enhancements is not needed.

## 4	References

- RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc.