3GPP TSG-RAN WG4 Meeting #114	R4-2502016

# Athens, Greece, 17 – 21 February 2025

Agenda item:		5.33.1

Source: 		Qualcomm Inc.

Title: 	(TEI18) On NTN NB-IoT in-band operation with NR [NTNNBIoT_inbandNTNNR]

Document for:		Approval

## 1	Introduction

In recent meetings there has been maintenance and TEI discussions on adding a new feature of NTN NB-IoT in-band operation into TS 36.102. This contribution discussed how to proceed with the work.

## 2	Discussion

NTN NB-IoT was originally introduced in release 18 work item [1]. The work item objectives limited the scope to standalone operation:

The work item was preceded by a study on NB-IoT and eMTC support for NTN. The study item is documented in TR 36.763 [2]. For IoT NTN scenarios the study recommended to prioritize standalone deployment for NB-IoT / eMTC for support in Rel-17 timeframe.

RAN1/2/3 did the relevant specification work in release 17, but due to workload concern the corresponding UE RF requirement specification TS 36.102 was introduced only in rel-18. Release independence specification TS 36.307 makes it clear that rel-17 UEs supporting operating bands specified in rel-18 TS 36.102 shall meet the requirements of the rel-18 specifications.

NTN NB-IoT in-band operation was scoped for rel-19 RAN package, but it was not included.

Observation 1: Only standalone operation is covered in current specification.

Observation 2: Both rel-17 and rel-18 UEs comply with rel-18 version of TS 36.102

Given that the rel-18 work item setting the requirements for rel-17 and rel-18 UEs was closed already in Dec 2022 and that release 18 has overall been frozen a long time already, it is not possible to introduce new mandatory features to release 18. Depending on the final scope of the specification changes also in other working groups, it could be acceptable to make support optional in release 18, e.g. by making the changes to rel-19 specification and applying release independence.

Proposal 1: NTN NB-IoT Inband operation can at most be optional in rel-18.

In RAN1 specifications NB-IoT in-band operation was never optimized for in-band operation with NR, instead the specifications consider E-UTRA. When operation mode of NB-IoT is set as in-band TS 36.211 and 36.213 set expectations on LTE CRS and rate matching around it. As CRS does not exist for NR, this functionality is not needed. If specification changes in RAN1 want to be avoided, the correct approach is to indicate operationModeInfo in MIB as guardband operation. Guardband operation is similar to standalone in the sense that no CRS accommodations are required, but instead only raster shifts need to be applied.

Indicating operationModeInfo as guardband can avoid the impact in RAN1, but otherwise it would be confusing to a specification reader why this is done. Therefore it would be at least necessary to clearly capture in RAN4 specifications that that MIB indicates guardband when operation in fact is in-band with NR, and that MDL is adjusted in this case.

Proposal 2: For NTN NB-IoT Inband operation with NR, operationModeInfo shall only be indicated as guardband-r13.

Proposal 3: operationModeInfo as inband-SamePCI-r13 and inband-DifferentPCI-r13 shall not be indicated for in-band operation with NR and UE requirements are not defined.

Mixed operation mode is specified for TN NB-IoT, and in mixed operation mode anchor and non-anchor carriers can have different operationmode, e.g. anchor could be a standalone carrier whereas non-anchor could be an in-band carrier. For TN this is feasible as the DL frequency formulas for all the modes are the same, only possible MDL values differ. For NTN, a change to standalone DL frequency was introduced. UE cannot differentiate whether the non-anchor carrier is intended to be standalone or in-band carrier due to missing signaling.

As changes to other working groups need to be avoided, it is recommended to scope out mixed operation mode. Multicarrier operation can still be supported, with the assumptions that all carriers configured to the UE have the same operationmode. As there is no capability signaling to indicate support of the optional feature for in-band operation with NR

Proposal 4: Mixed operation mode is not supported at least in rel-18 and rel-19 and requirements apply only for the case where all carriers configured to the UE have the same operation mode.

Finally, defining capability signaling could be further considered. In general, new capability signaling is still acceptable in scope of TEI. Having capability signaling in place would not impact initial access as capability information is exchanged only later, but if network is aware which UEs support in-band operation with NR, network can more efficiently hand UEs over from standalone cells to in-band cells. Without capability signaling this would be done blindly.

Proposal 5: RAN4 to re-consider introducing capability signaling for in-band operation with NR

## 3	Conclusion

In this contribution NTN NB-IoT in-band operation with NR was discussed. Following observations and proposals were made.

Observation 1: Only standalone operation is covered in current specification.

Observation 2: Both rel-17 and rel-18 UEs comply with rel-18 version of TS 36.102

Proposal 1: NTN NB-IoT Inband operation can at most be optional in rel-18.

Proposal 2: For NTN NB-IoT Inband operation with NR, operationModeInfo shall only be indicated as guardband-r13.

Proposal 3: operationModeInfo as inband-SamePCI-r13 and inband-DifferentPCI-r13 shall not be indicated for in-band operation with NR and UE requirements are not defined.

Proposal 4: Mixed operation mode is not supported at least in rel-18 and rel-19 and requirements apply only for the case where all carriers configured to the UE have the same operation mode.

Proposal 5: RAN4 to re-consider introducing capability signaling for in-band operation with NR

## 4	References

[1]	RP-223437, Revised WID on NB-IoT/eMTC core & performance requirements for NTN, MediaTek Inc

[2]	TR 36.763, Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)