3GPP TSG-RAN WG4 Meeting #114            	R4-2502029

Athens, GR, 17th-21st, February, 2024

Agenda item:	7.18.3

Source:	Samsung

Title:	View and simulation results for BS demodulation requirement with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the performance part of Rel-19 WI on demodulation performance requirements enhancement was further discussed. The general assumption for MMSE-IRC and interference profile, as well as the test parameters, were discussed. The related agreements are captured in the WF [1] as following

- Interference modelling - Interference profile - On how to account for HPUE in existing LTE profiles: - For interference profile not to consider HPUE scenario for BS MMSE-IRC requirements for this Rel-19 WI - Number of interferers for 2Rx - Option 1: Define BS MMSE-IRC requirements for 2Rx - Option 1A: 1 interferer - Option 1B: 2 interferers - Option 1C: Consider 2 interferers for HomNet and 1 interfere for HetNet - Option 2: Do not need to specify requirements for 2Rx - Companies are encouraged to provide simulation results for 1 or 2 interferers for 2Rx, to check the performance gain. - Test parameters and simulation assumptions - Channel Bandwidth for CP-OFDM - Option 1 - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2 - For 15kHz SCS, cover 5MHz, 10MHz - For 30kHz SCS, cover 10MHz, 40MHz - Option 3: minimum CBW - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz - Option 4: 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS - Channel Bandwidth for DFT-s-OFDM - 15kHz SCS, 5MHz - 30kHz SCS, 10MHz - MCS - Consider the following options for performance evaluation and as candidates for further down-selection: - For 2Rx - Option 1: MCS 2 - Option 2: MCS 16 - For 4Rx - Option 1: MCS16 - For 8RX - Option 1: MCS20 - Option 2: MCS16 - Note: Reuse the same FRC A.3, A.4 and A.5 with Additional DM-RS position = pos1 and 1 transmission layer defined in TS38.104 for MCS 2, MCS 16 and MCS 20 respectively. - Check whether the existing test applicability rule can be reused if different MCS is configured for different Rx configuration. - Propagation condition - For serving cell, consider the following options for performance evaluation and as candidates for further down-selection: - Option 1: TDLA30-10 HetNet and TDLB100-400 for HomNet - Option 2: Consider different channel for different MCS - Option 2A: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10 - For the interference cell, use TDLC300-100. - Waveform-whether to cover DFT-s-OFDM - Option 1: Cover both CP-OFDM and DFT-s-OFDM - Option 2: Cover CP-OFDM only - DMRS configuration - Cover DMRS configuration type 1. FFS whether to include DMRS configuration type 2. - Use DMRS port #0 for both serving and interfering PUSCH - On DMRS sequence generation, use different DMRS sequence for each gNB: - For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively - FFS DMRS sequence generation configuration for DFT-s-OFDM - Test requirement definition - Candidate options: - Option 1: FFS whether SINR based definition is needed - Option 2: Use the SNR based requirement definition - Manufacturer declarations and applicability rules - Applicability rule for different test scenarios - Proposals on applicability for Homnet and Hetnet scenarios: - Proposal 1: HomNet requirements are not applicable for Local Area BS - Proposal 2: HomNet requirements are only applied for Wide area BS, and HetNet requirements are only applied for Home BS and Local BS - Proposals on additional applicability rule - Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped - Proposal 2: Introduce the applicability rules that only one of HomNet and HetNet scenarios is tested - Manufacturer declarations - Proposal 1: Introduce new manufactory declaration for BS MMSE-IRC receiver

In this contribution, the view on the remaining issue of test setup for BS demodulation with MMSE-IRC are provided, meanwhile, the initial simulation results are provided for initial analysis and alignment.

# 2	General

## 2.1	Test scenario

Manufacture declaration and applicability rule

For test scenario, both HomNet and HetNet scenarios were agreed to cover the performance requirement of MMSE-IRC receiver. Regarding the applicability for requirement in different scenarios, it will be further discussed.

In LTE, for requirement in HomNet scenario was not appliable for Local Area BS and Home BS, since the HomNet is mainly targeting for macro cell only as following table

Table 1: Enhanced performance requirement type A for PUSCH, 1.4 MHz Channel Bandwidth

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Propagation conditions and correlation matrix (Annex B) (Note 2) |  |  | DIP set | FRC (Annex A) | Fraction of maximum throughput | SINR [dB] (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 | Interferer 2 |  |  |  |  |
| 1 | 2 | EPA 5 Low | ETU 5 Low | N/A | Set 2 | A12-1 | 70% | -4.8 |
|  |  | EVA 70 Low | ETU 70 Low | N/A | Set 1* | A12-1 | 70% | -2.0 |
|  | 4 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A13-1 | 70% | -4.1 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A13-1 | 70% | -0.1 |
|  | 8 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A4-3 | 70% | -4.7 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A4-3 | 70% | 0.0 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |

For NR, only three categories of BS class were defined as wide area base station, medium Range base station and local area range base station in current specification. Therefore, we think the similar approach can be applied for NR to differentiate the different BS classes.

Observation 1:  The requirement in HomNet (DIP set 1) is not applicable for local Area BS and Home BS in LTE system.

Observation 2:  Only three categories of BS class were defined in NR system as wide area base station, medium Range base station and local area range base station.

The requirement of HomNet scenario is only applied for wide area BS. Regarding the Home BS scenario, since it is not introduced in current specification, we do not need to mention it in current release. If home BS was introduced into future release, the applicable of requirement can be updated.

Proposal 1: Introduce the applicability rule for these two requirements testing. The following test applicability rule can be considered as

  - HomNet requirements are only applied for wide area BS, and HetNet requirements are only applied for local area BS.

Meanwhile, from the baseband processing perspective of MMSE-IRC, there is no difference foreseen. In our view there is no necessary to test both if BS declared to support MMSE-IRC receiver for these two scenarios, pending on the deployment, for example, in Marco cell or Micro cell or Pico cell.

To reduce the test effort, we propose to introduce the test applicability rule for these two requirements (DIP) testing. The following test applicability rule can be considered as

  - Unless other stated, if a BS declared to support [HomNet (DIP1)] or [HetNet (DIP2)] and pass the tests for [HomNet (DIP1)], can also consider the test for [HetNet (DIP2)] as passed.

Proposal 2: Introduce the additional applicability rule for these two requirements testing as following

  - Unless other stated, if a BS declared to support [HomNet (DIP1)] or [HetNet (DIP2)] and pass the tests for [HomNet (DIP1)], can also consider the test for [HetNet (DIP2)] as passed.

Regarding manufacturer declaration for BS with MMSR-IRC receiver, since it was specified in Rel-19 for advanced receiver to handle the interference, it was straight forward to introduce the new manufactory declaration for BS MMSE-IRC receiver. Therefore, the following manufacturer declaration for BS MMSE-IRC receiver can be considered

Table 2: manufactory declaration for BS with MMSE-IRC receiver

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

Proposal 3: Introduce new manufactory declaration for BS with MMSE-IRC receiver as following

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

## 2.2	Interference modeling

Number of Interferers

Regarding number of interferer UE modeling, it was agreed to check the performance with 1 interferer and 2 interferers for 2Rx. Based on our initial simulation, the gain can be achieved by MMSE-IRC receiver compared with MMSE receiver for both HomNet and HetNet scenario, similar gain achieved as 1 interferer. So, it was feasible to consider 2 interferers for specifying PUSCH requirement with MMSE-IRC receiver.

Regarding the proposed option 3c, different number of interferers are considered for HomNet and HetNet, since the test applicability rule for different scenario was under discussion. For option 3c, it will require new test applicability rule introduced to differentiate different number of antennas on top of different scenario. To simplify the test applicability, we do not think it was necessary to differentiate it, considering the gain can be achieved.

Observation 1: The gain can be achieved with MMSE-IRC receiver compared with MMSE receiver for the 2 interferers modeling, under the interference profile for both HomNet and HetNet scenarios.

Observation 2: New test applicability rule was required to be introduced to differentiate different number of antennas on top of different scenario for option 3c.

Proposal 4:  2 interferers can be considered for 2Rx to specify the PUSCH requirement with MMSE-IRC receiver.

# 3	Test parameters and simulation assumption

In this section, the remaining simulation assumptions are provided for evaluation.

Channel Bandwidth

Regarding the SCS and channel bandwidth, minimum CBW for each SCS has been considered for performance evaluation purpose. According to existing test applicability rule, the BS with declared to support large CBW can still be tested. To reduce the test effort, we prefer to select one CBW for requirement definition. Considering typical CBW with 10MHz for 15KHz SCS, and 40MHz for 30kHz SCS has already widely applied in current field, we are also open to discuss whether additional case will be added. Regarding the CBW with 50MHz for 15KHz SCS, considering there is no requirement specified in Rel-15 based on MMSE receiver without considering inter-cell inference. We prefer to not define the requirement for it.

Therefore, for CP-OFDM and DFT-s-OFDM waveform, the following channel bandwidth can be considered for requirement definition.

- For CP-OFDM

  - 15kHz SCS, 5MHz,

    - Additional CBW with 10MHz

  - 30KHz SCS, 10MHz

    - Additional CBW with 40MHz

- For DFT-s-OFDM

  - 15kHz SCS, 5MHz

  - 30KHz SCS, 10MHz

Proposal 5:  The following channel bandwidth can be considered to specify PUSCH requirement with MMSE-IRC receiver

- For CP-OFDM

  - 15kHz SCS, 5MHz, 10MHz

    - Additional CBW with 10MHz

  - 30KHz SCS, 10MHz, 40MHz

    - Additional CBW with 40MHz

- For DFT-s-OFDM

  - 15kHz SCS, 5MHz

  - 30KHz SCS, 10MHz

Propagation condition

To check the per-PRB interference covariance estimation, it is better to choose the channel with high delay spread for the interferers. In Rel-15, several channel models are specified to cover different delay spread and doppler spread, including TDLA30-10, TDLB400-100 and TDLC300-100, and different MCS will be applied for different channel. In LTE BS performance with MMSE-IRC receiver, different channel models were applied for target UE and interfered UE. Therefore, to align the similar principle, we slightly prefer to consider different channel for different MCS when specifying the PUSCH requirement with MMSE-IRC receiver.

Proposal 6: Reusing the existing channel models to specify the PUSCH requirement with MMSE-IRC receiver

The following candidate propagation channel with combining MCS can be considered as initial evaluation

  - Targeting cell and MCS:

    - TDLA30-10 with MCS 20

    - TDLC300-100 with MCS 16

    - TDLB100-400 with MCS 2

  - Interference cell: TDLC300-100

Test applicability for different MCS and different Rx configuration.

In the last meeting, one issue was raised whether the existing test applicability rule can be used if different MCS is configured for different Rx configuration. In Rel-15 test, the same MCS was specified for the same Rx number. Regarding the test applicability rule for different Rx antenna, the following was introduced in Rel-15

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the lowest and highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

Regarding MCS, in current test, up to MCS 20 with 64QAM was considered as candidate options, which was mandatory supported for UL transmission. So, we don’t think it was necessary to considerer new test applicability for different MCS and different Rx configuration, since there is no declaration or test applicabltity rule for different MCS excepting for 256QAM.

Proposal 7: Existing test applicability rule with different Rx number can still be reused, no need to introduce new test applicability for different MCS and different Rx antenna configuration.

MCS

For target cell, MCS 2, MCS 16 and MCS 20 were considered as candidate for performance evaluation and further down-selection based on simulation results. Based on our initial results, it was feasible to support MCS 16 with 2Rx, and MCS 20 with 8Rx. Therefore, the following MCS can be considered as requirement definition for PUSCH with MMSE-IRC receiver

Proposal 8: The following MCS can be considered as requirement definition

  - 2Rx

    - MCS 16 with TDLC300-100

  - 4Rx

    - MCS 16 with TDLC300-100

  - 8Rx

    - MCS 20 with TDLA30-10

Waveform

Considering only 1Tx UE scenario, both CP-OFDM and DFT-s-OFDM are feasible. From MMSE-IRC receiver perspective, there is no different with CP-OFDM and DFT-s-OFDM waveform. For test coverage perspective, both CP-OFDM and DFT-s-OFDM can be considered. In general, we are also open to discuss whether both waveforms should be considered for requirement definition. Considering the test effort, we are fine to only cover CP-OFDM wave as compromise.

Proposal 9: For requirement definition, only cover CP-OFDM waveform as compromise

DMRS configuration

Regarding the DMRS configuration, both type 1 and type 2 were considered as one of option. Type 2 is mainly targeting for multi user scenarios to support more ports multiplexing. Since only UE with 1 port was considered, the performance with type1 and type 2 is similar. Meanwhile, from MMSE-IRC receiver perspective, there is no different with DMRS type 1 and type 2. In legacy NR PUSCH requirement, RAN4 only specified the requirement with type 1. We do not think it is necessary to specify the requirement with DMRS type 2.

Proposal 10: Only specify the PUSCH requirement with MMSR-IRC for DMRS type 1.

DMRS sequence generation

For DMRS generation configuration with DFT-s-OFDM, similar as Rel-15, NID0=0, group hopping and sequence hopping are disabled. For interference cell, NID0= 1 and NID0=2 can be considered respectively

Proposal 11: For DFT-s-OFDM waveform, the DMRS sequence generation configuration can be considered as

  - nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively

  - Group hopping and sequence hopping are disabled.

Test requirement definition

In the last meeting, 70% of maximum throughput as metric to derive the requirement for PUSCH with MMSE-IRC receiver. Regarding the requirement definition, both SNR based and SINR based method can be considered. In LTE MMSE-IRC, the SINR is applied to derive the related requirement.

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Propagation conditions and correlation matrix (Annex B) (Note 2) |  |  | DIP set | FRC (Annex A) | Fraction of maximum throughput | SINR [dB] (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 | Interferer 2 |  |  |  |  |
| 1 | 2 | EPA 5 Low | ETU 5 Low | N/A | Set 2 | A12-1 | 70% | -4.8 |
|  |  | EVA 70 Low | ETU 70 Low | N/A | Set 1* | A12-1 | 70% | -2.0 |
|  | 4 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A13-1 | 70% | -4.1 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A13-1 | 70% | -0.1 |
|  | 8 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A4-3 | 70% | -4.7 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A4-3 | 70% | 0.0 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal, interferer 1 and interferer 2. Note 2: The propagation conditions for the tested signal, interferer 1 and interferer 2 are statistically independent. Note 3: SINR corresponds to of the tested signal as defined in clause 8.1. |  |  |  |  |  |  |  |  |

For NR PDSCH requirement with MMSE-IRC, SNR was applied to derive the related requirement

Table 5.2.2.1.15-3: Minimum performance for PDSCH with rank 1 and with inter-cell interference

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 70 | 15.4 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 12.5 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |

Observation 3: The SINR was applied for test requirement definition for PUSCH requirement with MMSE-IRC receiver in LTE

Observation 4: The SNR was applied for test requirement definition for PDSCH requirement with MMSE-IRC receiver in NR

Based on our initial simulation results, the performance gain over MMSE receiver is similar for both SNR and SINR-based requirement definition.  In general, either approach is fine, since the interference power is fixed. To reflect the performance with certain interference condition, SINR based requirement definition is more proper. Therefore, similar as LTE MMSE-IRC, we can still apply the SINR based requirement definition to derive the targeting value based on 70% of TP. While we are also open to further discuss whether SNR based metric is more proper from conformance test perspective, since TE vendor can transmit the PUSCH with target power. Without additional calculation to derive the signal power based on SINR and INR.

Proposal 12: Either SINR based or SNR based metric are fine to consider the test requirement definition.

# 4	Initial Simulation Results

In this section, the initial simulation results are provided to check the performance gain of MMSE-IRC over MMSE receiver. Based on the WF agreed in the last meeting, the candidate test cases are listed in the []. As 1st priority,  the simulation results for the following test cases are provided

- Number of Rx: 2/4/8Rx

- Number of Interferes: 1 or 2 for 2Rx, and 2 for 4Rx/8Rx

- PUSCH mapping type: type A

- Interferes scenario: HomNet and HetNet

- Channel of interferes: TDLC300-100

- Waveform:  CP-OFDM

- SCS and BW:

  - 15KHz:  5/20/50MHz

  - 30KHz: 10/100MHz

Table 3: PUSCH performance with HomNet scenario under 2Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -3.89 | -7.08 | 3.19 | 2.58 | -0.61 | 3.19 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -3.89 | -7.27 | 3.38 | 4.51 | 1.13 | 3.38 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS 16 | 8.18 | 5.42 | 2.76 | 14.65 | 11.89 | 2.76 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 9.73 | 7.26 | 2.47 | 16.2 | 13.72 | 2.48 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.21 | 5.32 | 2.89 | 16.61 | 13.72 | 2.89 |
| 6 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 9.77 | 7.18 | 2.59 | 18.17 | 15.59 | 2.58 |

Table 4.  PUSCH performance with HetNet scenario under 2Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -3.97 | -9.58 | 5.61 | 6.29 | 0.67 | 5.62 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.3 | -9.73 | 5.43 | 5.96 | 0.53 | 5.43 |
| 3 | 1 | 2 | TDLB100-400 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS2 | MCS16 | -3.97 | -9.76 | 5.79 | 8.84 | 3.05 | 5.79 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.29 | -9.96 | 5.67 | 8.52 | 2.84 | 5.68 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.14 | 3.25 | 4.89 | 18.4 | 13.51 | 4.89 |
| 6 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 7.3 | 2.12 | 5.18 | 17.56 | 12.38 | 5.18 |
| 7 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.17 | 3.21 | 4.96 | 20.98 | 16.02 | 4.96 |
| 8 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 7.28 | 2.01 | 5.27 | 20.09 | 14.81 | 5.28 |

Table 5.  PUSCH performance with HomNet scenario under 2Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -3.57 | -6.69 | 3.12 | 2.9 | -0.22 | 3.12 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -3.58 | -6.88 | 3.3 | 4.82 | 1.52 | 3.3 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.58 | 5.71 | 2.87 | 15.05 | 12.18 | 2.87 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 10.39 | 7.79 | 2.6 | 16.86 | 14.26 | 2.6 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.61 | 5.65 | 2.96 | 17.01 | 14.05 | 2.96 |
| 6 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 10.45 | 7.77 | 2.68 | 18.86 | 16.17 | 2.69 |

Table 6.  PUSCH performance with HetNet scenario under 2Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -3.76 | -9.17 | 5.41 | 6.5 | 1.09 | 5.41 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.93 | -10.29 | 5.36 | 5.32 | -0.03 | 5.35 |
| 3 | 1 | 2 | TDLB100-400 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS2 | MCS16 | -3.75 | -9.39 | 5.64 | 9.06 | 3.42 | 5.64 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.96 | -10.55 | 5.59 | 7.85 | 2.26 | 5.59 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.58 | 3.6 | 4.98 | 18.83 | 13.86 | 4.97 |
| 6 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 7.25 | 2.3 | 4.95 | 17.5 | 12.55 | 4.95 |
| 7 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.58 | 3.61 | 4.97 | 21.39 | 16.42 | 4.97 |
| 8 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 7.28 | 2.17 | 5.11 | 20.09 | 14.98 | 5.11 |

Table 7.  PUSCH performance with HomNet scenario under 2Rx, 15KHz SCS and 50MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -3.67 | -6.74 | 3.07 | 2.8 | -0.27 | 3.07 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -3.66 | -6.94 | 3.28 | 4.74 | 1.46 | 3.28 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS16 | MCS16 | 8.84 | 5.95 | 2.89 | 15.31 | 12.42 | 2.89 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 11.3 | 8.68 | 2.62 | 17.77 | 15.15 | 2.62 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.87 | 5.91 | 2.96 | 17.28 | 14.31 | 2.97 |
| 6 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 11.4 | 8.73 | 2.67 | 19.8 | 17.13 | 2.67 |

Table 8.  PUSCH performance with HetNet scenario under 2Rx, 15KHz SCS and 50MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -3.83 | -9.16 | 5.33 | 6.42 | 1.1 | 5.32 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -5.41 | -10.8 | 5.39 | 4.85 | -0.54 | 5.39 |
| 3 | 1 | 2 | TDLB100-400 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS2 | MCS16 | -3.8 | -9.39 | 5.59 | 9 | 3.42 | 5.58 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -5.4 | -11.03 | 5.63 | 7.4 | 1.78 | 5.62 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.87 | 3.89 | 4.98 | 19.12 | 14.15 | 4.97 |
| 6 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 7.28 | 2.3 | 4.98 | 17.54 | 12.67 | 4.87 |
| 7 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.87 | 3.89 | 4.98 | 21.68 | 16.7 | 4.98 |
| 8 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 7.27 | 2.27 | 5 | 20.07 | 15.07 | 5 |

Table 9.  PUSCH performance with HomNet scenario under 2Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.18 | -7.24 | 3.06 | 2.29 | -0.78 | 3.07 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.16 | -7.48 | 3.32 | 4.24 | 0.92 | 3.32 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS16 | MCS16 | 8.28 | 5.58 | 2.7 | 14.75 | 12.05 | 2.7 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.62 | 5.9 | 2.72 | 15.09 | 12.37 | 2.72 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.29 | 5.49 | 2.8 | 16.69 | 13.9 | 2.79 |
| 6 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.61 | 5.75 | 2.86 | 17.01 | 14.16 | 2.85 |

Table 10.  PUSCH performance with HetNet scenario under 2Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.32 | -9.75 | 5.43 | 5.94 | 0.5 | 5.44 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -5.49 | -10.72 | 5.23 | 4.77 | -0.46 | 5.23 |
| 3 | 1 | 2 | TDLB100-400 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS2 | MCS16 | -4.34 | -10.03 | 5.69 | 8.46 | 2.78 | 5.68 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -5.48 | -10.94 | 5.46 | 7.32 | 1.87 | 5.45 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.26 | 3.5 | 4.76 | 18.52 | 13.75 | 4.77 |
| 6 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 6.32 | 1.24 | 5.08 | 16.58 | 11.49 | 5.09 |
| 7 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.29 | 3.41 | 4.88 | 21.09 | 16.22 | 4.87 |
| 8 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 6.35 | 1.12 | 5.23 | 19.16 | 13.93 | 5.23 |

Table 11.  PUSCH performance with HomNet scenario under 2Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.28 | -7.4 | 3.12 | 2.19 | -0.93 | 3.12 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.26 | -7.63 | 3.37 | 4.15 | 0.78 | 3.37 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS16 | MCS16 | 8.58 | 5.79 | 2.79 | 15.05 | 12.26 | 2.79 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.7 | 5.73 | 2.97 | 15.17 | 12.2 | 2.97 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.61 | 5.73 | 2.88 | 17.01 | 14.14 | 2.87 |
| 6 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.72 | 5.66 | 3.06 | 17.12 | 14.06 | 3.06 |

Table 12.  PUSCH performance with HetNet scenario under 2Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.4 | -9.86 | 5.46 | 5.86 | 0.4 | 5.46 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -5.85 | -11.09 | 5.24 | 4.4 | -0.83 | 5.23 |
| 3 | 1 | 2 | TDLB100-400 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS2 | MCS16 | -4.4 | -10.06 | 5.66 | 8.41 | 2.75 | 5.66 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -5.84 | -11.37 | 5.53 | 6.96 | 1.44 | 5.52 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.57 | 3.71 | 4.86 | 18.82 | 13.96 | 4.86 |
| 6 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 6.54 | 1.56 | 4.98 | 16.8 | 11.82 | 4.98 |
| 7 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.58 | 3.67 | 4.91 | 21.39 | 16.48 | 4.91 |
| 8 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 6.52 | 1.43 | 5.09 | 19.33 | 14.24 | 5.09 |

Table 11.  PUSCH performance with HomNet scenario under 4Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 4.92 | -0.09 | 5.01 | 13.33 | 8.31 | 5.02 |
| 2 | 1 | 4 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.78 | 0.92 | 4.86 | 14.18 | 9.32 | 4.86 |

Table 12.  PUSCH performance with HetNet scenario under 4Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5 | -3.35 | 8.35 | 17.81 | 9.46 | 8.35 |
| 2 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.64 | -4.89 | 8.53 | 16.45 | 7.92 | 8.53 |

Table 13.  PUSCH performance with HomNet scenario under 4Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.28 | 0.28 | 5 | 13.68 | 8.68 | 5 |
| 2 | 1 | 4 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 6.36 | 1.47 | 4.89 | 14.76 | 9.88 | 4.88 |

Table 14.  PUSCH performance with HetNet scenario under 4Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.46 | -2.94 | 8.4 | 18.27 | 9.87 | 8.4 |
| 2 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.68 | -4.54 | 8.22 | 16.49 | 8.27 | 8.22 |

Table 15.  PUSCH performance with HomNet scenario under 4Rx, 15KHz SCS and 50MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.6 | 0.66 | 4.94 | 14 | 9.06 | 4.94 |
| 2 | 1 | 4 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 6.76 | 2.01 | 4.75 | 15.17 | 10.41 | 4.76 |

Table 16.  PUSCH performance with HetNet scenario under 4Rx, 15KHz SCS and 50MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.75 | -2.48 | 8.23 | 18.55 | 10.32 | 8.23 |
| 2 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.99 | -4.12 | 8.11 | 16.8 | 8.68 | 8.12 |

Table 17.  PUSCH performance with HomNet scenario under 4Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 4.97 | 0.13 | 4.84 | 13.37 | 8.54 | 4.83 |
| 2 | 1 | 4 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.14 | 0.16 | 4.98 | 13.54 | 8.57 | 4.97 |

Table 18.  PUSCH performance with HetNet scenario under 4Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.29 | -2.87 | 11.16 | 21.08 | 9.94 | 11.14 |
| 2 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.3 | -4.99 | 8.29 | 16.11 | 7.82 | 8.29 |

Table 19.  PUSCH performance with HomNet scenario under 4Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.4 | 0.54 | 4.86 | 13.8 | 8.95 | 4.85 |
| 2 | 1 | 4 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.47 | 0.34 | 5.13 | 13.87 | 8.74 | 5.13 |

Table 20.  PUSCH performance with HetNet scenario under 4Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.58 | -2.45 | 11.03 | 21.39 | 10.36 | 11.03 |
| 2 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.48 | -4.66 | 8.14 | 16.29 | 8.15 | 8.14 |

Table 21.  PUSCH performance with HomNet scenario under 8Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 2.29 | -4.07 | 6.36 | 10.69 | 4.34 | 6.35 |
| 2 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 2.98 | -3.24 | 6.22 | 11.38 | 5.17 | 6.21 |
| 3 | 1 | 8 | TDLA30-10 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 3.77 | -2.15 | 5.92 | 12.17 | 6.25 | 5.92 |
| 4 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 6.53 | 0.9 | 5.63 | 14.93 | 9.3 | 5.63 |

Table 22.  PUSCH performance with HetNet scenario under 8Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 2.56 | -7.73 | 10.29 | 15.37 | 5.08 | 10.29 |
| 2 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 0.5 | -9.22 | 9.72 | 13.31 | 3.59 | 9.72 |
| 3 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 3.79 | -5.97 | 9.76 | 16.6 | 6.83 | 9.77 |

Table 23.  PUSCH performance with HomNet scenario under 8Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 2.99 | -3.85 | 6.84 | 11.4 | 4.55 | 6.85 |
| 2 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.7 | -2.92 | 6.62 | 12.11 | 5.48 | 6.63 |
| 3 | 1 | 8 | TDLA30-10 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 3.88 | -1.98 | 5.86 | 12.29 | 6.43 | 5.86 |
| 4 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 7.4 | 1.79 | 5.61 | 15.81 | 10.2 | 5.61 |

Table 24.  PUSCH performance with HetNet scenario under 8Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.35 | -7.37 | 10.72 | 16.15 | 5.44 | 10.71 |
| 2 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 0.8 | -8.79 | 9.59 | 13.6 | 4.02 | 9.58 |
| 3 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 3.9 | -5.72 | 9.62 | 16.71 | 7.09 | 9.62 |

Table 25.  PUSCH performance with HomNet scenario under 8Rx, 15KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.38 | -3.52 | 6.9 | 11.79 | 4.88 | 6.91 |
| 2 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 4.24 | -2.19 | 6.43 | 12.65 | 6.21 | 6.44 |
| 3 | 1 | 8 | TDLA30-10 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 4.46 | -1.39 | 5.85 | 12.87 | 7.01 | 5.86 |
| 4 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 9.16 | 3.63 | 5.53 | 17.56 | 12.03 | 5.53 |

Table 26.  PUSCH performance with HetNet scenario under 8Rx, 15KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.73 | -7.03 | 10.76 | 16.54 | 5.78 | 10.76 |
| 2 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 0.93 | -8.46 | 9.39 | 13.74 | 4.35 | 9.39 |
| 3 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 4.52 | -5.18 | 9.7 | 17.33 | 7.62 | 9.71 |

Table 27.  PUSCH performance with HomNet scenario under 8Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 2.42 | -3.78 | 6.2 | 10.82 | 4.63 | 6.19 |
| 2 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 2.46 | -3.84 | 6.3 | 10.86 | 4.57 | 6.29 |
| 3 | 1 | 8 | TDLA30-10 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 3.67 | -2.15 | 5.82 | 12.07 | 6.25 | 5.82 |
| 4 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 5.36 | -0.55 | 5.91 | 13.76 | 7.85 | 5.91 |

Table 28.  PUSCH performance with HetNet scenario under 8Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 2.69 | -7.36 | 10.05 | 15.49 | 5.45 | 10.04 |
| 2 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 0.4 | -9.05 | 9.45 | 13.21 | 3.76 | 9.45 |
| 3 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 3.66 | -5.92 | 9.58 | 16.46 | 6.89 | 9.57 |

Table 29.  PUSCH performance with HomNet scenario under 8Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.2 | -3.53 | 6.73 | 11.6 | 4.87 | 6.73 |
| 2 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.37 | -3.9 | 7.27 | 11.77 | 4.5 | 7.27 |
| 3 | 1 | 8 | TDLA30-10 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 4.21 | -1.63 | 5.84 | 12.62 | 6.77 | 5.85 |
| 4 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 6.15 | 0.08 | 6.07 | 14.55 | 8.48 | 6.07 |

Table 30.  PUSCH performance with HetNet scenario under 8Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.57 | -6.9 | 10.47 | 16.37 | 5.9 | 10.47 |
| 2 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 0.58 | -8.72 | 9.3 | 13.39 | 4.09 | 9.3 |
| 3 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 4.27 | -5.3 | 9.57 | 17.08 | 7.51 | 9.57 |

Based on above initial simulation results, the following observations can be achieved

Observation 5: As number of interferers increasing from 1 to 2 for 2Rx condition, the achieved SNR@70% Throughput is increasing, while the achieved SINR @ 70%Throughput is decreasing

Observation 6: With low MCS 2, the performance gain compared with MMSE-IRC is large than MCS with 16 for 2Rx

Observation 7: With MCS 16, the performance gain compared with MMSE-IRC is large than MCS with 20 for 8Rx

Observation 8: Similar performance gain can be achieved with MMSE-IRC for different CBW

# 5	Conclusion

In this contribution, the view on the BS demodulation requirement with MMSE-IRC receiver. And the initial simulation results are provided to check the performance gain over MMSE based on LTE interference profile.

Observation 1:  The requirement in HomNet (DIP set 1) is not applicable for local Area BS and Home BS in LTE system.

Observation 2:  Only three categories of BS class were defined in NR system as wide area base station, medium Range base station and local area range base station.

Proposal 1: Introduce the applicability rule for these two requirements testing. The following test applicability rule can be considered as

  - HomNet requirements are only applied for wide area BS, and HetNet requirements are only applied for local area BS.

Proposal 2: Introduce the additional applicability rule for these two requirements testing as following

  - Unless other stated, if a BS declared to support [HomNet (DIP1)] or [HetNet (DIP2)] and pass the tests for [HomNet (DIP1)], can also consider the test for [HetNet (DIP2)] as passed.

Proposal 3: Introduce new manufactory declaration for BS with MMSE-IRC receiver as following

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

Observation 1: The gain can be achieved with MMSE-IRC receiver compared with MMSE receiver for the 2 interferers modeling, under the interference profile for both HomNet and HetNet scenarios.

Observation 2: New test applicability rule was required to be introduced to differentiate different number of antennas on top of different scenario for option 3c.

Proposal 4:  2 interferers can be considered for 2Rx to specify the PUSCH requirement with MMSE-IRC receiver.

Proposal 5:  The following channel bandwidth can be considered to specify PUSCH requirement with MMSE-IRC receiver

- For CP-OFDM

  - 15kHz SCS, 5MHz, 10MHz

    - Additional CBW with 10MHz

  - 30KHz SCS, 10MHz, 40MHz

    - Additional CBW with 40MHz

- For DFT-s-OFDM

  - 15kHz SCS, 5MHz

  - 30KHz SCS, 10MHz

Proposal 6: Reusing the existing channel models to specify the PUSCH requirement with MMSE-IRC receiver

The following candidate propagation channel with combining MCS can be considered as initial evaluation

  - Targeting cell and MCS:

    - TDLA30-10 with MCS 20

    - TDLC300-100 with MCS 16

    - TDLB100-400 with MCS 2

  - Interference cell: TDLC300-100

Proposal 7: Existing test applicability rule with different Rx number can still be reused, no need to introduce new test applicability for different MCS and different Rx antenna configuration.

Proposal 8: The following MCS can be considered as requirement definition

  - 2Rx

    - MCS 16 with TDLC300-100

  - 4Rx

    - MCS 16 with TDLC300-100

  - 8Rx

    - MCS 20 with TDLA30-10

Proposal 9: For requirement definition, only cover CP-OFDM waveform as compromise

Proposal 10: Only specify the PUSCH requirement with MMSR-IRC for DMRS type 1.

Proposal 11: For DFT-s-OFDM waveform, the DMRS sequence generation configuration can be considered as

  - nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively

  - Group hopping and sequence hopping are disabled.

Observation 3: The SINR was applied for test requirement definition for PUSCH requirement with MMSE-IRC receiver in LTE

Observation 4: The SNR was applied for test requirement definition for PDSCH requirement with MMSE-IRC receiver in NR

Proposal 12: Either SINR based or SNR based metric are fine to consider the test requirement definition.

Observation 5: As number of interferers increasing from 1 to 2 for 2Rx condition, the achieved SNR@70% Throughput is increasing, while the achieved SINR @ 70%Throughput is decreasing

Observation 6: With low MCS 2, the performance gain compared with MMSE-IRC is large than MCS with 16 for 2Rx

Observation 7: With MCS 16, the performance gain compared with MMSE-IRC is large than MCS with 20 for 8Rx

Observation 8: Similar performance gain can be achieved with MMSE-IRC for different CBW

# References

R4-2419780, WF on [113][321] NR_demod_Ph5_Part1_General_BS

R4-2502028, simulation results summary for BS requirement with MMSE-IRC receiver