3GPP TSG-RAN4 Meeting #114	R4-2502049

# Athens, GR, 17th –21th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR | 0059 | rev | <Rev#> | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction CR on SAN performance requirements in TS 38.181 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The performance part of Rel-18 NTN enhanced WI was completed in RAN4#111 meeting, the big CR has been implemented into the spec - The format of test requirement clause is not aligned with other channel, where two sub-sections are included to differentiate SAN type 1-O and SAN type 2-O for PUSCH - The manufacturer declaration in the table of AWGN power level at the SAN input for D.xx is not updated for each section of PUSCH, PUCCH and PRACH - [] for clause number reference is not removed in section 11.3 - The test parameters terminology of PUCCH format1 and multi-slot PUCCH are not aligned for First PRB after frequency hopping in section 11.3 - The Table 11.3.5.4.2-2 is duplicated in section 11.3.5 - The channel model for SAN PRACH requirement is not correct without inlcuding the doppler value in section 11.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new sections under test requirement section for SAN type 1-O and 2-O, and update the related table index for SAN PUSCH requirement - Update the note in the table of AWGN power level at the SAN input for each section - Remove [] for clause number referenece - Update the test parameters terminology of PUCCH format1 and multi-slot PUCCH for First PRB after frequency hopping in section 11.3 - Remove the duplicated table 11.3.5.4.2-2 in section 11.3.5 - Update the channel model for SAN PRACH requirement in section 11.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SAN performance requirement can not be verified well |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2, 11.3, 11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

11.2	OTA performance requirements for PUSCH

11.2.1	Performance requirements for PUSCH with transform precoding disabled

11.2.1.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.3.

11.2.1.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2] clause 11.2.1.1.

For SAN type 2-O, the minimum requirement is in TS 38.108 [2], clause 11.2.2.1.

11.2.1.3	Test Purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR.

11.2.1.4	Method of test

11.2.1.4.1	Initial Conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.2.1.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 11.2.1.4.2-1.

Table 11.2.1.4.2-1: Test parameters for testing PUSCH

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Transform precoding |  | SAN type 1-O | SAN type 2-O |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port | {0} |  |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |  |
| Time domain resource assignment | PUSCH mapping type | A, B | B |
|  | Start symbol | 0 |  |
|  | Allocation length | 14 | 10 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| Frequency domain resource assignment | RB assignment | N.A. | Disabled |
|  | Frequency hopping | N.A. | Disabled |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.2.1.5 and clause 11.2.1.6 for SAN type 1-O and SAN type 2-O respectively, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.2.1.4.2-2.

Table 11.2.1.4.2-2: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 1-O (Note 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1 NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	For reference channels applicable to the SAN, measure the throughput.

11.2.1.5	Test Requirement

11.2.1.5.1	Test requirement for SAN type 1-O

The throughput measured according to clause 11.2.1.4.2 shall not be below the limits for the SNR levels specified in table 11.2.1.5.1-1 to 11.2.1.5.1-4.

Table 11.2.1.5.1-1: Test requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-1 | pos1 | 3.8 |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3-1 | pos1 | 2.2 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-1 | pos1 | -0.1 |
|  |  | Normal | NTN-TDLC5-200 Low | 70% | G-FR1-NTN-A3-1 | pos1 | -0.6 |

Table 11.2.1.5.1-2: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-2 | pos1 | 3.5 |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3-2 | pos1 | 2.0 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-2 | pos1 | -0.4 |
|  |  | Normal | NTN-TDLC5-200 Low | 70% | G-FR1-NTN-A3-2 | pos1 | -0.8 |

Table 11.2.1.5.1-3: Test requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-1 | pos1 | 3.9 |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3-1 | pos1 | 2.2 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-1 | pos1 | 0.0 |
|  |  | Normal | NTN-TDLC5-200 Low | 70% | G-FR1-NTN-A3-1 | pos1 | -0.6 |

Table 11.2.1.5.1-4: Test requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-2 | pos1 | 3.5 |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3-2 | pos1 | 1.9 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-2 | pos1 | -0.4 |
|  |  | Normal | NTN-TDLC5-200 Low | 70% | G-FR1-NTN-A3-2 | pos1 | -0.8 |

11.2.1.5.2	Test requirement for SAN type 2-O

The throughput measured according to clause 11.2.1.4.2 shall not be below the limits for the SNR levels specified in table 11.2.1.5.2-1.

Table 11.2.1.5.2-1: Test requirements for PUSCH with 70% of maximum throughput, 50 MHz Channel Bandwidth, 120 kHz SCS in FR2-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-1200 Low | 70 % | G-FR2-NTN-A5-1 | pos1 | 0.6 |
|  |  | Normal | NTN-TDLC5-1200 Low | 70 % | G-FR2-NTN-A6-1 | pos1 | 9.5 |
|  | 2 | Normal | NTN-TDLC5-1200 Low | 70 % | G-FR2-NTN-A5-1 | pos1 | -2.8 |
|  |  | Normal | NTN-TDLC5-1200 Low | 70% | G-FR2-NTN-A6-1 | pos1 | 6.1 |

11.2.2	Performance requirements for PUSCH with transform precoding enabled

11.2.2.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.3.

11.2.2.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2] clause 11.2.1.2.

For SAN type 2-O, the minimum requirement is in TS 38.108 [2] clause 11.2.2.2.

11.2.2.3	Test Purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR.

11.2.2.4	Method of test

11.2.2.4.1	Initial Conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.2.2.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 11.2.2.4.2-1.

Table 11.2.2.4.2-1: Test parameters for testing PUSCH

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | SAN type 1-O | SAN type 2-O |
| Transform precoding |  | Enabled |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port | {0} |  |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |  |
| Time domain resource assignment | PUSCH mapping type | A, B | B |
|  | Start symbol | 0 |  |
|  | Allocation length | 14 | 10 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth | 30 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PTRS configuration | Frequency density (KPT-RS) | N.A. | Disabled |
|  | Time density (LPT-RS) | N.A. | Disabled |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.2.2.5 and clause 11.2.2.6 for SAN type 1-O and SAN type 1-O respectively, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.2.2.4.2-2.

Table 11.2.2.4.2-2: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 1-O (Note 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	For reference channels applicable to the SAN, measure the throughput.

11.2.2.5	Test Requirement

11.2.2.5.1	Test requirement for SAN type 1-O

The throughput measured according to clause 11.2.2.4.2 shall not be below the limits for the SNR levels specified in table 11.2.2.5.1-1 to 11.2.2.5.1-4.

Table 11.2.2.5.1-1: Test requirements for PUSCH with 70% of maximum throughput, PUSCH mapping Type A, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-3 | pos1 | 4.3 |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3-3 | pos1 | 2.2 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-3 | pos1 | 0.1 |
|  |  | Normal | NTN-TDLC5-200 Low | 70% | G-FR1-NTN-A3-3 | pos1 | -0.6 |

Table 11.2.2.5.1-2: Test requirements for PUSCH with 70% of maximum throughput, PUSCH mapping Type A, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-4 | pos1 | 4.1 |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3-4 | pos1 | 1.9 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-4 | pos1 | -0.1 |
|  |  | Normal | NTN-TDLC5-200 Low | 70% | G-FR1-NTN-A3-4 | pos1 | -0.8 |

Table 11.2.2.5.1-3: Test requirements for PUSCH with 70% of maximum throughput, PUSCH mapping Type B, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-3 | pos1 | 4.3 |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3-3 | pos1 | 2.2 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-3 | pos1 | 0.1 |
|  |  | Normal | NTN-TDLC5-200 Low | 70% | G-FR1-NTN-A3-3 | pos1 | -0.6 |

Table 11.2.2.5.1-4: Test requirements for PUSCH with 70% of maximum throughput, PUSCH mapping Type B, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-4 | pos1 | 4.1 |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3-4 | pos1 | 1.9 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-4 | pos1 | -0.1 |
|  |  | Normal | NTN-TDLC5-200 Low | 70% | G-FR1-NTN-A3-4 | pos1 | -0.8 |

11.2.2.5.2	Test requirement for SAN type 2-O

The throughput measured according to clause 11.2.2.4.2 shall not be below the limits for the SNR levels specified in table 11.2.2.5.2-1.

Table 11.2.2.5.2-1: Test requirements for PUSCH with 70% of maximum throughput, PUSCH mapping Type B, 50 MHz channel bandwidth, 120 kHz SCS in FR2-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-1200 Low | 70 % | G-FR2-NTN-A5-2 | pos1 | 0.7 |
|  | 2 | Normal | NTN-TDLC5-1200 Low | 70 % | G-FR2-NTN-A5-2 | pos1 | -2.6 |

11.2.3	Performance requirements for UL timing adjustment

11.2.3.1	Definition and applicability

The performance requirement of UL timing adjustment is determined by a minimum required throughput measured for the moving UE at given SNR. The performance requirements assume HARQ retransmissions.

In the tests for UL timing adjustment, two signals are configured, one being transmitted by a moving UE and the other being transmitted by a stationary UE. The transmission of SRS from UE is optional. FRC parameters in Table A.3-3 are applied for both UEs. The received power for both UEs is the same. The resource blocks allocated for both UEs are consecutive.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.3.

11.2.3.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2] clause 11.2.1.3.

11.2.3.3	Test Purpose

The test shall verify the receiver's ability to achieve throughput measured for the moving UE at given SNR under moving propagation conditions.

11.2.3.4	Method of test

11.2.3.4.1	Initial Conditions

Test environment: Normal, see annex B.2.

RF channels to be tested: M; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.2.3.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 11.2.3.4.2-1.

Table 11.2.3.4.2-1 Test parameters for testing UL timing adjustment for SAN type 1-O

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 12 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 12 |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | CSRS = 5, BSRS =0, for 20 RB |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1. The transmission of SRS is optional. The transmission comb is configured as KTC = 2. The SRS periodic is configured as TSRS = 10 for 15kHz SCS and 20 for 30kHz SCS respectively. |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G. Unless stated otherwise, the MIMO correlation matrices for the gNB are defined in annex G for low correlation.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.2.3.5 and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.2.3.4.2-2.

Table 11.2.3.4.2-2: AWGN power level at the SAN input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5MHz |
| 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |

8)	For reference channels applicable to the SAN, measure the throughput.

11.2.3.5	Test Requirement

11.2.3.5.1	Test requirement for SAN type 1-O

The throughput measured according to clause 11.2.3.4.2 shall not be below the limits for the SNR levels specified in table 11.2.3.5.1-1 to 11.2.3.5.1-4.

Table 11.2.3.5.1-1: Test requirements for UL timing adjustment with 70% of maximum throughput, PUSCH mapping Type A, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | Scenario X | 70 % | G-FR1-NTN-A3-5 | pos1 | 4.7 |
|  | 2 | Normal | Scenario X | 70 % | G-FR1-NTN-A3-5 | pos1 | 0.3 |

Table 11.2.3.5.1-2: Test requirements for UL timing adjustment with 70% of maximum throughput, PUSCH mapping Type A, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | Scenario X | 70 % | G-FR1-NTN-A3-6 | pos1 | 4.2 |
|  | 2 | Normal | Scenario X | 70 % | G-FR1-NTN-A3-6 | pos1 | 0.1 |

Table 11.2.3.5.1-3: Test requirements for UL timing adjustment with 70% of maximum throughput, PUSCH mapping Type B, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | Scenario X | 70 % | G-FR1-NTN-A3-5 | pos1 | 4.8 |
|  | 2 | Normal | Scenario X | 70 % | G-FR1-NTN-A3-5 | pos1 | 0.3 |

Table 11.2.3.5.1-4: Test requirements for UL timing adjustment with 70% of maximum throughput, PUSCH mapping Type B, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | Scenario X | 70 % | G-FR1-NTN-A3-6 | pos1 | 4.2 |
|  | 2 | Normal | Scenario X | 70 % | G-FR1-NTN-A3-6 | pos1 | 0.2 |

11.2.4	Performance requirements for PUSCH repetition Type A

11.2.4.1	Definition and applicability

The performance requirement of PUSCH with slot aggregation factor configured is determined by a maximum target BLER for a given SNR. The required BLER is defined as the probability of incorrectly decoding the PUSCH information when the PUSCH information is sent for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.3.

11.2.4.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2] clause 11.2.1.4.

For SAN type 2-O, the minimum requirement is in TS 38. 108 [2] clause 11.2.2.3.

11.2.4.3	Test Purpose

The test shall verify the receiver's ability to achieve 1% BLER with PUSCH repetition Type A under multipath fading propagation conditions for a given SNR.

11.2.4.4	Method of test

11.2.4.4.1	Initial Conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.2.4.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 11.2.4.4.2-1.

Table 11.2.4.4.2-1: Test parameters for testing PUSCH repetition Type A

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | SAN Type 1-O | SAN Type 2-O |
| Transform precoding |  | Disabled |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port | {0} |  |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |  |
| Time domain resource assignment | PUSCH mapping type | A, B | B |
|  | Start symbol | 0 |  |
|  | Allocation length | 14 | 10 |
|  | PUSCH aggregation factor | n2 |  |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PTRS configuration | Frequency density (KPT-RS) | N.A. | Disabled |
|  | Time density (LPT-RS) | N.A. | Disabled |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.2.4.5 and clause 11.2.4.6 for SAN type 1-O and SAN type 2-O respectively, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.2.4.4.2-2.

Table 11.2.4.4.2-2: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 1-O (NOTE 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| 2-O (NOTE 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	For reference channels applicable to the SAN, measure the throughput.

11.2.4.5	Test Requirement

11.2.4.5.1	Test requirement for SAN type 1-O

The BLER measured according to clause 11.2.4.4.2 shall not be above the limits for the SNR levels specified in table 11.2.4.5.1-1 to 11.2.4.5.1-4.

Table 11.2.4.5.1-1: Test requirements for PUSCH repetition TypeA, PUSCH mapping Type A, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Target BLER | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 1% (Note 1) | G-FR1-NTN-A3A-1 | pos1 | -4.5 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 1% (Note 1) | G-FR1-NTN-A3A-1 | pos1 | -7.9 |
| Note 1: BLER is defined as residual BLER; i.e., ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

Table 11.2.4.5.1-2: Test requirements for PUSCH, PUSCH mapping Type A, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Target BLER | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 1% (Note 1) | G-FR1-NTN-A3A-2 | pos1 | -4.5 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 1% (Note 1) | G-FR1-NTN-A3A-2 | pos1 | -7.9 |
| Note 1: BLER is defined as residual BLER; i.e., ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

Table 11.2.4.5.1-3: Test requirements for PUSCH, PUSCH mapping Type B, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Target BLER | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 1% (Note 1) | G-FR1-NTN-A3A-1 | pos1 | -4.5 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 1% (Note 1) | G-FR1-NTN-A3A-1 | pos1 | -7.9 |
| Note 1: BLER is defined as residual BLER; i.e., ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

Table 11.2.4.5.1-4: Test requirements for PUSCH, PUSCH mapping Type B, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Target BLER | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 1% (Note 1) | G-FR1-NTN-A3A-2 | pos1 | -4.5 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 1% (Note 1) | G-FR1-NTN-A3A-2 | pos1 | -7.9 |
| Note 1: BLER is defined as residual BLER; i.e., ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

11.2.4.5.2	Test requirement for SAN type 2-O

The BLER measured according to clause 11.2.4.4.2 shall not be above the limits for the SNR levels specified in table 11.2.4.5.2-1.

Table 11.2.4.5.2-1: Test requirements for PUSCH repetition TypeA, PUSCH mapping Type B, 50 MHz channel bandwidth, 120 kHz SCS in FR2-NTN

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Target BLER | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-1200 Low | 1% (Note 1) | G-FR2-NTN-A3A-1 | pos1 | -8.0 |
|  | 2 | Normal | NTN-TDLC5-1200 Low | 1% (Note 1) | G-FR2-NTN-A3A-1 | pos1 | -11.4 |
| Note 1: BLER is defined as residual BLER; i.e., ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

11.2.5	Performance requirements for PUSCH with DM-RS bundling

11.2.5.1	Definition and applicability

The performance requirement of PUSCH with DM-RS bundling is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.3.

11.2.5.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2], clause 11.2.2.5.

11.2.5.3	Test purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR with DM-RS bundling.

11.2.5.4	Method of test

11.2.5.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M, see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.2.5.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 11.2.5.4.2-1.

Table 11.2.5.4.2-1: Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence (Note 1) | 0,0,0,0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n4 for 15kHz SCS n8 for 30kHz SCS |
| pusch-TimeDomainWindowLength |  | 4 for 15kHz SCS 8 for 30kHz SCS |
| Frequency domain resource assignment | RB assignment | 6 RBs in the middle of the channel bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| SAN type |  | 1-O |
| NOTE 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.2.5.5.1 for SAN type 1-O, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.2.5.4.2-2.

Table 11.2.5.4.2-2: AWGN power level at the SAN input

| SAN Type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 1-O (NOTE 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 7.1. NOTE 2: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	For reference channels applicable to the SAN, measure the throughput.

11.2.5.5	Test Requirement

11.2.5.5.1	Test requirement for SAN type 1-O

The throughput measured according to clause 11.2.5.4.2 shall not be below the limits for the SNR levels specified in table 11.2.5.5.1-1 ~ 11.2.5.5.1.4.

Table 11.2.5.5.1-1: Test requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | -0.1 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | -3.6 |

Table 11.2.5.5.1-2: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | -2.9 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | -6.1 |

Table 11.2.5.5.1-3: Test requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | -0.0 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | -3.5 |

Table 11.2.5.5.1-4: Test requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | -2.8 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | -6.1 |

<End of Change 1>

<Start of Change 2>

11.3	OTA performance requirements for PUCCH

11.3.1	Performance requirements for PUCCH format 0

11.3.1.1	Definition and applicability

The performance requirement of single user PUCCH format 0 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

The transient period as specified in TS 38.101-5 [12] clause 6.3.3 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e., intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.3.

11.3.1.2	Minimum Requirement

For SAN type 1-O, the minimum requirements are in TS 38.108 [2] clause 11.3.1.1 and 11.3.1.2.

For SAN type 2-O, the minimum requirements are in TS 38.108 [2] clause 11.3.2.1 and 11.3.2.2.

11.3.1.3	Test purpose

The test shall verify the receiver's ability to detect ACK under multipath fading propagation conditions for a given SNR.

11.3.1.4	Method of test

11.3.1.4.1	Initial conditions

Test environment:	Normal, see Annex B.2.

RF channels to be tested:	single carrier  M; see clause 4.9.1.

Direction to be tested:	OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.3.1.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [8] and according to additional test parameters listed in table 11.3.1.4.2-1.

Table 11.3.1.4.2-1: Test parameters

| Parameter | SAN type 1-O | SAN type 2-O |
| --- | --- | --- |
| number of UCI information bits | 1 |  |
| Number of PRBS | 1 |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | Enabled |  |
| First PRB after frequency hopping | N/A |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Initial cyclic shift | 0 |  |
| First symbol | 12 for 2 symbols |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.2.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.3.1.5.1 and 11.3.1.5.2 for SAN type 1-O and SAN type 2-O respectively, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level quoted in table 11.3.1.4.2-2.

Table 11.3.1.4.2-2: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O (Note 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| SAN type 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	The signal generator sends a test pattern with the pattern outlined in figure 11.3.1.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

Figure 11.3.1.4.2-1: Test signal pattern for single user PUCCH format 0 demodulation tests

<End of Change 2>

<Start of Change 3>

11.3.2	Performance requirements for PUCCH format 1

11.3.2.1	NACK to ACK detection

11.3.2.1.1	Definition and applicability

The performance requirement of PUCCH format 1 for NACK to ACK detection is determined by the two parameters: probability of false detection of the ACK and the NACK to ACK detection probability. The performance is measured by the required SNR at probability of the NACK to ACK detection equal to 0.1% or less. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK at particular bit position when input is only noise. Each false bit detection is counted as one error.

The NACK to ACK detection probability is the probability of detecting an ACK bit when a NACK bit was sent on particular bit position. Each NACK bit erroneously detected as ACK bit is counted as one error. Erroneously detected NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e., NACK bits received when DTX is sent should not be considered.

The transient period as specified in TS 38.101-5 [12] clause 6.3.3 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e., intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.3.

11.3.2.1.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2], clause 11.3.1.3.

For SAN type 2-O, the minimum requirement is in TS 38.108 [2], clause 11.3.2.3.

11.3.2.1.3	Test purpose

The test shall verify the receiver's ability not to falsely detect NACK bits as ACK bits under multipath fading propagation conditions for a given SNR.

11.3.2.1.4	Method of test

11.3.2.1.4.1	Initial Conditions

Test environment: Normal, see Annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1

Direction to be tested: OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.3.2.1.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [8], and according to additional test parameters listed in table 11.3.2.1.4.2-1.

Table 11.3.2.1.4.2-1: Test parameters

| Parameter | Test |  |
| --- | --- | --- |
|  | SAN type 1-O | SAN type 2-O |
| Number of information bits | 2 |  |
| Number of PRBs | 1 |  |
| Number of symbols | 14 |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index - (Number of PRBs - 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Initial cyclic shift | 0 |  |
| First symbol | 0 |  |
| Index of orthogonal cover code (timeDomainOCC) | 0 |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.2.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.3.2.1.5.1 and 11.3.2.1.5.2 for SAN type 1-O and SAN type 2-O respectively, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.3.2.1.4.2-2.

Table 11.3.2.1.4.2-2: AWGN power level at the SAN input

| SAN type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O (Note 2) | 15 kHz | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 kHz | 10 | -83.6 – ΔOTAREFSENS dBm / 8.64 MHz |
| SAN type 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	The signal generator sends random codeword from applicable codebook, in regular time periods. The following statistics are kept: the number of ACK bits detected in the idle periods and the number of NACK bits detected as ACK.

<End of Change 3>

<Start  of Change 4>

11.3.3	Performance requirements for PUCCH format 2

11.3.3.1	ACK missed detection performance requirements

11.3.3.1.1	Definition and applicability

The performance requirement of PUCCH format 2 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK on the wanted signal. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as probability of detection of the ACK when the signal is present.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.3.

The transient period as specified in TS 38.101-5 [12] clause 6.3.3 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e., intra-slot frequency hopping is enabled.

11.3.3.1.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2] clause 11.3.1.4.

For SAN type 2-O, the minimum requirement is in TS 38.108 [2] clause 11.3.2.4.

11.3.3.1.3	Test Purpose

The test shall verify the receiver's ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

11.3.3.1.4	Method of test

11.3.3.1.4.1	Initial conditions

Test environment:	Normal, see Annex B.2.

RF channels to be tested for single carrier;	M; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.44 in table.4.6-1).

11.3.3.1.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [8], and according to additional test parameters listed in table 11.3.3.1.4.2-1.

Table 11.3.3.1.4.2-1: Test parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | SAN type 1-O | SAN type 2-O |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | N/A |  |
| First PRB after frequency hopping | The largest PRB index - (Number of PRBs-1) |  |
| Number of PRBs | 4 |  |
| Number of symbols | 1 |  |
| The number of UCI information bits | 4 |  |
| First symbol | 13 |  |
| DM-RS sequence generation | NID0=0 |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.2.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.3.3.1.5.1 and 11.3.3.1.5.2 for SAN type 1-O and SAN type 2-O respectively, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.3.3.1.4.2-2.

Table 11.3.3.1.4.2-2: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O (Note 2) | 15 kHz | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 kHz | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| SAN type 2-O (Note 5) | 120 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	The signal generator sends a test pattern with pattern outlined in figure 11.3.3.1.4.2-1. The following statistics are kept: the number of ACK bits detected in the idle periods and the number of missed ACKs.

<End  of Change 4>

<Start of Change 5>

11.3.4	Performance requirements for PUCCH format 3

11.3.4.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 1 and part 2.

The transient period as specified in TS 38.101-5 [12] clause 6.3.3 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.3.

11.3.4.2	Minimum requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2], clause 11.3.1.5.

For SAN type 2-O, the minimum requirement is in TS 38.108 [2], clause 11.3.2.5.

11.3.4.3	Test purpose

The test shall verify the receiver's ability to detect UCI under multipath fading propagation conditions for a given SNR.

11.3.4.4	Method of test

11.3.4.4.1	Initial conditions

Test environment: Normal, see Annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1

Direction to be tested:

-	OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.3.4.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [8], and according to additional test parameters listed in table 11.3.4.4.2-1.

Table 11.3.4.4.2-1: Test parameters

| Parameter | Test |  |
| --- | --- | --- |
|  | SAN type 1-O | SAN type 2-O |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 |  |
| Number of symbols | 14 |  |
| The number of UCI information bits | 16 |  |
| First symbol | 0 |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.2.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.3.4.5.1 and 11.3.4.5.2 for SAN type 1-O and SAN type 2-O respectively, and the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.3.4.4.2-2.

Table 11.3.4.4.2-2: AWGN power level at the SAN input

| SAN type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O (Note 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64MHz |
| SAN type 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

<End of Change 5>

<Start of Change 6>

Table 11.3.4.5.2-1: Required SNR for PUCCH format 3 with 120 kHz SCS 50MHz channel bandwidth

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | 2.9 |
|  |  |  |  | Additional DM-RS | 2.3 |
|  | 2 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | -1.3 |
|  |  |  |  | Additional DM-RS | -1.9 |

<End of Change 6>

<Start of Change 7>

11.3.5	Performance requirements for PUCCH format 4

11.3.5.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 1 and part 2.

The transient period as specified in TS 38.101-5 [12] clause 6.3.3 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e., intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.3.

11.3.5.2	Minimum requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2], clause 11.3.1.6.

For SAN type 2-O, the minimum requirement is in TS 38.108 [2], clause 11.3.2.6.

11.3.5.3	Test purpose

The test shall verify the receiver's ability to detect UCI under multipath fading propagation conditions for a given SNR.

11.3.5.4	Method of test

11.3.5.4.1	Initial conditions

Test environment: Normal, see Annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1

Direction to be tested:

-	OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.3.5.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [8], and according to additional test parameters listed in table 11.3.4.4.2-1.

Table 11.3.5.4.2-1: Test parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | SAN type 1-O | SAN type 2-O |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Number of PRBS | 1 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index - (Number of PRBS - 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of symbols | 14 |  |
| The number of UCI information bits | 22 |  |
| First symbol | 0 |  |
| Length of the orthogonal cover code | n2 |  |
| Index of the orthogonal cover code | n0 |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.2.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.3.5.5.1 and 11.3.5.5.2 for SAN type 1-O and SAN type 2-O respectively, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.3.5.4.2-2.

| SAN type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O (Note 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64MHz |
| SAN type 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

<End of Change 7>

<Start of Change 8>

Table 11.3.5.5.2-1: Required SNR for PUCCH format 4 with 120 kHz SCS 50MHz channel bandwidth

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | 4.7 |
|  |  |  |  | Additional DM-RS | 6.4 |
|  | 2 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | 0.4 |
|  |  |  |  | Additional DM-RS | 0.0 |

<End of Change 8>

<Start of Change 9>

11.4	OTA performance requirements for PRACH

11.4.1	PRACH false alarm probability and missed detection

11.4.1.1	Definition and applicability

The performance requirement of PRACH for preamble detection is determined by the two parameters: total probability of false detection of the preamble (Pfa) and the probability of detection of preamble (Pd). The performance is measured by the required SNR at probability of detection, Pd of 99%. Pfa shall be 0.1% or less.

Pfa is defined as a conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

Pd is defined as conditional probability of detection of the preamble when the signal is present. The erroneous detection consists of several error cases – detecting only different preamble(s) than the one that was sent, not detecting any preamble at all, or detecting the correct preamble but with the out-of-bounds timing estimation value. For AWGN, NTN-TDLA100-200, and NTN-TDLC5-1200, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than the time error tolerance values given in table 11.4.1.1-1.

Table 11.4.1.1-1: Time error tolerance for AWGN, NTN-TDLA100-200, and NTN-TDLC5-1200

| PRACH | PRACH SCS | Time error tolerance |  |  |
| --- | --- | --- | --- | --- |
| preamble | (kHz) | AWGN | NTN-TDLA100-200 | NTN-TDLC5-1200 |
| 0 | 1.25 | 1.04 us | 1.324 us | N/A |
| 2 | 1.25 | 1.04 us | 1.324 us | N/A |
| B4, C2 | 15 | 0.52 us | 0.804 us | N/A |
|  | 30 | 0.26 us | 0.544 us | N/A |
|  | 120 | N/A | N/A | 0.13 us |

The test preambles are listed in table A.4. Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.3.

11.4.1.2	Minimum requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2] clause 11.4.1.1 and 11.4.1.2.

For SAN type 2-O, the minimum requirement is in TS 38.108 [2] clause 11.4.2.1 and 11.4.2.2.

11.4.1.3	Test purpose

The test shall verify the receiver's ability to detect PRACH preamble under static conditions and multipath fading propagation conditions for a given SNR.

11.4.1.4	Method of test

11.4.1.4.1	Initial conditions

Test environment: Normal, see clause B.2.

RF channels to be tested for single carrier: M, see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.4.1.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameter msg1-FrequencyStart is set to 0.

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

7)	Adjust the AWGN generator, according to the SCS and channel bandwidth. The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.4.1.4.2-1.

Table 11.4.1.4.2-1: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O (Note 2) | 15 | 5 | -83.5 - ΔOTAREFSENS dBm / 4.5MHz |
|  | 30 | 10 | -80.6 - ΔOTAREFSENS dBm / 8.64MHz |
| SAN type 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS receiver target reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	Adjust the frequency offset of the test signal according to table 11.4.1.5.1-1 or 11.4.1.5.1-2 or 11.4.1.5.1-3 or 11.4.1.5.2-1.

9)	Adjust the equipment so that the SNR specified in table 11.4.1.5.1-1 or 11.4.1.5.1-2 or 11.4.1.5.1-3 or 11.4.1.5.2-1is achieved at the SAN input during the PRACH preambles.

10) The test signal generator sends a preamble and the receiver tries to detect the preamble. This pattern is repeated as illustrated in figure 11.4.1.4.2-1. The preambles are sent with certain timing offsets as described below. The following statistics are kept: the number of preambles detected in the idle period and the number of missed preambles.

Figure 11.4.1.4.2-1: PRACH preamble test pattern

Unless otherwise stated, the timing offset base value for PRACH preamble format 0 and 2 is set to 50% of Ncs. This offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.9us. Then the loop is being reset and the timing offset is set again to 50% of Ncs. The timing offset scheme for PRACH preamble format 0 and 2 is presented in Figure 11.4.1.4.2-2.

Figure 11.4.1.4.2-2: Timing offset scheme for PRACH preamble format 0 and 2

Unless otherwise stated, the timing offset base value for PRACH preamble format B4 and C2 is set to 0. This offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.8us. Then the loop is being reset and the timing offset is set again to 0. The timing offset scheme for PRACH preamble format B4 and C2 is presented in Figure 11.4.1.4.2-3.

Figure 11.4.1.4.2-3: Timing offset scheme for PRACH preamble format B4 and C2

11.4.1.5	Test requirement

11.4.1.5.1	Test requirement for SAN type 1-O

Pfa shall not exceed 0.1%. Pd shall not be below 99% for the SNRs in tables 11.4.1.5.1-1 to 11.4.1.5.1-3.

Table 11.4.1.5.1-1: PRACH missed detection test requirements, 1.25 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex G) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 2 |
| 1 | 1 | AWGN | 0 | -11.7 | -17.1 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | 1.3 | -9.1 |
|  | 2 | AWGN | 0 | -14.2 | -19.5 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | -6.2 | -14.3 |

Table 11.4.1.5.1-2: PRACH missed detection test requirements, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex G) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format B4 | Burst format C2 |
| 1 | 1 | AWGN | 0 | -14.3 | -8.9 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | -2.1 | 2.5 |
|  | 2 | AWGN | 0 | -16.5 | -12.2 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | -8.4 | -4.2 |

Table 11.4.1.5.1-3: PRACH missed detection test requirements, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex G) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format B4 | Burst format C2 |
| 1 | 1 | AWGN | 0 | -14.1 | -8.9 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | -3.7 | 0.7 |
|  | 2 | AWGN | 0 | -16.2 | -11.6 |
|  |  | NTN-TDLA100-200 Low | 200 Hz | -9.4 | -5.2 |

11.4.1.5.2	Test requirement for SAN type 2-O

Pfa shall not exceed 0.1%. Pd shall not be below 99% for the SNRs in table 11.4.1.5.2-1.

Table 11.4.1.5.2-1: PRACH missed detection test requirements, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (annex G) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format B4 | Burst format C2 |
| 1 | 1 | NTN-TDLC5-1200 Low | 3000 Hz | -6.1 | -2.7 |
|  | 2 | NTN-TDLC5-1200 Low | 3000 Hz | -11.3 | -8.0 |

<End of Change 9>