3GPP TSG-RAN WG4 Meeting #114                                            R4-2502050

# Athens, Greece, 17th – 21st of February 2025

Title: 	Discussion on UE RF requirements for NTN NB-IoT TDD

Agenda Item:	7.31.4 – UE RF requirements

Release:	Rel-19

Source: 	THALES, Iridium Satellite LLC

Document for:	Discussion

# For information

As described in the approved WID RP-242415 for New WID on introduction of IoT-NTN TDD mode (Iridium, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024), the study and work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band.

This work item includes the following objectives:

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2]

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

  - Specify band numbering

  - Specify SAN and UE RF characteristics

  - Specify DL and UL channelization.

  - Specify channel bandwidth as 200 kHz

  - Note1: No NTN-NTN coexistence study needed.

  - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

Current Work Plan from R4-2419663:

| RAN#106 (Dec. 2024) | RAN1#120 | 0.5 | - Based on the study outcome, discuss potential solutions - Identify potential specification impacts - Define the TDD pattern parameters |
| --- | --- | --- | --- |
|  | RAN2#129 | 0.5 | - Start discussion on TDD support for NB IoT NTN - Identify potential impacts on higher layers - Start the definition and configuration of signalling of the periodic pattern, and associated UE procedures. |
|  | RAN4#114 | 0.5+0.25 | - Decide on RF, discuss on RRM requirements that can be re-used from previously defined S/L band. Start preparing Draft CRs for the requirements that can be reused - Conclude on requirements stemming from introduction of TDD mode for NB-IoT NTN and potential impact of the periodic pattern: offset-subframes, DL-UL gap - Start discussion on demodulation performance and specific configuration related to the new TDD pattern |

Decisions from previous RAN4#113 IoT_NTN_TDD WF:

### Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements

Agreement:

- RAN4 to consider the following values for analysing the impact of requirements, as necessary:

  - N = 9, D = {8, 20, 30}, U = {8, 20, 30}

  - N = 8, D = 20, U = 20

- Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis.

Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision.

Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

Decisions from RAN plenary, December 2024, Madrid (WID update):

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with at least N=9 as baseline. No blind detection is assumed at the UE side. The value of N is fixed per band. Other parameters of the configuration of the periodic pattern (e.g. downlink to uplink guard period for TDD operation and the number of consecutive/contiguous DL and UL subframes) can be defined per cell.

- For this target band N=9 and minimum of D=8 and U=8 to be supported.

The RAN4#114 meeting agenda is:

7.31	Introduction of IoT-NTN TDD mode	[IoT_NTN_TDD]

7.31.1	General aspects	[IoT_NTN_TDD]

7.31.2	Band and system parameters (DL and UL channelization, channel bandwidth)	[IoT_NTN_TDD-Core]

7.31.3	SAN RF requirements	[IoT_NTN_TDD-Core]

7.31.4	UE RF requirements	[IoT_NTN_TDD-Core]

7.31.5	RRM core requirements	[IoT_NTN_TDD-Core]

7.31.6	Moderator summary and conclusions	[IoT_NTN_TDD-Core]

# General aspects for NTN NB-IoT TDD

Observation 1: First of all, RAN4 is kindly requested to take into account the WID update:

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with at least N=9 as baseline. No blind detection is assumed at the UE side. The value of N is fixed per band. Other parameters of the configuration of the periodic pattern (e.g. downlink to uplink guard period for TDD operation and the number of consecutive/contiguous DL and UL subframes) can be defined per cell.

- For this target band N=9 and minimum of D=8 and U=8 to be supported.

# UE Requirements for IoT_NTN_TDD

First, please recall the proposals submitted in 7.31.2	- Band and system parameters (DL and UL channelization, channel bandwidth) [IoT_NTN_TDD-Core] at RAN4#114:

Observation 2:

Table 5.4A.2-1: E-UTRA channel numbers E-UTRA Operating Band; ΔFRaster (kHz); Downlink; Uplink. FDL_low (MHz); NOffs-DL; Range of NDL (First – <Step size> – Last); FUL_low (MHz); NOffs-UL; Range of NUL (First – <Step size> – Last). 256; 100; 2170; 229076; 229076 -<1>- 229375; 1980; 261844; 261844 -<1>- 262143. 255; 100; 1525; 228736; 228736 -<1>- 229075; 1626.5; 261504; 261504 -<1>- 261843. 254; 100; 2483.5; 228571; 228571 -<1>- 228735; 1610; 261339; 261339 -<1>- 261503. 253; 100; 1518; 228501; 228501 -<1>- 228570; 1668; 261269; 261269 -<1>- 261338. 249; 100; 1616; 196607; 196607-<1>-196711; 1616; 196607; 196607-<1>-196711. NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. NOTE 2: Band 249 is defined only for NB-IoT operation..

Observation 3:

## 5.4B Channel arrangement for category NB1 and NB2 ### 5.4B.1 Channel spacing Nominal channel spacing for UE category NB1 and NB2 in stand-alone mode is 200 kHz. ### 5.4B.2 Channel raster, carrier frequency and EARFCN The channel raster of UE category NB1/NB2 shall be as defined in clause 5.4A.2, and the channel raster per-frequency band shall be as defined in table 5.4A.2-1. The carrier frequency of UE category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD and TDD band 249. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN. FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL) The carrier frequency of UE category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD and TDD band 249. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN. FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL) NOTE 1: Guard-band operation and in-band operation for NB-IoT are not supported in this version of the specification. NOTE 2: For the carrier including NPSS/NSSS for stand-alone operation, MDL = 0.

And please also check the agreements from the previous WF at RAN#113:

Agreement:

- The assessment of the impact on UE RF requirements (TS 36.102) shown in the table below will serve as a baseline for further discussion on the exact requirement definition.

| 36.102 Clause: Requirements | Band Agnostic | Re-use for TDD band [249] | Note |
| --- | --- | --- | --- |
| 6.2B.1: UE maximum output power | Yes | Yes | Support UE power class 3 (23 dBm) Support UE power class 5 (20 dBm) |
| 6.2B.1: UE maximum output power tolerance | No |  |  |
| 6.2B.2: UE maximum output power reduction | Yes | Yes |  |
| 6.2B.3: UE additional maximum output power reduction | No | - | Note: Reuse which band A-MPR, B254 A-MPR is not defined yet and the band [249] UL frequency range is not fully aligned with B254. |
| 6.2.B.4: Configured transmit power | Yes | Yes |  |
| 6.3: Output power dynamics | Yes | Yes FFS | Note: For 36.102 6.3B.4, 36.102 refers to 36.101 6.3.5F, the testing period for aggerate power control may depend on different UL pattern. |
| 6.4: Transmit signal quality | Yes | Yes |  |
| 6.5B.3.2: Spectrum emission mask | Yes | Yes |  |
| 6.5B.3.4: Adjacent Channel Leakage Ratio | Yes | Yes |  |
| 6.5B.4.2: General spurious emissions | Yes | Yes |  |
| 6.5B.4.3: Spurious emissions for UE co-existence | No |  | Band [249] needs to be added in Table 6.5A.4.3-1 Spurious emission band UE co-existence. |
| 6.5B.4.4: Additional spurious emissions | No |  | Additional spurious emissions requirement for B254 can be reused for the new IoT-NTN TDD L-band. UL frequency range of B254 is superset of [B249]. |
| 6.6: Transmit intermodulation for category NB1 and NB2 | Yes | Yes |  |
| 7.2: Diversity characteristics | Yes | Yes |  |
| 7.3: Reference sensitivity power level | No | No | To be studied for IoT-NTN TDD mode. |
| 7.4: Maximum input level | Yes | Yes |  |
| 7.5: Adjacent Channel Selectivity | Yes | Yes |  |
| 7.6B.2: In-band blocking | No | Yes | Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 255,254, and 253 apply the same IBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use IBB requirements from B255,254 and 253 |
| 7.6B.3: Out-of-band blocking | No |  | Different Out-of-band requirements for IoT NTN bands. Currently, bands 255,254, and 253 apply the same OBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use OBB requirements from B255, 254 and 253 |
| 7.6B.4: Narrow band blocking | No | Yes |  |
| 7.7: Spurious response | Yes | Yes |  |
| 7.8: Intermodulation characteristics | Yes | Yes |  |
| 7.9: RX spurious emission | No | Yes | Re-use spurious emission requirements from Table 7.9.1-1 of 36.101 |
| A.1 DL reference measurement channels |  |  | NPDSCH Reference Channel for TDD should be introduced. Fixed Reference Channel for TDD should be introduced. |
| A.2 OFDMA Channel Noise Generator (OCNG) |  |  | OCNG Patterns for TDD should be introduced. |
| A.3 Testing related to Satellite Access |  |  | No impact. |
| A.4 UL reference measurement channels |  |  | Reference measurement channels for TDD should be introduced. |

Therefore, we propose the following set of proposals:

Proposal 1: For UE TS 36.102, add extra 0.5 dB for the REFSENS TDD compared to FDD due to the lack of duplexer.

Proposal 2: RAN4 to adopt the following modification in the TS 36.102:

## 7.3B Reference sensitivity power level for UE category NB1 and NB2 The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise. The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2 with received signal level as specified in Table 7.3B-1. Requirement in Table 7.3B-1 applies for any uplink configuration. Table 7.3B-1: Reference sensitivity for UE category NB1 and NB2 Operating band; REFSENS [dBm]. According to subclause 5.2B; - 108.2. The UE TDD category NB1 and NB2 throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2 with received signal level as specified in Table 7.3B-2. Requirement in Table 7.3B-2 applies for any uplink configuration. Table 7.3B-2: Reference sensitivity for UE category NB1 and NB2 Operating band; REFSENS [dBm]. According to subclause 5.2B; - 108.7.

# NB-IoT Specific TDD performance demodulation requirements for TS 36.102

In TS 36.102, currently is described:

## 8.3 Demodulation performance requirements for UE category NB1 and NB2 ### 8.3.1 Half-duplex FDD #### 8.3.1.1 NPDSCH demodulation requirements The parameters specified in Table 8.3.1.1-1 and Table 8.3.1.1-2 are valid for all half-duplex FDD tests unless otherwise stated. Table 8.3.1.1-1: Common Test Parameters Parameter; Unit; Value. Number of HARQ processes per component carrier; Processes; 1. Maximum number of HARQ transmission; 4. Cyclic Prefix; Normal. eutraControlRegionSize-r13; N/A. downlinkBitmap-r13 and dl-Gap-r13; Not configured. dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13; Not configured. Unused REs or RB; OCNG. OCNG pattern; NB.OP.1. Table 8.3.1.1-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations Parameter; Unit; Value. DCI format; DCI format N1. scheduling delay field (); 1. (ack-NACK-NumRepetitions-r13); 1. ACK/NACK resource field; 0. Reference channel for NPDCCH; R.NB.3 FDD. (npdcch-Offset-USS-r13); 0. K_offset; ms; 8.

And further it can be found:

##### 8.3.1.1.1 Single-antenna port performance 8.3.1.1.1.1 Minimum Requirements for standalone The requirements are specified in Table 8.3.1.1.1.1-2, with the addition of the parameters in Table 8.3.1.1.1.1-1 and the downlink physical channel setup according to Annex B.3.3. The purpose of these tests is to verify the performance. Table 8.3.1.1.1.1-1: Test Parameters for NPDSCH under Standalone Parameter; Unit; Test 1, 2. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -99 (Note 2). NPDCCH repetition number; subframe; 32 for Test 1; 128 for Test 2.. (npdcch-NumRepetitions-r13); subframe; 64 for Test 1; 256 for Test 2.. (nPDCCH-startSF-USS-r13); 1.5. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission. Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.3.1.1.1.1-2: Minimum performance for NPDSCH under Standalone with 1 NRS port Test number; Bandwidth; Carrier Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Anchor; R.NB.1 FDD; 32; NTN-TDLC5-200; 1; 1x1; 70%; -4.7; NB1, NB2. 2; 200kHz; Non-anchor; R.NB.2 FDD; 128; NTN-TDLA100-10; 1; 1x1; 70%; -10.6; NB1, NB2.

Proposal 3: Introduce a new clause for TDD NB-IoT category for 249 band (currently only FDD configuration is defined) for NPDSCH minimum performance requirements.

Proposal 4: Further discuss repetition number to be considered for demodulation and performance aspects. Provide SNR values for less repetition numbers to align with U/L=8 and N=9 as per TDD NB-IOT NTN band 249 configuration.

Similarly, TS 36.101 currently describes:

## 8.12 Demodulation of Narrowband IoT ### 8.12.1 NPDSCH 8.12.1.1 Half-duplex FDD The parameters specified in Table 8.12.1.1-1 are valid for all half-duplex FDD tests unless otherwise stated. Table 8.12.1.1-1: Common Test Parameters Parameter; Unit; Value. Number of HARQ processes per component carrier (Note 2); Processes; 1 or 2. Maximum number of HARQ transmission; 4. Cyclic Prefix; Normal. eutraControlRegionSize-r13; 3 for In-band and N/A for Standalone/Guard-band unless otherwise stated. downlinkBitmap-r13 and dl-Gap-r13; Not configured. dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13; Not configured. Unused REs or RB (Note 1); OCNG. OCNG pattern; NB.OP.1. Note 1: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. Note 2: 1 is applicable for both UE Category NB1 and NB2; 2 is only applicable for UE Category NB2 if UE supports it.. Table 8.12.1.1-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations Parameter; Unit; Value. DCI format; DCI format N1. scheduling delay field (); For tests in 8.12.1.1.4: 1 For others: 1 for the first HARQ process; 2 for the second HARQ process if configured. (ack-NACK-NumRepetitions-r13); 1. ACK/NACK resource field; 0. Reference channel for NPDCCH; R.NB.3 FDD for one NRS antenna port; R.NB.4 FDD for two NRS antenna ports. (npdcch-Offset-USS-r13); 0. ##### 8.12.1.1.1 Minimum Requirements for In-band The requirements are specified in Table 8.12.1.1.1-2, with the addition of the parameters in Table 8.12.1.1.1 -1 and the downlink physical channel setup according to Annex C.3.2 and C.3.6. The purpose of these tests is to verify the performance. Table 8.12.1.1.1-1: Test Parameters for NPDSCH under In-band Parameter; Unit; Test 1, 2. Downlink power allocation of LTE signal; dB; -3. dB; -3(Note 1). dB; 0. at antenna port; dBm/15kHz; -93 (Note 2). dBm/15kHz; -99 (Note 3). LTE CRS port number (eutra-NumCRS-Ports-r13); 4. NPDCCH repetition number; subframe; 8 for Test 1; 16 for Test 2; 128 for Test 3.. (npdcch-NumRepetitions-r13); subframe; 8 for Test 1; 16 for Test 2; 128 for Test 3.. (nPDCCH-startSF-USS-r13); 4 for Test 1; 2 for Test 2; 1.5 for Test 3. Note 1: . Note 2: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 3: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.1.1-2: Minimum performance under In-band with 2 NRS ports Test number; Bandwidth; Carrier Type; Reference Channel; Repetition number; Propagation Condition; Correlation Matrix and Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Anchor; R.NB.5 FDD; 1; EPA5; 2x1 Low; 70%; 6.9; NB1, NB2. 2; 200kHz; Anchor; R.NB.5 FDD; 32; EPA5; 2x1 Low; 70%; -4.8; NB1, NB2. 3; 200kHz; Non-anchor; R.NB.5-1 FDD; 256; ETU1; 2x1 Low; 70%; -9.8; NB1, NB2. ##### 8.12.1.1.2 Minimum Requirements for Standalone/Guard-band The requirements are specified in Table 8.12.1.1.2-2, with the addition of the parameters in Table 8.12.1.1.2 -1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the performance. Table 8.12.1.1.2-1: Test Parameters for NPDSCH under Standalone/Guard-band Parameter; Unit; Test 1, 2. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -99 (Note 2). NPDCCH repetition number; subframe; 32 for Test 1; 256 for Test 2.. (npdcch-NumRepetitions-r13); subframe; 64 for Test 1; 512 for Test 2.. (nPDCCH-startSF-USS-r13); 1.5. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.1.2-2: Minimum performance for NPDSCH under Standalone/Guard-band with 1 NRS port Test number; Bandwidth; Carrier Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Anchor; R.NB.6 FDD; 32; EPA5; 1; 1x1; 70%; -3.4; NB1, NB2. 2; 200kHz; Non-anchor; R.NB.6-1 FDD; 256; ETU1; 1; 1x1; 70%; -10.2; NB1, NB2. ##### 8.12.1.1.3 Minimum Requirements for Standalone for UE Category NB2 The requirements are specified in Table 8.12.1.1.3-2, with the addition of the parameters in Table 8.12.1.1.3-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance. Table 8.12.1.1.3-1: Test Parameters for NPDSCH under Standalone Parameter; Unit; Test 1. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -96 (Note 2). NPDCCH repetition number; subframe; 4 for Test 1. (npdcch-NumRepetitions-r13); subframe; 8 for Test 1. (nPDCCH-startSF-USS-r13); 1.5. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.1.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port Test number; Bandwidth; Carrer Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Non-anchor; R.NB.7 FDD; 1; EPA5; 1; 1x1; 70%; 9.4; NB2. ##### 8.12.1.1.4 Minimum Requirements for Standalone for UE with multiple TBs interleaved transmission The requirements are specified in Table 8.12.1.1.4-2, with the addition of the parameters in Table 8.12.1.1.4-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify NPDSCH performance when multiple TBs with interleaved transmission are scheduled by one DCI as specified in TS 36.213[6]. These requirements are applicable for UE of UE-Category-NB NB2 supporting multiple TBs scheduling with interleaved transmission when multiple TBs are scheduled. Table 8.12.1.1.4-1: Test Parameters for NPDSCH under Standalone Parameter; Unit; Test 1. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -99 (Note 2). NPDCCH repetition number; subframe; 128 for Test 1. (npdcch-NumRepetitions-r13); subframe; 128 for Test 1. (nPDCCH-startSF-USS-r13); 1.5. Two HARQ processes (twoHARQ-ProcessesConfig-r14); true. Multiple TBs scheduling (multiTB-Config-r16); interleaved. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.1.4-2: Minimum performance for NPDSCH under Standalone with 1 NRS port Test number; Bandwidth; Carrer Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Non-anchor; R.NB.8 FDD; 32; ETU1; 1; 1x1; 70%; -6.0; NB2. ##### 8.12.1.1.5 Minimum Requirements for Standalone for UE with 16-QAM The requirements are specified in Table 8.12.1.1.5-2, with the addition of the parameters in Table 8.12.1.1.5-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance with 16-QAM. Table 8.12.1.1.5-1: Test Parameters for 16-QAM NPDSCH under Standalone Parameter; Unit; Test 1. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -96 (Note 2). NPDCCH repetition number; subframe; 1 for Test 1. (npdcch-NumRepetitions-r13); subframe; 4 for Test 1. (nPDCCH-startSF-USS-r13); 2. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.1.5-2: Minimum performance for 16-QAM NPDSCH under Standalone with 1 NRS port Test number; Bandwidth; Carrer Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Non-anchor; R.NB.9 FDD; 1; EPA5; 1; 1x1; 70%; 19.5; NB2. #### 8.12.1.2 TDD Table 8.12.1.2-1: Common Test Parameters Parameter; Unit; Value. Uplink downlink configuration (Note 1); 4. Special subframe configuration (Note 2); 4. Number of HARQ processes per component carrier (Note 4); Processes; 1 or 2. Maximum number of HARQ transmission; 4. Cyclic Prefix; Normal. eutraControlRegionSize-r13; 2 for In-band, N/A for Standalone/Guard-band unless otherwise stated. downlinkBitmap-r13 and dl-Gap-r13; Not configured. dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13; Not configured. Unused REs or RB (Note 3); OCNG. OCNG pattern; NB.OP.1. Note 1: As specified in Table 4.2-2 in TS 36.211 [4] Note 2: As specified in Table 4.2-1 in TS 36.211 [4] Note 3: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. Note 4: 1 is applicable for both UE Category NB1 and NB2; 2 is only applicable for UE Category NB2 if UE supports it.. Table 8.12.1.2-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations Parameter; Unit; Value. DCI format; DCI format N1. scheduling delay field (); 1 for the first HARQ process; 2 for the second HARQ process if configured. (ack-NACK-NumRepetitions-r13); 1. ACK/NACK resource field; 0. Reference channel for NPDCCH; R.NB.3 TDD for one NRS antenna port; R.NB.4 TDD for two NRS antenna ports. (npdcch-Offset-USS-r13); 0. ##### 8.12.1.2.1 Minimum Requirements for In-band The requirements are specified in Table 8.12.1.2.1-2, with the addition of the parameters in Table 8.12.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2 and C.3.6. The purpose of these tests is to verify the performance. Table 8.12.1.2.1-1: Test Parameters for NPDSCH under In-band Parameter; Unit; Test 1, 2. Downlink power allocation of LTE signal; dB; -3. dB; -3(Note 1). dB; 0. at antenna port; dBm/15kHz; -93 (Note 2). dBm/15kHz; -99 (Note 3). LTE CRS port number (eutra-NumCRS-Ports-r13); 4. NPDCCH repetition number; subframe; 8 for Test 1; 16 for Test 2; 128 for Test 3.. (npdcch-NumRepetitions-r13); subframe; 8 for Test 1; 16 for Test 2; 128 for Test 3.. (nPDCCH-startSF-USS-r13); 4. Note 1: . Note 2: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 3: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.2.1-2: Minimum performance under In-band with 2 NRS ports Test number; Bandwidth; Carrier Type; Reference Channel; Repetition number; Propagation Condition; Correlation Matrix and Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Anchor; R.NB.5 TDD; 1; EPA5; 2x1 Low; 70%; 6.9; NB1, NB2. 2; 200kHz; Anchor; R.NB.5 TDD; 32; EPA5; 2x1 Low; 70%; -4.8; NB1, NB2. 3; 200kHz; Non-anchor; R.NB.5-1 TDD; 256; ETU1; 2x1 Low; 70%; -9.8; NB1, NB2. ##### 8.12.1.2.2 Minimum Requirements for Standalone/Guard-band The requirements are specified in Table 8.12.1.2.2-2, with the addition of the parameters in Table 8.12.1.2.2-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the performance. Table 8.12.1.2.2-1: Test Parameters for NPDSCH under Standalone/Guard-band Parameter; Unit; Test 1, 2. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -99 (Note 2). NPDCCH repetition number; subframe; 32 for Test 1; 256 for Test 2.. (npdcch-NumRepetitions-r13); subframe; 32 for Test 1; 256 for Test 2.. (nPDCCH-startSF-USS-r13); 4. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.2.2-2: Minimum performance for NPDSCH under Standalone/Guard-band with 1 NRS port Test number; Bandwidth; Carrier Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Anchor; R.NB.6 TDD; 32; EPA5; 1; 1x1; 70%; -3.4; NB1, NB2. 2; 200kHz; Non-anchor; R.NB.6-1 TDD; 256; ETU1; 1; 1x1; 70%; -10.2; NB1, NB2. ##### 8.12.1.2.3 Minimum Requirements for Standalone for UE Category NB2 The requirements are specified in Table 8.12.1.2.3-2, with the addition of the parameters in Table 8.12.1.2.3-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance. Table 8.12.1.2.3-1: Test Parameters for NPDSCH under Standalone Parameter; Unit; Test 1. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -96 (Note 2). NPDCCH repetition number; subframe; 4 for Test 1. (npdcch-NumRepetitions-r13); subframe; 8 for Test 1. (nPDCCH-startSF-USS-r13); 4. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.2.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port Test number; Bandwidth; Carrer Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Non-anchor; R.NB.7 TDD; 1; EPA5; 1; 1x1; 70%; 9.4; NB2. ##### 8.12.1.2.4 Minimum Requirements for Standalone for UE with 16-QAM The requirements are specified in Table 8.12.1.2.4-2, with the addition of the parameters in Table 8.12.1.2.4-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance with 16-QAM. Table 8.12.1.2.4-1: Test Parameters for 16-QAM NPDSCH under Standalone Parameter; Unit; Test 1. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -96 (Note 2). NPDCCH repetition number; Subframe; 1 for Test 1. (npdcch-NumRepetitions-r13); Subframe; 8 for Test 1. (nPDCCH-startSF-USS-r13); 4. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.2.4-2: Minimum performance for 16-QAM NPDSCH under Standalone with 1 NRS port Test number; Bandwidth; Carrer Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Non-anchor; R.NB.8 TDD; 1; EPA5; 1; 1x1; 70%; 19.5; NB2. ### 8.12.2 NPDCCH The receiver characteristics of the NPDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg). #### 8.12.2.1 Half-duplex FDD The parameters specified in Table 8.12.2.1-1 and Table 8.12.2.1-2 are valid for all half-duplex FDD tests unless otherwise stated. Table 8.12.2.1-1: Test Parameters for NPDCCH Parameter; Unit; Single antenna port; Transmit diversity. Narrowband physical layer Cell ID; 0; 0. at antenna port; dBm/15kHz; -98; -98. Cyclic prefix; Normal; Normal. Number of CRS ports for in-band deployment mode; 4; 4. NPDCCH starting position (eutraControlRegionSize-r13) (Note 1); 3; 3. Maximum number of repetitions (npdcch-NumRepetitions-r13); 128 for Test 1; 1024 for Test 2.; 64 for Test 1; 512 for Test 2.. NPDCCH start subframe (npdcch-startSF-USS-r13); 2 for test 1, 1.5 for test 2; 2 for test 1, 1.5 for test 2. NPDCCH fractional period offset of starting subframe (npdcch-Offset-USS-r13); 0; 0. NB-IoT downlink subframe bitmap for anchor carrier (downlinkBitmap-r13); Not configured; Not configured. NB-IoT downlink subframe bitmap for non-anchor carrier (downlinkBitmapNonAnchor-r13); Not configured; Not configured. Downlink gap configuration for anchor carrier (dl-Gap-r13); Not configured; Not configured. Downlink gap configuration for non-anchor carrier (dl-GapNonAnchor-r13); Not configured; Not configured. Unused REs or RBs (Note 1); OCNG; OCNG. OCNG pattern; NB.OP.1; NB.OP.1. NOTE 1: Applicable only for in-band deployment mode.. Table 8.12.2.1-2: Test Parameters of related NPDSCH and NPUSCH format 2 configurations Parameter; Unit; Value. Scheduling delay field (); 0. NPDSCH Repetition number; 1. at antenna port for NPDSCH; dBm/15kHz; -98. (ack-NACK-NumRepetitions-r13); 1. ACK/NACK resource field; 0. Reference channel for NPDSCH; R.NB.6 FDD and R.NB.6-1 FDD for one NRS antenna port; R.NB.5 FDD and R.NB.5-1 FDD for two NRS antenna ports. ##### 8.12.2.1.1 Single-antenna performance For the parameters specified in Table 8.12.2.1.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.1.1-1. The downlink physical channel setup is in accordance with Annex C.3.6. Table 8.12.2.1.1-1: Minimum performance NPDCCH Test number; Deployment mode; Repetition number (R); Carrier Type; Reference Channel; Propagation Condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Pm-dsg (%); SNR (dB). 1; Stand-alone/Guard-band; 128; Anchor; R.NB.3 FDD; EPA5; 1; 1x1; 1; -4.9; NB1, NB2. 2; Stand-alone/Guard-band; 1024; Non-anchor; R.NB.3 FDD; ETU1; 1; 1x1; 1; -11.4; NB1, NB2. ##### 8.12.2.1.2 Transmit diversity performance For the parameters specified in Table 8.12.2.1.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.1.2-1. The downlink physical channel setup is in accordance with Annex C.3.6. Table 8.12.2.1.2-1: Minimum performance NPDCCH Test number; Deployment mode; Repetition number (R); Carrier Type; Reference Channel; Propagation Condition; Number of NRS ports; Antenna Configuration and correlation matrix; Reference value; UE Category. Pm-dsg (%); SNR (dB). 1; In-band; 64; Anchor; R.NB.4 FDD; EPA5; 2; 2x1 Low; 1; -3.9; NB1, NB2. 2; In-band; 512; Non-anchor; R.NB.4 FDD; ETU1; 2; 2x1 Low; 1; -10.0; NB1, NB2. #### 8.12.2.2 TDD The parameters specified in Table 8.12.2.2-1 and Table 8.12.2.2-2 are valid for all TDD tests unless otherwise stated. Table 8.12.2.2-1: Test Parameters for NPDCCH Parameter; Unit; Single antenna port; Transmit diversity. Uplink downlink configuration (Note 1); 4; 4. Special subframe configuration (Note 2); 4; 4. Narrowband physical layer Cell ID; 0; 0. at antenna port; dBm/15kHz; -98; -98. Cyclic prefix; Normal; Normal. Number of CRS ports for in-band deployment mode; 4; 4. NPDCCH starting position (eutraControlRegionSize-r13) (Note 1); 2; 2. Maximum number of repetitions (npdcch-NumRepetitions-r13); 128 for Test 1; 1024 for Test 2.; 64 for Test 1; 512 for Test 2.. NPDCCH start subframe (npdcch-startSF-USS-r13); 4; 4. NPDCCH fractional period offset of starting subframe (npdcch-Offset-USS-r13); 0; 0. NB-IoT downlink subframe bitmap for anchor carrier (downlinkBitmap-r13); Not configured; Not configured. NB-IoT downlink subframe bitmap for non-anchor carrier (downlinkBitmapNonAnchor-r13); Not configured; Not configured. Downlink gap configuration for anchor carrier (dl-Gap-r13); Not configured; Not configured. Downlink gap configuration for non-anchor carrier (dl-GapNonAnchor-r13); Not configured; Not configured. Unused REs or RBs (Note 1); OCNG; OCNG. OCNG pattern; NB.OP.1; NB.OP.1. NOTE 1: Applicable only for in-band deployment mode.. Table 8.12.2.2-2: Test Parameters of related NPDSCH and NPUSCH format 2 configurations Parameter; Unit; Value. Scheduling delay field (); 0. NPDSCH Repetition number; 1. at antenna port for NPDSCH; dBm/15kHz; -98. (ack-NACK-NumRepetitions-r13); 1. ACK/NACK resource field; 0. Reference channel for NPDSCH; R.NB.6 TDD and R.NB.6-1 TDD for one NRS antenna port; R.NB.5 TDD and R.NB.5-1 TDD for two NRS antenna ports. ##### 8.12.2.2.1 Single-antenna performance For the parameters specified in Table 8.12.2.2.1-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.2.1-1. The downlink physical channel setup is in accordance with Annex C.3.6. Table 8.12.2.2.1-1: Minimum performance NPDCCH Test number; Deployment mode; Repetition number (R); Carrier Type; Reference Channel; Propagation Condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Pm-dsg (%); SNR (dB). 1; Stand-alone/Guard-band; 128; Anchor; R.NB.3 TDD; EPA5; 1; 1x1; 1; -4.9; NB1, NB2. 2; Stand-alone/Guard-band; 1024; Non-anchor; R.NB.3 TDD; ETU1; 1; 1x1; 1; -11.4; NB1, NB2. ##### 8.12.2.2.2 Transmit diversity performance For the parameters specified in Table 8.12.2.2.2-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.2.2-1. The downlink physical channel setup is in accordance with Annex C.3.6. Table 8.12.2.2.2-1: Minimum performance NPDCCH Test number; Deployment mode; Repetition number (R); Carrier Type; Reference Channel; Propagation Condition; Number of NRS ports; Antenna Configuration and correlation matrix; Reference value; UE Category. Pm-dsg (%); SNR (dB). 1; In-band; 64; Anchor; R.NB.4 TDD; EPA5; 2; 2x1 Low; 1; -3.9; NB1, NB2. 2; In-band; 512; Non-anchor; R.NB.4 TDD; ETU1; 2; 2x1 Low; 1; -10.0; NB1, NB2. ### 8.12.3 Demodulation of NPBCH The receiver characteristics of the NPBCH are determined by the probability of miss-detection of the NPBCH (Pm-bch), which is defined as For the performance with single a NPBCH TTI decoding, A is the number of correctly decoded MIB-NB PDUs and B is the number of transmitted MIB-NB PDUs. For the performance with multiple NPBCH TTIs decoding, A is the number of 5120ms durations consisting of contiguous NPBCH TTIs where there is at least one correctly decoded MIB-NB PDU, and B is the number of 5120ms durations consisting of contiguous NPBCH TTIs during the test. #### 8.12.3.1 HD-FDD Table 8.12.3.1-1: Test Parameters for NPBCH Parameter; Unit; Single antenna port; Transmit diversity. Downlink power allocation; NPBCH_RA; dB; 0; -3. NPBCH_RB; dB; 0; -3. at antenna port; dBm/15kHz; -98; -98. Cyclic prefix; Normal; Normal. Cell ID; 0; 0. ##### 8.12.3.1.1 Single-antenna port performance with single NPBCH TTI For the parameters specified in Table 8.12.3.1-1 the average probability of a miss-detecting NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.1.1-1. The downlink physical setup is in accordance with Annex C.3.6. Table 8.12.3.1.1-1: Minimum performance NPBCH Test number; Bandwidth; Reference Channel; Propagation Condition; Antenna configuration; Reference value; UE Category. Pm-bch (%); SNR (dB). 1; 200 KHz; R.NB1.1; EPA1; 1 x 1; 1; -2.0; NB1, NB2. ##### 8.12.3.1.2 Transmit diversity performance 8.12.3.1.2.1 Minimum Requirement 2 Tx Antenna Port with multiple NPBCH TTIs For the parameters specified in Table 8.12.3.1-1 the average probability of a miss-detected NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.1.2.1-1. The downlink physical setup is in accordance with Annex C.3.6. Table 8.12.3.1.2.1-1: Minimum performance NPBCH Test number; Bandwidth; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value; UE Category. Pm-bch (%); SNR (dB). 1; 200 KHz; R.NB1.2; EPA1; 2 x 1 Low; 1; -11.5; NB1, NB2. #### 8.12.3.2 TDD Table 8.12.3.2-1: Test Parameters for NPBCH TDD Parameter; Unit; Single antenna port; Transmit diversity. Downlink power allocation; NPBCH_RA; dB; 0; -3. NPBCH_RB; dB; 0; -3. at antenna port; dBm/15kHz; -98; -98. Cyclic prefix; Normal; Normal. Cell ID; 0; 0. Uplink downlink configuration (Note 1); 4; 4. Special subframe configuration (Note 2); 4; 4. Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4].. ##### 8.12.3.2.1 Single-antenna port performance with single NPBCH TTI For the parameters specified in Table 8.12.3.2-1 the average probability of a miss-detecting NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.2.1-1. The downlink physical setup is in accordance with Annex C.3.6. Table 8.12.3.2.1-1: Minimum performance NPBCH Test number; Bandwidth; Reference Channel; Propagation Condition; Antenna configuration; Reference value; UE Category. Pm-bch (%); SNR (dB). 1; 200 KHz; R.NB1.1; EPA1; 1 x 1; 1; -2.0; NB1, NB2. ##### 8.12.3.2.2 Transmit diversity performance 8.12.3.2.2.1 Minimum Requirement 2 Tx Antenna Port with multiple NPBCH TTIs For the parameters specified in Table 8.12.3.2-1 the average probability of a miss-detected NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.2.2.1-1. The downlink physical setup is in accordance with Annex C.3.6. Table 8.12.3.2.2.1-1: Minimum performance NPBCH Test number; Bandwidth; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value; UE Category. Pm-bch (%); SNR (dB). 1; 200 KHz; R.NB1.2; EPA1; 2 x 1 Low; 1; -11.5; NB1, NB2.

Proposal 5: RAN4 to consider at least the introduction of the following performance requirements in TS 36.102:

## 8.3 Demodulation performance requirements for UE category NB1 and NB2 ### 8.3.1 Half-duplex FDD #### 8.3.1.1 NPDSCH demodulation requirements ##### 8.3.1.1.1 Single-antenna port performance 8.3.1.1.1.1 Minimum Requirements for standalone

### 8.3.2 TDD #### 8.3.2.1 NPDSCH demodulation requirements ##### 8.3.1.1.1 Single-antenna port performance 8.3.1.1.1.1 Minimum Requirements for standalone Table 8.12.1.2-1: Common Test Parameters Parameter; Unit; Value. Uplink downlink configuration (Note 1); 1 or 4 - TBD. Special subframe configuration (Note 2); 4. Number of HARQ processes per component carrier (Note 4); Processes; 1 or 2. Maximum number of HARQ transmission; 4. Cyclic Prefix; Normal. eutraControlRegionSize-r13; 2 for In-band, N/A for Standalone/Guard-band unless otherwise stated. downlinkBitmap-r13 and dl-Gap-r13; Not configured. dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13; Not configured. Unused REs or RB (Note 3); OCNG. OCNG pattern; NB.OP.1. Note 1: As specified in Table 4.2-2 in TS 36.211 [4] Note 2: As specified in Table 4.2-1 in TS 36.211 [4] Note 3: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. Note 4: 1 is applicable for both UE Category NB1 and NB2; 2 is only applicable for UE Category NB2 if UE supports it.. Table 8.12.1.2-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations Parameter; Unit; Value. DCI format; DCI format N1. scheduling delay field (); 1 for the first HARQ process; 2 for the second HARQ process if configured. (ack-NACK-NumRepetitions-r13); 1. ACK/NACK resource field; 0. Reference channel for NPDCCH; R.NB.3 TDD for one NRS antenna port; R.NB.4 TDD for two NRS antenna ports. (npdcch-Offset-USS-r13); 0. K_offset; ms; 8 (TBD). ##### 8.12.1.2.1 Minimum Requirements for In-band N/A in this specification ##### 8.12.1.2.2 Minimum Requirements for Standalone/Guard-band The requirements are specified in Table 8.12.1.2.2-2, with the addition of the parameters in Table 8.12.1.2.2-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the performance. Table 8.12.1.2.2-1: Test Parameters for NPDSCH under Standalone/Guard-band Parameter; Unit; Test 1, 2. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -99 (Note 2). NPDCCH repetition number; subframe; TBD (currently 32 for Test 1; 256 for Test 2.). (npdcch-NumRepetitions-r13); subframe; TBD (currently 32 for Test 1; 256 for Test 2.). (nPDCCH-startSF-USS-r13); 4. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.2.2-2: Minimum performance for NPDSCH under Standalone/Guard-band with 1 NRS port Test number; Bandwidth; Carrier Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Anchor; TBD (currently R.NB.6 TDD); TBD; NTN-TDLC5-200; 1; 1x1; 70%; TBD; NB1, NB2. 2; 200kHz; Non-anchor; TBD (currently R.NB.6-1 TDD); TBD; NTN-TDLC5-200; 1; 1x1; 70%; TBD; NB1, NB2. ##### 8.12.1.2.3 Minimum Requirements for Standalone for UE Category NB2 The requirements are specified in Table 8.12.1.2.3-2, with the addition of the parameters in Table 8.12.1.2.3-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance. Table 8.12.1.2.3-1: Test Parameters for NPDSCH under Standalone Parameter; Unit; Test 1. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -96 (Note 2). NPDCCH repetition number; subframe; TBD (currently 4 for Test 1). (npdcch-NumRepetitions-r13); subframe; TBD (currently 8 for Test 1). (nPDCCH-startSF-USS-r13); 4. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.2.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port Test number; Bandwidth; Carrer Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Non-anchor; TBD (currently R.NB.7 TDD); 1; NTN-TDLA100-10; 1; 1x1; 70%; TBD; NB2.

Proposal 6: Following FDD process for NB-IoT NTN, propose not to define NPDCCH and NPBCH performance requirements in TS 36.102 for TDD NB-IoT NTN definition.

Proposal 7: Following FDD process for NB-IoT NTN, propose to define only NPDSCH performance requirements in TS 36.102 for TDD NB-IoT NTN definition.

Note: Further discuss on the reference channel to be used for NPDSCH performance requirements.

Proposal 8: Add for EVM clause

“For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 10 subframes averaging length. TDD special fields (DwPTS and GP) are not included in the averaging.”

# Conclusion

Proposal 1: For UE TS 36.102, add extra 0.5 dB for the REFSENS TDD compared to FDD due to the lack of duplexer.

Proposal 2: RAN4 to adopt the following modification in the TS 36.102:

## 7.3B Reference sensitivity power level for UE category NB1 and NB2 The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise. The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2 with received signal level as specified in Table 7.3B-1. Requirement in Table 7.3B-1 applies for any uplink configuration. Table 7.3B-1: Reference sensitivity for UE category NB1 and NB2 Operating band; REFSENS [dBm]. According to subclause 5.2B; - 108.2. The UE TDD category NB1 and NB2 throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2 with received signal level as specified in Table 7.3B-2. Requirement in Table 7.3B-2 applies for any uplink configuration. Table 7.3B-2: Reference sensitivity for UE category NB1 and NB2 Operating band; REFSENS [dBm]. According to subclause 5.2B; - 108.7.

Proposal 3: Introduce a new clause for TDD NB-IoT category for 249 band (currently only FDD configuration is defined) for NPDSCH minimum performance requirements.

Proposal 4: Further discuss repetition number to be considered for demodulation and performance aspects. Provide SNR values for less repetition numbers to align with U/L=8 and N=9 as per TDD NB-IOT NTN band 249 configuration.

Proposal 5: RAN4 to consider at least the introduction of the following performance requirements in TS 36.102:

## 8.3 Demodulation performance requirements for UE category NB1 and NB2 ### 8.3.1 Half-duplex FDD #### 8.3.1.1 NPDSCH demodulation requirements ##### 8.3.1.1.1 Single-antenna port performance 8.3.1.1.1.1 Minimum Requirements for standalone

### 8.3.2 TDD #### 8.3.2.1 NPDSCH demodulation requirements ##### 8.3.1.1.1 Single-antenna port performance 8.3.1.1.1.1 Minimum Requirements for standalone Table 8.12.1.2-1: Common Test Parameters Parameter; Unit; Value. Uplink downlink configuration (Note 1); 1 or 4 - TBD. Special subframe configuration (Note 2); 4. Number of HARQ processes per component carrier (Note 4); Processes; 1 or 2. Maximum number of HARQ transmission; 4. Cyclic Prefix; Normal. eutraControlRegionSize-r13; 2 for In-band, N/A for Standalone/Guard-band unless otherwise stated. downlinkBitmap-r13 and dl-Gap-r13; Not configured. dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13; Not configured. Unused REs or RB (Note 3); OCNG. OCNG pattern; NB.OP.1. Note 1: As specified in Table 4.2-2 in TS 36.211 [4] Note 2: As specified in Table 4.2-1 in TS 36.211 [4] Note 3: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. Note 4: 1 is applicable for both UE Category NB1 and NB2; 2 is only applicable for UE Category NB2 if UE supports it.. Table 8.12.1.2-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations Parameter; Unit; Value. DCI format; DCI format N1. scheduling delay field (); 1 for the first HARQ process; 2 for the second HARQ process if configured. (ack-NACK-NumRepetitions-r13); 1. ACK/NACK resource field; 0. Reference channel for NPDCCH; R.NB.3 TDD for one NRS antenna port; R.NB.4 TDD for two NRS antenna ports. (npdcch-Offset-USS-r13); 0. K_offset; ms; 8 (TBD). ##### 8.12.1.2.1 Minimum Requirements for In-band N/A in this specification ##### 8.12.1.2.2 Minimum Requirements for Standalone/Guard-band The requirements are specified in Table 8.12.1.2.2-2, with the addition of the parameters in Table 8.12.1.2.2-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the performance. Table 8.12.1.2.2-1: Test Parameters for NPDSCH under Standalone/Guard-band Parameter; Unit; Test 1, 2. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -99 (Note 2). NPDCCH repetition number; subframe; TBD (currently 32 for Test 1; 256 for Test 2.). (npdcch-NumRepetitions-r13); subframe; TBD (currently 32 for Test 1; 256 for Test 2.). (nPDCCH-startSF-USS-r13); 4. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.2.2-2: Minimum performance for NPDSCH under Standalone/Guard-band with 1 NRS port Test number; Bandwidth; Carrier Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Anchor; TBD (currently R.NB.6 TDD); TBD; NTN-TDLC5-200; 1; 1x1; 70%; TBD; NB1, NB2. 2; 200kHz; Non-anchor; TBD (currently R.NB.6-1 TDD); TBD; NTN-TDLC5-200; 1; 1x1; 70%; TBD; NB1, NB2. ##### 8.12.1.2.3 Minimum Requirements for Standalone for UE Category NB2 The requirements are specified in Table 8.12.1.2.3-2, with the addition of the parameters in Table 8.12.1.2.3-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance. Table 8.12.1.2.3-1: Test Parameters for NPDSCH under Standalone Parameter; Unit; Test 1. at antenna port; dBm/15kHz; -93 (Note 1). dBm/15kHz; -96 (Note 2). NPDCCH repetition number; subframe; TBD (currently 4 for Test 1). (npdcch-NumRepetitions-r13); subframe; TBD (currently 8 for Test 1). (nPDCCH-startSF-USS-r13); 4. Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission.. Table 8.12.1.2.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port Test number; Bandwidth; Carrer Type; Reference Channel; Repetition number; Propagation condition; Number of NRS ports; Antenna Configuration; Reference value; UE Category. Fraction of Maximum Throughput (%); SNR (dB). 1; 200kHz; Non-anchor; TBD (currently R.NB.7 TDD); 1; NTN-TDLA100-10; 1; 1x1; 70%; TBD; NB2.

Proposal 6: Following FDD process for NB-IoT NTN, propose not to define NPDCCH and NPBCH performance requirements in TS 36.102 for TDD NB-IoT NTN definition.

Proposal 7: Following FDD process for NB-IoT NTN, propose to define only NPDSCH performance requirements in TS 36.102 for TDD NB-IoT NTN definition.

Note: Further discuss on the reference channel to be used for NPDSCH performance requirements.

Proposal 8: Add for EVM clause

“For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 10 subframes averaging length. TDD special fields (DwPTS and GP) are not included in the averaging.”

# Annex – previous meeting agreements

The WF covers the contributions submitted under the following AI:

- 7.28.1 General aspects and work plan	[IoT_NTN_TDD]

- 7.28.2 Study of impact due to periodic pattern	[IoT_NTN_TDD-Core]

### Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements

Agreement:

- RAN4 to consider the following values for analyzing the impact of requirements, as necessary:

  - N = 9, D = {8, 20, 30}, U = {8, 20, 30}

  - N = 8, D = 20, U = 20

- Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis.

Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision.

Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

### Issue 1-2: Impact on SAN RF requirements

Agreement:

- The assessment of the impact on SAN RF requirements (TS 36.108) shown in the table below will serve as a baseline for further discussion on the exact requirement definition. For the requirements to be impacted for [249] band, if any, NTN IoT 254 L-band Tx/Rx requirements in TS 36.108 will serve as a baseline. If identified, new requirements can be defined for SAN RF, e.g. transient period.

| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| --- | --- |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Add this specification to SAN starting from Rel-19 |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> FFS: Specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

### Issue 1-3: Impact on UE RF requirements

Agreement:

- The assessment of the impact on UE RF requirements (TS 36.102) shown in the table below will serve as a baseline for further discussion on the exact requirement definition.

| 36.102 Clause: Requirements | Band Agnostic | Re-use for TDD band [249] | Note |
| --- | --- | --- | --- |
| 6.2B.1: UE maximum output power | Yes | Yes | Support UE power class 3 (23 dBm) Support UE power class 5 (20 dBm) |
| 6.2B.1: UE maximum output power tolerance | No |  |  |
| 6.2B.2: UE maximum output power reduction | Yes | Yes |  |
| 6.2B.3: UE additional maximum output power reduction | No | - | Note: Reuse which band A-MPR, B254 A-MPR is not defined yet and the band [249] UL frequency range is not fully aligned with B254. |
| 6.2.B.4: Configured transmit power | Yes | Yes |  |
| 6.3: Output power dynamics | Yes | Yes FFS | Note: For 36.102 6.3B.4, 36.102 refers to 36.101 6.3.5F, the testing period for aggerate power control may depend on different UL pattern. |
| 6.4: Transmit signal quality | Yes | Yes |  |
| 6.5B.3.2: Spectrum emission mask | Yes | Yes |  |
| 6.5B.3.4: Adjacent Channel Leakage Ratio | Yes | Yes |  |
| 6.5B.4.2: General spurious emissions | Yes | Yes |  |
| 6.5B.4.3: Spurious emissions for UE co-existence | No |  | Band [249] needs to be added in Table 6.5A.4.3-1 Spurious emission band UE co-existence. |
| 6.5B.4.4: Additional spurious emissions | No |  | Additional spurious emissions requirement for B254 can be reused for the new IoT-NTN TDD L-band. UL frequency range of B254 is superset of [B249]. |
| 6.6: Transmit intermodulation for category NB1 and NB2 | Yes | Yes |  |
| 7.2: Diversity characteristics | Yes | Yes |  |
| 7.3: Reference sensitivity power level | No | No | To be studied for IoT-NTN TDD mode. |
| 7.4: Maximum input level | Yes | Yes |  |
| 7.5: Adjacent Channel Selectivity | Yes | Yes |  |
| 7.6B.2: In-band blocking | No | Yes | Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 255,254, and 253 apply the same IBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use IBB requirements from B255,254 and 253 |
| 7.6B.3: Out-of-band blocking | No |  | Different Out-of-band requirements for IoT NTN bands. Currently, bands 255,254, and 253 apply the same OBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use OBB requirements from B255, 254 and 253 |
| 7.6B.4: Narrow band blocking | No | Yes |  |
| 7.7: Spurious response | Yes | Yes |  |
| 7.8: Intermodulation characteristics | Yes | Yes |  |
| 7.9: RX spurious emission | No | Yes | Re-use spurious emission requirements from Table 7.9.1-1 of 36.101 |
| A.1 DL reference measurement channels |  |  | NPDSCH Reference Channel for TDD should be introduced. Fixed Reference Channel for TDD should be introduced. |
| A.2 OFDMA Channel Noise Generator (OCNG) |  |  | OCNG Patterns for TDD should be introduced. |
| A.3 Testing related to Satellite Access |  |  | No impact. |
| A.4 UL reference measurement channels |  |  | Reference measurement channels for TDD should be introduced. |

### Issue 2-1: Impact on RRM requirements

Agreement:

- From the RRM requirement point of view, no impact on the following requirements is expected due to the new periodic pattern(s):

  - UE uplink timing

  - PUR

Agreement:

- For the following RRM requirements, modifications are expected due to the new periodic pattern(s):

  - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.)

  - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.)

  - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.)

  - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.)

Note 1: The exact requirement modifications are subject to further discussion in future RAN4 meetings based on RAN1 conclusion/progress on this feature.

Note 2: Different modifications can be made for different TDD patterns, if multiple, DRX configurations, SNR values, etc.

Note 3: Not all existing requirements for NB-IoT NTN may be applicable to the IoT_NTN_TDD feature, which is subject to further discussions in future RAN4 meetings.

Note 4: It is assumed that UE will be made aware of the periodic pattern of frequency layers to be measured.

Note 5: UE power consumption aspect may be considered when modifying the above requirements as needed.

Note 6: The requirement applicability rule defined in Rel-17/18 applies to the IoT_NTN_TDD feature as well. For instance, the above requirements may not be applicable for inter-satellite scenarios if the UE is not provided with the target inter-satellite’s information.

Demodulation aspect:

No observation in this meeting.

# Annex for information – previous meeting discussions

### Issue 1-1: Operating Bands

<Band number>

- RAN4 to consider the following proposals for the requirement definition during the normative work item phase in future RAN4 meetings.

  - Assign “[249]” E-UTRA operating band number for the new TDD [1616-1626.5] MHz NTN NB-IoT frequency band introduction.

E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| [249] | [1616] MHz | - | [1626.5] MHz | [1616] MHz | - | [1626.5] MHz | TDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |

  - Use a channel raster of 100 kHz and a Step size of <1>

  - Both FDL_low and FUL_low equal 1616 MHz.

  - For the range of both NDL and NUL:

    - Option 1: Starts from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL) or 2^18-2^14-1=245759 (in-between FDD NTN NOffs-DL and FDD NTN NOffs-UL)

    - Option 2: {198196 - 198300}