3GPP TSG RAN WG4 Meeting #114	R4-2502059

Athens, Greece, 17 Feb. 2025 – 21 Feb. 2025

Agenda Item:	5.27.1

Source: 	Ericsson

Title:	Discussions on the remaining open issues for Rel-18 NES

Document for:	Discussion

# Introduction

In last meeting, RAN4 achieved agreements on most open issues. In this contribution, we continue to discuss the remaining issue on NES.

- SSB less SCell activation for inter-band collocated CA

In last meeting following agreement is reached for the power difference side condition to support SSB less SCell on inter-band CA.

Agreement: - For EPRE difference within X dB, keep the existing requirement. - X equals to or larger than 12 dB - For EPRE difference beyond X dB and no larger than Y dB, add one more P-TRS occurrence. - Further discuss whether the upper bound of Y is needed. If needed, Y is larger than 25dB. - From RAN4 perspective, AGC compensation for carrier frequency difference can be optionally implemented by UE. Remove “[after pre-compensation]” from the spec, i.e., not specify “AGC compensation” for carrier frequency difference in the spec. - Further discuss the X and Y values in this meeting. Try to conclude in the 2nd round on Thursday afternoon. Additional Agreement: X = 12; Y = [30].

In last meeting, the wording precompensation was agreed to be removed from the spec. Following two scenarios can be considered for deriving the EPRE difference.

- Scenario 1: When the CC are close by

- Scenario 2: When the CC are far apart

In last meeting for scenario 1 (i.e., when the reference cell and SSB less SCell are very close in frequency and using same BW), companies mentioned that there is no precompensation UE can perform. For scenario 1, it was agreed that 12 dB is absolute maximum EPRE UE can support. That means the 12 dB is the maximum power difference UE can support without any propagation delay difference arising from the frequency separation. This can be taken as baseline for the scenario 2.

NR CA band combination supports wide range of frequency separation such as n71, n77 (630 MHz, 3.75Ghz), which corresponds to free space propagation delay difference of around 16 dB. Since the 16 dB is free space propagation delay difference, we might need to add additional margin on top of free space propagation delay difference to consider the realistic propagation conditions. Towards that end we can consider additional margin as 6 dB. Combinaing 12 dB (which UE can support when the CC are close by), additional margin and the 16dB power difference due to propagation delay difference due to frequency separation, we think 34 dB can be maximum EPRE for the scenario 2. We further think that 34 dB seems reasonable as we are allowing one additional P-TRS sample for the AGC.

- When EPRE difference is beyond 12 dB and below 34 dB, UE is allowed one additional P-TRS for the AGC settling.

  - Neighbour cells on carrier of SSB-less SCell

One issue discussed in last meeting without agreement was how does the measurements on SSB less carrier was categorized. There were two set of views presented during the discussion, one view is, since there is no SSB on the serving cell, as per the existing definition, neighbour cells on SSB less SCell carrier are measured as inter-frequency measurements. Other view is that, though SSB is not present on SSB less SCell, as per RAN2, there is no restriction on configuring or providing SSB center frequency. If the SSB less SCell MO is provided, then the measurements can be treated as per existing defintion. Otherwise, all the measurements are inter-frequency measurements.

This can be solved by following solutions based on whether there is an agreement to introduce the servingCellMO or not.

If servingCellMO is introduced:

One issue brought forward with the configuration of servingCellMO for the SSB less SCell is, it contradicts with the previously agreed definition of SSB less SCell in RAN4. As per RAN4, a SCell can be defined as SSB less SCell if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell. At the same time, following relationship highlighted in yellow is captured in the TS 38.331.

servingCellMO

measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. For this MeasObjectNR, the following relationship applies between this MeasObjectNR and frequencyInfoDL in ServingCellConfigCommon/ServingCellConfigCommonSIB of the serving cell: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB and if csi-rs-ResourceConfigMobility is configured, the value of its subcarrierSpacing is present in one entry of the scs-SpecificCarrierList, csi-RS-CellListMobility includes an entry corresponding to the serving cell (with cellId equal to physCellId in ServingCellConfigCommon) and the frequency range indicated by the csi-rs-MeasurementBW of the entry in csi-RS-CellListMobility is included in the frequency range indicated by in the entry of the scs-SpecificCarrierList.

We think RAN2 may need to clarify that the above restriction does not apply for the SSB less SCells. We think RAN2 can do this by simply creating a new IE to indicate the concerning measobject ID belong to SSB less SCell instead of using the legacy IE of servingCellMO. We think how to capture the measObjectID belong to SSB less SCell is upto RAN2 discussion. Upon our internal checking with RAN2, we heard that it can be easily indicated without impacting the legacy indication of the servingCellMO.

If the information of measObjectID belongs to SSB less SCell frequency is indicated to UE, then the legacy requirements can be reused without much change to RAN4 requirements. We think the change in RAN2 is minor compared to the change in RAN4 as servingCellMO is nothing but a measurement object ID.

If the measObject ID of SSB less SCell frequency is not indicated:

If RAN2 do not introdcue the measObject ID of SSB less SCell frequency for inter-band SSB-less SCell, all the measurements will be considered as an inter-freqeuncy measurements as per the existing definition. We think the main disadvantages of performing the neighbour cell measurements on SSB less SCell carrier as inter-frequnecny measurements is measurement delay is longer for inter-frequency measurements and a smaller number of cells can be measured on the inter-frequency. We think this alternative is not desirable for the measurement performance. This can be avoided with following alternatives.

- Alt1: Change the definition of intra-frequnecy measurement for SSB less SCell

- Alt2: All the measurements are defined as inter-frequncy measurements but introduce different requirements for SSB less SCells (e.g., the number of cells, etc.)

We think agreeing on Alt 1 and Alt 2 requires additional RAN4 efforts to figure out the details. At the current stage of the WI, we think it is not desirable.

Considering the workload on RAN2 and RAN4, we think introducing a SSB less SCell frequency’s measurement object ID indication to the UE is simpler from workload and system performance impact point of view.   Hence, we suggest sending LS to RAN2 capturing these observations.

- RAN4 to send LS to RAN2 informing about disadvanges of not indicating the SSB less SCells frequnecy’s measurement object ID.

  - The measurement object on SSB less SCell frequency is treated as inter-frequency

  - Measurement requirements are different for inter-frequency measurement, which might impact the measurement performance.

- Summary

In this contribution, we have discussed the SSB less SCell requirements. Based on the discussions, we have made following proposals and observations:

- When EPRE difference is beyond 12 dB and below 34 dB, UE is allowed one additional P-TRS for the AGC settling.

- RAN4 to send LS to RAN2 informing about disadvanges of not indicating the SSB less SCells frequnecy’s measurement object ID.

  - The measurement object on SSB less SCell frequency is treated as inter-frequency

  - Measurement requirements are different for inter-frequency measurement, which might impact the measurement performance.

- References

[1]. R4-2410269, “WF on RRM requirements for NR network energy saving”, Huawei, HiSilicon