3GPP TSG RAN WG4 Meeting #114	R4-2502066

Athens, Greece, 17 Feb. 2025 – 21 Feb. 2025

Agenda Item:	7.22.3.2

Source: 	Ericsson

Title:	Simulation results for CJTC reporting

Document for:	Information

# Introduction

In this contribution we provide the simulation results for the CJT calibration reporting.

# Discussion

CJTC simulation results

- Agree with the following table for delay/frequency offset reporting. FFS on phase offset.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm, 0.2ppm 2nd priority: 0.02 ppm, 0.05ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 2nd priority: 2, 4, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |

In the following sections we present the simulation results based on the above simulation assumptions

### Simulation results for frequency offset

| Number of TRS resource sets in a measurement | 30kHz SCS, A_FO=100 ppb, 500 UEs; FO error (absolute) in ppb estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 5.854 | 1 | 8 | 26 | 2.849 | 1 | 4 | 12 |
| 2 TRS | 4.853 | 1 | 7 | 18 | 2.165 | 1 | 3 | 8 |
| 3 TRS | 3.794 | 1 | 5 | 16 | 1.916 | 1 | 3 | 7 |
| 5 TRS | 3.106 | 1 | 4 | 12 | 1.433 | 1 | 2 | 6 |
| 10 TRS | 2.234 | 1 | 3 | 9 | 1.030 | 1 | 2 | 4 |

| Number of TRS resource sets in a measurement | 15kHz SCS, A_FO=100 ppb, 500 UEs; FO error (absolute) in ppb estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 3.265 | 1 | 5 | 14 | 1.553 | 1 | 2 | 7 |
| 2 TRS | 2.396 | 1 | 4 | 9 | 1.107 | 1 | 2 | 5 |
| 3 TRS | 1.905 | 1 | 3 | 8 | 0.991 | 1 | 2 | 4 |
| 5 TRS | 1.547 | 1 | 2 | 6 | 0.794 | 1 | 2 | 3 |
| 10 TRS | 2.234 | 1 | 3 | 9 | 1.030 | 1 | 2 | 4 |

### Simulation results for delay offset

| Number of TRS resource sets in a measurement | 30kHz SCS, A_DO=0.5CP, 500 UEs; DO error (absolute) in CP estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 0.012 | 1 | 4 | 10 | 0.012 | 1 | 3 | 9 |
| 2 TRS | 0.010 | 1 | 3 | 8 | 0.010 | 1 | 3 | 8 |
| 3 TRS | 0.009 | 1 | 3 | 7 | 0.008 | 1 | 3 | 7 |
| 5 TRS | 0.007 | 1 | 2 | 6 | 0.007 | 1 | 2 | 6 |
| 10 TRS | 0.005 | 1 | 2 | 4 | 0.005 | 1 | 2 | 4 |

| Number of TRS resource sets in a measurement | 15kHz SCS, A_DO=0.5CP, 500 UEs; DO error (absolute) in CP estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) | 50%ile | 5%ile in # levels (128) | 50%ile in # levels (128) | 95%ile in # levels (128) |
| 1 TRS | 0.009 | 1 | 3 | 7 | 0.008 | 1 | 2 | 6 |
| 2 TRS | 0.007 | 1 | 2 | 6 | 0.006 | 1 | 2 | 5 |
| 3 TRS | 0.006 | 1 | 2 | 6 | 0.005 | 1 | 2 | 5 |
| 5 TRS | 0.006 | 1 | 2 | 4 | 0.004 | 1 | 2 | 4 |
| 10 TRS | 0.004 | 1 | 2 | 3 | 0.003 | 1 | 1 | 3 |

### Simulation assumptions and results for Phase offset

Table below summarizes the simulation parameters for reciprocity-based CJTC phase offset reporting:

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz TDD |
| SCS | 30kHz SCS |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| TRP antenna configuration | 1x2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | cjtc-p, periodic CSI-RS using 1 or 2 ports. CSI-RS is precoded by phase of channel estimate from latest SRS with a 1-1 map between SRS and CSI-RS port number |
| CSI-RS periodicity | 20 slots, with slot offset 2 and 3 |
| SNR | -3dB, 3dB |
| Initial UL/DL phase offset between TRPs | 0 degree, 60 degree, 90 degree |
| SRS configuration | Periodic SRS every 10 slots, 48 RB bandwidth |

Simulation results:

| Initial phase offset (deg), number of CSI-RS ports | 180 UEs, 20 CSI reports per UE (400 slots); 50%ile of error (absolute) in deg over all reports |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 50%ile error | 50%ile in # levels (16) | 50%ile in # levels (32) | 50%ile in # levels (64) | 50%ile error | 50%ile in # levels (16) | 50%ile in # levels (32) | 50%ile in # levels (64) |
| 0 deg, 1 port | 7.595 | 1 | 1 | 2 | 5.255 | 1 | 1 | 1 |  |
| 60 deg, 1 port | 7.674 | 1 | 1 | 2 | 5.422 | 1 | 1 | 1 |  |
| 90 deg, 2 ports | 5.026 | 1 | 1 | 1 | 3.483 | 1 | 1 | 1 |  |

| Initial phase offset (deg), number of CSI-RS ports | 180 UEs, 20 CSI reports per UE (400 slots); 95%ile of error (absolute) in deg over all reports |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | 95%ile error | 95%ile in # levels (16) | 95%ile in # levels (32) | 95%ile in # levels (64) | 95%ile error | 95%ile in # levels (16) | 95%ile in # levels (32) | 95%ile in # levels (64) |
| 0 deg, 1 port | 23.001 | 2 | 3 | 5 | 14.757 | 1 | 2 | 3 |  |
| 60 deg, 1 port | 22.372 | 1 | 2 | 4 | 14.977 | 1 | 2 | 3 |  |
| 90 deg, 2 ports | 15.261 | 1 | 2 | 3 | 10.229 | 1 | 1 | 2 |  |